3GPP TSG RAN meeting #89e										RP-201882

Electronic Meeting, September 14th -18th, 2020

## Status Report to TSG

Agenda item: 9.2.17

| WI / SI Name | NR RRM enhancement in R16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_Enh |  |  |  |  |
| Unique ID | 840195 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201112 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2020 | Performance part: 03/2021 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 5% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Qiming Li |
|  | Company | Intel Corporation |
|  | Email | qiming.li@intel.com |
| Rapporteur | Name | Qian Yang |
|  | Company | ZTE Corporation |
|  | Email | yang.qian35@zte.com.cn |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Inc. |
|  | Email | jie_cui@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

#### 2.1.1	Agreements

##### RAN4 #96e meeting (August 2020)

###### SRS carrier-based switching requirements

- Interruption requirements for CA case 1, case 2 and case 3

  - The same interruption requirements as for async case in the spec shall apply

###### CGI reading requirements with autonomous gap

- MIB decoding delay in FR2:

  - 3 * N * TSMTC, where N = 8 and TSMTC is SMTC periodicity of target cell

- SIB1 decoding delay and side condition

  - 6 samples and -3 dB SNR

- Value of timer T321 for FR2

  - 5 seconds

- Further Agreement on impact to L1 RRM measurement

  - When intra-frequency CGI reading is performed on FR2 carrier, longer measurement delay is expected for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency.

###### Multiple Scell activation/deactivation

Maintenance CR on core requirement was agreed in R4-2012165

Principle to decide test case coverage

- P1: test the multiple SCell activation for unknown to-be-activated SCells, and test the multiple SCell activation for mixed unknown and known to-be-activated SCells.

- P2: only test the multiple SCell activation under EN-DC and NR-DC mode.

- P3: only test the multiple SCell activation under non-DRX mode and have different test requirements for both per-FR MG capable UE and per-UE capable UE.

- P4: only test the multiple SCell activation with periodic CSI-RS for CSI reporting for FR2 to-be-activated SCells.

Test case list for Rel-16 multiple SCell activation of eRRM:

- TC 1: EN-DC of LTE+FR1 NR without DRX with single MAC CE

  - 2 FR1 unknown to-be-activated SCells, where

    - first FR1 unknown SCell is intra-band contiguous to active FR1 NR PSCell (meet the exception condition of N1 counting)

    - second FR1 unknown SCell is inter-band to active FR1 NR PSCell

- TC 2: EN-DC of LTE +FR1 NR (the existing activated serving cell) without DRX (test both per-FR MG capable UE and per-UE MG capable UE) with single MAC CE

  - 1 FR2 known to-be-activated SCell and 1 FR2 unknown to-be-activated SCell

  - Both to-be-activated SCells are configured with periodic CSI-RS for CSI reporting

- TC 3: NR-DC without DRX (test per-FR MG capable UE) with dual MAC CEs

  - one inter-band FR1 unknown to-be-activated SCells + one FR2 unknown to-be-activated SCells with periodic CSI-RS for CSI reporting

###### Inter-frequency measurement without MG

Maintenance CR on core requirement

- Add the UE supports capability and network indicated IE to the definition of inter-frequency measurement without MGs

- Add the NeedForGap for inter-frequency measurement to the definition of inter-frequency measurement without MGs.

- When part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap and interFrequencyConfig-NoGap-r16 is indicated by network, UE performs inter-frequency measurement without gap. The condition “interFrequencyConfig-NoGap-r16 is indicated by network” is missing.

- Add the CSSF for inter-frequency measurement without gap in FR2 inter-band CA scenario.

- Adding the measurement type for within gap in section 9.1.5.2 : when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap and interFrequencyConfig-NoGap-r16 is not indicated by network.

###### FR2 inter-band CA requirements

- No CBM-specific RRM requirements will be specified in Rel-16.

- Assumption for IBM UE in Rel-16:

  - Assumption of deployment and band pair for IBM UE should follow the RF session conclusions, and no restriction on deployment and band pair shall be assumed in RRM requirement right now unless RF session concluded on those restrictions.

- Beam management requirements for IBM UE:

- To follow the conclusions in NR eMIMO on how to perform BFD/CBD measurements.

- Tentative agreement FFS:

  - SCell activation delay requirements for IBM UE

  - Not necessary to specify the requirements for ‘SCell being activated belongs to FR2 and there is an active serving cell on that FR2 band and the PCell or PSCell is in FR2 and the PCell or PSCell and SCell being activated are in a band pair with independent beam management’

- Necessity of clarification on “there is no scheduling restriction if UE uses independent beam”

  - The scheduling availability requirements for FR2 inter-band CA scenario shall be introduced to clarify there is no scheduling restriction if UE uses independent beam.

- Scheduling restrictions requirements for IBM UE in case 1~3

  - Scheduling availability requirement shall not apply for the Case 1 and Case 2

    - Case 1 and Case 2 are network configuration errors.

    - How to handle those cases is up to UE implementation without any specified requirement.

  - There is no scheduling restriction allowed for IBM capable UE when network configures mixed numerology between SSB on one FR2 band and data on the other FR2 band and UE is configured for IBM operation for the band pair.

- Measurement restrictions requirements for IBM UE in cases 1 and 2

  - Measurement requirement shall not apply for the Case 1

    - Case 1 is network configuration error.

    - How to handle this case is up to UE implementation without any specified requirement.

  - There is no measurement restriction allowed for IBM UE when network configures mixed numerology between SSB on one FR2 band and CSI-RS on the other FR2 band and UE is configured for IBM operation for the band pair.

###### BWP switching on multiple CCs

Simultaneous BWP switching on multiple CCs

- Delay requirements for DCI/timer based BWP switch:

TBWPSwitchDelay+D*(N-1); D is incremental delay for BWP switch processing on additional CCs

  - D: adding D=200us for type 2 UE capability

  - Definition of N: For UE which is capable of per-FR gap, and no BWP switch involves SCS change, N is the number of simultaneous BWP switching on CCs within the same frequency range; For UE which is not capable of per-FR gap, or the BWP switches on multiple CCs involves SCS changing, N is the number of simultaneous BWP switching on both FR.

- Delay requirements for RRC based BWP switch

TRRCprocessing+TBWPswitchDelayRRC +DRRC*(N-1);

  - For type 1 UE, DRRC = 0ms; For type 2 UE, DRRC = D

Partial Overlap BWP switching on multiple CCs

- Delay requirement for Timer based BWP switch

if UE is capable of per-FR gap and the timer based BWP switch happens in two frequency range, UE handles timer-based BWP switch sequentially

TMultipleBWPswitchDelayTotal = TDelay + TMultipleBWPswitchDelay

Where:

- TDelay is the time required to complete the ongoing timer-based BWP switching on other CCs.

- TMultipleBWPswitchDelay is the timer-based BWP switch delay on current single CC defined in clause 8.6.2 or simultaneously triggered on multiple CCs defined in clause 8.6.2B.1.

- Delay requirement for RRC based BWP switch

Additional waiting time for RRC based BWP switch is upper bounded by the multiple BWP switch time in CG1.

Note: UE is not precluded to process the 2nd RRC message before the first RRC based BWP switch completes and this is up to UE implementation (no spec impact)

###### UL spatial relation change

- No consensus to define requirements for UL signal that has spatial relation to an unknown DL RS. Companies can further bring contributions as a part of maintenance.

- Do not consider timing tracking when associated DL-RS is known but QCLed with a different qcl-Type1 RS

- Define delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH and SP-SRS

- For known spatial relation but the DL RS is not in the active TCI list: (THARQ +3ms)/NR slot length

- Delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS

- For known spatial relation but the DL RS is not in the active TCI list: TRRCprocessing/NR slot length

- When the UL signal has spatial relation to an unknown DL RS,

  - Option 1: Do not define requirements

  - Option 2: Define requirements

    - Define delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH and SP-SRS for unknown spatial relation

      - Option 1: THARQ + 3ms+ TL1-RSRP

      - Option 2: THARQ + 3ms+ TL1-RSRP + time for time tracking if applicable

    - Delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS for unknown spatial relation

      - Option 1: TRRCprocessing + TL1-RSRP

      - Option 2: TRRCprocessing + TL1-RSRP + time for time tracking if applicable

    - Sub 1. Whether to define UE behavior during the transition period when UL signal is configured with unknown DL-RS? (Where the transition period is the period after THARQ + 3ms or TRRCprocessing to UE finished spatial relation switch.)

      - Option 1: Define

      - Option 2: Do not define

    - Sub 2. If the answer of the sub 1 is Option 1,

      - What’s the UE behavior during the transition period when UL signal is configured with unknown DL-RS?

        - Option 1: UE transmits using previous TX beam

        - Option 2: Drop UL transmission until spatial relation info is known

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #96e meeting

- R4-2009596	NR CGI measurements with autonomous gaps for 36.133 Ericsson Limited, Nokia,	 Nokia Shanghai Bell

- R4-2009607	Requirements for BWP switching on multiple CCs	Apple

- R4-2009608	Requirements for UL spatial relation info switch	Apple

- R4-2009708	Discussion on spatial relation switch for uplink	NTT DOCOMO, INC.

- R4-2009709	Discussion on inter-band CA requirement for FR2	NTT DOCOMO, INC.

- R4-2009745	Discussion of RRM requirements for BWP switching on multiple CCs Intel Corporation

- R4-2009752	Discussion on requirements for spatial relation info switch	Intel Corporation

- R4-2009769	Discussion on the remaining issues for BWP switching on multiple CCs	Xiaomi Technology

- R4-2009863	RRM requirements for inter-band CA in FR2	Intel Corporation

- R4-2009864	CR on RRM requirements for BWP switching delay on multiple CCs	Intel Corporation

- R4-2009865	CR on uplink spatial relation switch delay (section 8.12)	Intel Corporation

- R4-2009899	CR on TS38.133 for inter-frequency measurement requirement without gap (Section 9.1.5, 9.3.1)	 MediaTek inc.

- R4-2009907	Draft CR on UE behavior for UE specific CBW change	Apple

- R4-2009915	Test case list for multiple SCell activation for R16 eRRM	Apple

- R4-2009980	Delay requirement for switching of multiple BWPs	Qualcomm

- R4-2009986	RRM requirements with CBM and IBM in FR2 inter band CA	Qualcomm

- R4-2009987	Spatial relation switch for uplink	Qualcomm

- R4-2010040	Remaining issues on Interruption at SRS carrier switch	MediaTek inc.

- R4-2010041	Remaining issues on CGI reading requirement	MediaTek inc.

- R4-2010042	Discussion on BWP switch on multiple CCs	MediaTek inc.

- R4-2010043	Remaining issues on active spatial relation switch	MediaTek inc.

- R4-2010044	CR on multiple SCells activation (section 8.3.7)	MediaTek inc.

- R4-2010107	CR on definition of inter-frequency measurements without measurement gap (9.3.1)	CMCC

- R4-2010120	Multiple SCell activation in NR	Qualcomm Incorporated

- R4-2010189	Requirements for BWP switching on multiple CCs	Apple

- R4-2010190	Requirements for UL spatial relation info switch	Apple

- R4-2010197	CR on BWP switch on multiple CCs	Apple

- R4-2010221	Discussion on Inter-band CA requirement for FR2	MediaTek inc.

- R4-2010361	Consideration on remaining issues for BWP switching over multiple CCs	vivo

- R4-2010362	CR for RRC based simultaneously BWP switch over multiple CCs	vivo

- R4-2010363	Further considerations on FR2 inter CA requirements.	vivo

- R4-2010364	On remaining issues for UL Spatial Relation Info Switching	vivo

- R4-2010375	Further considerations on RRM requirements for interband CA on FR2	Ericsson

- R4-2010376	Discussion on remaining issues for NR CGI reading	Ericsson

- R4-2010377	Impact of CGI reading on RLM	Ericsson

- R4-2010378	Impact of CGI reading on RLM and BM	Ericsson

- R4-2010571	FR2 inter-band CA requirements	Nokia, Nokia Shanghai Bell

- R4-2010572	CR for FR2 inter-band CA requirements	Nokia, Nokia Shanghai Bell

- R4-2010573	Discussion on UL spatial relation switch	Nokia, Nokia Shanghai Bell

- R4-2010666	On Spatial Relation Switching Delay Requirements	Ericsson

- R4-2010668	On BWP switching on multiple CCs	Ericsson

- R4-2010711	On RRM requirements for BWP switching on multiple CCs	OPPO

- R4-2010712	On FR2 inter-band CA RRM requirement for CBM and/or IBM UE	OPPO

- R4-2010759	Discussion on requirements for BWP switch delay on multiple CC	NEC

- R4-2011063	Discussion on RRM remaining issues for FR2 inter-band CA	Huawei, HiSilicon

- R4-2011064	CR on maintaining measurement restriction requirements for NR CA	Huawei, HiSilicon

- R4-2011069	CR on BWP switching delay on multiple CCs	Huawei, Hisilicon

- R4-2011070	Discussion on BWP switching on multiple CCs	Huawei, Hisilicon

- R4-2011121	Discussion on SRS carrier switching interruption	Huawei, Hisilicon

- R4-2011122	Correction on the interruption requirements due to SRS carrier switching	Huawei, Hisilicon

- R4-2011123	Discussion on CSSF for inter-frequency measurement without gap in FR2 inter-band CA scenario	Huawei, Hisilicon

- R4-2011124	CSSF for inter-frequency measurement without gap in FR2 inter-band CA scenario Huawei, Hisilicon

- R4-2011125	Correction on inter-frequency without gap measurement requirements	Huawei, Hisilicon

- R4-2011126	Discussion on spatial relation switch for uplink channels and SRS	Huawei, Hisilicon

- R4-2011169	Discussion on NR CGI reading requirements	Huawei, Hisilicon

- R4-2011170	CR to 36.133 for CGI reading	Huawei, Hisilicon

- R4-2011171	CR on multiple SCell activation requirements	Huawei, Hisilicon

- R4-2011248	Partial overlap timer-based and RRC-based BWP switching delay on multiple CCs	Ericsson

- R4-2011310	Remaining open issues on NR CGI reading with autonomous gaps	ZTE

- R4-2011311	CR to 38.133 on CGI reading of NR cell	ZTE

- R4-2011312	CR to 36.133 on CGI reading of E-UTRA cell in NE-DC	ZTE

- R4-2011313	Remaining open issues on NR SRS carrier switching RRM requirements	ZTE

- R4-2011385	CGI reading core requirement discussion	Qualcomm, Inc.

- R4-2011386	SCS carrier switching core requirement discussion	Qualcomm, Inc.

- R4-2011426	discussion on CGI reading with autonomous gap	Nokia, Nokia Shanghai Bell

- R4-2011427	Response LS on CGI reading with autonomous gaps	Nokia, Nokia Shanghai Bell

- R4-2011428	discussion on BWP switch on multiple CCs	Nokia, Nokia Shanghai Bell

- R4-2012052	Email discussion summary for [96e][221] NR_RRM_Enh_RRM_1	Moderator (Intel)

- R4-2012053	Email discussion summary for [96e][222] NR_RRM_Enh_RRM_2	Moderator (ZTE)

- R4-2012054	Email discussion summary for [96e][223] NR_RRM_Enh_RRM_3	Moderator (Apple)

- R4-2012063	Email discussion summary for [96e][221] NR_RRM_Enh_RRM_1	Moderator (Intel)

- R4-2012152	WF on NR RRM requirements enhancements - BWP switching on multiple CCs	Intel Corporation

- R4-2012153	CR on BWP switching delay on multiple CCs	Huawei, Hisilicon

- R4-2012154	WF on NR RRM requirements enhancements - UL spatial relation info switch	MediaTek

- R4-2012155	WF on NR RRM requirements enhancements - SRS carrier based  switching	ZTE

- R4-2012156	Reply LS on CGI reading with autonomous gaps	RAN4

- R4-2012157	NR CGI measurements with autonomous gaps for 36.133	Ericsson Limited, Nokia, Nokia Shanghai Bell

- R4-2012158	Impact of CGI reading on L1 and L3 measurement	ZTE

- R4-2012159	CR to 36.133 for CGI reading	Huawei, Hisilicon

- R4-2012160	Email discussion summary for [96e][223] NR_RRM_Enh_RRM_3	Moderator (Apple)

- R4-2012161	WF on NR RRM requirements enhancements - inter-band FR2 CA RRM	Huawei

- R4-2012162	CR for FR2 inter-band CA requirements	Nokia, Nokia Shanghai Bell

- R4-2012163	CR on maintaining measurement restriction requirements for NR CA	Huawei, HiSilicon

- R4-2012164	WF on NR RRM requirements enhancements	Apple

- R4-2012165	CR on multiple SCells activation (section 8.3.7)	MediaTek inc.

- R4-2012166	CR on definition of inter-frequency measurements without measurement gap (9.3.1)	CMCC

- R4-2012167	CSSF for inter-frequency measurement without gap in FR2 inter-band CA sceneario	Huawei, Hisilicon

- R4-2012221	Email discussion summary for [96e][221] NR_RRM_Enh_RRM_1	Moderator (Intel)

- R4-2012222	Email discussion summary for [96e][222] NR_RRM_Enh_RRM_2	Moderator (ZTE)

- R4-2012223	Email discussion summary for [96e][223] NR_RRM_Enh_RRM_3	Moderator (Apple)

- R4-2012233	LS on multiple BWP switch impact on HARQ design in dormancy SCell	MediaTek

- R4-2012238	Correction on the interruption requirements due to SRS carrier switching	Huawei, Hisilicon

- R4-2012258	WF on NR RRM requirements enhancements - UL spatial relation info switch	MediaTek

- R4-2012266	CR on BWP switching delay on multiple CCs	Huawei, Hisilicon

- R4-2012269	LS on multiple BWP switch impact on HARQ design in dormancy SCell	RAN4

- R4-2012271	WF on NR RRM requirements enhancements - BWP switching on multiple CCs	Intel Corporation

- R4-2012272	CR on uplink spatial relation switch delay (section 8.12)	Intel Corporation

- R4-2012273	CR for FR2 inter-band CA requirements	Nokia, Nokia Shanghai Bell

- R4-2012300	CR on BWP switching delay on multiple CCs	Huawei, Hisilicon

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template