3GPP TSG RAN Meeting #89e																		RP-202081

Electronic Meeting, September 14 - 18, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.306 | CR | 0525 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of REL-16 parameter value for Access Stratum release indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 11/09/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | REL-16 is not included in the parameter value range of the Access Stratum release indicator. The Access Stratum release indicator is defined in TS 25.306, subclause 4.10 as follows: “Access stratum release indicator: This is defined as the release of the UTRA layer 1, 2, and 3 specifications that is applicable for the UE e.g. R'99, Rel-4.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | REL-16 is added to the parameter value range of the Access Stratum release indicator in Table 5.1. Impact Analysis: Impacted functionality: UE Access Stratum Release Indicator If the UE is implemented according to the CR and the network is not, or vice versa, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value range of the Access Stratum release indicator in Table 5.1 is not complete. A REL-16 compliant UE cannot be distinguished from REL-15 compliant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 25.331 CR 5962 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This issue is inherited from RAN6 and was discussed on the RAN6 reflector. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CR is a revision of RP-201976, which was revised from RP-201693. |  |  |  |  |  |  |  |  |

1st Modification

# 5	Possible UE radio access capability parameter settings

## 5.1	Value ranges

Table 5.1: UE radio access capability parameter value ranges

|  |  | UE radio access capability parameter | Value range |
| --- | --- | --- | --- |
| PDCP parameters |  | Support for RFC 2507 | Yes/No |
|  |  | Support for RFC 3095 | Yes/No |
|  |  | Support for RFC 3095 context relocation | Yes/No |
|  |  | Support for loss-less SRNS relocation | Yes/No |
|  |  | Support for loss-less DL RLC PDU size change | Yes/No |
|  |  | Maximum header compression context space | 1024, 2048, 4096, 8192, 16384, 32768, 65536, 131072 bytes |
|  |  | Maximum number of ROHC context sessions | 2, 4, 8, 12, 16, 24, 32, 48, 64, 128, 256, 512, 1024, 16384 |
|  |  | Support for Reverse Decompression | Not supported, 1..65535 |
|  |  | Support for CS voice over HSPA | Yes/No |
| RLC, MAC-hs, MAC-ehs and MAC-i/is parameters |  | Total RLC AM, MAC-hs and MAC-ehs buffer size | 2, 10, 50, 100, 150, 200, 300, 400, 500, 750, 1000, 1150, 1250, 1800, 2000, 2300, 2550, 3400, 3500, 4400, 4500, 5000 kBytes |
|  |  | Maximum number of AM entities | 3, 4, 5, 6, 8, 16, 30 |
|  |  | Maximum RLC AM window size | 2047, 4095 |
|  |  | Support for MAC-ehs | Yes/No |
|  |  | Support for two logical channels | Yes/No |
|  |  | Support of MAC-i/is | Yes/No |
|  |  | Support of MAC-ehs window size extension | Yes/No |
|  |  | Support of UM RLC re-establishment via reconfiguration | Yes/No |
| PHY parameters | Transport channel parameters in downlink | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum number of simultaneous transport channels | 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo decoding | Yes/No |
|  | Transport channel parameters in uplink | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 2, 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH of DCH type (TDD only) | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks transmitted within TTIs that start at the same time | 2, 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo encoding | Yes/No |
|  | FDD Physical channel parameters in downlink | Maximum number of DPCHcodes to be simultaneously received | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH) | 600, 1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800 |
|  |  | Support for SF 512 and 80 ms TTI for DPCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_DCH | Yes/No |
|  |  | Support of HS-SCCHless HS-DSCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_PCH and URA_PCH | Yes/No |
|  |  | Support of Enhanced F-DPCH | Yes/No |
|  |  | Support of Target Cell Pre-Configuration | Yes/No |
|  |  | Support of Enhanced Serving Cell Change for Event 1C | Yes/No |
|  |  | Support of HS-DSCH DRX operation | Yes/No |
|  |  | Support of Node B triggered HS-DPCCH transmission | Yes/No |
|  |  | Support of HS-DSCH DRX operation with second DRX cycle | Yes/No |
|  |  | Support for Two DRX schemes in URA_PCH and CELL_PCH | Yes/No |
|  |  | Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | Yes/No |
|  |  | Support for cell-specific Tx diversity configuration for dual-cell operation | Yes/No |
|  |  | Support of MIMO only with single-stream restriction | Yes/No |
|  |  | Support for dual cell with MIMO operation in different bands | Yes/No |
|  |  | Support of MIMO mode with four transmit antennas operation only with dual-stream restriction | Yes/No |
|  |  | Support of dual band operation (Radio Access Capability Band Combination List) | 1..16 |
|  |  | >Band Combination | 1..256 |
|  |  | >Supported Carrier Combination List |  |
|  |  | >>Carrier Combination (1,2) | Yes/No |
|  |  | >>Carrier Combination (2,1) | Yes/No |
|  |  | >>Carrier Combination (1,3) | Yes/No |
|  |  | >>Carrier Combination (3,1) | Yes/No |
|  |  | >>Carrier Combination (2,2) | Yes/No |
|  |  | >>Carrier Combination (1,4) | Yes/No |
|  |  | >>Carrier Combination (4,1) | Yes/No |
|  |  | >>Carrier Combination (1,5) | Yes/No |
|  |  | >>Carrier Combination (5,1) | Yes/No |
|  |  | >>Carrier Combination (1,6) | Yes/No |
|  |  | >>Carrier Combination (6,1) | Yes/No |
|  |  | >>Carrier Combination (1,7) | Yes/No |
|  |  | >>Carrier Combination (7,1) | Yes/No |
|  |  | >>Carrier Combination (2,3) | Yes/No |
|  |  | >>Carrier Combination (3,2) | Yes/No |
|  |  | >>Carrier Combination (2,4) | Yes/No |
|  |  | >>Carrier Combination (4,2) | Yes/No |
|  |  | >>Carrier Combination (2,5) | Yes/No |
|  |  | >>Carrier Combination (5,2) | Yes/No |
|  |  | >>Carrier Combination (2,6) | Yes/No |
|  |  | >>Carrier Combination (6,2) | Yes/No |
|  |  | >>Carrier Combination (3,3) | Yes/No |
|  |  | >>Carrier Combination (3,4) | Yes/No |
|  |  | >>Carrier Combination (4,3) | Yes/No |
|  |  | >>Carrier Combination (4,4) | Yes/No |
|  |  | >>Carrier Combination (3,5) | Yes/No |
|  |  | >>Carrier Combination (5,3) | Yes/No |
|  |  | Support for Multiflow | Yes/No (per frequency band) |
|  |  | >Support for Multiflow and MIMO | No/Single stream/Dual stream NOTE: If a UE supports single-stream MIMO transmission with Multiflow, then single-stream MIMO is supported in all the frequency bands where Multiflow operation is supported. The dual-stream MIMO transmission with Multiflow support is per frequency band. |
|  |  | >Support for Multiflow with non-contiguous carrier allocation | 5, 10, any gap size (per frequency band) |
|  |  | Support for Multiflow in different bands | Yes/No (per frequency band combination) |
|  |  | Support for Multiflow with MIMO operation in different bands | No/Single stream/Dual stream NOTE1: If a UE does not support "Multiflow with MIMO operation in different bands", then no MIMO is supported in any of the band combinations where Multiflow is supported. NOTE2: If a UE supports single-stream "Multiflow with MIMO operation in different bands", then single-stream MIMO is supported in all the band combinations where Multiflow is supported. NOTE3: If a UE supports dual-stream "Multiflow with MIMO operation in different bands", then dual-stream MIMO transmission is supported in all the band combinations, in which the UE supports Multiflow and dual-stream MIMO in both bands of the corresponding band combination. Otherwise the UE supports single-stream MIMO in that band combination. |
|  |  | Support of MIMO mode with four transmit antennas operation | Yes/No (per frequency band) |
|  |  | Non-contiguous multi-cell | 1..3 |
|  |  | >Aggregated cells | 2, 3, 4 |
|  |  | >Gap size | 5, 10, any gap size |
|  |  | >Non-contiguous multi-cell Combination (2,2) | Yes/No |
|  |  | >Non-contiguous multi-cell Combination (3,1) (1,3) | Yes/No |
|  |  | Support of HS-DPCCH power offset extension | Yes/No |
|  |  | Support of STTD on DL Control Channels when Multiflow operation is active | Yes/No |
|  |  | Non-contiguous multi-cell with MIMO | Yes/No |
|  |  | Support of multi-cell configuration in inter-RAT handover | Yes/No |
|  |  | Support of DPCCH2 | Yes/No |
|  |  | Support for DCH Enhancements | Basic/Full |
|  |  | Simultaneous support for DCH Enhancements and Compressed Mode operation | Yes/No |
|  |  | Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission | Yes/No |
|  |  | DRX enhancements | Yes/No |
|  |  | HS-DPCCH overhead reduction | Yes/No |
|  |  | Support of F-TPICH feedback from the Multiflow assisting cell | Yes/No |
|  |  | Support of power control algorithm 3 | Yes/No |
|  |  | Support of blind HARQ retransmissions for HSDPA | Yes/No |
|  |  | Support of HS-SCCH DRX operation | Yes/No |
|  |  | Support of simplified HS-SCCH type 1 operation | Yes/No |
|  | FDD Physical channel parameters in uplink | Maximum number of DPDCH bits transmitted per 10 ms | 600, 1200, 2400, 4800, 9600, 19200, 28800, 38400, 48000, 57600 |
|  |  | Support of E-DPDCH | Yes/No |
|  |  | Support of Discontinuous Transmission in CELL_DCH | Yes/No |
|  |  | Support of Slot Format #4 | Yes/No |
|  |  | Support for E-DPCCH power interpolation formula | Yes/No |
|  |  | Support for E-DPCCH power boosting | Yes/No |
|  |  | Support of common E-DCH | Yes/No |
|  |  | Support of Common E-RGCH based interference control | Yes/No |
|  |  | Support of Fallback to R99 PRACH | Yes/No |
|  |  | Support of Concurrent deployment | Yes/No |
|  |  | Support of TTI alignment and Per HARQ process | Yes/No |
|  |  | Support of uplink open loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of uplink closed loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of Uplink MIMO | Yes/No (per frequency band) |
|  |  | Support of Serving E-DCH cell decoupling | Yes/No |
|  |  | Support of Radio Links without DPCH/F-DPCH | Yes/No |
|  |  | Support of Cell Reselection Indication Reporting | Yes/No |
|  |  | Access Groups based access control | Yes/No |
|  |  | Enhanced TTI switching | Yes/No |
|  |  | Implicit Grant handling | Yes/No |
|  |  | DTX enhancements | Yes/No |
|  |  | Support of Dual Cell E-DCH transmission with DPDCH | Yes/No |
|  |  | Support for Dual Band Dual Cell E-DCH operation | Yes/No |
|  |  | Support of Dual Cell E-DCH operation enhancements | Yes/No |
|  | TDD 3.84 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..224 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  | TDD 3.84 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 7.68 Mcps physical channel parameters in downlink TDD 7.68 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..448 |
|  |  | Minimum SF | 32, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..32 |
|  | TDD 7.68 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 32, 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in downlink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per subframe | 1, 2, 3, …, 96 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  |  | Support 8PSK | Yes/No |
|  |  | UE specific capability Information LCR TDD | Enumerated (NF, TriRxUniTx, TriRxTriTx, HexRxUniTx, HexRxTriTx, HexRxHexTx, TwoRxUniTxDiscontiguous, TwoRxTwoTxDiscontiguous, TwoRxUniTxContiguous, TwoRxTwoTxContiguous) NOTE: If three frequencies are supported, the three frequencies shall be configured within 5 MHz; if six frequencies are supported, the six frequencies shall be configured within 10MHz; TwoRxUniTxDiscontiguous and TwoRxTwoTxDiscontiguous mean that the UE is capable of supporting two non-adjacent carriers; TwoRxUniTxContiguous and TwoRxTwoTxContiguous mean that the UE is only capable of supporting two adjacent carriers; if two non-adjacent carriers are supported, the two carriers can be in the same band or in two different bands. |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of SPS | Yes/No |
|  |  | Support of HS-SCCH/E-AGCH Discontinuous Reception | Yes/No |
|  |  | Support of SF Mode For HS-PDSCH dual stream | Yes/No |
|  |  | Support of Enhanced TS0 | Yes/No |
|  |  | Support of Non-rectangular Resource Allocation | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in uplink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of 8PSK | Yes/No |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  |  | Support of E-DCH in CELL_FACH | Yes/No |
| RF parameters | FDD RF parameters | UE power class | 3, 4 NOTE: Only power classes 3 and 4 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [4]. UEs that support band XIX shall also support band VI |
|  |  | Tx/Rx frequency separation | Defined in [4] for the respective supported radio frequency band |
|  |  | Support of Multiple Frequency Band Indicators | Yes/No |
| RF parameters | TDD 3.84 Mcps RF parameters | UE power class | 2, 3 NOTE: Only power classes 2 and 3 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
|  | TDD 1.28 Mcps RF parameters | UE power class | 2, 3 |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
| Multi-mode related parameters |  | Support of UTRA FDD | Yes/No |
|  |  | Support of UTRA TDD 3.84 Mcps | Yes/No |
|  |  | Support of UTRA TDD 1.28 Mcps | Yes/No |
| Multi-RAT related parameters |  | Support of GSM | Yes/No (per GSM frequency band) |
|  |  | Support of multi-carrier | Yes/No |
|  |  | Support of UTRAN to GERAN Network Assisted Cell Change | Yes/No |
|  |  | Support of Handover to GAN | Yes/No |
|  |  | Support of Inter-RAT PS Handover | Yes/No |
|  |  | Support of PS Handover to GAN | Yes/No |
|  |  | Support of E-UTRA FDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA FDD | Yes/No |
|  |  | Support of E-UTRA FDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Support of E-UTRA TDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA TDD | Yes/No |
|  |  | Support of E-UTRA Multiple Frequency Band Indicators | Yes/No |
|  |  | Support of E-UTRA TDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Support of RAN-assisted WLAN interworking based on RAN rules | Yes/No |
|  |  | Support of RAN-assisted WLAN interworking based on ANDSF policies | Yes/No |
|  |  | Support of extended E-UTRA frequency priority | Yes/No |
| Security parameters |  | Support of ciphering algorithm UEA0 | Yes |
|  |  | Support of ciphering algorithm UEA1 | Yes |
|  |  | Support of ciphering algorithm UEA2 | Yes |
|  |  | Support of integrity protection algorithm UIA1 | Yes |
|  |  | Support of integrity protection algorithm UIA2 | Yes |
| UE positioning related parameters |  | Standalone location method(s) supported | Yes/No |
|  |  | UE based OTDOA supported | Yes/No |
|  |  | Network assisted GPS support | Network based / UE based / Both/ None |
|  |  | Network Assisted GANSS support List: | per GANSS |
|  |  | >GANSS ID | Galileo / SBAS / Modernized GPS / QZSS / GLONASS/ BDS |
|  |  | >SBAS IDs | WAAS / EGNOS / MSAS / GAGAN |
|  |  | >GANSS mode | Network based / UE based / Both/ None |
|  |  | >GANSS Signal ID | 0..7 |
|  |  | >GANSS Signal IDs | Yes/No (per GANSS signal) |
|  |  | >Support for GANSS timing of cell frames measurement | Yes/No |
|  |  | >Support for GANSS Carrier-Phase Measurement | Yes/No |
|  |  | >Support for non-native assistance choices | Yes/No |
|  |  | Support for GPS timing of cell frames measurement | Yes/No |
|  |  | Support for IPDL | Yes/No |
|  |  | Support for Rx-Tx time difference type 2 measurement | Yes/No |
|  |  | Support for UE Positioning assisted GPS measurement validity in CELL_PCH and URA_PCH RRC states | Yes |
|  |  | Support for SFN-SFN observed time difference type 2 measurement | Yes/No |
|  |  | Additional Positioning method support List | Per Additional Positioning method |
|  |  | >AddPos ID | Barometric Pressure, WLAN, Bluetooth, MBS |
|  |  | >AddPos mode | Standalone / UE assisted / Both |
| Measurement related capabilities |  | Need for downlink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Need for uplink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Support for absolute priority based cell re-selection in UTRAN | Yes/No |
|  |  | Support for System Information Block type 11bis | Yes |
|  |  | Adjacent Frequency measurements without compressed mode | Yes/No |
|  |  | Inter-band Frequency measurements without compressed mode | Yes/No |
|  |  | Enhanced inter-frequency measurements without compressed mode | Yes/No |
|  |  | Extended measurements ID support | Yes |
|  |  | Frequency specific compressed mode | Yes/No |
|  |  | Frequency specific compressed mode for non-contiguous operation | Yes/No |
|  |  | Inter-frequency detected set measurements | Yes/No |
|  |  | Inter-frequency measurements on configured carriers without compressed mode | Yes/No |
|  |  | Cells excluded from detected set measurements | Yes/No |
|  |  | Wideband RSRQ FDD measurements | Yes/No |
|  |  | Wideband RSRQ TDD measurements | Yes/No |
|  |  | Event 2g reporting on a configured secondary downlink frequency | Yes/No |
|  |  | Enhanced UPH reporting | Yes/No |
|  |  | RRC measurement events 6h and 6i for UPH reporting | Yes/No |
|  |  | Increased UE carrier monitoring UTRA | Yes/No |
|  |  | Increased UE carrier monitoring E-UTRA | Yes/No |
|  |  | Extended RSRQ lower value range | Yes/No |
|  |  | RSRQ measurement on all symbols | Yes/No |
|  |  | QoE Measurement Collection for streaming services | Yes/No |
|  |  | QoE Measurement Collection for MTSI services | Yes/No |
| General capabilities |  | Access Stratum release indicator | R99, REL-4, REL-5, REL-6, REL-7, REL-8, REL-9, REL-10, REL-11, REL-12, REL-13, REL-14, REL-15, REL-16 |
|  |  | Device type | Benefits from NW-based battery consumption optimisation / Does not benefit from NW-based battery consumption optimisation |
|  |  | Support of DSAC and PPAC update in CELL_DCH | Yes/No |
|  |  | Support of retrievable configurations | Yes/No |
|  |  | Support of URA_PCH with seamless transition | Yes/No |
|  |  | Support of improved synchronized RRC procedures | Yes/No |
|  |  | Support of enhanced state transition | Yes/No |
|  |  | Support of simultaneous setup and release of RABs and RBs | Yes/No |
| DL capabilities with simultaneous HS-DSCH |  | DL capability with simultaneous HS-DSCH configuration | 32 kbps, 64 kbps, 128 kbps, 384 kbps |
| UL capabilities with simultaneous E-DCH |  | UL capabilities with simultaneous E-DCH | 64 kbps |
| UE based network performance measurements parameters |  | Support of logged measurements in Idle mode and PCH States | Yes/No |
|  |  | Support of UTRAN ANR | Yes/No |
| Home Node B Inbound Mobility Related Parameters | CSG Proximity Indication capabilities | Support of intra-frequency proximity indication | Yes/No |
|  |  | Support of inter-frequency proximity indication | Yes/No |
|  |  | Support of E-UTRA proximity indication | Yes/No |
|  | Neighbour Cell SI Acquisition capabilities | Support of intra-frequency SI acquisition for HO | Yes/No |
|  |  | Support of inter-frequency SI acquisition for HO | Yes/No |
|  |  | Support of E-UTRA SI acquisition for HO | Yes/No |
| IMS Voice Parameters |  | Voice over UTRA PS HS Support | Yes/No |
|  |  | SRVCC Support from UTRA to UTRA | Yes/No |
|  |  | SRVCC Support from UTRA to GERAN | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN FDD | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN TDD | Yes/No |

End of Modifications