3GPP TSG-RAN Meeting #89-e	RP-201849

# 14 – 18 September 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0008 | rev | 19 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of NRPPa positioning CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-09-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add support of NR positioning in NRPPa |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following functions are added: - Introduction of the Assistance Information Control and Assistance Information Feedback procedures (both Class 2) - Positioning information request/response/failure/update - UL-SRS Activation (Class 1) and UL-SRS Deactivation (Class2) procedures to extend the Positioning Information Transfer function - TRP information request/response/failure - Measurement request/response/failure/report/update/abort/failure indication - Support of UL NR E-CID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for NR positioning over NRPPa |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 7, 8.1, 8.2.3.2, 8.x, 8.x.2, 8.2.x, 8.2.y, 8.2.z, 8.z, 9.1.1.a, 9.1.1.b, 9.1.1.c, 9.1.1.d, 9.1.1.e, 9.1.1.f, 9.1.1.g, 9.1.1.i, 9.1.1.j, 9.1.1.k, 9.1.1.l, 9.1.a, 9.1.x, 9.2.5, 9.2.a, 9.2.b, 9.2.c, 9.2.d, 9.2.9, 9.2.aa, 9.2.bb, 9.2.cc, 9.2.x, 9.2.y, 9.2.ya, 9.2.yb, 9.2.y1, 9.2.y2, 9.2.y3, 9.2.y5, 9.2.y6, 9.2.z, 9.2.z1, 9.2.z2, 9.2.z3, 9.2.z4, 9.2.z5, 9.2.z6, 9.2.z7, 9.2.z8, 9.2.z9, 9.2.z9a, 9.2.z9b, 9.2.z9c, 9.2.z9d, 9.2.z9e, 9.2.z10, 9.2.z11, 9.2.z12, 9.2.aa1, 9.2.bb1, 9.2.bb2, 9.2.xx1, 9.2.xx2, 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0373 TS 38.305 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev6: added agreed TP in R3-197688 Rev9: added asn.1 with FFS related to editor’s notes Rev10: allocated to R3-200784, withdrawn Rev11: merged TP in R3-201290 Rev12: submitted to RAN3#107bis-e Rev13: captured R3-202872, R3-201694, R3-202755 and R3-202776 Rev14: submitted to RAN3#108-e Rev15: merged TPs R3-204208, R3-204216, R3-204207, R3-204211, R3-204212, R3-204331, R3-204299, R3-204213 Rev16: rapporteur editorial corrections and addition of co-sourcing companies Rev17: incorporation of TP R3-205153 Rev18: merged TPs R3-205632, R3-205633, R3-205634, R3-205773, R3-205784, R3-205786, ASN.1 check and e-mail revision Rev19: correction of missing extension with criticality for ResultNR IE and ResultEUTRA IEs and asn.1 alignment |  |  |  |  |  |  |  |  |

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[x]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[y]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[z]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[a]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[b]	3GPP TS 36:214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer (PHY); Measurements".

NEXT CHANGE

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

NG-RAN node: as defined in TS 38.300 [3].

ng-eNB: as defined in TS 38.300 [3].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

NEXT CHANGE

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |

NEXT CHANGE

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |

NEXT CHANGE

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

NEXT CHANGE

## 8.x	Assistance Information Transfer Procedures

### 8.x.1	Assistance Information Control

#### 8.x.1.1	General

The purpose of the Assistance Information Control procedure is to allow the LMF to signal positioning assistance information to the NG-RAN Node for assistance information broadcasting.

#### 8.x.1.2	Successful Operation

# Figure 8.x.1.2-1: Assistance Information Control procedure

The LMF initiates the procedure by sending an ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall replace any previously stored assistance information and use the received information to configure assistance information broadcasting.

If the Broadcast Priority IE is included in the Assistance Information IE, the NG-RAN Node may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the NG-RAN Node or not broadcast).

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", the NG-RAN Node may start broadcasting the assistance information. If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "stop", the NG-RAN Node may stop broadcasting the assistance information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN shall, if supported, consider that the received assistance information is applicable to the cells in this IE.

#### 8.x.1.3	Abnormal Conditions

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", and no assistance information is available, the NG-RAN Node shall consider the procedure as failed.

If neither the Assistance Information IE nor the Broadcast IE are included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall consider the procedure as failed.

### 8.x.2	Assistance Information Feedback

#### 8.x.2.1	General

The purpose of the Assistance Information Feedback procedure is to allow the NG-RAN Node to give feedback to the LMF on assistance information broadcasting.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall consider that assistance information broadcasting could not be configured for the relevant information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall, if supported, consider that the feedback provided is applicable to the cells in this IE.

#### 8.x.2.3	Abnormal Conditions

Void.

NEXT CHANGE

### 8.2.x	Positioning Information Exchange

#### 8.2.x.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

#### 8.2.x.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.x.4	Abnormal Conditions

Void.

### 8.2.y	Positioning Information Update

#### 8.2.y.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration.

#### 8.2.y.2	Successful Operation

Figure 8.2.y.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. This message shall include the SRS configuration information for all cells where the SRS configuration has changed.

#### 8.2.y.3	Unsuccessful Operation

Not Applicable.

#### 8.2.y.4	Abnormal Conditions

Void.

NEXT CHANGE

### 8.2.Z	TRP Information Exchange

#### 8.2.Z.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node.

#### 8.2.Z.2	Successful Operation

Figure 8.2.Z.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the LMF includes the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the LMF does not include the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

#### 8.2.Z.3	Unsuccessful Operation

Figure 8.2.Z.3-1: TRP Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any TRP, the NG-RAN node shall respond with a TRP INFORMATION FAILURE message.

NEXT CHANGE

### 8.2.q	Positioning Activation

#### 8.2.q.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE.

#### 8.2.q.2	Successful Operation

Figure 8.2.q.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

The message includes an indication of the UL SRS resource set to be activated. For semi-persistent UL SRS, the message also indicates the spatial relation for the semi-persistent UL SRS resource to be activated.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message.

#### 8.2.q.3	Unsuccessful Operation

Figure 8.2.q.3-1: Positioning Activation procedure, unsuccessful operation

If the NG-RAN node is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

#### 8.2.q.4	Abnormal Conditions

Void.

### 8.2.r	Positioning Deactivation

#### 8.2.r.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE.

#### 8.2.r.2	Successful Operation

Figure 8.2.y.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set to be deactivated or release all the related resources.

#### 8.2.y.3	Unsuccessful Operation

Not Applicable.

#### 8.2.y.4	Abnormal Conditions

Void.

NEXT CHANGE

## 8.z	Measurement Information Transfer

### 8.z.1	Measurement

#### 8.z.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements.

#### 8.z.1.2	Successful Operation

Figure 8.z.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List  IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the Measurement Result IE of the MEASUREMENT RESPONSE message.

#### 8.z.1.3	Unsuccessful Operation

Figure 8.z.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

#### 8.z.1.4	Abnormal Conditions

Not applicable.

### 8.z.2	Measurement Report

#### 8.z.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF.

#### 8.z.2.2	Successful Operation

Figure 8.z.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

### 8.z.3	Measurement Update

#### 8.z.3.1	General

The Measurement Update Procedure allows the LMF to notify the NG-RAN node of a change in a previously configured measurement.

#### 8.z.3.2	Successful Operation

# Figure 8.z.3.2.1: Measurement Update: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT UPDATE message. Upon receiving the message, the NG-RAN node shall overwrite the previously received measurement configuration.

#### 8.z.3.3	Unsuccessful Operation

Not applicable.

#### 8.z.3.4	Abnormal Conditions

If the NG-RAN node cannot identify the previously requested measurement to be modified, it shall consider the procedure as failed and initiate local error handling.

### 8.z.4	Measurement Abort

#### 8.z.4.1	General

The purpose of the Measurement Abort Procedure is to enable the LMF to abort an on-going measurement.

#### 8.z.4.2	Successful Operation

# Figure 8.z.4.2.1: Measurement Abort Procedure: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT ABORT message.

Upon receiving this message, the NG-RAN node shall terminate the on-going measurement identified by the LMF Measurement ID IE and may release any resources previously allocated for the same measurement.

#### 8.z.4.3	Unsuccessful Operation

Not applicable.

#### 8.z.4.4	Abnormal Conditions

If the NG-RAN node cannot identify the previously requested measurement to be aborted, it shall ignore the MEASUREMENT ABORT message.

### 8.z.5	Measurement Failure Indication

#### 8.z.2.1	General

The Measurement Failure Indication procedure allows the NG-RAN node to notify the LMF that the measurements previously requested with the Measurement procedure can no longer be reported.

#### 8.z.2.2	Successful Operation

Figure 8.z.2.2.1: Measurement Report procedure. Successful operation.

Upon reception of the MEASUREMENT FAILURE INDICATION message, the LMF shall consider that the indicated measurements have been terminated by the NG-RAN node.

NEXT CHANGE

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Location Information Transfer Procedures

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival) |  | - | - |
| Other-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Other-RAT Measurement Quantities Item | M |  | ENUMERATED (GERAN, UTRAN ,…, NR, EUTRA) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

#### 9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement is successfully initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Other-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.14 |  | YES | ignore |

#### 9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement cannot be initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by NG-RAN node to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

#### 9.1.1.5	E-CID MEASUREMENT REPORT

This message is sent by NG-RAN node to report the results of the requested E-CID measurement.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.5 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |

#### 9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the LMF to terminate the requested E-CID measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 256) |  | YES | reject |

NEXT CHANGE

#### 9.1.1.a	POSITIONING INFORMATION REQUEST

This message is sent by LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.x |  | YES | ignore |

#### 9.1.1.b	POSITIONING INFORMATION RESPONSE

This message is sent by NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.2.y5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.c	POSITIONING INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the positioning information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.d	POSITIONING INFORMATION UPDATE

This message is sent by NG-RAN node to indicate that a change in the SRS configuration has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.2.y5 |  | YES | ignore |

NEXT CHANGE

#### 9.1.1.e	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  |  |  |
| >TRP List Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| TRP Information Type List |  | 1 |  |  |  |  |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

#### 9.1.1.f	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF. This message only applies when the NG-RAN node is a gNB.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item | M | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP Information | M |  | 9.2.bb |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535. |

#### 9.1.1.g	TRP INFORMATION FAILURE

This message is sent by an NG-RAN node to indicate that the requested TRP information cannot be provided to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

#### 9.1.1.i	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.y1 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.y2 |  | - | - |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.2.y3 |  | - | - |
| Activation Time | O |  | SFN Initialisation Time 9.2.y5 | indicates the start time when the SRS activation is requested | YES | ignore |

#### 9.1.1.j	POSITIONING ACTIVATION RESPONSE

This message is sent by NG-RAN node to confirm successful UL SRS activation in the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

#### 9.1.1.k	POSITIONING ACTIVATION FAILURE

This message is sent by NG-RAN node to indicate that activation of UL SRS transmission in the UE was unsuccessful.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.l	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | YES | reject |
| CHOICE Abort Transmission | M |  |  |  | YES | Ignore |
| >SRS Resource Set ID deactivation | M |  | 9.2.y1 |  | YES | Ignore |
| >Release ALL |  | NULL |  | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose | YES | Ignore |

NEXT CHANGE

### 9.1.a	Messages for Assistance Information Transfer Procedures

#### 9.1.a.1	ASSISTANCE INFORMATION CONTROL

This message is sent by the LMF to transfer assistance information.

Direction: LMF  NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information | O |  | 9.2.a |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, …) |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.xx2 | The cell(s) that are requested to broadcast posSIB(s) according to the Assistance Information IE. | YES | reject |

#### 9.1.a.2	ASSISTANCE INFORMATION FEEDBACK

This message is sent by the NG-RAN Node to give feedback on assistance information broadcasting.

Direction: NG-RAN Node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information Failure List | O |  | 9.2.e |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.xx2 | The cells associated to the feedback provided in the Assistance Information Failure List IE. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

### 9.1.x	Messages for Measurement Information Transfer Procedures

#### 9.1.x.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| >>Search Window Information | O |  | 9.2.cc |  |  |  |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES |  |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [a] |  |  |
| SFN initialisation Time | O |  | 9.2.y5 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.x.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.2.aa |  |  |  |
| >>Measurement Result | M |  | 9.2.z1 |  |  |  |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.x.3	MEASUREMENT FAILURE

This message is sent by the NG-RAN node to report measurement failure.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

#### 9.1.x.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| TRP Measurement Report List |  | 1 |  |  | YES | reject |
| >TRP Measurement Report Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH |  |
| >>TRP ID | M |  | 9.2.aa |  | - | - |
| >>Measurement Result | M |  | 9.2.z1 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.x.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |

#### 9.1.x.6	MEASUREMENT ABORT

This message is sent by the LMF to request the NG-RAN node to abort a measurement.

Direction: LMF   NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |

#### 9.1.x.7	MEASUREMENT FAILURE INDICATION

This message is sent by the NG-RAN node to indicate that the previously requested measurements can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

NEXT CHANGE

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0 .. <maxnoMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] | - |  |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.214 [b] | - |  |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36.214 [b] | - |  |
| >>Result RSRP EUTRA |  | 1 .. <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0.. 262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.6 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  | - |  |
| >>Result RSRQ EUTRA |  | 1 . <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | 9.2.7 | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0..262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  | - |  |
| >>Result SS-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>SS-RSRP per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>Result SS-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>SS-RSRQ per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>Result CSI-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value CSI-RSRP Cell | O |  | INTEGER (0..127) | CSI-RSRP measurement aggregated at cell level | - |  |
| >>>CSI-RSRP per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRP | M |  | INTEGER (0..127) | CSI-RSRP measurement per CSI-RS resource | - |  |
| >>Result CSI-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NG-RAN CGI | O |  | 9.2.6 |  | - |  |
| >>>Value CSI-RSRQ Cell | O |  | INTEGER (0..127) | CSI-RSRQ measurement aggregated at cell level | - |  |
| >>>CSI-RSRQ per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRQ | M |  | INTEGER (0..127) | CSI-RSRQ measurement per CSI-RS resource | - |  |
| >>Angle of Arrival NR | M |  | UL Angle of Arrival 9.2.z2 |  | YES | ignore |
| Geographical Coordinates | O |  | 9.2.z9 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |
| maxCellReportNR | Maximum no. of NR cells that can be reported with one message. Value is 9. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

NEXT CHANGE

### 9.2.13	Other-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Other-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |  |  |
| >CHOICE Other-RAT Measured Results Value | M |  |  |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |  |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |  |  |
| >>>RSSI | M |  | INTEGER (0..63, ...) |  |  |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |  |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |  |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |  |  |
| >>>UTRA RSCP | O |  | INTEGER (-5..91, ...) |  |  |  |
| >>>UTRA EcNo | O |  | INTEGER (0..49, ...) | This IE applies to FDD only. |  |  |
| >>Result NR |  | 1..<maxNRMeas> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..1007) |  | - |  |
| >>>SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>SS-RSRP per SSB Resource |  | 0 .. <maxnoIndexesToReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>>SS-RSRQ per SSB Resource |  | 0 .. <maxnoIndexesToReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>>CGI NR | O | 9.2.cc |  | Cell Global Identifier of the reported NR cell | - |  |
| >>Result EUTRA |  | 1..<maxEUTRAMeas> |  |  | YES | ignore |
| >>>PCI EUTRA | M |  | 9.2.7 |  | - |  |
| >>>EARFCN | M |  | INTEGER (0..262143). |  | - |  |
| >>>RSRP EUTRA | O |  | INTEGER (0..97) |  | - |  |
| >>>RSRQ EUTRA | O |  | INTEGER (0..34) |  | - |  |
| >>>CGI EUTRA | O | 9.2.7 |  | Cell Global Identifier of the reported E-UTRA cell | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 8. |
| maxEUTRAMeas | Maximum no. of EUTRA cells that can be reported with one message. Value is 8. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

NEXT CHANGE

### 9.2.a	Assistance Information

This IE contains the assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Broadcast Periodicity for the Pos SIBs, see TS 38.331 [x] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | 9.2.d |  |
| >>>PosSIB Segments | M |  | 9.2.b |  |
| >>>Assistance Information Meta Data | O |  | 9.2.c |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

NEXT CHANGE

### 9.2.b	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | TS 37.355 [y] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 37.355 [y]). Value is 64. |

NEXT CHANGE

### 9.2.c	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) |  |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, navic ...) |  |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) |  |

NEXT CHANGE

### 9.2.d	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 37.355 [y].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ... ) |  |

NEXT CHANGE

### 9.2.e	Assistance Information Failure List

This parameter identifies the assistance information for which the NG-RAN Node failed to configure broadcasting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information Failure List |  | 1..<maxnoAssistInfoFailureListItems> |  |  |
| >PosSIB-Type | M |  | 9.2.d |  |
| >Outcome | M |  | ENUMERATED (failed, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoAssistInfoFailureListItems | Maximum no. of assistance information failure list items that can be signaled with one message. Value is 32. |

NEXT CHANGE

### 9.2.aa	TRP ID

The TRP ID IE is used to identify a TRP uniquely within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER (1..65535,…) | Identifies a TRP within an NG-RAN node |

### 9.2.bb	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.aa |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  |
| >CHOICE TRP Information Item | M |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NG-RAN CGI | M |  | 9.2.6 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>PRS Configuration | M |  | 9.2.z6 |  |
| >>SSB Information | M |  | 9.2.z7 |  |
| >>SFN Initialisation Time | M |  | 9.2.y5 |  |
| >>Spatial Direction Information | M |  | 9.2.z8 |  |
| >>Geographical Coordinates | M |  | 9.2.z9 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

### 9.2.9	NR CGI

The Cell Global Identifier NR is used to globally identify an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.8 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

### 9.2.cc	Search Window Information

This information element contains search window information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Propagation Delay | M |  | INTEGER (-3841..3841,…) | Indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Reference Time. The UL RTOA Reference Time for a target SRS is defined as T0+tSRS, where - T0 is the SFN Initialisation Time - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Granularity 4Ts, where Ts=1/(15103 2048) seconds. Centre of the search window. |
| Delay Uncertainty | M |  | INTEGER (1..246,…) | Indicates the uncertainty of the expected SRS arrival time at the TRP Granularity 4Ts, where Ts=1/(151032048) seconds. Single-sided search window. |

### 9.2.x	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | O |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. This IE is applicable only if the Resource Type IE is set to “periodic”, otherwise, it is ignored. |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |
| CHOICE Bandwidth | M |  |  |  |
| >FR1 |  |  | ENUMERATED (5kHz, 10kHz, 20kHz, 40kHz, 50kHz, 80kHz, 100kHz, ...) |  |
| >FR2 |  |  | ENUMERATED (50kHz, 100kHz, 200kHz, 400kHz,…) |  |
| SRS Resource Set List |  | 0.. 1 |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |
| >>Periodicity List |  | 0.. 1 |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |
| >>Spatial Relation Information | O |  | 9.2.y2 |  |
| >>Pathloss Reference Information | O |  | 9.2.y6 |  |
| SSB Information | O |  | 9.2.z7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.y	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1..<maxnoSRS-Carriers> |  |  |
| >Point A | M |  | INTEGER (0..3279165,…) NR ARFCN |  |
| >Uplink Channel BW-PerSCS-List |  | 1..<maxnoSCSs> |  | SCS-SpecificCarrier TS 38.331 [x] |
| >>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…) |  |
| >>Carrier Bandwidth | M |  | INTEGER(0..275,…) |  |
| >Active UL BWP | M |  |  | Only the configuration in the active UL BWP is needed. |
| >>Location And Bandwidth | M |  | INTEGER(0..37949,…) | BWP TS 38.331 [x] |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…) |  |
| >>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  |
| >>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  |
| >>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  |
| >>SRS Config | M |  |  |  |
| >>>SRS Resource List |  | 0..1 |  |  |
| >>>>SRS Resource |  | 1..<maxnoSRS-Resources> | 9.2.ya |  |
| >>>Positioning SRS Resource List |  | 0..1 |  |  |
| >>>>Positioning SRS Resource |  | 1..<maxnoSRS-PosResources> | 9.2.yb |  |
| >>>SRS Resource Set List |  | 0..1 |  |  |
| >>>>SRS Resource Set |  | 1..<maxnoSRS-ResourceSets> | 9.2.yc |  |
| >>>Positioning SRS Resource Set List |  | 0..1 |  |  |
| >>>>Positioning SRS Resource Set |  | 1..<maxnoSRS-PosResourceSets> | 9.2.yd |  |
| >PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

### 9.2.ya	SRS Resource

This information element contains the SRS resource.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER(0..63) |  |
| Number of Ports | M |  | ENUMERATED(port1, ports2, ports4) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(n1,n2,n4) |  |
| Repetition Factor | M |  | ENUMERATED(r1,r2,r4) |  |
| Frequency Domain Position | M |  | INTEGER(0..67) |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| B-SRS | M |  | INTEGER(0..3) |  |
| B-Hop | M |  | INTEGER(0..3) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type | M |  |  |  |
| >Periodic |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  |
| >Semi-persistent |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  |
| >Aperiodic |  |  |  |  |
| >>Aperiodic Resource Type | M |  | EMUNERATED(true,…) |  |
| Slot Offset | M |  | INTEGER(0..2559) |  |
| Sequence ID | M |  | INTEGER(0..1023) |  |

### 9.2.yb	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12} |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type Positioning | M |  |  |  |
| >periodic |  |  |  |  |
| >>Periodicity |  |  | ENUMERATED(slot, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920,…) |  |
| >>Offset |  |  | INTEGER(0..81919,…) |  |
| >semi-persistent |  |  |  |  |
| >>Periodicity |  |  | ENUMERATED(1,2,4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >>Offset |  |  | INTEGER(0..81919,…) |  |
| >aperiodic |  |  |  |  |
| >>slot offset |  |  | INTEGER(1..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation Positioning | O |  |  |  |
| >SSB | M |  |  |  |
| >>PCI | O |  | INTEGER (0..1007) |  |
| >>SSB index | M |  | INTEGER(0..63) |  |
| >PRS | M |  |  |  |
| >>PRS ID | O |  | INTEGER(0..255) | This IE is absent if the SRS Configuration IE is included in the MEASUREMENT REQUEST. It is optionally present if the SRS Configuration IE is included in the POSITIONING INFORMATION RESPONSE/UPDATE messages |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |

### 9.2.yc	SRS Resource Set

This information element indicates an SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Set Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>periodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>semi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |
| >>Slot offset | M |  | INTEGER(1..32) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourcePerSet | Maximum no of SRS resources per SRS resource set. Value is 16. |

### 9.2.yd	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

### 9.2.y1	SRS Resource Set ID

This information element indicates a resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER (0..15) | According to TS 38.331 [x] |

### 9.2.y2	Spatial Relation Information

This information element indicates a spatial relation for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation for Resource ID |  | 1..<maxnoSpatialRelations> |  | According to TS 38.321 [z] and TS 38.331 [x] |
| CHOICE Reference Signal | M |  |  |  |
| >NZP CSI-RS |  |  |  |  |
| >>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >SRS |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >Positioning SRS |  |  |  |  |
| >>SRS Pos Resource ID | M |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSpatialRelations | Maximum no. of Spatial Relations that can be configured. Value is 64. |

### 9.2.y3	SRS Resource Trigger

This information element indicates a DCI code point according to a SRS resource set configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Aperiodic SRS Resource Trigger List |  | 1..<maxnoSRS-TriggerStates> |  | According to TS 38.331 [x] |
| >Aperiodic SRS Resource Trigger |  |  | INTEGER (1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRSTriggerStates | Maximum no. of SRS trigger states. Value is 3. |

### 9.2.y5	SFN Initialisation Time

This information element indicates the SFN initialisation time.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN initialisation time | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

### 9.2.z1	Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.z2 |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..127) |  |
| >>UL RTOA | M |  | 9.2.z |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.z10 |  |
| >Time Stamp | M |  | 9.2.z3 |  |
| >Measurement Quality | O |  | 9.2.z4 |  |
| >Measurement Beam Information | O |  | 9.2.aa1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

### 9.2.z2	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [a] |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [a] |
| Angle Coordinate System | O |  | ENUMERATED(LCS, GCS) |  |

### 9.2.z	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [a] |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [a] |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [a] |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [a] |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [a] |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [a] |
| Additional Path List | O |  | 9.2.z11 |  |

### 9.2.z10	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [a] |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [a] |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [a] |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [a] |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [a] |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [a] |
| Additional Path List | O |  | 9.2.z11 |  |

### 9.2.z11	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |
| >>k0 | M |  | INTEGER(0..16351,…) |  |
| >>k1 | M |  | INTEGER(0..8176,…) |  |
| >>k2 | M |  | INTEGER(0..4088,…) |  |
| >>k3 | M |  | INTEGER(0..2044,…) |  |
| >>k4 | M |  | INTEGER(0..1022,…) |  |
| >>k5 | M |  | INTEGER(0..511,…) |  |
| >Path Quality | O |  | Measurement Quality 9.2.z4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

### 9.2.z3	Time Stamp

This information element contains the time stamp associated with the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| CHOICE Slot Index | M |  |  |  |
| >SCS-15 | M |  | INTEGER(0..9) |  |
| >SCS-30 | M |  | INTEGER(0..19) |  |
| >SCS-60 | M |  | INTEGER(0..39) |  |
| >SCS-120 | M |  | INTEGER(0..79) |  |
| Measurement time | O |  | SFN Initialisation Time 9.2.y5 |  |

### 9.2.z4	Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  |  |
| >>Measurement Quality | M |  | INTEGER(0..31) | TS 37.355 [y] |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) | TS 37.355 [y] |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |

### 9.2.z6	PRS Configuration

This information element contains the DL PRS configuration for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List |  | 1..<maxnoofPRSresourceSet> |  |  |
| >PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Comb Size | M |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >CP Type | M |  | ENUMERATED(normal, extended, …) |  |
| >Resource Set Periodicity | M |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >Resource Set Slot Offset | M |  | INTEGER(0..81919,…) |  |
| >Resource Repetition Factor | M |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >Resource Time Gap | M |  | ENUMERATED(tg1,tg2,tg4,tg8,tg16,tg32,…) |  |
| >Resource Number of Symbols | M |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >PRS Muting | O |  |  |  |
| >>Option1 | O |  |  |  |
| >>>Muting Pattern | M |  | 9.2.z15 |  |
| >>>Muting Bit Repetition Factor | M |  | ENUMERATED(1,2,4,8,…) |  |
| >>Option2 | O |  |  |  |
| >>>Muting Pattern | M |  | DL-PRS Muting Pattern 9.2.z15 |  |
| >PRS Resource Transmit Power | M |  | INTEGER(-60..50) |  |
| >PRS Resource List | M | 1..<maxnoofPRSresources> |  |  |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |
| >>Sequence ID | M |  | INTEGER(0..4095,…) |  |
| >>RE Offset | M |  | INTEGER(0..11) |  |
| >>Resource Slot Offset | M |  | INTEGER(0..511,…) |  |
| >>Resource Symbol Offset | M |  | INTEGER(0..12,…) |  |
| >>QCL Info | O |  |  |  |
| >>>QCL Source SSB Index | O |  | INTEGER(0..63) |  |
| >>>QCL Source PRS Info | O |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) | If it is absent, the QCL source PRS resource ID is the same as the PRS resource ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.z8	Spatial Direction Information

This information element contains the spatial direction information of the DL PRS resources for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information | M |  | 9.2.xx1 | The spatial directions of DL-PRS Resources for TRP |

### 9.2.z9	Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  |
| >Direct |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  |
| >>>normal accuracy |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. |
| >>>high accuracy |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.bb1 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Referenced |  |  |  |  |
| >>Reference Point | M |  | 9.2.z9d | The reference point is used to derive the TRP position |
| >>CHOICE Type | M |  |  |  |
| >>>Geodetic |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.z9b | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP |
| >>>Cartesian |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.z9c | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |
| DL-PRS Resource Coordinates | O |  | 9.2.z9a | DL-PRS Resource Coordinates relative to the TRP coordinate |

### 9.2.z9a	DL-PRS Resource Coordinates

This information element contains the geographical coordinates of the antenna reference points (ARP) for the DL-PRS Resources of a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL-PRS Resource Set ARP List | M | 1..<maxPRS-ResourceSets> |  |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >CHOICE DL-PRS Resource Set ARP Location | M |  |  | Relative to the geographical coordinates for the TRP. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource Set ID. |
| >>Geodetic |  |  |  |  |
| >>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.z9b |  |
| >>Cartesian |  |  |  |  |
| >>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.z9c |  |
| >DL-PRS Resource ARP List | M | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |
| >>CHOICE DL-PRS Resource ARP Location | M |  |  | Relative to the DL-PRS Resource Set ARP Location. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource ID. |
| >>>Geodetic |  |  |  |  |
| >>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.z9b |  |
| >>>Cartesian |  |  |  |  |
| >>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.z9c |  |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

### 9.2.z9b	Relative Geodetic Location

This information element provides a location relative to some known reference location in a relative geodetic coordinate system.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Milli-Arc-Second Units | M |  | ENUMERATED (0.03, 0.3, 3, ...) | Units and scale factor for the delta-latitude and delta-longitude fields. 0.03, 0.3, 3, milliarcseconds. TS 37.355 [y]. |
| Height Units | M |  | ENUMERATED (mm, cm, m, ...) | Units and scale factor for the delta-height field. 10-3 metre, 10-2 metre, TS 37.355 [y]. |
| Delta Latitude | M |  | INTEGER (-1024..1023) | Delta value in latitude in the unit provided in Milli-Arc-Second Units. TS 37.355 [y]. |
| Delta Longitude | M |  | INTEGER (-1024..1023) | Delta value in longitude in the unit provided in Milli-Arc-Second Units. TS 37.355 [y]. |
| Delta Height | M |  | INTEGER (-1024..1023) | Delta value in ellipsoidal height in the unit provided in Height Units. TS 37.355 [y]. |
| Location uncertainty | M |  | 9.2.z9e |  |

### 9.2.bb1	NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

### 9.2.z9c	Relative Cartesian Location

This information element provides a location relative to some known reference location in a relative Cartesian coordinate system.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| XYZ unit | M |  | ENUMERATED (mm, cm, dm,..) |  |
| X value | M |  | INTEGER (-216.. 216-1) | Positive value represents northing from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-216.. 216-1) | Positive value represents easting from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-215.. 215-1) | Positive value represents height above reference point in units of XYZ Unit IE. |
| Location uncertainty | M |  | 9.2.z9e |  |

### 9.2.z9d	Reference Point

This information element provides a reference point information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ReferencePoint | M |  |  | Reference point to which relative location information is related to |
| >Coordinate ID |  |  |  |  |
| >>Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>Reference Point Position | M |  | NG-RAN Access Point Position 9.2.10 |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>Reference Point High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.bb1 |  |

### 9.2.z9e	Location Uncertainty

This information element provides the location uncertainty information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| >Horizontal Uncertainty | M |  | INTEGER (0..255) | Horizontal uncertainty of the ARP latitude/longitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| >Horizontal Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |
| >Vertical Uncertainty | M |  | INTEGER (0..255) | Vertical uncertainty of the ARP altitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| >Vertical Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |

### 9.2.y6	Pathloss Reference Information

This information element indicates a pathloss reference for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Pathloss Reference Signal | M |  |  |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

### 9.2.z7	SSB Information

This information element contains the SSB time/frequency information for the TRPs.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB Info List |  | 1…<maxNoSSBs> |  |  |
| >SSB Configuration | M |  | SSB Time/Frequency Configuration 9.2.z12 |  |
| >PCI | M |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxNoSSBs | Maximum no of SSBs for which the configuration can be provided. Value is 255. |

### 9.2.z12	SSB Time/Frequency Configuration

This information element contains the time and frequency configuration of an SSB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz, 240kHz,...) |  |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, 160ms, …) |  |
| SSB half frame index | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| CHOICE SSB Position in Burst | O |  |  |  |
| >Short Bitmap |  |  | BIT STRING (SIZE(4)) |  |
| >Medium Bitmap |  |  | BIT STRING (SIZE(8)) |  |
| >Long Bitmap |  |  | BIT STRING (SIZE(64)) |  |
| SFN initialisation time | O |  | 9.2.y5 |  |

### 9.2.z15	DL-PRS Muting Pattern

This information element contains the DL-PRS muting pattern.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE DL-PRS Muting Pattern | M |  |  |  |
| >Two | M |  | BIT STRING (SIZE(2)) |  |
| >Four | M |  | BIT STRING (SIZE(4)) |  |
| >Six | M |  | BIT STRING (SIZE(6)) |  |
| >Eight | M |  | BIT STRING (SIZE(8)) |  |
| >Sixteen | M |  | BIT STRING (SIZE(16)) |  |
| >Thirty-two | M |  | BIT STRING (SIZE(32)) |  |

### 9.2.aa1	Measurement Beam Information

This information element contains the receiving beam information when measuring UL signals.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource ID | O |  | INTEGER(0..63) |  |
| PRS Resource Set ID | O |  | INTEGER(0..7) |  |
| SSB Index | O |  | INTEGER(0..63) |  |

### 9.2.xx1	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 .. < maxPRS-ResourceSets > |  |  |
| >PRS Resource Set ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. |
| >PRS Angle Item |  | 1..< maxPRS-ResourcesPerSet > |  |  |
| >>NR PRS Azimuth | M |  | INTEGER (0..359) |  |
| >>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles |
| >>NR PRS Elevation | O |  | INTEGER (0..180) |  |
| >>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles |
| LCS to GCS Translation |  | 1 .. <maxnolcs-gcs-translation> |  | Only the single value, 1, shall be used in this version of the specifications |
| >Alpha | M |  | INTEGER (0..359) |  |
| >Alpha-fine | O |  | INTEGER (0..9) | Fine angles |
| >Beta | M |  | INTEGER (0..359) |  |
| >Beta-fine | O |  | INTEGER (0..9) | Fine angles |
| >Gamma | M |  | INTEGER (0..359) |  |
| >Gamma-fine | O |  | INTEGER (0..9) | Fine angles |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. |

### 9.2.xx2	Positioning Broadcast Cells

This IE is used to indicate the cells that are requested to broadcast, or failed to broadcast, the associated posSIB(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Broadcast Cells |  | 1 .. <maxnoBcastCell> |  |  |
| >NG-RAN-CGI | M |  | 9.2.6 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoBcastCells | Maximum no. of cells broadcasting a posSIB in a NG-RAN node. Value is 16384. |

NEXT CHANGE

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

NRPPA-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

NRPPATransactionID

FROM NRPPA-CommonDataTypes

ErrorIndication,

PrivateMessage,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

OTDOAInformationRequest,

OTDOAInformationResponse,

OTDOAInformationFailure,

AssistanceInformationControl,

AssistanceInformationFeedback,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningInformationUpdate,

MeasurementRequest,

MeasurementResponse,

MeasurementFailure,

MeasurementReport,

MeasurementUpdate,

MeasurementAbort,

MeasurementFailureIndication,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation

FROM NRPPA-PDU-Contents

id-errorIndication,

id-privateMessage,

id-e-CIDMeasurementInitiation,

id-e-CIDMeasurementFailureIndication,

id-e-CIDMeasurementReport,

id-e-CIDMeasurementTermination,

id-oTDOAInformationExchange,

id-assistanceInformationControl,

id-assistanceInformationFeedback,

id-positioningInformationExchange,

id-positioningInformationUpdate,

id-Measurement,

id-MeasurementReport,

id-MeasurementUpdate,

id-MeasurementAbort,

id-MeasurementFailureIndication,

id-tRPInformationExchange,

id-positioningActivation,

id-positioningDeactivation

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NRPPA-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURES NRPPA-ELEMENTARY-PROCEDURE ::= {

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1			|

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementInitiation	|

oTDOAInformationExchange	|

positioningInformationExchange	|

measurement						|

tRPInformationExchange		|

positioningActivation,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementFailureIndication		|

e-CIDMeasurementReport					|

e-CIDMeasurementTermination				|

errorIndication							|

privateMessage							|

assistanceInformationControl			|

assistanceInformationFeedback 			|

positioningInformationUpdate			|

measurementReport						|

measurementUpdate						|

measurementAbort						|

measurementFailureIndication		|

positioningDeactivation,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

e-CIDMeasurementInitiation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-e-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-e-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-e-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-e-CIDMeasurementTermination

CRITICALITY				reject

}

oTDOAInformationExchange NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OTDOAInformationRequest

SUCCESSFUL OUTCOME		OTDOAInformationResponse

UNSUCCESSFUL OUTCOME	OTDOAInformationFailure

PROCEDURE CODE			id-oTDOAInformationExchange

CRITICALITY				reject

}

assistanceInformationControl NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationControl

PROCEDURE CODE			id-assistanceInformationControl

CRITICALITY				reject

}

assistanceInformationFeedback NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationFeedback

PROCEDURE CODE			id-assistanceInformationFeedback

CRITICALITY				reject

}

errorIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY			ignore

}

privateMessage			NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY			ignore

}

positioningInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-positioningInformationExchange

CRITICALITY				reject

}

positioningInformationUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-positioningInformationUpdate

CRITICALITY				ignore

}

measurement	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementRequest

SUCCESSFUL OUTCOME		MeasurementResponse

UNSUCCESSFUL OUTCOME	MeasurementFailure

PROCEDURE CODE			id-Measurement

CRITICALITY				reject

}

measurementReport	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementReport

PROCEDURE CODE			id-MeasurementReport

CRITICALITY				ignore

}

measurementUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementUpdate

PROCEDURE CODE			id-MeasurementUpdate

CRITICALITY				ignore

}

measurementAbort	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementAbort

PROCEDURE CODE			id-MeasurementAbort

CRITICALITY				ignore

}

measurementFailureIndication	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementFailureIndication

PROCEDURE CODE			id-MeasurementFailureIndication

CRITICALITY				ignore

}

tRPInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-tRPInformationExchange

CRITICALITY				reject

}

positioningActivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-positioningActivation

CRITICALITY				reject

}

positioningDeactivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-positioningDeactivation

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa.

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

UE-Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

TRPMeasurementQuantities,

TrpMeasurementResult,

TRP-ID,

TRPInformationTypeList,

TRPInformationList,

TRP-MeasurementRequestList,

TRP-MeasurementResponseList,

MeasurementBeamInfoRequest,

PositioningBroadcastCells,

ActivationTime,

SRSResourceSetID,

SRSSpatialRelation,

SRSResourceTrigger,

TRPList,

AbortTransmission,

SystemFrameNumber,

SlotNumber,

SFNInitialisationTime

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-Measurement-ID,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-Measurement-ID,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-TRPMeasurementQuantities,

id-MeasurementResult,

id-TRP-ID,

id-TRPInformationTypeList,

id-TRPInformationList,

id-TRP-MeasurementRequestList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList,

id-SRSSpatialRelation,

id-AbortTransmission,

id-SystemFrameNumber,

id-SlotNumber,

id-SRSResourceTrigger,

id-SFNInitialisationTime

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION REQUEST

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID			CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics				PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity				PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” --

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities				PRESENCE mandatory}|

{ ID id-OtherRATMeasurementQuantities	CRITICALITY ignore	TYPE OtherRATMeasurementQuantities		PRESENCE optional}|

{ ID id-WLANMeasurementQuantities		CRITICALITY ignore	TYPE WLANMeasurementQuantities			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION RESPONSE

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID					PRESENCE optional}|

{ ID id-OtherRATMeasurementResult	CRITICALITY ignore	TYPE OtherRATMeasurementResult		PRESENCE optional}|

{ ID id-WLANMeasurementResult		CRITICALITY ignore	TYPE WLANMeasurementResult			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION FAILURE

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT TERMINATION

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION REQUEST

--

-- **************************************************************

OTDOAInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationRequest-IEs}},

...

}

OTDOAInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Group		CRITICALITY reject	TYPE OTDOA-Information-Type			PRESENCE mandatory},

...

}

OTDOA-Information-Type ::= SEQUENCE (SIZE(1..maxnoOTDOAtypes)) OF ProtocolIE-Single-Container { { OTDOA-Information-TypeIEs} }

OTDOA-Information-TypeIEs	NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Item		CRITICALITY reject	TYPE OTDOA-Information-Type-Item	PRESENCE mandatory},

...

}

OTDOA-Information-Type-Item ::= SEQUENCE {

oTDOA-Information-Type-Item		OTDOA-Information-Item,

iE-Extensions					ProtocolExtensionContainer { { OTDOA-Information-Type-ItemExtIEs} } OPTIONAL,

...

}

OTDOA-Information-Type-ItemExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION RESPONSE

--

-- **************************************************************

OTDOAInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationResponse-IEs}},

...

}

OTDOAInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOACells						CRITICALITY ignore	TYPE OTDOACells					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION FAILURE

--

-- **************************************************************

OTDOAInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{OTDOAInformationFailure-IEs}},

...

}

OTDOAInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION CONTROL

--

-- **************************************************************

AssistanceInformationControl ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationControl-IEs}},

...

}

AssistanceInformationControl-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Assistance-Information	CRITICALITY reject	TYPE Assistance-Information		PRESENCE optional}|

{ ID id-Broadcast				CRITICALITY reject	TYPE Broadcast					PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION FEEDBACK

--

-- **************************************************************

AssistanceInformationFeedback ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationFeedback-IEs}},

...

}

AssistanceInformationFeedback-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-AssistanceInformationFailureList	CRITICALITY reject	TYPE AssistanceInformationFailureList	PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs NRPPA-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE SFNInitialisationTime		PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION FAILURE

--

-- **************************************************************

PositioningInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PositioningInformationFailure-IEs}},

...

}

PositioningInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationUpdate-IEs}},

...

}

PositioningInformationUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE SFNInitialisationTime		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REQUEST

--

-- **************************************************************

MeasurementRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementRequest-IEs}},

...

}

MeasurementRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList		CRITICALITY reject	TYPE TRP-MeasurementRequestList PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics		PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” –

{ ID id-TRPMeasurementQuantities			CRITICALITY reject	TYPE TRPMeasurementQuantities		PRESENCE mandatory}|

{ ID id-SRSConfiguration				CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime			CRITICALITY ignore	TYPE SFNInitialisationTime		PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest		CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional}|

{ ID id-SystemFrameNumber				CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional}|

{ ID id-SlotNumber						CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT RESPONSE

--

-- **************************************************************

MeasurementResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementResponse-IEs}},

...

}

MeasurementResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementResponseList	CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE

--

-- **************************************************************

MeasurementFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailure-IEs}},

...

}

MeasurementFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REPORT

--

-- **************************************************************

MeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementReport-IEs}},

...

}

MeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementReportList			CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT UPDATE

--

-- **************************************************************

MeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementUpdate-IEs}},

...

}

MeasurementUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-SRSConfiguration		CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT ABORT

--

-- **************************************************************

MeasurementAbort ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementAbort-IEs}},

...

}

MeasurementAbort-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

MeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailureIndication-IEs}},

...

}

MeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION REQUEST

--

-- **************************************************************

TRPInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationRequest-IEs}},

...

}

TRPInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPList						CRITICALITY reject	TYPE TRPList				PRESENCE mandatory}|

{ ID id-TRPInformationTypeList		CRITICALITY reject	TYPE TRPInformationTypeList	PRESENCE mandatory},	...

}

-- **************************************************************

--

#### -- TRP INFORMATION RESPONSE

--

-- **************************************************************

TRPInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationResponse-IEs}},

...

}

TRPInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPInformationList				CRITICALITY ignore	TYPE TRPInformationList			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION FAILURE

--

-- **************************************************************

TRPInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{TRPInformationFailure-IEs}},

...

}

TRPInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Activation Request

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	} |

{ ID id-ActivationTime			CRITICALITY ignore	TYPE ActivationTime				PRESENCE optional	},

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

sRSType-extension				ProtocolIE-Single-Container { { SRSType-ExtIEs} }

}

SRSType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelation	CRITICALITY ignore	EXTENSION SRSSpatialRelation PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

aperiodic					ENUMERATED{true,...},

sRSResourceTrigger			SRSResourceTrigger OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- Positioning Activation Response

--

-- **************************************************************

PositioningActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationResponseIEs} },

...

}

PositioningActivationResponseIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-SystemFrameNumber			CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional }|

{ ID id-SlotNumber					CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Activation Failure

--

-- **************************************************************

PositioningActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationFailureIEs} },

...

}

PositioningActivationFailureIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING DEACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Deactivation

--

-- **************************************************************

PositioningDeactivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningDeactivationIEs} },

...

}

PositioningDeactivationIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-AbortTransmission			CRITICALITY ignore	TYPE AbortTransmission		PRESENCE mandatory	} ,

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-SRSSpatialRelation,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource

FROM NRPPA-Constants

Criticality,

NRPPATransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NRPPA-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers;

#### -- A

AbortTransmission ::= CHOICE {

sRSResourceSetID		SRSResourceSetID,

releaseALL				NULL,

choice-extension							ProtocolIE-Single-Container { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,...},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions				ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL,

...

}

ActiveULBWP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ActivationTime ::= SFNInitialisationTime

AdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPath)) OF AdditionalPathListItem

AdditionalPathListItem ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TrpMeasurementQuality	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPathListItem-ExtIEs} } OPTIONAL,

...

}

AdditionalPathListItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (0..3, ...)

Assistance-Information ::= SEQUENCE {

systemInformation				SystemInformation,

iE-Extensions					ProtocolExtensionContainer { { Assistance-Information-ExtIEs} } OPTIONAL,

...

}

Assistance-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationFailureList ::= SEQUENCE (SIZE (1..maxnoAssistInfoFailureListItems)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

outcome							Outcome,

iE-Extensions					ProtocolExtensionContainer { {AssistanceInformationFailureList-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationFailureList-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationMetaData ::= SEQUENCE {

encrypted			ENUMERATED {true, ...}	OPTIONAL,

gNSSID				ENUMERATED {gps, sbas, qzss, galileo, glonass, bds, navic, ...}	OPTIONAL,

sBASID				ENUMERATED {waas, egnos, msas, gagan, ...}					OPTIONAL,	iE-Extensions		ProtocolExtensionContainer { { AssistanceInformationMetaData-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationMetaData-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {kHz5, kHz10, kHz20, kHz40, kHz50, kHz80, kHz100, ...},

fR2		ENUMERATED {kHz50, kHz100, kHz200, kHz400, ...},

...

}

BCCH ::= INTEGER (0..1023, ...)

Broadcast ::= ENUMERATED {

start,

stop,

...

}

BroadcastPeriodicity ::= ENUMERATED {

ms80,

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

...

}

PositioningBroadcastCells ::= SEQUENCE (SIZE (1..maxnoBcastCell)) OF NG-RAN-CGI

BSSID ::= OCTET STRING (SIZE(6))

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

protocol			CauseProtocol,

misc				CauseMisc,

cause-Extension	ProtocolIE-Single-Container {{ Cause-ExtensionIE }}

}

Cause-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

requested-item-not-supported,

requested-item-temporarily-not-available,

...

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CGI-EUTRA ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRAcellIdentifier			EUTRACellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-EUTRA-ExtIEs} } OPTIONAL,

...

}

CGI-EUTRA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CGI-NR ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier			NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-NR-ExtIEs} } OPTIONAL,

...

}

CGI-NR-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CPLength-EUTRA ::= ENUMERATED {

normal,

extended,

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality					OPTIONAL,

nrppatransactionID				NRPPATransactionID				OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

DL-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

DL-PRS ::= SEQUENCE {

prsid 					INTEGER (0..255),

dl-PRSResourceSetID		INTEGER (0..7),

dl-PRSResourceID		INTEGER (0..63)		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {DL-PRS-ExtIEs} }	OPTIONAL,

...

}

DL-PRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSMutingPattern ::= CHOICE {

two					BIT STRING (SIZE(2)),

four				BIT STRING (SIZE(4)),

six					BIT STRING (SIZE(6)),

eight				BIT STRING (SIZE(8)),

sixteen				BIT STRING (SIZE(16)),

thirty-two			BIT STRING (SIZE(32)),

choice-extension							ProtocolIE-Single-Container { { DL-PRSMutingPattern-ExtIEs } }

}

DL-PRSMutingPattern-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

DLPRSResourceCoordinates ::= SEQUENCE {

listofDL-PRSResourceSetARP		SEQUENCE (SIZE(1.. maxPRS-ResourceSets)) OF DLPRSResourceSetARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceCoordinates-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceSetARP ::= SEQUENCE {

dl-PRSResourceSetID				INTEGER (0..7),

dL-PRSResourceSetARPLocation	DL-PRSResourceSetARPLocation,

listofDL-PRSResourceARP			SEQUENCE (SIZE(1.. maxPRS-ResourcesPerSet)) OF DLPRSResourceARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceSetARP-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceSetARP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceSetARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-Single-Container { { DL-PRSResourceSetARPLocation-ExtIEs } }

}

DL-PRSResourceSetARPLocation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

DLPRSResourceARP ::= SEQUENCE {

dl-PRSResourceID			INTEGER (0..63),

dL-PRSResourceARPLocation	DL-PRSResourceARPLocation,

iE-Extensions				ProtocolExtensionContainer { { DLPRSResourceARP-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceARP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-Single-Container { { DL-PRSResourceARPLocation-ExtIEs } }

}

DL-PRSResourceARPLocation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

E-CID-MeasurementResult-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-GeographicalCoordinates	CRITICALITY ignore	EXTENSION GeographicalCoordinates PRESENCE optional},

...

}

EUTRACellIdentifier ::= BIT STRING (SIZE (28))

EARFCN ::= INTEGER (0..262143, ...)

#### -- F

#### -- G

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL,

...

}

GeographicalCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff		GNBRxTxTimeDiffMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }	OPTIONAL,

...

}

GNB-RxTxTimeDiff-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

GNBRxTxTimeDiffMeas ::= CHOICE {

k0			INTEGER (0.. 1970049),

k1			INTEGER (0.. 985025),

k2			INTEGER (0.. 492513),

k3			INTEGER (0.. 246257),

k4			INTEGER (0.. 123129),

k5			INTEGER (0.. 61565),

choice-extension		ProtocolIE-Single-Container { { GNBRxTxTimeDiffMeas-ExtIEs } }

}

GNBRxTxTimeDiffMeas-ExtIEs		NRPPA-PROTOCOL-IES ::= {

...

}

#### -- H

HESSID ::= OCTET STRING (SIZE(6))

#### -- I

#### -- J

#### -- K

#### -- L

LCG-to-GCS-TranslationItem::= SEQUENCE {

alpha				INTEGER (0..359) 	OPTIONAL,

alphaFine			INTEGER (0..9) 		OPTIONAL,

beta				INTEGER (0..359)	OPTIONAL,

betaFine			INTEGER (0..9) 		OPTIONAL,

gamma				INTEGER (0..359) 	OPTIONAL,

gammaFine			INTEGER (0..9) 		OPTIONAL,

...

}

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty		INTEGER (0..255),

horizontalConfidence		INTEGER (0..100),

verticalUncertainty			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { LocationUncertainty-ExtIEs} } OPTIONAL,

...

}

LocationUncertainty-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- M

Measurement-ID ::= INTEGER (1.. 65536)

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID		OPTIONAL,

pRS-Resource-Set-ID			PRS-Resource-Set-ID	OPTIONAL,

sSB-Index					SSB-Index			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL,

...

}

MeasurementBeamInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue					MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

... ,

sS-RSRP,

sS-RSRQ,

cSI-RSRP,

cSI-RSRQ,

angleOfArrivalNR

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

measuredResultsValue-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	},

...

}

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NG-RANAccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NG-RANAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NG-RANAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100),

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANHighAccuracyAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NGRANHighAccuracyAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-CGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nG-RANcell		NG-RANCell,

iE-Extensions				ProtocolExtensionContainer { {NG-RAN-CGI-ExtIEs} } OPTIONAL,

...

}

NG-RAN-CGI-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RANCell ::= CHOICE {

eUTRA-CellID	EUTRACellIdentifier,

nR-CellID		NRCellIdentifier,

nG-RANCell-Extension			ProtocolIE-Single-Container {{ NG-RANCell-ExtensionIE }}

}

NG-RANCell-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

NR-ARFCN ::= INTEGER (0..3279165)

NRCellIdentifier ::= BIT STRING (SIZE (36))

NR-PCI ::= INTEGER (0..1007)

NR-PRS-Beam-Information ::= SEQUENCE {

nR-PRS-Beam-InformationList SEQUENCE (SIZE(1.. maxPRS-ResourceSets)) OF NR-PRS-Beam-InformationItem,

lCG-to-GCS-TranslationList SEQUENCE (SIZE(1..maxnolcs-gcs-translation)) OF LCG-to-GCS-TranslationItem,

iE-Extensions	ProtocolExtensionContainer { { NR-PRS-Beam-Information-IEs} } OPTIONAL,

...

}

NR-PRS-Beam-Information-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NR-PRS-Beam-InformationItem ::= SEQUENCE {

pRSresourceSetID 	INTEGER (0..7),

pRSAngleItem 		SEQUENCE (SIZE(1..maxPRS-ResourcesPerSet)) OF PRSAngleItem,

...

}

NumberOfAntennaPorts-EUTRA ::= ENUMERATED {

n1-or-n2,

n4,

...

}

NumberOfDlFrames-EUTRA ::= ENUMERATED {

sf1,

sf2,

sf4,

sf6,

...

}

NumberOfDlFrames-Extended-EUTRA ::= INTEGER (1..160,...)

NumberOfFrequencyHoppingBands ::= ENUMERATED {

twobands,

fourbands,

...

}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191, ...)

#### -- O

OTDOACells ::= SEQUENCE (SIZE (1.. maxCellinRANnode)) OF SEQUENCE {

oTDOACellInfo					OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {OTDOACells-ExtIEs} } OPTIONAL,

...

}

OTDOACells-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

OTDOACell-Information-Item ::= CHOICE {

pCI-EUTRA									PCI-EUTRA,

cGI-EUTRA									CGI-EUTRA,

tAC											TAC,

eARFCN										EARFCN,

pRS-Bandwidth-EUTRA							PRS-Bandwidth-EUTRA,

pRS-ConfigurationIndex-EUTRA				PRS-ConfigurationIndex-EUTRA,

cPLength-EUTRA								CPLength-EUTRA,

numberOfDlFrames-EUTRA						NumberOfDlFrames-EUTRA,

numberOfAntennaPorts-EUTRA					NumberOfAntennaPorts-EUTRA,

sFNInitialisationTime-EUTRA					SFNInitialisationTime-EUTRA,

nG-RANAccessPointPosition					NG-RANAccessPointPosition,

pRSMutingConfiguration-EUTRA				PRSMutingConfiguration-EUTRA,

prsid-EUTRA									PRS-ID-EUTRA,

tpid-EUTRA									TP-ID-EUTRA,

tpType-EUTRA								TP-Type-EUTRA,

numberOfDlFrames-Extended-EUTRA				NumberOfDlFrames-Extended-EUTRA,

crsCPlength-EUTRA							CPLength-EUTRA,

dL-Bandwidth-EUTRA							DL-Bandwidth-EUTRA,

pRSOccasionGroup-EUTRA						PRSOccasionGroup-EUTRA,

pRSFrequencyHoppingConfiguration-EUTRA		PRSFrequencyHoppingConfiguration-EUTRA,

oTDOACell-Information-Item-Extension		ProtocolIE-Single-Container {{ OTDOACell-Information-Item-ExtensionIE }}

}

OTDOACell-Information-Item-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-TDD-Config-EUTRA-Item		CRITICALITY 	ignore	TYPE 	TDD-Config-EUTRA-Item		PRESENCE 	optional },

...

}

OTDOA-Information-Item ::= ENUMERATED {

pci,

cGI,

tac,

earfcn,

prsBandwidth,

prsConfigIndex,

cpLength,

noDlFrames,

noAntennaPorts,

sFNInitTime,

nG-RANAccessPointPosition,

prsmutingconfiguration,

prsid,

tpid,

tpType,

crsCPlength,

dlBandwidth,

multipleprsConfigurationsperCell,

prsOccasionGroup,

prsFrequencyHoppingConfiguration,

...,

tddConfig

}

OtherRATMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {OtherRATMeasurementQuantities-ItemIEs} }

OtherRATMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OtherRATMeasurementQuantities-Item	CRITICALITY reject	TYPE OtherRATMeasurementQuantities-Item PRESENCE mandatory}}

OtherRATMeasurementQuantities-Item ::= SEQUENCE {

otherRATMeasurementQuantitiesValue			OtherRATMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { OtherRATMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

OtherRATMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OtherRATMeasurementQuantitiesValue ::= ENUMERATED {

geran,

utran,

... ,

nR,

eUTRA

}

OtherRATMeasurementResult ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF OtherRATMeasuredResultsValue

OtherRATMeasuredResultsValue ::= CHOICE {

resultGERAN									ResultGERAN,

resultUTRAN									ResultUTRAN,

otherRATMeasuredResultsValue-Extension		ProtocolIE-Single-Container {{ OtherRATMeasuredResultsValue-ExtensionIE }}

}

OtherRATMeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultNR		CRITICALITY 	ignore	TYPE 	ResultNR		PRESENCE 	mandatory }|

{ ID id-ResultEUTRA		CRITICALITY 	ignore	TYPE 	ResultEUTRA		PRESENCE 	mandatory },

...

}

Outcome ::= ENUMERATED {

failed,

...

}

#### -- P

PathlossReferenceInformation ::= SEQUENCE {

pathlossReferenceSignal					PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { { PathlossReferenceInformation-ExtIEs } } OPTIONAL,

...

}

PathlossReferenceInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceSignal ::= CHOICE {

sSB-Reference					SSB,

dL-PRS-Reference				DL-PRS,

choice-Extension				ProtocolIE-Single-Container {{ PathlossReferenceSignal-ExtensionIE }}

}

PathlossReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

PCI-EUTRA ::= INTEGER (0..503, ...)

PhysCellIDGERAN ::= INTEGER (0..63, ...)

PhysCellIDUTRA-FDD ::= INTEGER (0..511, ...)

PhysCellIDUTRA-TDD ::= INTEGER (0..127, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

PeriodicityList ::= SEQUENCE (SIZE (1.. maxnoSRS-ResourcePerSet)) OF PeriodicityItem

PeriodicityItem ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4dot, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80m, ms160, ms320, ms640m, ms1280, ms2560, ms5120, ms10240, ...}

PosSIBs ::= SEQUENCE (SIZE (1.. maxNrOfPosSIBs)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

posSIB-Segments					PosSIB-Segments,

assistanceInformationMetaData	AssistanceInformationMetaData	OPTIONAL,

broadcastPriority				INTEGER (1..16,...)				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSIBs-ExtIEs} }	OPTIONAL,

...

}

PosSIBs-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Segments ::= SEQUENCE (SIZE (1.. maxNrOfSegments)) OF SEQUENCE {

assistanceDataSIBelement			OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { PosSIB-Segments-ExtIEs} }	OPTIONAL,

...

}

PosSIB-Segments-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Type ::= ENUMERATED {

posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType1-8,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType2-20,

posSibType2-21,

posSibType2-22,

posSibType2-23,

posSibType2-24,

posSibType2-25,

posSibType3-1,

posSibType4-1,

posSibType5-1,

posSibType6-1,

posSibType6-2,

posSibType6-3,

...

}

PosSRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF PosSRSResource-Item

PosSRSResource-Item ::= SEQUENCE {

srs-PosResourceId				SRSPosResourceID,

transmissionCombPos				TransmissionCombPos,

startPosition                   INTEGER (0..13),

nrofSymbols                     ENUMERATED {n1, n2, n4, n8, n12},

freqDomainShift                 INTEGER (0..268),

c-SRS	                        INTEGER (0..63),

groupOrSequenceHopping          ENUMERATED { neither, groupHopping, sequenceHopping },

resourceTypePos					ResourceTypePos,

sequenceId                      INTEGER (0.. 65535),

spatialRelationPos				SpatialRelationPos OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceID-List			PosSRSResourceID-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePeriodic,

semi-persistent		PosResourceSetTypeSemi-persistent,

aperiodic			PosResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PosResourceSetTypePeriodic ::= SEQUENCE {

posperiodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeSemi-persistent ::= SEQUENCE {

possemi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger		 	INTEGER(1..3),

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

PRSAngleItem  ::= SEQUENCE {

pRSresourceAngleID		INTEGER (0..63),

nRPRSAzimuth			INTEGER (0..359),

nRPRSAzimuthFine		INTEGER (0..9) OPTIONAL,

nRPRSElevation			INTEGER (0..180) OPTIONAL,

nRPRSElevationFine		INTEGER (0..9) OPTIONAL,

...

}

PRSInformationPos  ::= SEQUENCE {

pRS-IDPos					INTEGER(0..255) OPTIONAL,

pRS-Resource-Set-IDPos		INTEGER(0..7),

pRS-Resource-IDPos			INTEGER(0..63),

iE-Extensions					ProtocolExtensionContainer { { PRSInformationPos-ExtIEs} } OPTIONAL,

...

}

PRSInformationPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSConfiguration ::= SEQUENCE {

pRSResourceSet-List				PRSResourceSet-List,

iE-Extensions					ProtocolExtensionContainer { { PRSConfiguration-ExtIEs} } OPTIONAL,

...

}

PRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-ConfigurationIndex-EUTRA ::= INTEGER (0..4095, ...)

PRS-ID-EUTRA	::= INTEGER (0..4095, ...)

PRSMutingConfiguration-EUTRA ::= CHOICE {

two 												BIT STRING (SIZE (2)),

four 												BIT STRING (SIZE (4)),

eight 												BIT STRING (SIZE (8)),

sixteen 											BIT STRING (SIZE (16)),

thirty-two											BIT STRING (SIZE (32)),

sixty-four											BIT STRING (SIZE (64)),

one-hundred-and-twenty-eight						BIT STRING (SIZE (128)),

two-hundred-and-fifty-six							BIT STRING (SIZE (256)),

five-hundred-and-twelve								BIT STRING (SIZE (512)),

one-thousand-and-twenty-four						BIT STRING (SIZE (1024)),

pRSMutingConfiguration-EUTRA-Extension			ProtocolIE-Single-Container {{ PRSMutingConfiguration-EUTRA-ExtensionIE }}

}

PRSMutingConfiguration-EUTRA-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

PRSOccasionGroup-EUTRA ::= ENUMERATED {

og2,

og4,

og8,

og16,

og32,

og64,

og128,

...

}

PRSFrequencyHoppingConfiguration-EUTRA ::= SEQUENCE {

noOfFreqHoppingBands		NumberOfFrequencyHoppingBands,

bandPositions				SEQUENCE(SIZE (1..maxnoFreqHoppingBandsMinusOne)) OF NarrowBandIndex,

iE-Extensions				ProtocolExtensionContainer { { PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs} } OPTIONAL,

...

}

PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMuting::= SEQUENCE {

pRSMutingOption1			PRSMutingOption1,

pRSMutingOption2			PRSMutingOption2,

iE-Extensions					ProtocolExtensionContainer { { PRSMuting-ExtIEs} } OPTIONAL,

...

}

PRSMuting-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption1 ::= SEQUENCE {

mutingPattern						DL-PRSMutingPattern,

mutingBitRepetitionFactor			ENUMERATED{n1,n2,n4,n8,...},

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption1-ExtIEs} } OPTIONAL,

...

}

PRSMutingOption1-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption2 ::= SEQUENCE {

mutingPattern						DL-PRSMutingPattern,

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption2-ExtIEs} } OPTIONAL,

...

}

PRSMutingOption2-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresource)) OF PRSResource-Item

PRSResource-Item  ::= SEQUENCE {

pRSResourceID			INTEGER(0..63),

sequenceID				INTEGER(0..4095,...),

rEOffset				INTEGER(0..11),

resourceSlotOffset		INTEGER(0..511,...),

resourceSymbolOffset	INTEGER(0..12,...),

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-Item-ExtIEs} } OPTIONAL,

...

}

PRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLInfo  ::= SEQUENCE {

qCLSourceSSBIndex		INTEGER(0..63) 					OPTIONAL,

qCLSourcePRSInfo		PRSResource-QCLSourcePRSInfo	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-QCLInfo-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLSourcePRSInfo ::= SEQUENCE {

qCLSourcePRSResourceSetID		INTEGER(0..7),

qCLSourcePRSResourceID 			INTEGER(0..63) OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-QCLSourcePRSInfo-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLSourcePRSInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSet)) OF PRSResourceSet-Item

PRSResourceSet-Item ::= SEQUENCE {

pRSResourceSetID				INTEGER(0..7),

subcarrierSpacing				ENUMERATED{kHz15, kHz30, kHz60, kHz120, ...},

pRSbandwidth					INTEGER(1..63),

startPRB						INTEGER(0..2176),

pointA							INTEGER (0..3279165),

combSize						ENUMERATED{n2, n4, n6, n12, ...},

cPType							ENUMERATED{normal, extended, ...},

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,...},

resourceSetSlotOffset			INTEGER(0..81919,...),

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...},

resourceTimeGap					ENUMERATED{tg1,tg2,tg4,tg8,tg16,tg32,...},

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...},

pRSMuting						PRSMuting 		OPTIONAL,

pRSResourceTransmitPower		INTEGER(-60..50),

pRSResource-List				PRSResource-List,

iE-Extensions					ProtocolExtensionContainer { { PRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

PRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-ID ::= INTEGER (0..63)

PRS-Resource-Set-ID ::= INTEGER(0..7)

#### -- Q

#### -- R

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

choice-Extension				ProtocolIE-Single-Container {{ReferenceSignal-ExtensionIE }}

}

ReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

ReferencePoint ::= CHOICE {

relativeCoordinateID			CoordinateID,

referencePointCoordinate		NG-RANAccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-Single-Container { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

RelativeGeodeticLocation ::= SEQUENCE {

milli-Arc-SecondUnits		ENUMERATED {zerodot03, zerodot3, three, ...},	heightUnits					ENUMERATED {mm, cm, m, ...},

deltaLatitude				INTEGER (-1024.. 1023),

deltaLongitude				INTEGER (-1024.. 1023),

deltaHeight					INTEGER (-1024.. 1023),

locationUncertainty			LocationUncertainty,

iE-extensions						ProtocolExtensionContainer {{RelativeGeodeticLocation-ExtIEs }}	OPTIONAL,

...

}

RelativeGeodeticLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativeCartesianLocation ::= SEQUENCE {

xYZunit						ENUMERATED {mm, cm, dm, ...},

xvalue						INTEGER (-65536..65535),

yvalue						INTEGER (-65536..65535),

zvalue						INTEGER (-32768..32767),

locationUncertainty			LocationUncertainty,

iE-Extensions				ProtocolExtensionContainer { { RelativeCartesianLocation-ExtIEs} } OPTIONAL,

...

}

RelativeCartesianLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativePathDelay ::= CHOICE {

k0			INTEGER(0..16351,...),

k1			INTEGER(0..8176,...),

k2			INTEGER(0..4088,...),

k3		INTEGER(0..2044,...),

k4			INTEGER(0..1022,...),

k5			INTEGER(0..511,...),

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...)							OPTIONAL,

resourceType			ENUMERATED {periodic, semi-persistent, aperiodic, ...},

bandwidth				BandwidthSRS,

listOfSRSResourceSet	SEQUENCE (SIZE (1.. maxnoSRS-ResourceSets)) OF SRSResourceSet-Item	OPTIONAL,

sSBInformation			SSBInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedSRSTransmissionCharacteristics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSet-Item ::= SEQUENCE {

numberOfSRSResourcePerSet		INTEGER (1..16, ...)			OPTIONAL,

periodicityList					PeriodicityList					OPTIONAL,

spatialRelationInformation		SpatialRelationInfo		OPTIONAL,

pathlossReferenceInformation	PathlossReferenceInformation	OPTIONAL,

...

}

ResourceSetType  ::= CHOICE {

periodic			ResourceSetTypePeriodic,

semi-persistent		ResourceSetTypeSemi-persistent,

aperiodic			ResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ ResourceSetType-ExtIEs }}

}

ResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceSetTypePeriodic ::= SEQUENCE {

periodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeSemi-persistent ::= SEQUENCE {

semi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger		 	INTEGER(1..3),

slotoffset					INTEGER(1..32),

iE-Extensions				ProtocolExtensionContainer { { ResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= CHOICE {

periodic						ResourceTypePeriodic,

semi-persistent					ResourceTypeSemi-persistent,

aperiodic						ResourceTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ ResourceType-ExtIEs }}

}

ResourceType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodic ::= SEQUENCE {

periodicity		 		  ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset					INTEGER(0..2559, ...),

iE-Extensions			ProtocolExtensionContainer { { ResourceTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

ResourceTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistent ::= SEQUENCE {

periodicity			ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodic ::= SEQUENCE {

aperiodicResourceType	   ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypePos ::= CHOICE {

periodic			ResourceTypePeriodicPos,

semi-persistent		ResourceTypeSemi-persistentPos,

aperiodic			ResourceTypeAperiodicPos,

choice-extension				ProtocolIE-Single-Container {{ ResourceTypePos-ExtIEs }}

}

ResourceTypePos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodicPos ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920, ...},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodicPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypePeriodicPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistentPos ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920, ...},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistentPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeSemi-persistentPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset          INTEGER (1..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeAperiodicPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRP-Item

ResultCSI-RSRP-Item ::= SEQUENCE {

nR-PCI					NR-PCI,

nR-ARFCN				NR-ARFCN,

nG-RAN-CGI				NG-RAN-CGI													OPTIONAL,

valueCSI-RSRP-Cell		ValueRSRP-NR												OPTIONAL,

cSI-RSRP-PerCSI-RS		ResultCSI-RSRP-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRP-PerCSI-RS-Item

ResultCSI-RSRP-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRQ-Item

ResultCSI-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI														OPTIONAL,

valueCSI-RSRQ-Cell	ValueRSRQ-NR													OPTIONAL,

cSI-RSRQ-PerCSI-RS		ResultCSI-RSRQ-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRQ-PerCSI-RS-Item

ResultCSI-RSRQ-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultEUTRA ::= SEQUENCE (SIZE (1.. maxEUTRAMeas)) OF ResultEUTRA-Item

ResultEUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

valueRSRP-EUTRA		ValueRSRP-EUTRA												OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA												OPTIONAL,

cGI-EUTRA			CGI-EUTRA													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultEUTRA-Item-ExtIEs} }	OPTIONAL,

...

}

ResultEUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRP-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-EUTRA-Item

ResultRSRP-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-EUTRA			CGI-EUTRA OPTIONAL,

valueRSRP-EUTRA		ValueRSRP-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRP-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-EUTRA-Item

ResultRSRQ-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-UTRA			CGI-EUTRA OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRQ-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRP-Item

ResultSS-RSRP-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI													OPTIONAL,

valueSS-RSRP-Cell	ValueRSRP-NR												OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRP-PerSSB-Item

ResultSS-RSRP-PerSSB-Item ::= SEQUENCE {

sSB-Index			INTEGER (0..63),

valueSS-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRQ-Item

ResultSS-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

nG-RAN-CGI			NG-RAN-CGI													OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR												OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRQ-PerSSB-Item

ResultSS-RSRQ-PerSSB-Item ::= SEQUENCE {

sSB-Index			INTEGER (0..63),

valueSS-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRMeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

valueSS-RSRP-Cell	ValueRSRP-NR											OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR											OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB									OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB									OPTIONAL,

cGI-NR				CGI-NR													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} }	OPTIONAL,

...

}

ResultNR-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SCS-SpecificCarrier ::= SEQUENCE {

offsetToCarrier                     INTEGER (0..2199,...),

subcarrierSpacing                   ENUMERATED {kHz15, kHz30, kHz60, kHz120,...},

carrierBandwidth                    INTEGER (0..275,...),

iE-Extensions					ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL,

...

}

SCS-SpecificCarrier-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL,

...

}

Search-window-information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SFNInitialisationTime-EUTRA ::= BIT STRING (SIZE (64))

SlotNumber ::= INTEGER (0..79)

SpatialDirectionInformation ::= SEQUENCE {

nR-PRS-Beam-Information			NR-PRS-Beam-Information,

iE-Extensions					ProtocolExtensionContainer { { SpatialDirectionInformation-ExtIEs } } OPTIONAL,

...

}

SpatialDirectionInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationInfo ::= SEQUENCE {

spatialRelationforResourceID					SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationInfo-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationforResourceID ::= SEQUENCE (SIZE(1..maxnoSpatialRelations)) OF SpatialRelationforResourceIDItem

SpatialRelationforResourceIDItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationforResourceIDItem-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationforResourceIDItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPos ::= CHOICE {

sSBPos					SSBPos,

pRSInformationPos		PRSInformationPos,

choice-extension				ProtocolIE-Single-Container {{ SpatialInformationPos-ExtIEs }}

}

SpatialInformationPos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SRSConfig  ::= SEQUENCE {

sRSResource-List			SRSResource-List OPTIONAL,

posSRSResource-List			PosSRSResource-List OPTIONAL,

sRSResourceSet-List			SRSResourceSet-List OPTIONAL,

posSRSResourceSet-List		PosSRSResourceSet-List OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSConfig-ExtIEs } } OPTIONAL,

...

}

SRSConfig-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSCarrier-List ::= SEQUENCE (SIZE(1.. maxnoSRS-Carriers)) OF SRSCarrier-List-Item

SRSCarrier-List-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165,...),

uplinkChannelBW-PerSCS-List		UplinkChannelBW-PerSCS-List,

activeULBWP						ActiveULBWP,

pCI								INTEGER (0..1007)		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSCarrier-List-Item-ExtIEs } } OPTIONAL,

...

}

SRSCarrier-List-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,	iE-Extensions			ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSPosResourceID ::= INTEGER (0..63, ...)

SRSResource::= SEQUENCE {

sRSResourceID                   SRSResourceID,

nrofSRS-Ports                   ENUMERATED {port1, ports2, ports4},

transmissionComb				TransmissionComb,

startPosition                   INTEGER (0..13),

nrofSymbols                     ENUMERATED {n1, n2, n4},

repetitionFactor              	ENUMERATED {n1, n2, n4},

freqDomainPosition              INTEGER (0..67),

freqDomainShift                 INTEGER (0..268),

c-SRS                           INTEGER (0..63),

b-SRS                           INTEGER (0..3),

b-hop                           INTEGER (0..3),

groupOrSequenceHopping          ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType					ResourceType,

slotOffset						INTEGER (0..2559),

sequenceId                      INTEGER (0..1023),

iE-Extensions					ProtocolExtensionContainer { { SRSResource-ExtIEs } } OPTIONAL,

...

}

SRSResource-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceID ::= INTEGER (0..63, ...)

SRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-Resources)) OF SRSResource

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceID-List::= SEQUENCE (SIZE (1..maxnoSRS-ResourcePerSet)) OF SRSResourceID

SRSResourceSet::= SEQUENCE {

sRSResourceSetID                INTEGER(0..15),

sRSResourceID-List				SRSResourceID-List,

resourceSetType					ResourceSetType,

iE-Extensions					ProtocolExtensionContainer { { SRSResourceSet-ExtIEs } } OPTIONAL,

...

}

SRSResourceSet-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSetID ::= INTEGER (0..15, ...)

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL,

...

}

SRSResourceTrigger-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSSpatialRelation ::= SEQUENCE {

spatialRelationforResourceID				SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SRSSpatialRelation-ExtIEs} }	OPTIONAL,

...

}

SRSSpatialRelation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBInfo ::= SEQUENCE {

listOfSSBInfo		SEQUENCE (SIZE (1..maxNoSSBs)) OF SSBInfoList,

iE-Extensions		ProtocolExtensionContainer { {SSBInfo-ExtIEs} }	OPTIONAL,

...

}

SSBInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBInfoList ::= SEQUENCE {

pCI-NR				INTEGER  (0..1007),

sSB-Configuration	TF-Configuration,

iE-Extensions		ProtocolExtensionContainer { { SSBInfoList-ExtIEs} }	OPTIONAL,

...

}

SSBInfoList-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSB ::= SEQUENCE {

pCI-NR				INTEGER  (0..1007),

ssb-index			INTEGER  (0..63)	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SSB-ExtIEs} }	OPTIONAL,

...

}

SSB-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBPos ::= SEQUENCE {

pCI-NR				INTEGER  (0..1007)	OPTIONAL,

ssb-index			INTEGER  (0..63),

iE-Extensions		ProtocolExtensionContainer { {SSBPos-ExtIEs} }	OPTIONAL,

...

}

SSBPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBBurstPosition ::= CHOICE {

shortBitmap			BIT STRING (SIZE(4)),

mediumBitmap		BIT STRING (SIZE(8)),

longBitmap			BIT STRING (SIZE(64)),

...

}

SSB-Index ::= INTEGER(0..63)

SSID ::= OCTET STRING (SIZE(1..32))

SystemFrameNumber ::= INTEGER (0..1023)

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TAC ::= OCTET STRING (SIZE(3))

TDD-Config-EUTRA-Item ::= SEQUENCE {

subframeAssignment			ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6, ... },

iE-Extensions				ProtocolExtensionContainer { { TDD-Config-EUTRA-Item-Item-ExtIEs } } OPTIONAL,

...

}

TDD-Config-EUTRA-Item-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz120, kHz240, ...},

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sSB-BurstPosition			SSBBurstPosition	OPTIONAL,

sFN-initialisation-time		SFNInitialisationTime	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TF-Configuration-ExtIEs} }	OPTIONAL,

...

}

TF-Configuration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			SFNInitialisationTime	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL,

...

}

TimeStamp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-Single-Container { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TP-ID-EUTRA ::= INTEGER (0..4095, ...)

TP-Type-EUTRA ::= ENUMERATED { prs-only-tp, ... }

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-Single-Container { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TransmissionCombPos ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

n8    SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5)

},

choice-extension				ProtocolIE-Single-Container { { TransmissionCombPos-ExtIEs} }

}

TransmissionCombPos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementQuantities-Item,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuantities-Item ::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...

}

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOAMeasurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension							ProtocolIE-Single-Container { { TrpMeasuredResultsValue-ExtIEs } }

}

TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TrpMeasurementQuality ::= CHOICE {

timingMeasQuality		TrpMeasurementTimingQuality,

angleMeasQuality		TrpMeasurementAngleQuality,

choice-Extension		ProtocolIE-Single-Container {{ TrpMeasurementQuality-ExtIEs}}

}

TrpMeasurementQuality-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TrpMeasurementTimingQuality ::= SEQUENCE {

measurementQuality		INTEGER (0..31),

resolution				ENUMERATED {m0dot1, m1, m10, m30, ...},

...

}

TrpMeasurementAngleQuality ::= SEQUENCE {

azimuthQuality		INTEGER (0..255),

zenithQuality		INTEGER (0..255)	OPTIONAL,

resolution			ENUMERATED {deg0dot1, ...},

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRP-ID							TRP-ID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementRequestItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-MeasurementResponseList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementResponseItem

TRP-MeasurementResponseItem ::= SEQUENCE {

tRP-ID							TRP-ID,

measurementResult				TrpMeasurementResult,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementResponseItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementResponseItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationList ::= SEQUENCE (SIZE (1.. maxnoTRPs)) OF SEQUENCE {

tRP-ID							TRP-ID,

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { {TRPInformation-ExtIEs} } OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPInformation ::= SEQUENCE (SIZE (1..maxnoTRPInfoTypes)) OF TRPInformationItem

TRPInformationItem ::= CHOICE {

pCI-NR										INTEGER  (0..1007),

nG-RAN-CGI									NG-RAN-CGI,

aRFCN										INTEGER (0..3279165),

pRSConfiguration							PRSConfiguration,

sSBinformation								SSBInfo,

sFNInitialisationTime						SFNInitialisationTime,

spatialDirectionInformation					SpatialDirectionInformation,

geographicalCoordinates						GeographicalCoordinates,

choice-extension							ProtocolIE-Single-Container { { TRPInformationItem-ExtIEs } }

}

TRPInformationItem-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPInformationTypeList ::= SEQUENCE (SIZE(1.. maxnoTRPInfoTypes)) OF TRPInformationTypeItem

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBInfo,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF TRPItem

TRPItem ::= SEQUENCE {

tRP-ID		TRP-ID,

...

}

TRP-ID ::= INTEGER (1.. maxnoTRPs, ...)

TRPPositionDefinitionType ::= CHOICE {

direct		TRPPositionDirect,

referenced	TRPPositionReferenced,

choice-extension							ProtocolIE-Single-Container { { TRPPositionDefinitionType-ExtIEs } }

}

TRPPositionDefinitionType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPPositionDirect ::= SEQUENCE {

accuracy	TRPPositionDirectAccuracy,

iE-extensions		ProtocolExtensionContainer { { TRPPositionDirect-ExtIEs } }	OPTIONAL,

...

}

TRPPositionDirect-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDirectAccuracy ::= CHOICE {

tRPPosition			NG-RANAccessPointPosition						,

tRPHAposition		NGRANHighAccuracyAccessPointPosition	,

choice-extension			ProtocolIE-Single-Container { { TRPPositionDirectAccuracy-ExtIEs } }

}

TRPPositionDirectAccuracy-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPPositionReferenced ::= SEQUENCE {

referencePoint					ReferencePoint,

referencePointType				TRPReferencePointType,

iE-extensions							ProtocolExtensionContainer { { TRPPositionReferenced-ExtIEs } }	OPTIONAL,

...

}

TRPPositionReferenced-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPReferencePointType ::= CHOICE {

tRPPositionRelativeGeodetic			RelativeGeodeticLocation,

tRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension							ProtocolIE-Single-Container { { TRPReferencePointType-ExtIEs } }

}

TRPReferencePointType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UARFCN ::= INTEGER (0..16383, ...)

UE-Measurement-ID ::= INTEGER (1..15, ..., 256)

UTRA-EcN0 ::= INTEGER (0..49, ...)

UTRA-RSCP ::= INTEGER (-5..91, ...)

UL-AoA ::= SEQUENCE {

azimuthAoA				INTEGER (0..3599),

zenithAoA				INTEGER (0..1799)	OPTIONAL,

angleCoordinateSystem	ENUMERATED {lCS, gCS}	OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }	OPTIONAL,

...

}

UL-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UL-RTOAMeasurement ::= SEQUENCE {

uLRTOAmeas			ULRTOAMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

...

}

ULRTOAMeas::= CHOICE {

k0		INTEGER (0.. 1970049),

k1		INTEGER (0.. 985025),

k2		INTEGER (0.. 492513),

k3		INTEGER (0.. 246257),

k4		INTEGER (0.. 123129),

k5		INTEGER (0.. 61565),

...

}

UL-SRS-RSRP ::= INTEGER (0..127)

UplinkChannelBW-PerSCS-List ::= SEQUENCE (SIZE (1..maxnoSCSs)) OF SCS-SpecificCarrier

#### -- V

ValueRSRP-EUTRA ::= INTEGER (0..97, ...)

ValueRSRQ-EUTRA ::= INTEGER (0..34, ...)

ValueRSRP-NR ::= INTEGER (0..127)

ValueRSRQ-NR ::= INTEGER (0..127)

#### -- W

WLANMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {WLANMeasurementQuantities-ItemIEs} }

WLANMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-WLANMeasurementQuantities-Item	CRITICALITY reject	TYPE WLANMeasurementQuantities-Item PRESENCE mandatory}}

WLANMeasurementQuantities-Item ::= SEQUENCE {

wLANMeasurementQuantitiesValue			WLANMeasurementQuantitiesValue,

iE-Extensions							ProtocolExtensionContainer { { WLANMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasurementQuantitiesValue ::= ENUMERATED {

wlan,

...

}

WLANMeasurementResult ::= SEQUENCE (SIZE (1..maxNoMeas)) OF WLANMeasurementResult-Item

WLANMeasurementResult-Item ::= SEQUENCE {

wLAN-RSSI			WLAN-RSSI,

sSID				SSID					OPTIONAL,

bSSID				BSSID					OPTIONAL,

hESSID				HESSID					OPTIONAL,

operatingClass		WLANOperatingClass		OPTIONAL,

countryCode			WLANCountryCode			OPTIONAL,

wLANChannelList		WLANChannelList			OPTIONAL,

wLANBand			WLANBand				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementResult-Item-ExtIEs } }	OPTIONAL,

...

}

WLANMeasurementResult-Item-ExtIEs	NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLAN-RSSI ::= INTEGER (0..141, ...)

WLANBand ::= ENUMERATED {band2dot4, band5, ...}

WLANChannelList ::= SEQUENCE (SIZE (1..maxWLANchannels)) OF WLANChannel

WLANChannel ::= INTEGER (0..255)

WLANCountryCode ::= ENUMERATED {

unitedStates,

europe,

japan,

global,

...

}

WLANOperatingClass ::= INTEGER (0..255)

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

NRPPA-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

NRPPATransactionID		::= INTEGER (0..32767)

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local			INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-assistanceInformationControl									ProcedureCode ::= 7

id-assistanceInformationFeedback								ProcedureCode ::= 8

id-positioningInformationExchange								ProcedureCode ::= 9

id-positioningInformationUpdate									ProcedureCode ::= 10

id-Measurement													ProcedureCode ::= 11

id-MeasurementReport											ProcedureCode ::= 12

id-MeasurementUpdate											ProcedureCode ::= 13

id-MeasurementAbort												ProcedureCode ::= 14

id-MeasurementFailureIndication									ProcedureCode ::= 15

id-tRPInformationExchange										ProcedureCode ::= 16

id-positioningActivation										ProcedureCode ::= 17

id-positioningDeactivation										ProcedureCode ::= 18

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxIndexesReport							INTEGER ::= 64

maxNoMeas									INTEGER ::= 64

maxCellReport								INTEGER ::= 9

maxCellReportNR								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxEUTRAMeas								INTEGER ::= 8

maxGERANMeas								INTEGER ::= 8

maxNRMeas									INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNoPath									INTEGER ::= 2

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNoOfMeasTRPs								INTEGER ::= 64

maxnoTRPs									INTEGER ::= 65535

maxnoTRPInfoTypes							INTEGER ::= 64

maxnoofAngleInfo							INTEGER ::= 65535

maxnolcs-gcs-translation					INTEGER ::= 3

maxnoBcastCell								INTEGER ::= 16384

maxnoSRSTriggerStates						INTEGER ::= 3

maxnoSpatialRelations						INTEGER ::= 64

maxnoPosMeas								INTEGER ::= 16384

maxnoSRS-Carriers							INTEGER ::= 32

maxnoSCSs									INTEGER ::= 5

maxnoSRS-Resources							INTEGER ::= 64

maxnoSRS-PosResources						INTEGER ::= 64

maxnoSRS-ResourceSets						INTEGER ::= 16

maxnoSRS-ResourcePerSet						INTEGER ::= 16

maxnoSRS-PosResourceSets					INTEGER ::= 16

maxnoSRS-PosResourcePerSet					INTEGER ::= 16

maxPRS-ResourceSets							INTEGER ::= 2

maxPRS-ResourcesPerSet						INTEGER ::= 64

maxNoSSBs									INTEGER ::= 255

maxnoofPRSresourceSet						INTEGER ::= 8

maxnoofPRSresource							INTEGER ::= 64

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 23

id-Broadcast													ProtocolIE-ID ::= 24

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 25

id-SRSConfiguration												ProtocolIE-ID ::= 26

id-MeasurementResult											ProtocolIE-ID ::= 27

id-TRP-ID														ProtocolIE-ID ::= 28

id-TRPInformationTypeList										ProtocolIE-ID ::= 29

id-TRPInformationList											ProtocolIE-ID ::= 30

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 31

id-ResultSS-RSRP												ProtocolIE-ID ::= 32

id-ResultSS-RSRQ												ProtocolIE-ID ::= 33

id-ResultCSI-RSRP												ProtocolIE-ID ::= 34

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 35

id-AngleOfArrivalNR												ProtocolIE-ID ::= 36

id-GeographicalCoordinates										ProtocolIE-ID ::= 37

id-PositioningBroadcastCells									ProtocolIE-ID ::= 38

id-LMF-Measurement-ID											ProtocolIE-ID ::= 39

id-RAN-Measurement-ID											ProtocolIE-ID ::= 40

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

NRPPA-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Containers (5)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NRPPA-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

NRPPA-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

NRPPA-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { NRPPA-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		NRPPA-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					NRPPA-PROTOCOL-EXTENSION.&id			({ExtensionSetParam}),

criticality			NRPPA-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		NRPPA-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { NRPPA-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { NRPPA-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PRIVATE-IES.&id				({IEsSetParam}),

criticality		NRPPA-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

END OF CHANGES