3GPP TSG RAN meeting #89e 																RP-20xxxx

Electronic Meeting, 14 – 18 Sep., 2020

Agenda item:			9.1.2

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for Rel-17 Demodulation working area

Document for:	Information

# Introduction

In RAN#88e meeting the work areas of RAN4 R17 non-spectrum related WI/SIs was endorsed in [1]. One of the working area is Rel-17 Demodulation. Before RAN#89E, the email discussion for this working area was triggered and the summary was provided in [2].

# Topic #0: Objectives prioritization

## Initial round

### Candidate objectives

UE Demodulation/CSI (see detailed objectives description in Topic #1)

- Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

- Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

- Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

- Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios

- Objective 1-5: Requirements for Rel-15 multi-TRP DPS TX scheme

- Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas

- Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78)

BS Demodulation (see detailed objectives description in Topic #2)

- Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

- Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

- Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

- Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

Link adaptation requirements (see detailed objectives description in Topic #2)

- Objective 3-1: Link adaptation throughput requirements

### Companies views’ collection

Companies are encouraged to provide views on the support and prioritization of the candidate objectives takin into account RAN4 workload and system impacts.

| Whether support on proposals | 1-1 | 1-2 | 1-3 | 1-4 | 1-5 | 1-6 | 1-7 | 2-1 | 2-2 | 2-3 | 2-4 | 3-1 | FR2 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Verizon | x |  | x |  |  |  |  | x |  | x | x |  | x |
| SoftBank |  |  |  |  |  | x | x |  |  |  |  |  |  |
| Qualcomm | x |  |  |  |  | x |  |  |  |  |  |  | x |
| Samsung | x | x | x |  |  | x | x | x |  | x | x |  |  |
| MTK | x |  |  |  |  | x |  |  |  |  |  |  |  |
| Apple | x | x |  |  |  |  |  |  |  |  |  |  |  |
| China Telecom | x | x | x | x |  |  |  | x |  | x | x |  |  |
| CMCC | x | x | x |  |  |  |  | x |  |  | x |  |  |
| ZTE | x |  |  | x |  |  |  | x |  |  |  |  | x |
| Ericsson | x |  | x |  |  |  | x | x |  |  | x |  |  |
| KDDI |  |  |  |  |  |  | x |  |  |  |  |  |  |
| ORANGE | x | x | x | x |  |  |  | x | x | x |  |  |  |
| Intel | x |  | x | x | x |  |  | x |  |  | x |  |  |
| Nokia | x |  |  |  |  |  |  |  |  |  | x |  |  |
| Vodafone | x |  |  |  |  |  |  |  |  |  |  |  |  |
| Huawei | x | x | x |  |  |  | x | x |  |  | x |  |  |
| Note: Please use “x” to indicate the support on proposals. Moderator: added Vodafone preference based on comments for UE demodulation work area |  |  |  |  |  |  |  |  |  |  |  |  |  |

Companies are encouraged to provide additional comments (if any) on the support and prioritization of the candidate objectives in the table below:

| Company | Comments |
| --- | --- |
| SoftBank | 1-7 has the highest priority for us. |
| QC | In addition to the opinion collection in the above table, the technical comments/discussions should be taken into consideration when deciding prioritization/scope of R17 demod objectives. |
| MTK | We suggest to conclude on which objectives are included in Rel-17 early. So that we can still have time to fine tune to details in the agreed objective(s) |
| China Telecom | We agree with MTK’s suggestion to conclude on which objectives are included in Rel-17 early. So that we can still have time to fine tune to details in the agreed objective(s). |
| Ericsson | The x subject to objective revisions as discussed above. |
| Samsung | 1)Timeline for this WI if approved: we suggest to start the work after Rel-16 performance part be stable i.e. Q12021 or Q2 2021 pending on the progress on Rel-16 demod requirements. 2) Rel-16 demod leftover topics: if possible, we may need to reopen the discussion on Rel-16 demod leftover topics in Dec RAN-P or March RAN-P 2021. |
| KDDI | 1-7 is requested as high priority item. |
| Nokia, Nokia Shanghai Bell | 1-1: The topic description is mature enough to start work and there is near unanimous support. 2-4: We see this as a higher priority proposal. With regards to the workload of this WI, the mature UE advanced receiver scenarios and FR1 PUSCH 256QAM should be exclusively prioritized. |
| Huawei | Except for 1-1, 1-2, 1-7 is our high priority. We support the operator request. |

### Summary and recommendation for further discussion

Summary of views:

- The following objectives got the most support (>5 supporting companies)

  - UE demodulation / CSI reporting

    - Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

    - Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

    - Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

  - BS demodulation / CSI reporting

    - Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

    - Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

- The following objectives got medium support (4-5 supporting companies)

  - Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios

  - Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas

  - Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78)

  - Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

- Other objectives got less support

|  | 1-1 | 1-2 | 1-3 | 1-4 | 1-5 | 1-6 | 1-7 | 2-1 | 2-2 | 2-3 | 2-4 | 3-1 | FR2 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total | 14 | 6 | 8 | 4 | 1 | 4 | 5 | 9 | 1 | 4 | 8 |  | 3 |

- WI timelines:

  - One company suggested to discuss WI timelines.

- Rel-16 demodulation leftover topics

  - One company suggested to discuss how to handle the Rel-16 leftovers. Same topic was discussed in the pre-plenary email discussion

Moderator’s proposal

- From moderator perspective the WI can fit ~5 objectives (50% of the total number of objectives)

- Propose to select 2-4 candidate objectives for UE demodulation work area and 2 candidate objectives for the BS demodulation based on majority view to ensure reasonable work scope and fair split between UE and BS enhancements.

- Proposal 1: Prioritize the following candidate UE demodulation/CSI and BS demodulation objectives and include in the WI scope

  - UE demodulation / CSI reporting

    - Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

    - Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

  - BS demodulation / CSI reporting

    - Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

    - Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

- Proposal 2: Further discuss in the next round whether any further reduction or extension of objectives is needed.

- Proposal 3: Further discuss WI timelines in the next round

  - Option 1: start in Q1’20

  - Option 2: start in Q2’20

- Proposal 4: Further discuss Rel-16 demodulation leftover topics in Dec’20 or March’21 RAN plenary meetings

- No further discussion on Objective 3-1 and it will be handled in a separate email thread

## Intermediate round

### Companies views’ collection

#### WI objectives prioritization

Proposal 1: Prioritize the following candidate UE demodulation/CSI and BS demodulation objectives and include in the WI scope

- UE demodulation / CSI reporting

  - Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

  - Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

  - Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

- BS demodulation / CSI reporting

  - Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

  - Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

Proposal 2:	Further discuss in the next round whether any further reduction or extension of objectives is needed.

Companies are encouraged to provide comments on the proposals

| Company | Comments |
| --- | --- |

#### WI timelines

Companies are encouraged to further discuss WI timelines in the next round

- Option 1: start in Q1’20

- Option 2: start in Q2’20

| Company | Comments |
| --- | --- |

### Summary and recommendation for further discussion

## Fine-tuning round

# Topic #1: UE Demodulation/CSI requirements

## Initial round

### Candidate objectives

The following candidate objectives were identified based on pre-plenary e-mail discussion:

Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

    - Type of requirements:

    - Define PDSCH demodulation requirements

    - Further decide whether to introduce the corresponding CQI reporting requirements during the WI

    - SCS and slot duration

    - Scenario 1: Slot-based transmission and aligned SCS among cells

    - [Scenario 2: Non-slot-based transmission and/or different SCSs among cells]

    - Further discuss the assumptions for requirements definition

    - Reference receiver:

    - For scenario 1: MMSE-IRC with DMRS based interference covariance estimation

      - Note: use the DRMS for target UE’s PDSCH.

    - [For scenario 2: MMSE-IRC. Interference covariance estimation method is FFS]

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx

    - Interference profile

    - Reuse LTE interference profiles as a starting point

    - Other interference profiles are not precluded

    - TRS/CSI-RS/DMRS configuration

    - Further discuss and decide whether TRS/CSI-RS are collided among cells during the WI

    - Further discuss and decide whether DMRS is colliding with TRS/CSI-RS of interfering cell during the WI

    - [Further discuss and decide whether suppression or cancellation on interfering cell’s CSI-RS/TRS is needed during the WI]

    - As baseline, avoid network assistance and/or restriction.

Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

  - Phase I: Study on the reference receiver structure for (iterative) soft IC and evaluate the performance benefit over R-ML, by taking into account the implementation complexity and processing delay

    - Phase II: Define the requirements if needed based on the outcome of phase I

    - Target frequency: FR1, FR2

    - Number of data layers: up to 4 for FR1; 2 for FR2

    - Rx antenna number: 2Rx and 4Rx for FR1; 2Rx for FR2

    - Modulation order and spatial correlation level: further discuss and decide during the WI

Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Phase I: Evaluate the performance under practical MU-MIMO interference profile for the candidate reference receiver including MMSE-IRC receiver.

    - Further discuss and decide the interference covariance estimation method during the WI

    - Phase II: Define the requirements if needed based on the outcome of phase I

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx for FR1

Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios

    - Candidate reference receivers include neighboring cell CRS-IC [and LLR de-weighting]

    - Further discuss and decide if network assistance information including cell ID, number of ports, MBSFN configuration of the neighbouring cell is required

Objective 1-5: Demodulation requirements for Rel-15 multi-TRP DPS TX

    - Demodulation requirements for Rel-15 multi-TRP DPS TX scheme under normal propagation conditions

Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas

    - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas

    - Specify the static channel model

    - Define PDSCH demodulation requirements for the rank lower than or equal to 4.

    - Further discuss and decide whether to define PDSCH demodulation requirements for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

    - Define SDR requirements for 8Rx

    - Define CSI reporting requirements for 8Rx.

    - No PDCCH demodulation requirement is expected for 8Rx.

    - Considering the test coverage of 8Rx, test applicability rule is needed to define

    - Define applicability rule of existing performance requirements for 8Rx capable UEs.

    - After finalizing the single carrier requirements, the following CA scenarios will be specified

    - Intra-band contiguous CA with 2CC and 8Rx with up to 8-layers supported per CC

    - Inter-band CA with 8Rx/8-layers supported on one or two contiguous CCs on the identified operating bands, and with 2Rx or 4Rx supported on the other CC(s).

    - Note 1: if agreeable to be included in Rel-17 package, the RF/RRM impact needs to be included in the FR1 RF/RRM work areas.

Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78)

  - First investigate the applicable MRTD and power imbalance level, considering the network deployment scenario and UE implementation feasibility.

  - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values.

  - Introduce a UE capability report that indicates the support of dual Rx chain in a band of interest

  - Note 1: if agreeable to be included in Rel-17 package, the RF/RRM impact needs to be included in the FR1 RF/RRM work areas.

### Companies views’ collection

Companies are encouraged to provide comments on the suggested refinement of the individual WI objectives in the table below:

| Company | Comments |
| --- | --- |
| Verizon | We should prioritize the objectives 1-1 and 1-3 in Rel-17. As the objectives of Rel-17 UE demodulation, the FR2 4Rx should be in scope based on the available RAN1 and RAN2 (multi-TRP) specs requirements in Rel-16. |
| SoftBank | Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas We are interested in 8Rx deployment. Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) As shown in our motivation paper RP-201599, this proposal is motivated by the problems observed from our practical network, and we want to address it in Rel-17. During the offline discussion with some of the companies including operators, we received comments that the target bands should be generic. We are fine with it. |
| QC | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference In our opinion, Scenario 2 in SCS and slot duration should be removed. As we commented before, this scenario is a corner case, we do not see the point of further upscoping this item that is already very big. Scenario 2 in Reference receiver is not clear what it is referring to and what enhancement can possibly achieved by including this scenario. If this is not clarified, this scenario should be removed. For TRS/CSI-RS/DMRS configurations, in our opinion this is just part of the configuration to be specified in the scenario profile, none of these colliding/non-colliding information pieces can be taken into consideration for UE interference mitigation scheme, since in absence of network assistant information, blind detection of all these different configuration is infeasible given the huge number of combinations. Therefore, the most common case should be taken into interference scenario profile but it is not needed to study all difference scenarios and receiver enhancement doesn’t take this configuration into consideration. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO We don’t support studying this item. Information exchange across layers in LDPC decoder with single codeword through check nodes on parity check graph effectively performs inter-layer interference mitigation already. If a strong layer bit connected to weak layer bit through check node, interference mitigation was effectively performed by check node decoding operation. Therefore, unlike LTE, additional performance gain is small, not worth the added complexity and stretching HARQ timeline. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO For receiver architecture, we agree to keep just MMSE-IRC. Other techniques require blind detection of interference at the UE which leads to high complexity with questionable gains. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios Interference mitigation requires detection of CRS existence, power/interference channel estimation, and cancellation in channel estimation. All these algorithm requires resource and computation, therefore channel estimation and demodulation timeline can be extending by adding all these operations. The performance gain achievable by this is not obviously large enough to justify the complexity and HARQ timeline impact. LTE study shows that gain exists only when neighboring cell is lightly loaded, even in lightly loaded scenario, the amount of achievable gain is controversial. In NR, given the flexibility of configurations, different alignment of RS/data between serving and neighboring cell can degrade the achievable gain. Additionally, although the coexistence scenario may not disappear 100%, occasions will definitely decrease significantly. Moreover, R15 LTE introduce CRS-muting scheme which is a better feature to address this issue. To summarize, system performance improvement might be small, the occasions might also be limited, and there is a better feature to address this issue, therefore, we don’t think this study is needed for R17. Objective 1-5: Requirements for Rel-15 multi-TRP DPS TX scheme For demod, RAN4 has already agreed to define the requirements for m-TRP schemes in Rel-16 which cover tracking of different loops and separate channel estimation for each TRP with frequency/timing offset between two TPs. We think that Rel-16 m-TRP schemes can have much better performance than LTE schemes (designed for similar purpose) since those schemes are already designed specifically for NR. Also, in DPS scheme, only one TP transmits at a time while Rel-16 m-TRP schemes also allow for receiving from both TRPs at the same time. So, we believe that Rel-15 DPS scheme may not provide much benefit for Rel-17 NR UE. Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas This is our proposal, we support it. Rel.15 NR already introduced 800MHz with 2 layers for FR2, the processing requirements should be at least comparable with 8Rx with 100MHz. We are also talking about Rel.17 UEs so we do not see the argument that power consumption or UE over-heat would be an issue. The antenna implementation issue should be the same for LTE and our understanding is that there are already commercial devices with 8Rx for LTE. The main target does not necessarily have to be smartphones, this can be useful for other form factors such as CPEs or laptops/tablets. We do see a need for such requirements. In low correlation channel, 8L can obviously improve throughput by a lot. It’s true that when Rx correlation gets larger, the gain with 8L decreases, but 8Rx can still outperform 4Rx by configuring <8 layers. All these adaptation/design consideration are implemented on LTE 8Rx devices, NR should be able to do the same to achieve performance gain. Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) We believe that such large MRTD and power imbalance will lead to performance issues. For the power imbalance issue, it would be good to see how the 25dB was derived. With such large number, one of the links should have very low SINR so the usefulness of EN-DC is questionable as this is considered for Rel.17 when we believe SA will be available. MRTD larger than CP will also introduce performance degradation for demod. |
| MTK | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference Support this objective in general. Some detail comments below: - [Type of requirements] Enhancement on CQI can be considered only if network can guarantee same interference is experienced by CSI-RS and PDSCH. Otherwise, there is always a mismatch no mater UE enhances its CQI reporting or not. - [SCS and slot duration] Scenario 2 is not preferred. Non-slot based transmission requires blind detection which is rather complicated in NR, if there is no network assistance information. Also, we are not clear what UE can extra do for different SCS. - [Reference receiver] Remove scenario 2 which is too open and unclear. - [TRS/CSI-RS/DMRS configuration] It will be beneficial if network can provide information about the CSI-RS/TRS configuration from neighoring cells. Then UE can do something such as cancellation or suppression to improve the performance. If assistance information is not provided, we suggest to remove this sub-objective because the blind detection complexity is too high. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO No This type of UE receiver is demanding too high UE complexity and the improvement over R-ML receiver is not clear to us. Note that in LTE 2 codewords are configured with different MCS and encoded by different turbo encoder. Therefore iteration between demapper and decoders can help exchanging extrinsic information between 2 Trubo decoders. Codeword-specific CRS can also help provide additional information for iteration. But now the encoding structure is different in NR. Another critical issue is the HARQ timeline, which is shorter in NR now, making this soft IC receiver even more challenging in implementation. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO Neutral 2 key issues to make this feature works are how network pairs the UE and whether assistance information (MCS, PRB bundling, scrambling sequence) can be provided. Both parts needs some more study. At least from our understanding, blindly detecting the scrambling sequence of the interfering UE is impossible. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios Neutral LTE CRS is consistent in both time and frequency domain. Therefore, it is easier for UE to handle it. Cancellation is not the only solution. Some existing LTE assistance information can also be re-used. The only question is whether this is an urgent issue in Rel-17. Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas Support This is a CPE type UE which is customized for special purposes. Therefore we do not see complexity is a concern here. Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) We understanding Operator’s deployment concern. It would be better to discuss the UE architecture in RF session first. Given current architecture (single RF chain), we expect unacceptable performance degradation due to large power imbalance and large MRTD. |
| Apple | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference We support this. Scenario 1 should be prioritized. On RS configuration, it is suggest to remove CSI-RS cancellation related objectives. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO The gain of the advanced receiver over R-ML should be justified. Considering only single CW is defined for NR, CWIC in LTE cannot be reused. We are OK to further investigate this issue. A SI item is more appropriate. The wording in WI looks OK to us  Scenario b): Inter-layer interference for SU-MIMO – Phase I: Study on the reference receiver structure for (iterative) soft IC and evaluate the performance benefit over R-ML, by taking into account the implementation complexity and processing delay − Phase II: Define the requirements if needed based on the outcome of phase Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO Since NW assistant information is needed, a SI is more appropriate. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios Before studying this in RAN4 it would be helpful to see some results demonstrating potential gain from this scheme. Objective 1-5: Requirements for Rel-15 multi-TRP DPS TX scheme There is no benefit of introducing requirements with DPS transmission schemes. This mode has been introduced in Rel-15 with multiple TCI states. The test will verify UE capability of switching TCI sate which is already covered in RRM performance test and no additional demodulation capability. The Rel-16 mTRP transmission schemes already cover UE capability/ processing of receiving simultaneously with 2 TCI states from 2 TRP where as DPS scheme would cover reception from one TRP at a time which is not really enhancement to UE demod performance. Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas We would like to understand the market need to introduce this in Rel-17 given the limited scope of demod enhancements WI. This would also entail some RF work to specify bands that supports 8RX. Would this be allowed and feasible? Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) This objective also impacts RF and RRM core part. From UE architecture point of view, LNA can be shared for intra-band non-contiguous CCs. None collocated scenario can become quite challenging. Feasibility from UE RF perspectives should be investigated first. |
| China Telecom | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference For the SCS and slot duration, we are also ok to prioritize scenario 1 in Rel-17 to reduce the workload. It is important to avoid network assistance and restriction for the MMSE-IRC receiver, so that this receiver can be more widely used in the real network. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO Support to study the performance gain. From the operator’s perspective, we would like to seek for some receiver more advanced than LTE. Especially for this SU-MIMO scenario, the UE has full knowledge of the interference parameters, which means more robust performance gain and no network signalling. There is already a study phase. Based on the outcome of study, with consideration on both performance benefit and implementation complexity, we can then decide whether to define the requirements. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO With massive MIMO deployed, we see MU-MIMO as an important scenario for NR. Although some more advanced receivers are preferred, to reduce the spec impact, we are fine to start with MMSE-IRC. Currently we are not sure how much gain can be brought by IRC receiver, it depends on the multi-user modelling and DMRS configuration. A study phase is needed. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios This is new proposal from Intel. After internal checking, we support this proposal based on our deployment of DSS in band n1. For DSS scenario, in LTE non-MBSFN sub-frame, the LTE CRS from neighboring cell will impact the NR PDSCH performance. For LTE, CRS-IC requirement has already defined in Rel-13. Therefore, if the neighboring cell LTE CRS cannot be handled by UE in DSS scenario, there is a potential risk that NR performance would be even worse than LTE in some cases. |
| Samsung | In general, we think objective 1-1, 1-2 and 1-3 shall be prioritized. Based on operators’ request, we are also fine to include 1-7 in the Rel-17 scope. Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference Interference covariance estimation we would like to exclude the data-based interference covariance estimation which extremely complicated especially for PDSCH. Explict note shall be added by removing data-based estimation For TRS/CSI-RS colliding, not favor of interference cancellation receiver for TRS/CSI-RS measurement unless NW assistance signaling exists ,as UE cannot guarantee the enough accuracy for TRS interference estimation without NW-side assistance, and the TRS measurement error will impact the overall UE performance. Therefore, we suggest to remove this bullet totally. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO The objective is not so clear. In our understanding, RAN4 does not have definition on the interference profile for MU-MIMO yet. The study phase is to conclude such interference profile for further requirements. If above is the common understanding on the scope of study phase, it is better to revise the objective as - Phase I: Study MU-MIMO interference profile considering the practical deployment scenario for the candidate reference receiver including MMSE-IRC receiver. - Further discuss and decide the interference covariance estimation method during the WI |
| ZTE | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference Scenario 1 should have a higher priority than scenario 2. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO Since end-to-end latency demands of NR is much lower than that in LTE, e.g., user plane latency 4ms, and for URLLC 1ms, and under such restriction, the allowed headroom for the iterative IC-like method may be limited. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO For methods other than MMSE-IRC, is UE provided any network assisted information intended in this WI, or pure blind detection assumed? Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios We support this objective. |
| Ericsson | Objective 1-1: Support to include with MMSE-IRC interference aware. The scope is generally OK and we think that CQI can be discussed technically and decided during the WI. Scenario 2 can be removed to focus the scope. Objective 1-2: We do not support to include this as the complexity and benefits are not clear and, together with other more important objectives there is insufficient time for a proper study at this stage. Objective 1-3: This is OK but the scope should be limited to MMSE-IRC receiver. As Samsung point out, there will need to be work to determine the interference profile, and so we should focus the target receiver to keep the scope reasonable. Objective 1-4: The scope should be narrowed to deciding the feasibility of this CRS mitigation without network signaling. We can also de-prioritize this though. Objective 1-5: In our view the rel-16 eMIMO and HST provide sufficient requirement coverage. Objective 1-6: Technically 8RX is OK, however it is not clear that this should be prioritized considering workload. Possible could be considered if there is strong prioritization in other areas. Objective 1-7: In principle OK, but we should consider the best approach considering splitting across WI considering the need to determine maximum expected degradation. |
| KDDI | Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) In accordance with the spectrum allocation in Japan, we have the scenario as in Objective 1-7 and request it to be treated with priority to be finalized in Rel-17. |
| ORANGE | We generally agree with China Telecom views, that is to say, objectives 1-1 (scenario 1), 1-2 (with emphasis on soft-IC study), 1-3 and 1-4 should be prioritized. Some comments: In Rel. 12 the comparison of R-ML and (hard) CWIC is not valid anymore for SU-MIMO in NR since in LTE two code-words (2 CQIs) are transmitted for 2 or more spatial layers while in NR only one code-word (1 CQI) is transmitted. As a result, the CSI at Tx is degraded for managing the interference between layers in SU-MIMO. Moreover, the comparison between R-ML and CWIC was performed assuming highly correlated Rx antennas. This latter point should be revisited with more realistic Rx antenna correlation models. The argument that advanced receivers are negative towards latency should be balanced. For eMBB in FR1, the main challenge is spectral efficiency. Therefore we believe that the gains expected from advanced receivers on macro deployments exceed the slight impact on latency. We also think that DSS by supporting the control signals of both LTE and NR has a huge control overhead which result in a significant degradation of the spectral efficiency. We cannot allow additional degradation due to NR UE badly managing the neighbor cells’ CRS interference. Therefore, we strongly support LTE CRS-IC for NR UEs (objective 1.4). |
| Vodafone | 1-1 is the priority for us. For 1-3 would be interesting to understand realistic potential gains before agreeing the work. |
| Intel | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference We think that Scenario 2 is one of the typical scenarios in NR taking into account flexibility of NR design. Therefore, we think that it is rather important to ensure UEs, which support operation in such scenarios, can handle interference properly. As for TRS/CSI-RS/DMRS configuration, we think that it is mainly related to detailed discussion on simulation assumptions which is usually done during WI stage. Probably it is better to remove this information from WID. Same time, we are fine to keep it if other companies have strong position to keep it. As for network assistance, we suggest to keep it open. We can decide it during WI stage based on outcome of reference receiver discussion. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO We don’t have any technical concern to have study on this topic. Same time, taking into account limited time budget, this topic has less priority for us in comparison to other topics due to performance benefits of Soft IC receiver over R-ML are not obvious. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO Taking into account that we consider only MMSE-IRC receiver, the only open question is to define MU-MIMO interference model which will be feasible for TE vendors (because we have some issues with MU-MIMO modelling in Rel-16 eMIMO WI). Therefore, study stage on MMSE-IRC performance probably is not needed and we can focus on study of feasibility of MU-MIMO interference modelling. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios CRS-IC receiver is widely used in LTE because it provides performance benefits in typical scenarios. We expect similar observations can be done in case CRS-IC receiver will be used for NR in DSS scenarios. Same time, complexity of CRS-IC receiver is comparable to existing SU-MIMO advanced receiver (R-ML) which is supported from Rel-15. @QC: As for R15 LTE CRS muting, this feature can be activated only on some dedicated carriers on which UEs, which support this feature, can only operate. Therefore, this feature will be activated everywhere. Objective 1-5: Requirements for Rel-15 multi-TRP DPS TX scheme As we commented before, Multi-TRP DPS transmission scheme is supported from Rel-15. Features, which are required for this scenario are 2-4 “TCI states for PDSCH” and 2-4a “Additional active TCI state for PDCCH”, which are mandatory with capability signalling. Same time, all requirements for Rel-16 Multi-TRP Tx schemes are optional. Same time, this feature is only covered by HST scenarios. We think that it is rather to verify support of this feature under normal conditions. Taking into account, that a lot of technical discussions were done before, we don’t expect significant workload to define such requirements. Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas Based on our previous comments, we don’t expect significant improvement in the maximum achievable throughput. Same time we understand that 8 Rx allows to improve SNR operating point and, as result, improve coverage. Therefore, we would like to understand whether coverage is issue for CPE or laptops/tablets devices? Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) As we commented before, we think that this topic should be discussed in RF and RRM rooms first. If no issue will be observed than we can continue discussion in Demod room |
| Nokia, Nokia Shanghai Bell | Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference We support scenario A (inter-cell interference with MMSE-IRC) with DM-RS based interference covariance estimation. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO The details of the other scenarios are not clear and the advanced receiver implementation gains are highly depending on the chose interference profile and network assistance/restriction. Significant further study is needed, with corresponding time investment. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO The details of the other scenarios are not clear and the advanced receiver implementation gains are highly depending on the chose interference profile and network assistance/restriction. Significant further study is needed, with corresponding time investment. |
| Huawei | Objective 1-1: UE interference-aware receiver for scenario (a) We propose to define PDSCH demodulation requirement but open to CSI requirements. We would like to focus on slot-based scenario but open to discuss how to deal with non-slot-based scenario. Regarding receiver, we would like to consider DMRS based interference covariance estimation. Focus on FR1. For FR2, since the beamforming will be used, there is less interference expected. Rx antenna: 2Rx and 4Rx. Interference model: we would like to reuse LTE DIP profile. Regarding TRS/CSI-RS/DMRS configuration, we can further discuss the configurations in WI. There is no need to make decision now. Regarding the baseline assumption to avoid assistance and/or restriction, we would like to keep the possible solution on table when approving WI. Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO We are with Phase I and Phase II proposal. But about the exact objective, we would like to have more discussion. Focus on FR1 first. But open to FR2. UP to 4 layer for FR1 and 2 layer for FR2. Up to 4Rx for FR1. 2Rx for FR2. We can discuss MCS in WI phase. Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO We are OK with phase I and Phase II. Focus on FR1. Consider both 2Rx and 4Rx for FR1. Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios We are not in favor of doing CRS-IC for NR. But if most operators thought it would be a typical case, we would like to consider it on the condition that there is no assistance signaling introduced to avoid the impact on network. UE could do it based on blind detection. Objective 1-5: Demodulation requirements for Rel-15 multi-TRP DPS TX Not support We thought the frequency and time tracking functionality and performance has already been verified via Rel-16 requirement including requirements for HST and requirements for multiple TRP. Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas Not support. Would like to postpone it to the next release. We understood that in LTE there are such 8Rx requirements for certain TDD bands. But for NR, can we postpone specifying such requirement? Compared to LTE, NR is mandated to support 100MHz and 256QAM for DL. Combining 8Rx with them, UE needs to support MIMO equalizer up to 8-layer in 100MHz channel bandwidth. Let alone that UE may need support RLM algorithm for up to 8-layer with 100MHz channel bandwidth. UE seems more complicated, the power consumption will be increased significantly, which is not linearly proportional to bandwidth or Rx antenna number, and UE would be over-heat. And for smart phone, due to the limit of form factor, it seems difficult to guarantee the isolation between Rx antenna elements. We wonder if 8Rx is such urgent in terms of meeting the DL service requirement in the real life at the current stage. Maybe 4Rx + 100MHz for downlink would be sufficient at the current stage. Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78) Support. We think it is based on the practical deployment and would like to address the issue in the real life. The issue is caused by shared LNA or other RF chains for intra-band non-contiguous spectrum combinations. We see some commonality between EN-DC and CA. Fortunately we have the LTE demodulation performance requirements to address the similar issue for LTE intra-band non-contiguous combinations. We may consider following the similar approach to address the issue to avoid ping-pong between RF, RRM and demod sessions. |

### Summary and recommendation for further discussion

Summary of views:

Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

    - SCS and slot duration

    - Seven companies propose to prioritize Scenario 1 or only consider Scenario 1

    - Several companies open to consider Scenario 2

    - TRS/CSI-RS/DMRS configuration

    - Three companies suggest to discuss this as a part of test configuration

    - One company suggest to remove bullet 3 in case assistance information is not provided

    - Several companies suggested to remove bullet 3 (CSI-RS/TRS cancellation)

    - Network assistance

    - Two companies suggest to keep open discussion on Network assistance

    - One company suggest to avoid network assistance and/or restriction

    - Interference profile

    - One company suggest to reuse LTE profiles

Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

    - Three companies have concern to have study in Rel-17

    - Four companies support to have study.

    - Two companies suggest to consider this study with lower priority.

    - One company have concern about latency impact

    - One company have concern about workload

Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Eight companies support these requirements

    - One company is neutral

    - One company proposes to use SI for this topic

    - Two companies propose to consider study of MU-MIMO interference profile as Phase I

    - One company has question on network assistance and blind detection

    - One company suggests to understand realistic potential gains before agreeing the work.

Objective 1-4: NR PDSCH demodulation requirements for handling neighboring cell CRS in LTE-NR coexistence scenarios

    - Four companies support definition of these requirements

    - Two companies suggest not to define requirements for this scenario

    - One company is neutral

    - One company prefers to see performance benefits for this scenario, first

    - One company is fine with this scenario only in case no assistance signaling is introduced

Objective 1-5: Demodulation requirements for Rel-15 multi-TRP DPS TX

    - Four companies don’t see the benefits of these requirements

    - One company support to define requirements

Objective 1-6: Demodulation/CSI reporting requirements for downlink 8Rx antennas

    - Three companies support definition of these requirements

    - Two companies would like to understand the market need to introduce this in Rel-17

    - One companies doesn’t support definition of these requiremnets

    - One company has question about priority of this topic

Objective 1-7: Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78)

    - Two companies have concern on performance impact

    - Three companies suggest to discuss this topic in other rooms, first (RF and/or RRM)

Moderator proposal

- No further discussion on details of Objectives 1-4, 1-5, 1-6 and 1-7 based on overall objectives prioritization. Discussion can resume if companies have consensus to add more objectives into the scope.

- Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

  - Further discuss the objectives in the next round

  - Proposed revised objectives

    - Added prioritization of Scenario 1 and 2

    - Removed proposal on additional IRC receivers

    - Removed statement on network assistance and this can be discussed in the WI stage

    - Removed CSI-RS/TRS suppression receivers and this can be discussed in the WI stage

- Type of requirements: - Define PDSCH demodulation requirements - Further decide whether to introduce the corresponding CQI reporting requirements during the WI - SCS and slot duration - Scenario 1: Slot-based transmission and aligned SCS among cells (first priority) - Scenario 2: Non-slot-based transmission and/or different SCSs among cells (second priority) - Further discuss the assumptions for requirements definition - Reference receiver: - MMSE-IRC with DMRS based interference covariance estimation - Note: use the DRMS for target UE’s PDSCH. - Target frequency: FR1 - Rx antenna number: 2Rx and 4Rx - Interference profile - Reuse LTE interference profiles as a starting point - Other interference profiles are not precluded - TRS/CSI-RS/DMRS configuration - Further discuss and decide whether TRS/CSI-RS are collided among cells during the WI - Further discuss and decide whether DMRS is colliding with TRS/CSI-RS of interfering cell during the WI

- Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

  - Further discuss the objectives in the next round (no changes vs initial round)

- Phase I: Study on the reference receiver structure for (iterative) soft IC and evaluate the performance benefit over R-ML, by taking into account the implementation complexity and processing delay - Phase II: Define the requirements if needed based on the outcome of phase I - Target frequency: FR1, FR2 - Number of data layers: up to 4 for FR1; 2 for FR2 - Rx antenna number: 2Rx and 4Rx for FR1; 2Rx for FR2 - Modulation order and spatial correlation level: further discuss and decide during the WI

- Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

  - Further discuss the objectives in the next round (no changes vs 1st round)

- Phase I: Evaluate the performance under practical MU-MIMO interference profile for the candidate reference receiver including MMSE-IRC receiver. - Further discuss and decide the interference covariance estimation method during the WI - Phase II: Define the requirements if needed based on the outcome of phase I - Target frequency: FR1 - Rx antenna number: 2Rx and 4Rx for FR1

## Intermediate round

### Companies views’ collection

#### Objective 1-1: UE interference-aware receivers for Scenario a): Inter-cell interference

Candidate objectives

- Type of requirements: - Define PDSCH demodulation requirements - Further decide whether to introduce the corresponding CQI reporting requirements during the WI - SCS and slot duration - Scenario 1: Slot-based transmission and aligned SCS among cells (first priority) - Scenario 2: Non-slot-based transmission and/or different SCSs among cells (second priority) - Further discuss the assumptions for requirements definition - Reference receiver: - MMSE-IRC with DMRS based interference covariance estimation - Note: use the DRMS for target UE’s PDSCH. - Target frequency: FR1 - Rx antenna number: 2Rx and 4Rx - Interference profile - Reuse LTE interference profiles as a starting point - Other interference profiles are not precluded - TRS/CSI-RS/DMRS configuration - Further discuss and decide whether TRS/CSI-RS are collided among cells during the WI - Further discuss and decide whether DMRS is colliding with TRS/CSI-RS of interfering cell during the WI

Companies are encouraged to provide comments on further objective refinement:

| Company | Comments |
| --- | --- |

#### Objective 1-2: UE interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

Candidate objectives

- Phase I: Study on the reference receiver structure for (iterative) soft IC and evaluate the performance benefit over R-ML, by taking into account the implementation complexity and processing delay - Phase II: Define the requirements if needed based on the outcome of phase I - Target frequency: FR1, FR2 - Number of data layers: up to 4 for FR1; 2 for FR2 - Rx antenna number: 2Rx and 4Rx for FR1; 2Rx for FR2 - Modulation order and spatial correlation level: further discuss and decide during the WI

Companies are encouraged to provide comments on further objective refinement:

| Company | Comments |
| --- | --- |

#### Objective 1-3: UE interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

Candidate objective

- Phase I: Evaluate the performance under practical MU-MIMO interference profile for the candidate reference receiver including MMSE-IRC receiver. - Further discuss and decide the interference covariance estimation method during the WI - Phase II: Define the requirements if needed based on the outcome of phase I - Target frequency: FR1 - Rx antenna number: 2Rx and 4Rx for FR1

Companies are encouraged to provide comments on further objective refinement:

| Company | Comments |
| --- | --- |

## Fine-tuning round

# Topic #2: BS Demodulation requirements

## Candidate objectives

Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

    - Type of requirements: PUSCH requirement

    - Reference receiver:

    - As starting point, use MMSE-IRC with DMRS based interference covariance estimation

    - FFS Soft-IC/CW-IC for cell sectors belonging to the same site

    - Target frequency: FR1

    - Rx antenna number: further discuss and decide in the WI

    - Interference profile

    - Reuse LTE interference profiles as a starting point

    - Other interference profiles are not precluded

    - As baseline, avoid network assistance and/or restriction.

Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

    - Type of requirements: PUSCH requirement

    - Candidate reference receiver:

    - Soft IC

    - SL-IC

  - Evaluate the performance gain for different reference receivers, by taking into account the implementation complexity

    - Target frequency: FR1, FR2

    - Number of data layers: 2 for FR1 and FR2

    - Rx antenna number: 2Rx, 4Rx and 8Rx for FR1; 2Rx for FR2

Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Type of requirements: PUSCH requirement

    - Reference receiver:

    - As starting point, use CWIC

    - FFS hybrid-IC  (mixing hard-IC and soft-IC)

    - Target frequency: FR1

    - Rx antenna number: 2Rx, 4Rx and 8Rx for FR1

Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

    - Define PUSCH demodulation requirements for FR1 256QAM

## Initial round

### Companies views’ collection

Companies are encouraged to provide comments on the individual WI objectives in the table below:

| Company | Comments |
| --- | --- |
| Verizon | We would prioritize the objectives 2-1, 2-3 and 2-4 in Rel-17. |
| Samsung | In general, we think objective 2-1, 2-3 and 2-4. Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference We do not see any use cases for soft-IC/CW-IC. Such objective shall be removed (even with FFS wording) Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO For hybrid-IC, it is premature to define the hybrid-IC at this stage. We suggest to remove this bullet from 2-3 |
| China Telecom | Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference Rx antenna number: Suggest to include 2Rx, 4Rx and 8Rx. Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO Similar to the comment to UE SU-MIMO. We understand the complexity here, but there is already a study phase. Based on the outcome of study, with consideration on both performance benefit and implementation complexity, we can then decide whether to define the requirements. Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO CW-IC for MU-MIMO has been specfied for LTE. Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements In the pre-meeting discussion, all interested companies are ok to this proposal. We think this objective will not take much efforts compard to other proposals for advanced receivers. |
| ZTE | As a generic comment, we may have to keep in mind one thing that the power consumption of 5G networks might become an unexpected issue. Reports are often seen that operators can shut down BSs during nights in order to save energies. So trade-off between complexity/power consumption and performance is always the critical key in this regard. Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference MMSE-IRC can be the focus. As commented in topic #1, NR systems have a more stringent end-to-end latency demands than LTE, thus it might limit the use of iterative IC-like methods under certain complexity constraint. Interference profiles of NR may be different from that in LTE. Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO Similar comments on iterative IC-like methods. Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO Similar comments on iterative IC-like methods. Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements We support this objective. |
| Ericsson | Objective 2-1: We are OK for MMSE-IRC, but the FFS soft IC etc. should be removed to keep the scope focused. Objective 2-2, objective 2-3: We do not support to introduce these objectives since the amount of study required is large and we should develop the MMSE-IRC requirements first. Objective 2-4: We should introduce these requirements. |
| ORANGE | The outcome of the NOMA SI was indeed the acknowledgement that advanced receiver improves significantly the spectral efficiency of the UL. As suggested by China Telecom we should try to tighten the performance requirements in FR1 compared to LTE. We think objective 2-1/2-2/2-3 should be prioritized. |
| Vodafone | We would prioritise UE demod enhancements unless there is protocol work to support BS demod enhancements. |
| Intel | Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference We are fine to define requirements for MMSE-IRC receiver. Same time, definition of requirements for Soft-IC/CW-IC receiver requires more feasibility, complexity and performance benefits study. If it will be agreed to include some advanced receivers in the scope for further study, then we prefer to have the list of reference receiver candidate open and include R-ML and SL-IC receivers Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO Similar to UE, we don’t have technical concern, but consider this scenario with low priority. If it will be agreed, then we suggest to have study stage in WID (i.e. similar to UE) Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO We suggest to have study stage of complexity and performance benefits of different receiver algorithms (CW-IC and hybrid-IC) and, based on outcome of this study, decide the definition of this requirements and reference receiver. |
| Nokia, Nokia Shanghai Bell | Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference While the scenario A (inter-cell interference with MMSE-IRC) with DM-RS based interference covariance estimation is quite mature at this point, with regards to the work load in RAN4 demod, the BS advanced receivers should be down-prioritized w.r.t. to UE advanced receivers and FR1 PUSCH 256QAM Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO The details of this scenario are not clear and the advanced receiver implementation gains are highly depending on the chose interference profile and network assistance/restriction. Significant further study is needed, with corresponding time investment. Furthermore, the reference receiver architecture should be constrained as little as possible to allow performance input in line with the practical receiver performance of the real product implementations. In other words, all reference signals should be configured, and interference profile should be chosen, but the receiver implementation should be left open. Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO The details of this scenario are not clear and the advanced receiver implementation gains are highly depending on the chose interference profile and network assistance/restriction. Significant further study is needed, with corresponding time investment. Furthermore, the reference receiver architecture should be constrained as little as possible to allow performance input in line with the practical receiver performance of the real product implementations. Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements We see this as a higher priority proposal. However, Nokia’s concerns (minimal mobility is to be assumed; realistic phase noise modelling is left up to the contributing entities) should, at least, be captured as a note in the WID. Objective 3-1: Link adaptation throughput requirements The scope of introducing link adaptation throughput requirements in RAN4 is not clear and there is a risk that the effort becomes infeasibly large. Further studies are required to evaluate the potential impact and feasibility in the first place. The introduction of requirements themselves seems difficult in Rel-17, and no support is observed in the first place. |
| Huawei | Objective 2-1: Support. PUSCH requirement is preferred. Prefer MMSE-IRC with DMRS based interference covariance estimation. FR1 Further discuss how many Rx is utilized. Prefer to reuse LTE interference profile. We prefer not to preclude the solution of network assistance and/or restriction. Objective 2-2: Prefer to postpone it to the future release. Objective 2-3: Prefer to postpone it to the future release. Objective 2-4: Support |

### Summary and recommendation for further discussion

Summary of views:

- Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

  - Most companies agree to have the objectives in the WI scope

  - Four companies suggested to remove IC receiver from the scope

  - One company suggested to clarify the number of Rx antennas (include 2Rx, 4Rx and 8Rx). One company suggest to keep it open.

  - One company suggested to further look into interference profiles since they can be different from LTE. One company suggest to focus on LTE profiles.

  - One company suggest not to put restriction on NW assistance / restrictions.

- Objective 2-2: BS interference-aware receivers for Scenario b): Inter-layer interference for SU-MIMO

  - Five companies raised concerns on receiver complexity

  - One company mentioned impact on latency for iterative IC receivers

  - Four companies are against further work in Rel-17

  - Three companies suggested to have studies in Rel-17

- Objective 2-3: BS interference-aware receivers for Scenario c): Intra-cell inter-user interference for MU-MIMO

  - Several raised concerns on receiver complexity

  - One company mentioned impact on latency for iterative IC receivers

  - Four companies are against further work in Rel-17

  - One company suggested to remove hybrid-IC

  - Further studies supported by at least 4 companies

- Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

  - Supported by almost all companies

  - One company proposed to capture a note that minimal mobility is to be assumed and realistic phase noise modelling is left up to the contributing entities

Moderator proposal

- Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

  - Further discuss the objectives in the next round

  - Proposed revised objectives

    - Removed IC receivers

    - Left interference profiles discussion up to WI stage

    - Removed the statement on NW assistances and restrictions. This can be discussed in WI stage.

Define BS demodulation requirements for Inter-cell interference scenario - Type of requirements: PUSCH requirement - Reference receiver: - MMSE-IRC with DMRS based interference covariance estimation - Target frequency: FR1 - Rx antenna number: further discuss and decide in the WI - Identify the interference profiles for requirements definition

- Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

  - Further discuss the objectives in the next round

  - Proposed revised objectives

Define PUSCH demodulation requirements for FR1 256QAM - Note 1: low mobility is considered for requirements definition - Note 2: Realistic phase noise modelling is left up to the contributing entities

- No further discussion for Objectives 2-2 and 2-3 based on overall objectives prioritization

## Intermediate round

### Companies views’ collection

#### Objective 2-1: BS interference-aware receivers for Scenario a): Inter-cell interference

Candidate objectives

Define BS demodulation requirements for Inter-cell interference scenario - Type of requirements: PUSCH requirement - Reference receiver: - MMSE-IRC with DMRS based interference covariance estimation - Target frequency: FR1 - Rx antenna number: further discuss and decide in the WI - Identify the interference profiles for requirements definition

Companies are encouraged to provide comments on further objective refinement:

| Company | Comments |
| --- | --- |

#### Objective 2-4: BS FR1 PUSCH 256QAM demodulation requirements

Candidate objective

Define PUSCH demodulation requirements for FR1 256QAM - Note 1: low mobility is considered for requirements definition - Note 2: Realistic phase noise modelling is left up to the contributing entities

Companies are encouraged to provide comments on further objective refinement:

| Company | Comments |
| --- | --- |

## Fine-tuning round

# References

[1]	RP-201331 “Work areas of RAN4 R17 non-spectrum related WI/SIs”, RAN4 Chair, RAN#88e.

[2]	RP‑201967 “Email discussion summary for RAN4 Rel-17 demodulation scope”, China Telecom, RAN#89e