3GPP TSG-RAN Meeting # 89																	RP-20xxxx

Electronic Meeting, 14th -18th Sep, 2020

Agenda item:			x.x.x

Source:	Moderator (Samsung)

Title:	Email discussion summary for FR2 HST

Document for:	Information

Introduction

The working area for RAN4 Rel-17 non-spectrum WI/SI proposals were agreed in RP-201331. The scope of this email discussion is to stabilize the scope of FR2 HST based on the input from RP-200846 (Nokia) and RP-200896 (Samsung).

List of candidate topics for this assigned e-mail discussion

- Topic #1: Target scenarios

- Topic #2: Objectives for RF core parts

- Topic #3: Objectives for RRM core parts

- Topic #4: Objective for performance part

- Topic #5: Details of drafting WID, including justification, time scale and expected output (new specifications, impacted new specifications)

Topic #1: Target scenarios

Companies’ contributions summary

| Company | Proposals / Observations |
| --- | --- |
| Nokia | - This work item focuses on train car-mounted high-power devices only. - Single-panel and/or multi-panel? - NR SA with PCell in FR2. - The channel model: - HST-SFN scenarios, i.e. multiple RRHs connecting to one BBU. The channel model for HST-SFN will be discussed in this WI. - HST single tap channel model open-space - Other channel models are not precluded - The maximum Doppler frequency will be investigated and determined based on operating frequency, velocity and the NR design limitations for all UL/DL physical channels. - Target band n261. - The feasibility of supporting speeds of up to a maximum of 250km/h will be investigated. The actual maximum supported velocity will be decided in this WI. |
| Samsung | - Vehicle-roof mounted customer-premises equipment (CPE), which are expected to communicate with track-side deployed gNBs for the backhaul link and to further provide on-board broadband connections to user terminals and/or for other train-specific demands as access link. - NR SA single carrier scenario in FR2 - Detailed frequency bands in FR2 will be decided in this WI - Further study the channel model for FR2 HST - Both uni-directional SFN and bi-directional SFN shall be studied - Other channel model is not precluded - The maximum Doppler frequency will be investigated and determined based on operating frequency, velocity and the NR design limitations for all UL/DL physical channels. - The feasibility of supporting speeds of up to a maximum of 500km/h will be investigated. The actual maximum supported velocity in Rel-16 FR2 frequency bands will be decided in this WI. |

Open issues summary

Sub-topic 1-1: Whether below target scenarios for FR2 HST can be confirmed?

  - Train-roof mounted customer-premises equipment (CPE), which are expected to communicate with track-side deployed gNBs for the backhaul link and to further provide on-board broadband connections to user terminals and/or for other train-specific demands as access link.

  - FR2 single carrier, i.e., NR SA with PCell in FR2

  - Maximum Doppler frequency will be investigated and determined based on operating frequency, velocity and the NR Rel-16 design limitation for all UL/DL physical channels

    - The feasibility of supporting speeds of up to a maximum of 2500km/h will be investigated. The actual maximum supported velocity in Rel-16 FR2 frequency bands will be decided in this WI.

Sub-topic 1-2: For candidate FR2 frequency bands, is there any other bands request except band n261?

Sub-topic 1-3: Whether the HST single tap channel model shall be included in the WI phase, two options:

  - Option 1: Yes

  - Option 2: No

Sub-topic 1-4, For HST SFN, whether the uni-directional and bi-directional SFN shall be considered as baseline?

  - Option 1: Yes

  - Option 2: No

Sub-topic 1-5: For train-mounted CPE, any implementation assumption for Rel-17? Also, companies’ interpretation on the implementation assumptions are encouraged to facilitate the further discussions.

  - Option 1: Single-panel only

  - Option 2: Single-panel and/or multi-panel

Companies views’ collection

| Company | Comments |
| --- | --- |

Summary for target scenarios

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#1 | Tentative agreements: Candidate options: Recommendations for wording in the WID: |

Topic #2: Objectives for RF core parts

Companies’ contributions summary

| Company | Proposals / Observations |
| --- | --- |
| Samsung | - Specify the UE RF core requirements for - Existing power class 2 or power class 4 UE can be investigated as starting point - New power class is not precluded - Further enhancement for beam correspondence requirements - Further study the feasibility of more restrict requirements for transmit signal quality for FR2 HST applicable SCS |
| Nokia | No RF requirements |

Open issues summary

Sub-topic 2-1: Whether the RF requirements is required to support FR2 HST train-roof-mounted CPE?

  - Option 1: Yes

  - Option 2: No

Sub-topic 2-2: If the answer for sub-topic 2-1 is yes, whether existing PC4 can be used for baseline?

  - Option 1: PC4 can be used as baseline, no other power class shall be considered in the WI phase

  - Option 2: PC4 can be used as baseline, other power class including existing power class the new power class, is not precluded in WI phase

  - Option 3: No, new power class shall be defined for FR2 HST train-roof-mounted CPE

Sub-topic 2-3: If answer for sub-topic 2-1 is yes, are we going to further enhance the beam correspondence requirements for the designed power class?

  - Option 1: Yes

  - Option 2: No

Sub-topic 2-4: If answer for sub-topic 2-1 is yes, are we going to further study the feasibility of more restrict requirements for transmit signal quality for FR2 HST applicable SCS?

  - Option 1: Yes

  - Option 2: No

Companies views’ collection

| Company | Comments |
| --- | --- |

Summary for RF core requirements

|  | Status summary |
| --- | --- |
| Topic#2 | Tentative agreements: Candidate options: Recommendations for wording in the WID: |

Topic #3: Objectives for RRM core parts

Companies’ contributions summary

| Company | Proposals / Observations |
| --- | --- |
| Samsung | - Specify the UE RRM core requirements for - Idle/inactive mode cell reselection requirements enhancement - Connected mode - Handover delay requirement enhancement - Measurement requirements enhancement including both L1 and L3 measurement - Beam management requirements enhancement including beam failure detection, candidate beam detection performance requirements |
| Nokia | - Investigate and specify the UE RRM core requirements for: - Connected mode - Cell identification requirements - Measurement delay requirements - Beam management related requirements, e.g. L1-RSRP measurement - Impact on RLM and UL timing. - If needed other requirements are not precluded. If needed, signalling impact should be discussed in RAN2. |

Open issues summary

Sub-topic 3-1: Whether the IDLE mode cell reselections requirements shall be specified?

  - Option 1: Yes

  - Option 2: No

Sub-topic 3-2: For connected mode, whether the following objectives shall be included in the WID?

  - Cell identification requirements

  - Handover delay requirement enhancement

  - Measurement requirements enhancement including both L1 and L3 measurement

  - Beam management requirements enhancement including beam failure detection, candidate beam detection performance requirements

  - Impact on RLM and UL timing.

Sub-topic 3-3: Any other missing RRM requirements from the objectives in sub-topic 3-2?

Companies views’ collection

| Company | Comments |
| --- | --- |

Summary for RRM core requirements

|  | Status summary |
| --- | --- |
| Topic#3 | Tentative agreements: Candidate options: Recommendations for wording in the WID: |

Topic #4: Objective for performance part

Companies’ contributions summary

| Company | Proposals / Observations |
| --- | --- |
| Samsung | - Specify the RRM performance requirements of measurement accuracy if identified. - Specify the RRM test cases related to new core requirements. - Specify the UE demodulation/CSI and BS demodulation requirements based on outcome of channel model and maximum Doppler frequency discussions |
| Nokia | - Investigate and specify the RRM performance requirements of measurement accuracy. - Specify the RRM test cases related to new core requirements (if defined): - Connected mode - Cell identification requirements - Measurement delay requirements - Measurement accuracy requirements - Beam management requirements - Other test cases are not precluded, if the core requirements are defined, e.g., BFD, RLM, UL timing, etc. - Specify the UE demodulation requirements and test cases at least for - PDSCH - Other requirements are not precluded if needed. - Specify the BS demodulation requirements and test cases at least for - PUSCH - Other requirements are not precluded if needed |

Open issues summary

Sub-topic 4-1: Whether the following objectives for performance part can be confirmed?

  - Specify the RRM performance requirements of measurement accuracy if identified.

  - Specify the RRM test cases related to new core requirements if defined

  - Specify the UE demodulation requirements and test cases at least for PDSCH, other requirements are not precluded

  - Specify the BS demodulation requirements and test cases at least for PUSCH, other requirements are not precluded

Sub-topic 4-2: Whether the UE CSI requirements shall be specified in the WI phase if identified

  - Option 1: Yes

  - Option 2: No

Companies views’ collection

| Company | Comments |
| --- | --- |

Summary for performance part requirements

|  | Status summary |
| --- | --- |
| Topic#4 | Tentative agreements: Candidate options: Recommendations for wording in the WID: |

Topic #5: Other details of drafting WID

Companies’ contributions summary

| Company | Proposals / Observations |
| --- | --- |
| Samsung | See RP-200896 |
| Nokia | See RP-200846 |

Open issues summary

Sub-topic 5-1: Can we confirm the time scale for FR2 HST in Rel-17, i.e.,

  - WI starts from Q4 2020. WI is expected to be completed aligning with REl-17 schedule for both core part and performance part

Sub-topic 5-2: Whether a new Technical Report for FR2 HST is required?

  - Option 1: Yes

  - Option 2: No

Sub-topic 5-3: Any other comments for wording suggestions for justification parts in WID?

Companies views’ collection

| Company | Comments |
| --- | --- |

Summary for details of drafting WID

|  | Status summary |
| --- | --- |
| Topic#5 | Tentative agreements: Candidate options: Recommendations for wording in the WID: |