3GPP TSG-RAN2 Meeting #111e	 R2-2008513

# 17 – 28. August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0285 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mandatory support of full rate user plane integrity protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, BT, Huawei, HiSilicon, Nokia, T-Mobile USA, Siemens, Telecom Italia, ORANGE, BMWi, Telstra, Telefonica, Swift Navigation, Vodafone, NCSC, KPN, Erillisverkot, AT&T, Intel Corporation, Ericsson, Telia Company, BOSCH, A.S.T.R.I.D. S.A., NTT DOCOMO INC, QUALCOMM, SEQUANS, Broadcom, FirstNet, Bell Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020.08.25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With the current limitation of UE signalling the support of user plane integrity protection for only 64kbps and full data rate, the security of UEs operating in NR connected to the 5GC (both NR SA and NR-DC) cannot be guaranteed. GSMA has been informed about security breaches, and subsequently informed 3GPP, where 3GPP SA#88-e decided to mandate full rate user plane integrity protection of UEs operating in NR connected to 5GC starting from Rel-16. This applies not only to NR SA but also to NR-DC, where both MN and SN terminated DRBs of a PDU session can be integrity protected. A corresponding CR to TS 33.501 has been approved at SA#88e in SP-200628. SA#88e tasked relevant WGs to agree corresponding CRs to their specifications at their next meeting for Rel-16 accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The limitation of UE signalling the support of user plane integrity protection for only 64kbps and full data rate has been removed to mandate Rel-16 UEs operating in NR connected to 5GC (both NR SA and NR-DC) to always support user plane integrity protection at any data rate, up to and including the highest data rate supported by the UE for UL and DL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security vulnerability remains for UEs operating in NR connected to the 5GC (both NR SA and NR-DC) and end-to-end 5GS would not be protected against new security attacks arising. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 3GPP TS 24.501, 3GPP TS 37.340 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START CHANGE

# 13	Security

## 13.1	Overview and Principles

The following principles apply to NR connected to 5GC security, see TS 33.501 [5]:

-	For user data (DRBs), ciphering provides user data confidentiality and integrity protection provides user data integrity;

-	For RRC signalling (SRBs), ciphering provides signalling data confidentiality and integrity protection signalling data integrity;

NOTE:	Ciphering and integrity protections are optionally configured except for RRC signalling for which integrity protection is always configured. Ciphering and integrity protection can be configured per DRB but all DRBs belonging to a PDU session for which the User Plane Security Enforcement information indicates that UP integrity protection is required (see TS 23.502 [22]), are configured with integrity protection.

-	For key management and data handling, any entity processing cleartext shall be protected from physical attacks and located in a secure environment;

-	The gNB (AS) keys are cryptographically separated from the 5GC (NAS) keys;

-	Separate AS and NAS level Security Mode Command (SMC) procedures are used;

-	A sequence number (COUNT) is used as input to the ciphering and integrity protection and a given sequence number must only be used once for a given key (except for identical re-transmission) on the same radio bearer in the same direction.

The keys are organised and derived as follows:

-	Key for AMF:

-	KAMF is a key derived by ME and SEAF from KSEAF.

-	Keys for NAS signalling:

-	KNASint is a key derived by ME and AMF from KAMF, which shall only be used for the protection of NAS signalling with a particular integrity algorithm;

-	KNASenc is a key derived by ME and AMF from KAMF, which shall only be used for the protection of NAS signalling with a particular encryption algorithm.

Key for gNB:

-	KgNB is a key derived by ME and AMF from KAMF. KgNB is further derived by ME and source gNB when performing horizontal or vertical key derivation.

Keys for UP traffic:

-	KUPenc is a key derived by ME and gNB from KgNB, which shall only be used for the protection of UP traffic between ME and gNB with a particular encryption algorithm;

-	KUPint is a key derived by ME and gNB from KgNB, which shall only be used for the protection of UP traffic between ME and gNB with a particular integrity algorithm.

Keys for RRC signalling:

-	KRRCint is a key derived by ME and gNB from KgNB, which shall only be used for the protection of RRC signalling with a particular integrity algorithm;

-	KRRCenc is a key derived by ME and gNB from KgNB, which shall only be used for the protection of RRC signalling with a particular encryption algorithm.

Intermediate keys:

-	NH is a key derived by ME and AMF to provide forward security.

-	KgNB* is a key derived by ME and gNB when performing a horizontal or vertical key derivation.

The primary authentication enables mutual authentication between the UE and the network and provide an anchor key called KSEAF. From KSEAF, KAMF is created during e.g. primary authentication or NAS key re-keying and key refresh events. Based on KAMF, KNASint and KNASenc are then derived when running a successful NAS SMC procedure.

Whenever an initial AS security context needs to be established between UE and gNB, AMF and the UE derive a KgNB and a Next Hop parameter (NH). The KgNB and the NH are derived from the KAMF. A NH Chaining Counter (NCC) is associated with each KgNB and NH parameter. Every KgNB is associated with the NCC corresponding to the NH value from which it was derived. At initial setup, the KgNB is derived directly from KAMF, and is then considered to be associated with a virtual NH parameter with NCC value equal to zero. At initial setup, the derived NH value is associated with the NCC value one. On handovers, the basis for the KgNB that will be used between the UE and the target gNB, called KgNB*, is derived from either the currently active KgNB or from the NH parameter. If KgNB* is derived from the currently active KgNB, this is referred to as a horizontal key derivation and is indicated to UE with an NCC that does not increase. If the KgNB* is derived from the NH parameter, the derivation is referred to as a vertical key derivation and is indicated to UE with an NCC increase. Finally, KRRCint, KRRCenc, KUPint and KUPenc are derived based on KgNB after a new KgNB is derived. This is depicted on Figure 13.1-1 below:

Figure 13.1-1: 5G Key Derivation

With such key derivation, a gNB with knowledge of a KgNB, shared with a UE, is unable to compute any previous KgNB that has been used between the same UE and a previous gNB, therefore providing backward security. Similarly, a gNB with knowledge of a KgNB, shared with a UE, is unable to predict any future KgNB that will be used between the same UE and another gNB after n or more handovers (since NH parameters are only computable by the UE and the AMF).

The AS SMC procedure is for RRC and UP security algorithms negotiation and RRC security activation. When AS security context is to be established in the gNB, the AMF sends the UE 5G security capabilities to the gNB. The gNB chooses the ciphering algorithm which has the highest priority from its configured list and is also present in the UE 5G security capabilities. The gNB also chooses the integrity algorithm which has the highest priority from its configured list and is also present in the UE 5G security capabilities. The chosen algorithms are indicated to the UE in the AS SMC and this message is integrity protected. RRC downlink ciphering (encryption) at the gNB starts after sending the AS SMC message. RRC uplink deciphering (decryption) at the gNB starts after receiving and successful verification of the integrity protected AS security mode complete message from the UE. The UE verifies the validity of the AS SMC message from the gNB by verifying the integrity of the received message. RRC uplink ciphering (encryption) at the UE starts after sending the AS security mode complete message. RRC downlink deciphering (decryption) at the UE shall start after receiving and successful verification of the AS SMC message. The RRC Connection Reconfiguration procedure used to add DRBs shall be performed only after RRC security has been activated as part of the AS SMC procedure.

A UE connected to 5GC, shall support integrity protected DRBs at any data rate, up to and including the highest data rate supported by the UE for both UL and DL. In case of failed integrity check (i.e. faulty or missing MAC-I), the concerned PDU shall be discarded by the receiving PDCP entity.

Key refresh is possible for KgNB, KRRC-enc, KRRC-int, KUP-enc, and KUP-int and can be initiated by the gNB when a PDCP COUNTs are about to be re-used with the same Radio Bearer identity and with the same KgNB. Key re-keying is also possible for the KgNB, KRRC-enc, KRRC-int, KUP-enc, and KUP-int and can be initiated by the AMF when a 5G AS security context different from the currently active one shall be activated.

END OF CHANGES