3GPP TSG-RAN Meeting #90e 	RP-202274

# Electronic Meeting, December 7-11, 2020

Source:		Ericsson

Title:			TP to TR 37.890 – CEPT updates

Agenda item:		9.7.1

Document for:	Approval

# Introduction

On November 16th-20th, CEPT ECC approved CEPT Report 75 and ECC Decision(20)01, based on updates made by WG FM  after Public Enquiry. This TP is capturing this update.

# Text proposal

<Start of changes>

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	RP-172804: “Feasibility Study on 6 GHz for LTE and NR”, Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]					ITU-R Radio Regulations, Articles, Edition 2016;

[4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017;

[5]	FCC 17-104, Notice of Inquiry, “Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz”;

[6]	Comments of IEEE 802, in GN Docket No. 17-183;

[7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183;

[8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183;

[9]	Comments of Ericsson, in GN Docket No. 17-183;

[10]	Comments of T-Mobile USA, in GN Docket No. 17-183;

[11]	Comments of Verizon, in GN Docket No. 17-183;

[12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183;

[13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183;

[14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183;

[15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183;

[16]	Comments of CTIA, in GN Docket No. 17-183;

[17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183;

[18] 	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183;

[19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations;

[20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	Draft ETSI TR 103 524 V0.0.10 (2018-01) - ERM System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”;

[23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

[24]	Draft ECC Report, “Compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5 925 – 6 425 MHz”, December 2017.

[25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018

[26]	ETSI TR 103 612, MFCN in the band 6425-7125 MHz

[27]	ETSI TR 103 631, Characteristics of WAS/RLANs in 6 725 to 7 125 MHz

[28]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz”

[29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

[30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[31]				Report and order and further notice of proposed rulemaking, FCC 20-51

[32]	ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz”, 21 May 2020

[33]	CEPT Report 075, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation. Not yet published

[34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)” Not yet published

[35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz"

[36]	EN 303 687, “"6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

[37]	doc 2.1_LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits), [to be updated]

[38]	CEPT Report 073, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

<End of changes>

<Start of changes>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BAS		Broadcast Auxiliary Service

CARS		Cable Television Relay Service

CBTC	Communication-based Train Control systems

CEPT		European Conference of Postal and Telecommunications

DA2GC	Direct Air-to-Ground Communications

EC		European Commission

ECC		European Electronic Communications Committee

EESS		Earth Exploration-Satellite Service

eLAA		enhanced LAA

ERC		European Radio-communication Commitee

ESV		Earth Stations on Vessel

FCC		Federal Communication Commission

FDP	Fractional Degradation in Performance

FM		Frequency Management – ECC Working Group

FS		Fixed Service

FSS		Fixed Satellite Service

IMT		International Mobile Telecommunications

ITS		Intelligent Transport System

LAA		Licensed Assisted Access

LPI	Low Power Indoor

LPR		Level Probing Radar

LTTS 		Local TV Transmission Service

MBR		Maritime Broadband Radio Links

MCL	Maximum Coupling Loss

NOI		Notice of Inquiry

NPRM		Notice of Proposed Rule Making

NR		New Radio

NR-U		NR Unlicensed

PMSE		Programme-Making and Special Events

RLAN		Radio Local Area Network

RR		Radio Regulation

SDARS	Sirius Satellite Radio Service

SE		Spectrum Engineering – ECC Working Group

SRD		Short Range Device

TLPR 		Tank Level Probing Radar

UNII		Unlicensed National Information Infrastructure

UWB		Ultra Wideband

VLBI		Very Long Baseline Interferometry (Radio Astronomy)

VLP	Very Low Power

WAS		Wireless Access System

WIA		Wireless Industrial Applications

<End of changes>

<Start of changes>

###### 4.1.1.3.1c	CEPT Report 075

The CEPT Report 075 [33] addresses the possible designation of the frequency band 5925-6425 MHz for the implementation of Wireless Access Systems (WAS) including Radio Local Area Networks (RLAN), in accordance with Task 2 of the EC Mandate to CEPT.

Based on ECC Report 316 and the complementary studies done in parallel with the public consultation, the in-band Power Spectral Density (PSD) for VLP devices and out-of-band emission limits below 5935 MHz for LPI and VLP devices were  further assessed.

The recommended framework distinguishes the following use cases for use by WAS/RLAN in the 5945-6425 MHz frequency band:

- Low power indoor (LPI) use with max 200 mW EIRP, only indoor.

- Very low power (VLP) portable use with max 25 mW EIRP, indoor and outdoor.

The Report proposes that WAS/RLAN transmissions should be constrained to above 5945 MHz to take into account CBTC operations.

CEPT concludes that coexistence between WAS/RLAN operating in the 5945-6425 MHz band and existing services and systems within the band and in adjacent band would be technically feasible under the generic technical conditions detailed the annex of the Report and also given in ECC Decision(20)01, see clause 4.1.1.3.1d.

In addition, to maintain the protection of point-to-point FS, national administrations should consider an effective enforcement of the indoor restriction for LPI.

[Note: The CEPT Report 075 [33] has been approved by ECC and will be published by end of 2020].

###### 4.1.1.3.1d	ECC Decision (20)01

The ECC Decision (20)01 [34] harmonized the 5.945-6.425 GHz frequency band for WAS/RLAN use, restricted to:

- Indoor use only for LPI equipment. This LPI equipment could be:

  - LPI Access Point or bridge, power supplied from a wired connection, not battery powered, with an integrated antenna.

  - LPI client connected to a LPI access point or another LPI client. This LPI client could be battery powered or not.

- Indoor and outdoor use for VLP equipment. The VLP device is a portable device, drones are prohibited.

Those equipment shall support an adequate spectrum sharing mechanism.

Moreover, LPI equipment shall comply with the limits captured in Table 4.1.1.3.1d-1 and VLP equipment with limits captured in Table 4.1.1.3.1d-2.

Table 4.1.1.3.1d -1: LPI WAS/RLAN equipment limits

| Parameter | Technical conditions |
| --- | --- |
| Maximum mean e.i.r.p. for in-band emissions (note 1) | 23 dBm |
| Maximum mean e.i.r.p. density for in-band emissions (note 1) | 10 dBm/MHz |
| Maximum mean e.i.r.p. density for out-of-band emissions below 5935 MHz (note 1) | -22 dBm/MHz |
| Note 1: The "mean e.i.r.p." refers to the e.i.r.p. during the transmission burst, which corresponds to the highest power, if power control is implemented. |  |

Table 4.1.1.3.1d -2: VLP WAS/RLAN equipment limits

| Parameter | Technical conditions |
| --- | --- |
| Maximum mean e.i.r.p. for in-band emissions (note 1) | 14 dBm |
| Maximum mean e.i.r.p. density for in-band emissions (note 1) | 1 dBm/MHz |
| Narrowband usage maximum mean e.i.r.p. density for in-band emissions (note 1) (note 2) | 10 dBm/MHz |
| Maximum mean e.i.r.p. density for out-of-band emissions below 5935 MHz (note 1) | -45 dBm/MHz (note 3) |
| Note 1: The "mean e.i.r.p." refers to the e.i.r.p. during the transmission burst, which corresponds to the highest power, if power control is implemented. Note 2: Narrowband (NB) devices are devices that operate in channels bandwidths below 20 MHz. Narrowband devices also require a frequency hopping mechanism based on at least 15 hop channels to operate at a PSD value above 1 dBm/MHz. Note 3: ECC will study the appropriateness of this level of OOBE by 31/12/2024. In absence of the justified evidence, a value of -37 dBm/MHz will be adopted from 01/01/2025. |  |

[Note: The ECC Decision 20(01) [34] has been approved by ECC and will be published by end of 2020].

###### 4.1.1.3.1e	CEPT Report 073

This CEPT Report 073 [38] contains the summaries of the technical studies and assessment of coexistence scenarios for Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) systems with incumbent systems in the 5925-6425 MHz band and adjacent bands, in response to Task 1 (Assessment and study of compatibility and coexistence scenarios in the band 5925-6425 MHz) of the EC Mandate on WAS/RLAN in the 5925-6425 MHz band.

Based on the studies described in ECC Report 302 (see clause 4.1.1.3.1b), the CEPT Report 073 concludes that coexistence between WAS/RLAN with existing services and systems within and adjacent to the band 5925-6425 MHz would be technically feasible under certain conditions.

For scenarios with indoor only WAS/RLAN deployment with a maximum e.i.r.p. in the range of 200-250 mW, the studies indicated that coexistence was feasible with FS when considering the long-term aggregated interference protection criterion and with FSS. Also, coexistence with CBTC systems and Road-ITS would be technically feasible assuming suitable measures such as a guard-band and requirements on WAS/RLAN in band and/or out-of-band emissions. These elements would limit the available spectrum to WAS/RLAN to less than the entire 5925-6425 MHz band.

Complementary studies and analyses  are covered by CEPT Report 075. These studies address FS short-term protection between point-to-point applications and WAS/RLAN indoor only deployments as well as potential WAS/RLAN portable devices that operate outdoor with power levels significantly lower than that for indoor use.

High power WAS/RLAN devices and outdoor WAS/RLAN devices other than those mentioned in the Report could present a risk of harmful interference to the Fixed Service , Fixed-Satellite Service, Communication based train control  systems and Road Intelligent Transport System  incumbents when sharing the band without restrictions.

Based on this risk of interference, the feasibility of outdoor WAS/RLAN deployment and high-power WAS/RLAN access points (APs) would require additional studies to address the interference to the incumbent systems.

<End of changes>