3GPP TSG RAN meeting #90e										RP-202291

Electronic Meeting, December 7-11, 2020

## Status Report to TSG

Agenda item:			9.8.5

| WI / SI Name | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IIOT_URLLC_enh |  |  |  |  |
| Unique ID | 860045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201310 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2021 | Performance part: 03/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 20% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Ping-Heng Wallace Kuo |
|  | Company | Nokia |
|  | Email | Ping-Heng.Kuo@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreements from RAN1#103-e meeting (Online, 26 October – 13 November 2020):

| UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1) | Agreements: To address the issue of SPS HARQ-ACK dropping for TDD systems, focus on the following two options: - Option 1: Deferring HARQ-ACK until a next (e.g., first) available PUCCH - FFS: Details including the definition of a next (e.g, first) available PUCCH, CB construction / multiplexing - Option 2: Dynamic triggering of a one-shot / Type-3 CB type of re-transmission - FFS: Details on triggering and/or CB construction (incl. potential Type-3 CB optimizations) / multiplexing Agreements: In the studies on PUCCH carrier switching for HARQ-ACK, PUCCH carrier switching for different cells operated is considered only for cells that are part of the active UL CA configuration. Agreements: For the studies on SPS HARQ skipping for skipped SPS PDSCH, the further discussions should focus on the following reduced sets methods: - ‘NACK skipping’ for (skipped) SPS PDSCH (Alt. 1) - FFS: details including at least when to skip the HARQ-ACK as well as NACK skipping configuration details (per SPS or group of SPS configurations etc.) - Note: this alternative assumes inherently no identification of a skipped SPS PDSCH by the UE - Dynamic indication of skipped SPS PDSCH occasions (Alt. 3) - FFS: details including dynamic indication methods such as e.g. DCI, MAC CE, specific DM-RS instead of SPS DM-RS, … Agreements: For the studies on SPS HARQ payload size reduction (of non-skipped SPS PDSCH), the further discussions should focus on the following reduced sets of methods: - ACK skipping (NACK-only) (Alt. 1) - FFS: Details - NACK skipping (ACK-only) (Alt. 2) - FFS: Details - HARQ bundling / compression (Alt. 3) - FFS: Details including HARQ bundling / compression window, bundling / compression technique - HARQ-ACK disabling /skipping for certain SPS configurations (Alt. 4) - The skipping / disabling is higher-layer configured per SPS configuration - FFS: HARQ-ACK skipping behaviour for Type 1 CB |
| --- | --- |
| CSI feedback enhancements (RAN1 AI 8.3.1.2) | Agreements - No change of CSI processing time relative to Rel-16 CSI in this WI - CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied Agreement: - For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH. Agreements: For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation. - Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g., - CQI/SINR statistics (e.g., mean, variance, etc.) - CSI prediction - Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.) - Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g., - CQI reporting considering the worst subbands - Subband CQI granularity enhancement - Scheme 1d: New reporting quantity related to CSI expiration time - Scheme 1e: New reporting quantity with partial information update, e.g., - CSI reporting with interference update only Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e |
| Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2) | Agreements: - In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms Conclusion: - For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB. Agreements: - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported Conclusion: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. Agreements: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. o Note: Any value for the period, shall be at least 1ms and at most 10ms. o Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. |
| Intra-UE multiplexing enhancements (RAN1 AI 8.3.3) | Agreements: For multiplexing UCIs of different priorities in a PUCCH in R17, - Support of multiplexing between different resources not confined within a sub-slot if conditions are met - FFS: Details - Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met - FFS details Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e: l Option 1: Support joint coding. l Option 2: Support separate coding. l Option 3: Combination of Option1 and 2. l FFS the details For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e: · Multiplexing on a PUCCH format 0 · Multiplexing on a PUCCH format 1 Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17. - FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details. - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered. - FFS the timeline requirements. - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled. - FFS UE capability for this feature. - Note: The main bullet has been agreed in the WID by RAN Plenary. |
| Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4) | Agreements: - Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control. - Asymmetry between downlink and uplink channel for smart grid scenario is not considered. - TA adjustment accuracy is not considered for the evaluation of time synchronization error. - errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error. Agreements: TA adjustment accuracy is not considered for the evaluation of time synchronization error. Agreements: For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error: - Option 1: 200 ns - Option 2: 65 ns |

#### 2.1.2	Remaining Open issues

- UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1)

  - Specification of methods to support transmission of dropped SPS HARQ-ACK due to cancelled PUCCH for TDD (2 options still under discussion)

  - Study and specify if needed method(s) to enable the following:

    - SPS HARQ-ACK skipping for ‘skipped’ SPS PDSCH

    - SPS HARQ-ACK payload size reduction

    - Re-transmission of cancelled HARQ-ACK

    - Type 1 HARQ-ACK codebook for sub-slot PUCCH

    - PUCCH carrier switching for HARQ-ACK feedback

    - PUCCH repetition enhancements

- CSI feedback enhancements (RAN1 AI 8.3.1.2)

  - Study further following method(s) to enable accurate MCS selection,

    - New triggering methods for A-CSI and/or SRS

    - New CSI reporting based on channel/interference measurement including Scheme 1a, 1b, 1c,1d and 1e for case-1 new reporting

    - New reporting type (i.e., Case 2 new reporting) based on PDSCH decoding for OLLA performance enhancement

  - Specify support (if needed) for one or more method(s) from the above.

- Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2)

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

- Intra-UE multiplexing enhancements (RAN1 AI 8.3.3)

  - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

  - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

- Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4)

  - Finalization of evaluation assumptions for propagation delay compensation

  - Specify propagation delay compensation enhancements (if any)

## 2.2	RAN2

#### 2.2.1	Agreements

Agreements from RAN2#112-e meeting (Online, 2 – 13 November 2020):

| Time Synchronization | 1: RAN2 should consider the following three scenarios, with a focus on Scenario 2 and 3: • Scenario 1: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the CN. The 5GS introduced error is caused by the relative time-stamping inaccuracy at the NW-TT and the DS-TTs. • Scenario 2: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs. • Scenario 3: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT. 2 RAN2 should evaluate the synchronicity budget by dividing the 5GS E2E path into three parts: Network, Device, and Uu interface. Where the Uu interface is understood as the maximum 5GS time synchronization error between the UE and the gNB-DU (i.e. DU-CU interface error is not included) 3 RAN2 assumes the two Uu interfaces in Scenario 2 have the same time synchronization error budget. 4 The Uu interface budget for Scenario 1, 2 and 3 are respectively calculated as following: • Scenario 1: Uu budget = 900ns – Device – Network scenario1 • Scenario 2: Uu budget = (900ns – 2xDevice – 2xNetwork scenario2)/2 (assumption is based on GPTP) • Scenario 3: Uu budget = 1000ns – Device – Networkscenario3 (baseline assumption that this is based on GNSS) 5 The Device part time synchronization accuracy budget is assumed to be in the range ±50 to ±100ns, this applies to all three scenarios 6 The error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is to be included in the network part budget, and RAN1 should be informed not to include this error in Uu interface. 7 The Network part time synchronization accuracy budget for Scenario 1, 2, and 3 are assumed to be the following: • Scenario 1: ±120 to ±200ns (NetworkScenario1) (assuming 3-5 hops worst case scenario • Scenario 2: ±240 to ±400ns (2xNetworkScenario2) (assuming 6-10hops worst case scenario) • Scenario 3: ±100ns (NetworkScenario3) 8 Based on Proposal 4, 5, 6 and 7, the per Uu interface time synchronization accuracy for Scenario 1, 2 and 3 are as following: • Scenario 1: ±595ns to ±725ns • Scenario 2: ±145ns to ±275ns • Scenario 3: ±795ns to ±845ns 9 LS to RAN1 providing the scenarios and values. Indicate to RAN1 that they should aim to meet the most stringest requirements, but a number within the range is also acceptable 10 It is up to RAN1 to decide which PDC options should be supported for Scenario 1, 2 and 3 in Release-17. |
| --- | --- |
| URLLC in Unlicensed | - From RAN2 perspective - 1 It is assumed that LBT failures only happen infrequently in UCE (unlicensed controlled environment). A formal definition of UCE and its relationship to semi-static or dynamic access mode is not necessary in RAN2 specifications. - 2 cg-RetransmissionTimer can be configured optionally for shared spectrum - 3 When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. - 4 When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection. - 5 As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U. - 6 HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured. - 7 FFS if LCH based prioritization can be configured with cg-RetransmissionTimer - 8 The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell. No optimizations will be pursued to allow the two features be configured together in Rel-16. No CR is needed for this for now. - 9 If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it. FFS how to achieve it (using existing mechanisms should be considered as baseline) |
| RAN enhancement for New QoS | - => Time period during which “message loss” can be tolerated is adopted as the preferred format for Survival time. FFS how this will be achieved and what message loss means in RAN2 |

#### 2.2.2	Remaining Open issues

- Support of enhancement (if any) for propagation delay compensation, with consideration of mobility issues if needed.

- Identification of mechanisms that should be used to support autonomous (re-)transmission of deprioritized/pending MAC PDUs for URLLC in NR-U.

- Support of RAN enhancement for new TSCAI such as survival time and burst spread, if needed.

- Stage-2/Stage-3 specification changes for the above where applicable.

## 2.3	RAN3

#### 2.3.1	Agreements

The related work of this WI has not started in RAN3 yet.

#### 2.3.2	Remaining Open issues

- Propagation delay compensation enhancements (including mobility issues, if any).

- RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2.

## 2.4	RAN4

#### 2.4.1	Agreements

The related work of this WI has not started in RAN4 yet.

#### 2.4.2	Remaining Open issues

Specification of necessary RRM performance requirements (if any) for all the related objectives.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

FS_IIOT: SA2 captured the following Key Issues with potential impact on RAN in TR 23.700-20 v1.3.0:

- Key Issue #1 (Uplink Time Synchronization)

  - Solutions and conclusions updated.

  - Resolution for preferred User plane BMCA method selected for generating announce message.

  - Next step: normative CR for specification impact based on agreed conclusion.

- Key Issue #2 (UE-UE TSC communication)

  - Conclusion agreed for forwarding method in case of UE-UE communication for all PDU Sessions

  - Next step: normative CR for specification impact based on agreed conclusion.

- Key Issue #3A (Exposure of TSC Services: Exposure of deterministic QoS)

  - Conclusion agreed with the basic framework finally.

  - Pending ENs related to burst spread and jitter need resolution.

  - Next steps:

    - normative CR for specification impact based on agreed conclusion,

    - Send LS out to SA1 for H&F for Ethernet PDU sessions.

    - Resolve other open issues in TR

- Key Issue #3B (Exposure of TSC Services: Exposure of Time Synchronization):

  - Conclusions updated to include uplink time sync, BMCA method supported for PTP, Time Sync services policy control and charging

  - Open item related to validity period.

  - Next step: Resolution of validity period.

- Key Issue #4 (fully distributed configuration model for TSN)

  - Downscoped. Will not be progressed during Rel-17

- Key Issue #5 (Use of Survival Time for Deterministic Applications in 5GS)

  - Update conclusion based on RAN2/3 feedback.

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Whether signalling Survival Time to NG-RAN is sufficient to address the Communication service availability targets defined by SA1 in TS 22.104 Table 5.2-1 depends on the feedback from RAN WG.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#103-e Tdocs List

R1-2007539		CSI feedback enhancements for URLLC	FUTUREWEI

R1-2007551		UE initiated COT for FFP	FUTUREWEI

R1-2007565		UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2007566		CSI feedback enhancements	Huawei, HiSilicon

R1-2007567		Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2007568		Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2007655		HARQ-ACK enahncements for Rel-17 URLLC	vivo

R1-2007656		CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2007657		Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2007658		Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2007659		Discussion on propagation delay compensation enhancements	vivo

R1-2007707		HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2007708		CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2007709		Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2007710		Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2007711		Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2007789		UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2007849		UE feedback enhancements for HARQ-ACK	CATT

R1-2007850		CSI feedback enhancements	CATT

R1-2007851		Enhancements for unlicensed band URLLC/IIoT	CATT

R1-2007852		Intra-UE multiplexing and prioritization	CATT

R1-2007853		Discussion on propagation delay compensation enhancements	CATT

R1-2007885		Enhancements for unlicensed band URLLC/IIoT	TCL Communication Ltd.

R1-2007900		HARQ feedback enhancement for URLLC	Beijing Xiaomi Software Tech

R1-2007901		Intra-UE multiplexing prioritization	Beijing Xiaomi Software Tech

R1-2007902		Enhancement for unlicensed band URLLC IIoT	Beijing Xiaomi Software Tech

R1-2008007		Discussion on UE feedback enhancements for HARQ-ACK	CMCC

R1-2008008		Discussion on CSI feedback enhancements	CMCC

R1-2008009		Discussion on intra-UE multiplexing/prioritization	CMCC

R1-2008057		Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2008058		Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2008059		Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2008060		Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2008061		Discussion on propagation delay compensation enhancements	LG Electronics

R1-2008107		Discussion on CSI feedback enhancements	Spreadtrum Communications

R1-2008108		Discussion on enhancements for unlicensed band URLLCIIoT	Spreadtrum Communications

R1-2008159		HARQ-ACK feedback enhancements for Rel-17 URLLC/IIoT	Samsung

R1-2008160		CSI feedback enhancements for URLLC	Samsung

R1-2008161		Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2008162		Uplink intra-UE multiplexing and prioritization	Samsung

R1-2008163		Discussion for propagation delay compensation enhancements	Samsung

R1-2008279		HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2008280		CSI feedback enhancements for URLLC	OPPO

R1-2008281		Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2008282		Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2008283		Enhancements for Propagation Delay Compensation	OPPO

R1-2008318		Enhancements for support of time synchronization	Huawei, HiSilicon

R1-2008355		Considerations in HARQ-ACK enhancements for URLLC	Sony

R1-2008356		Considerations in CSI feedback enhancements	Sony

R1-2008357		Considerations in unlicensed URLLC	Sony

R1-2008358		Considerations in intra-UE UL multiplexing	Sony

R1-2008460		Discussion on UE feedback enhancements for HARQ-ACK	Apple

R1-2008461		Discussion on CSI feedback enhancements for URLLC/IIoT	Apple

R1-2008462		URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2008463		Discussion on Intra-UE Multiplexing/Prioritization	Apple

R1-2008464		Discussion on Orphan symbol handling for unlicensed spectrum	Apple

R1-2008495		CSI Feedback Enhancements for IIoT/URLLC	III

R1-2008568		UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2008821		Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2008822		Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2008823		Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2008824		Discussion on enhanced intra-UE multiplexing	ZTE

R1-2008825		Discussion on propagation delay compensation enhancements	ZTE

R1-2008834		Unlicensed aspects for IIoT	Charter Communications

R1-2008842		HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2008843		On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2008844		Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

R1-2008847		CSI feedback enhancement	NEC

R1-2008848		Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2008862		CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2008891		Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2008936		CSI feedback enhancements for enhanced URLLC/IIoT	InterDigital, Inc.

R1-2008937		Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2008941		UE feedback enhancements for HARQ-ACK	NEC Corporation

R1-2008952		Discussion on UE feedback enhancements for HARQ-ACK	Panasonic Corporation

R1-2008953		Discussion on CSI feedback enhancements	Panasonic Corporation

R1-2008954		Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2008955		Discussion on Intra-UE multiplexing and prioritization of different priority	Panasonic Corporation

R1-2008984		Discussion on prioritized UE HARQ feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2008985		Discussion on prioritized CSI feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2008986		Enhancements to Enable URLLC/IIoT in Unlicensed Band	Intel Corporation

R1-2008987		On Intra-UE Multiplexing and Prioritization for Release 17 URLLC/IIoT	Intel Corporation

R1-2008988		On propagation delay compensation for enhanced timing synchronization	Intel Corporation

R1-2009011		UE feedback enhancements for HARQ-ACK	ETRI

R1-2009012		Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2009013		Intra-UE Multiplexing/Prioritization	ETRI

R1-2009014		Processing time for COT sharing in FBE	ETRI

R1-2009045		Summary#1 on Intra-UE Multiplexing/Prioritization for R17	OPPO

R1-2009053		Discussion on UE feedback enhancements for HARQ-ACK	Asia Pacific Telecom co. Ltd

R1-2009063		On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2009064		CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2009065		On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2009066		Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2009083		HARQ-ACK enhancements for DL SPS	InterDigital, Inc.

R1-2009084		Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2009101		HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2009102		CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

R1-2009103		Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2009104		Intra-UE multiplexing enhancement for IIoT/URLLC		Lenovo, Motorola Mobility

R1-2009133		UE feedback enhancements for HARQ-ACK		Sharp

R1-2009134		CSI feedback enhancements for eURLLC	Sharp, NICT

R1-2009135		Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2009136		Enhancements on intra-UE UCI multiplexing and PUSCH prioritization		Sharp

R1-2009140		UE feedback enhancements for HARQ-ACK	China Unicom

R1-2009148		Discussion on necessity and support of Physical Layer feedback enhancements	Spreadtrum Communications

R1-2009149		Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2009182		Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009183		Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009184		Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2009185		Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009214		Discussion on intra-UE multiplexing	ITRI

R1-2009246		Discussion on HARQ-ACK enhancement for URLLC/IIoT	WILUS Inc.

R1-2009247		Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

R1-2009248		Discussion on Intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

R1-2009257		HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2009258		CSI enhancement for IOT and URLLC		Qualcomm Incorporated

R1-2009259		Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2009260		Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2009261		Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

R1-2009455		Feature lead summary #1 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2009492		Summary#1 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009546		Summary#1 of email thread [103-e-NR-IIOT_URLLC_enh-04]	Moderator (OPPO)

R1-2009551		Feature lead summary on propagation delay compensation enhancements (AI 8.3.4)	Moderator (Huawei)

R1-2009558		Feature lead summary #2 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2009566		Moderator summary #1 on Rel-17 HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT (AI 8.3.1.1)	Moderator (Nokia)

R1-2009601		Summary#2 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009649		Feature lead summary #3 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2009710		Summary#3 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009768		Discussion on CSI feedback enhancements for URLLC/IIoT	Apple

R1-2009775		Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2009781		Summary#4 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009789		Moderator summary on Rel-17 HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT (AI 8.3.1.1) – end of meeting	Moderator (Nokia)

RAN2#112-e Tdocs List

R2-2009755	Summary of email discussion [Post111-e][924][R17 URLLC/IIoT] Propagation delay for TSN (Nokia)	Nokia, Nokia Shanghai Bell

R2-2008855	Discussion on enhancements for support of time synchronization	Huawei, HiSilicon

R2-2008856	Draft Reply LS on propagation delay compensation enhancements	Huawei, HiSilicon

R2-2008880	Propagation Delay Compensation Enhancements		Ericsson

R2-2008972	Propagation Delay Compensation for TSN		Qualcomm Incorporated

R2-2009060	Further consideration on time synchronization and PDC in TSN	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

R2-2009118	On propagation delay compensation		MediaTek Inc.

R2-2009270	Enhancements for Propagation Delay Compensation and Mobility	Intel Corporation

R2-2009561	Consideration of time synchronization enhancement for TSN	OPPO

R2-2009672	Mobility related issues for the propagation delay compensation	Beijing Xiaomi Mobile Software

R2-2009756	[DRAFT] Reply LS on propagation delay compensation enhancements	Nokia, Nokia Shanghai Bell

R2-2009757	Discussion on propagation delay compensation mechanisms	Nokia, Nokia Shanghai Bell

R2-2009865	Considerations on time synchronization enhancement	Lenovo, Motorola Mobility

R2-2009915	Discussion on enhancements for TSN time synchronization	China Telecommunications

R2-2010173	Mobility aspects of time synchronization	Sequans Communications

R2-2010211	Discussion on the propagation delay compensation	vivo

R2-2010381	Enhancements for support of time synchronization for TSN	CMCC

R2-2010413	Discussion on propagation delay compensation for support of time synchronization	LG Electronics Inc.

R2-2010523	RAN2 Aspects on Timing Synchronization	Samsung

R2-2010532	Uplink time synchronization 	NTT DOCOMO, INC.

R2-2010836   Summary of [AT112-e][501][IIoT] Summary of URLLC in unlicensed (Qualcomm)

R2-2008853	Discussion about uplink enhancements for URLLC in unlicensed controlled environment		Huawei, HiSilicon

R2-2008859	Co-existence of NR-U and IIOT in R16	CATT

R2-2008860	Protocol selection for IIoT on unlicensed spectrum	CATT

R2-2008881	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson

R2-2008974	CG Harmonization in Unlicensed Controlled Environment	Qualcomm Incorporated

R2-2008976	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation

R2-2009117	On configured grant harmonization	MediaTek Inc.

R2-2009501	Potential UL enhancements for URLLC in unlicensed environments	Apple

R2-2009562	Consideration on URLLC over NRU	OPPO

R2-2009598	Enhancements for URLLC in unlicensed controlled environments	Lenovo, Motorola Mobility

R2-2009758	Uplink CG Harmonization for NR-U and URLLC	Nokia, Nokia Shanghai Bell

R2-2009900	Considerations in unlicensed URLLC  	Sony Europe B.V.

R2-2009912	Considerations on the harmonization of enhanced configured grant on shared spectrum channel	ZTE Corporation, Sanechips

R2-2009914	Discussion on CG harmonization for IIoT in unlicensed spectrum	Google Inc.

R2-2010110	IIoT operation in unlicensed controlled environments	InterDigital

R2-2010212	Harmonizing CG enhancements in NR-U and URLLC/IIoT	vivo

R2-2010374	Discussion on CG harmonization for URLLC in unlicensed controlled environments	CMCC

R2-2010437	Consideration on timers for URLLC/IIoT in unlicensed controlled environments	III

R2-2010439	Harmonized support of IIOT on unlicensed band	LG Electronics Inc.

R2-2010524	Uplink Enhancements for Unlicensed Spectrum	Samsung

R2-2009179	Concept and use of survival timer	Samsung Electronics GmbH

R2-2009759   RAN Enhancement for Survival Time		Nokia, Nokia Shanghai Bell

R2-2008882	  RAN enhancements based on new QoS related parameters	Ericsson

R2-2009062	New QoS related parameters in TSN	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

R2-2008854	Discussion on RAN enhancements based on new QoS related parameters	Huawei, HiSilicon

R2-2008861	RAN enhancement based on New QoS	CATT

R2-2008985	RAN Enhancements to Support New QoS Parameters for TSN		Intel Corporation

R2-2009130	U-plane aspect for RAN enhancement to support new QoS	Fujitsu

R2-2009563	Consideration on RAN enhancement based on new QoS	OPPO

R2-2009671	RAN impacts of the IIOT QoS parameters	Beijing Xiaomi Mobile Software

R2-2009870	Discuss on the mechanism to guarantee the survival time	Lenovo, Motorola Mobility

R2-2010111	Enhancements based on new QoS requirements	InterDigital

R2-2010213	Discussion on IIOT QoS impacts in RAN	vivo

R2-2010375	Discussion on the support of RAN enhancement for new QoS parameters	CMCC

R2-2010438	Discussion on RAN enhancements based on Survival Time	III

R2-2010444	Support of determinstic IIOT Traffic	LG Electronics UK

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template