3GPP TSG-RAN WG3 Meeting #110-e	R3-207134

# Electronic Meeting, November 2 – 12, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.474 | CR | 0008 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in F1-U on DSCP Derivation in IAB-donor node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In all cases except when a gNB operates as in IAB-donor, the DSCP marking of downlink user plane packets over the F1-U interface is conducted at the gNB-CU-UP per TS 38.474 (Rel-16). However, in support of IAB at the IAB-donor gNB, the DSCP marking responsibility is transferred to the gNB-CU-CP and the DSCP marking is signalled over the E1 interface to the gNB-CU-UP per TS 38.463 (Rel-16). In order to support priority services (e.g. MPS), DSCP marking at the gNB-CU-CP should be based on 5G QoS Identifier (5QI) , the Priority Level (if explicitly signalled), and other NG-RAN traffic parameters (e.g. ARP). The CR adds the necessary statements in TS 38.474 to be used for the DSCP derivation at the gNB-CU-CP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the DSCP derivation at the gNB-CU-CP is based on 5G QoS Identifier (5QI) , the Priority Level (if explicitly signalled), and other NG-RAN traffic parameters (e.g. ARP) for priority services (e.g. MPS) user plane traffic, when the gNB supports IAB and operates as an IAB-donor, as specified in the non-IAB case. Impact Analysis: Impacted 5G architecture option: NR-SA Impacted functionality: IAB operation at the IAB-donor node. Inter-operability: - No impact to the UE. - If the network is not implemented according to the CR; the UE does not receive user plane traffic prioritized per MPS requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the network is not implemented according to the CR; the UE does not receive user plane traffic prioritized per priority services (e.g. MPS) requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.463 CR 0556 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.4	Diffserv code point marking

IP Differentiated Services code point marking (IETF RFC 2474 [4]) shall be supported. Except the downlink F1-U to the IAB node, the mapping between traffic categories and Diffserv code points shall be configurable by O&M based on 5G QoS Identifier (5QI) , the Priority Level (if explicitly signalled), and other NG-RAN traffic parameters (e.g. ARP). Traffic categories are implementation-specific and may be determined from the application parameters. In the IAB system, the Diffserv code points used for the downlink F1-U shall be configurable by the CU-CP based on the O&M-configured mapping between traffic categories and Diffserv code points.

End of changes