3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2017345

# Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1191 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Interruption requirement for NR V2X synchronization source change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  |  | Date: |  | 2020/10/23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correction of synchronization source change interruption requirement, since current version doesn’t capture the UE side interruption correctly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of synchronization source change interruption requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE side interruption is not captured correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | It is a revision of R4-2014635. |
| --- | --- |

<<< Start of changed sections>>>

### 12.7.2	V2X Sidelink Communication Dropping due to synchronization source change

This clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

For NR V2X  UE not supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of  LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to syncRef UE that has the lowest priority

For NR V2X UE supporting gNB/eNB as synchronization reference source, UE is allowed to drop LTE and NR V2X SL transmission or reception for up to 1ms when synchronization source is changed, where the drop of  LTE V2X SL transmission or reception applies only to in-device coexistence scenario in TS38.213 [3]:

-	From GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to GNSS directly/in-directly

-	to GNSS

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From gNB or eNB

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to eNB or gNB

-	to syncRef UE that is synchronized to gNB or eNB directly

-	to syncRef UE that is synchronized to gNB or eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that is synchronized to gNB/eNB in-directly

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly/in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

-	From syncRef UE that has the lowest priority

-	to GNSS

-	to syncRef UE that is synchronized to GNSS directly

-	to syncRef UE that is synchronized to GNSS in-directly

-	to gNB/eNB

-	to syncRef UE that is synchronized to gNB/eNB directly

-	to syncRef UE that is synchronized to gNB/eNB in-directly

-	to syncRef UE that has the lowest priority

UE is allowed to interruption any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals.

<<< End of changed sections>>>