3GPP TSG-RAN WG4 Meeting #97-e	R4-2017193

Online, 2 – 13 November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6996 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR on NR CGI reading in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2020-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Update on the requirements for NR CGI reading in 36.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the reference to correct clause. - Remove NE-DC operation mode for the requirements in clause 8.1.2.4.27 & 8.1.2.4.28. - Correct the MIB decoding delay to 25 samples with the agreement that 1 sample for AGC/AFC is assumed for MIB decoding in RAN4#94ebis. - Add TSMTC definition - Update NR cell known condition in NR CGI reading requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NR CGI reading is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.2.15, 7.36.2.14, 7.37.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

#### 7.32.2.15	Interruption during NR measurement with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps as configured by PCell or NR PSCell, the UE is allowed on PCell or any activated SCell in MCG if configured

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB specified in clause 9.11 of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TMIB_NR specified in clause 8.17.18 and 8.17.19 if the CGI reading is configured by LTE PCell,

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.11 of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TSIB1_NR specified in clause 8.17.18 and 8.17.19 if the CGI reading is configured by LTE PCell, for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.11 of TS 38.133 [50] if the CGI reading is configured by NR PSCell, or TSIB1_NR specified in clause 8.17.18 and 8.17.19 if the CGI reading is configured by LTE PCell, for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.16 of TS 38.133 [50] assuming μ=0 for victim cell.

# < End of change 1>

# < Start of change 2>

#### 7.36.2.14	Interruption during NR measurement with autonomous gaps

When UE is configured by NR PCell to identify CGI of an NR cell, UE may cause interruptions on PSCell, and SCell(s) in SCG if configured.

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB specified in clause 9.11 of TS 38.133 [50],

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.11 of TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes, during SIB1 decoding time period TSIB1 specified in clause 9.11 of TS 38.133 [50], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.16 of TS 38.133 [50] assuming μ=0 for victim cell.

# < End of change 2>

# < Start of change 3>

## 7.37	Interruptions during NR measurement with autonomous gaps

### 7.37.1	Introduction

This section contains the requirements for the interruptions on PCell and activated SCells if configured, when performing inter-RAT NR measurement with autonomous gaps cells.

### 7.37.2	Requirements

When a UE is identifying CGI of an NR cell with autonomous gaps as configured by PCell, the UE is allowed on PCell or any activated SCell if configured

-	up to K1 interruptions, each with up to X1 interrupted subframes, during MIB decoding time period TMIB_NR specified in clause 8.1.2.4.27.2 and 8.1.2.4.28.2,

-	up to L1 interruptions, each with up to Y1 interrupted subframes, during SIB1 decoding time period TSIB1_NR specified in clause 8.1.2.4.27.2 and 8.1.2.4.28.2, for SSB and CORESET for RMSI scheduling multiplexing patterns 1,

-	up to L2 interruptions, each with up to Y2 interrupted subframes, during SIB1 decoding time period TSIB1_NR specified in clause 8.1.2.4.27.2 and 8.1.2.4.28.2, for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3,

where K1, L1, L2, X1, Y1 and Y2 are as defined in 8.2.1.2.16 of TS 38.133 [50] assuming μ=0 for victim cell.

# < End of change 3>