3GPP TSG-RAN WG4 Meeting #97-e	R4-2015574

# Electronic Meeting, 2nd Nov – 11th Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1305 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133: Correction to relaxed measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS38.331 v16.2.0, the combineRelaxedMeasCondition-r16 is defined as follows. relaxedMeasurement-r16 SEQUENCE { … combineRelaxedMeasCondition-r16 ENUMERATED {true} OPTIONAL, -- Need R … } The IE is either absent or configured as true. However in TS38.133 v16.5.0 the requirement is specified as follows. “…and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, …” The IE cannot be set to FALSE so the requirement is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected configuration of IE combineRelaxedMeasCondition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements is incorrect in terms of IE combineRelaxedMeasCondition configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.9.2, 4.2.2.9.3, 4.2.2.10.2, 4.2.2.10.3, 4.2.2.11.2, 4.2.2.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

#### 4.2.2.9 	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

##### 4.2.2.9.1 	Introduction

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

##### 4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on intra-frequency NR carrier and

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and UE has fulfilled only the lowMobilityEvalutation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.1-1.

Table 4.2.2.9.1-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |  |  |

##### 4.2.2.9.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on intra-frequency NR carrier and

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criteria and combineRelaxedMeasCondition [2] not configured, and UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.3-1.

Table 4.2.2.9.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

##### 4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on intra-frequency NR carrier, and

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1.

#### 4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

##### 4.2.2.10.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells when the UE is configured with any of following relaxed measurement criteria:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4. 9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4. 9.1 and 5.2.4.X.2 in [1] respectively.

##### 4.2.2.10.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.10.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

##### 4.2.2.10.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then te requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.3-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.3-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.3-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7

Table 4.2.2.10.3-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

##### 4.2.2.10.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	Has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.4-1.

##### 4.2.2.11 	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

4.2.2.11.1 	Introduction

This clause contains the requirements for measurements on inter-RAT E-UTRAN cells when the UE is configured with any of following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

##### 4.2.2.11.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.2-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

##### 4.2.2.11.3 	Measurements for UE fulfilling with not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the cellEdgeEvaluation [2] criterion.

When Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.3-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-RAT E-UTRAN frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.3-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

##### 4.2.2.11.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed,

In this case the UE is not required to meet Tdetect,EUTRAN , Tmeasure,EUTRAN  and Tevaluate,EUTRAN as defined in Table 4.2.2.5-1.

< End of change #1 >