3GPP TSG RAN meeting #90-e								              RP-202501

Electronic Meeting, December 14-18, 2020

## Status Report to TSG

Agenda item:	9.8.11 - NR small data transmissions in INACTIVE state [RAN2 WI: NR_SmallData_INACTIVE]

| WI / SI Name | NR small data transmissions in INACTIVE state |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_SmallData_INACTIVE |  |  |  |  |
| Unique ID | 860051 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201305 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2021 | Performance part: 03/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: Overall: 15% RAN2: 20% RAN3: 0% RAN4: 0% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Eswar Vutukuri |
|  | Company | ZTE Corporation |
|  | Email | eswar.vutukuri@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

In general, the TU allocation excel sheet is attached as a placeholder to be able to update this during the plenary to reflect the potential new Rel-17 timeline.

More specifically, the total TUs agreed in RAN#86 for RAN 2 and RAN 3 are unchanged but needs to be discussed as part of the overall R17 timeline discussion. The RAN4 TUs need to be officially endorsed in the detailed RAN4 TU allocation to be published at RAN#90e. It is requested to start RAN4 core activities in Q2 2021 with a specific time budget allocation reflected in the TUs excel sheet attached (considering the original Rel-17 timeline). This may also be discussed and updated as part of the overall R17 timeline discussion.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

- RAN1#103-e

[General]

None  (NR_SmallData_INACTIVE was not discussed)

#### 2.1.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 2.2	RAN2

#### 2.2.1	Agreements

- RAN2#112-e

[General]

Agreements 1 For small data, for RACH and CG based solutions when the UE receives RRC release with Suspend config, the UE at least performs the following actions (i.e. same action as in legacy): - MAC is reset and default MAC cell group configuration is released - RLC entities for SRB1 are re-established - SRBs and DRBs are suspended except SRB0 NOTE: SDT termination will be discussed with later papers 2 For both RACH and CG based solutions, upon initiating RESUME procedure for SDT initiation (i.e. for first SDT transmission), the UE shall re-establish at least the SDT PDCP entities and resume the SDT DRBs that are configured for small data transmission (along with the SRB1). FFS for non-SDT DRBs. FFS on implicit vs. explicit. FFS on whether we a new Resume cause. FFS on whether we need to deal with suppressing PDCP status report 3 The first UL message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG) may contain at least the following contents (depending on the size of the message): - CCCH message (needs to be included) LCP can be used to determine to priority of the content below that may be included - DRB data from one or more DRBs which are configured by the network for small data transmission - MAC CEs – (e.g. BSR). FFS other MAC CEs - Padding bits FFS if we need to ensure that SDT data only is included. Depends on whether the UE initiates legacy/normal resume 4 For RACH and CG, the existing UAC procedure to determine whether access attempt is allowed, will be reused for SDT. 5 SDT is transparent to NAS layer (i.e. NAS generates one of the existing resume causes and AS decides SDT vs non-SDT access) 6 In case of RRC-based solution, for both RACH and CG based solutions, the CCCH message contains ResumeMAC-I generated using the stored security key for RRC integrity protection – i.e same as Rel-16. 7 For both RACH and CG based solutions, new keys are generated using the stored security context and the NCC value received in the previous RRCRelease message (i.e. same as legacy procedure) and these new keys are used for generating the data of DRBs that are configured for SDT. 8 For RACH based solutions, upon successful completion of contention resolution, the UE shall monitor the C-RNTI. 9 Determine if RAN1 LS is needed later – current list of possible questions input on the coreset/search space for the C-RNTI (i.e. is it common or dedicated) 10: As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT - If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed. - If ROs for SDT and non SDT are same, preamble partitioning is needed FFS if common configuration should be allowed 11: If the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT then there is no further need for any differentiation between MSG2/MSGB for SDT vs non-SDT 12: Define a new timer. FFS whether it has the same definition as T319 or it is restarted every UL/DL Agreements for CG-SDT 1 The configuration of configured grant resource for UE uplink small data transfer is contained in the RRCRelease message. FFS if other dedicated messages can configure CG in INACTIVE CG. Configuration is only type 1 CG with no contention resolution procedure for CG. 2 The configuration of configured grant resource can include one type 1 CG configuration. FFS if multiple configured CGs are allowed 3 A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced. FFS on the procedure, the validity of TA, and how to handle expiration of TA timer. The TA timer is configured together with the CG configuration in the RRCRelease message. 4 The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell. FFS for other CG validity criteria (e.g. timer, UL/SUL aspect, etc) 5 The UE can use configured grant based small data transfer if at least the following criteria is fulfilled (1) user data is smaller than the data volume threshold; (2) configured grant resource is configured and valid; (3) UE has valid TA. FFS for the candidate beam criteria. 6 From RAN2 point of view: An association between CG resources and SSBs is required for CG-based SDT. FFS up to RAN1 how the association is configured or provided to the UE. Send an LS to RAN1 to start the discussion on how the association can be made. Mention that one option RAN2 considered was explicit configuration with RRC Release message 7 A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

- An LS has been sent to RAN1 (R2-2010841) informing RAN1 of some of the relevant agreements reached so far and requesting RAN1 to provide input for:

  - Configuration of the coreset and search space for monitoring the PDCCH addressed to the C-RNTI after successful completion of the RACH procedure during RA-SDT.

  - Configuration of association between the type 1 CG resource(s) for CG-SDT and SSB(s)

- An LS has also been sent to RAN3 (R2-2010839) informing about some of the relevant agreements and requesting RAN3 to consider the context fetch and data forwarding related procedures to support SDT data transmission with and without anchor relocation and provide any feedback on the RAN2 agreements

- The following 2 email discussions will be run until the next RAN2 meeting to further progress the details of solutions:

[POST112-e][550][SDT]  Further details of CG aspects (Lenovo)

Scope: Get input on: (+) Resource configuration, reconfiguration and release of CG resources (i.e. validity during cell reselection other validity criteria such as RSRP etc). (+) TA procedure, (+) Single vs multiple CG configurations. (+) Which BWP to use (initial or dedicated)

Output: Report, set of agreeable proposals

[POST112-e][551][SDT]  Common aspects between CG and RACH (ZTE)

Scope: Get input on: (+) RACH configuration – (preamble groups, TB sizes?), (+) Subsequent SDT transmission and indication, (+) SDT vs non SDT selection (Thresholds and order of selection), (+) Switching between CG-SDT and RA-SDT, (+) Cell Reselection handling

Output: Report, set of agreeable proposals

#### 2.2.2	Remaining Open issues

- Stage-2 design work for RACH and CG based solutions for small data in INACTIVE

- Stage-3 work

- See the WI objectives for general overview of remaining open issues

## 2.3	RAN3

#### 2.3.1	Agreements

- RAN3#110-e

[General]

None (NR_SmallData_INACTIVE was not discussed)

#### 2.3.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- RAN4#97-e

[General]

None (NR_SmallData_INACTIVE was not discussed)

#### 2.4.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### N/A

#### 3.1.1	Agreements with cross-TSG impacts

N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

N/A

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

Nothing to report

## 4.2	RAN2

###### RAN2#112-e, 02 – 13 Nov 2020, e-meeting

| R2-2008935 | Handling of subsequent small data transmission in RRC_INACTIVE | PANASONIC R&D Center Germany |
| --- | --- | --- |
| R2-2008958 | Discussion on the Security for Small Data Transmission | vivo |
| R2-2008959 | Duscussion on RRC-Controlled Small Data Transmission | vivo |
| R2-2008960 | Supporting Small Data Transmission via RA Procedure | vivo |
| R2-2008961 | Supporting Small Data Transmission via CG configuration | vivo |
| R2-2008992 | Security aspect for SDT | Intel Corporation |
| R2-2008993 | SDT control plane procedures and failure handling | Intel Corporation |
| R2-2008994 | RACH selection and User plane aspects with and without anchor relocation | Intel Corporation |
| R2-2008995 | Handling of Configured grant for SDT | Intel Corporation |
| R2-2009012 | Security aspects for small data transmission in inactive state | OPPO |
| R2-2009013 | Discussion on Control plane aspects for small data transmission | OPPO |
| R2-2009014 | Discussion on RACH based small data transmission | OPPO |
| R2-2009015 | Discussion on CG based small data transmission | OPPO |
| R2-2009055 | RRC-less SDT over CG | MediaTek Inc., Apple |
| R2-2009056 | RA-based SDT | MediaTek Inc. |
| R2-2009057 | CG-based SDT | MediaTek Inc. |
| R2-2009094 | Configured Grant based Small Data Transmission | Samsung Electronics Co., Ltd |
| R2-2009095 | Control Plane Aspects of SDT | Samsung Electronics Co., Ltd |
| R2-2009096 | Criteria for performing 2 step or 4 step RACH based SDT | Samsung Electronics Co., Ltd |
| R2-2009097 | RACH configuration for Small Data Transmission | Samsung Electronics Co., Ltd |
| R2-2009119 | Timer issues for subsequent data transmissions | PANASONIC R&D Center Germany |
| R2-2009131 | Open issue in [Post111-e][926]: TAT handling | Fujitsu |
| R2-2009132 | Identified issue in [Post111-e][926]: CA and PDCP CA duplication | Fujitsu |
| R2-2009151 | Discussion on the general aspects for small data transmission | Spreadtrum Communications |
| R2-2009152 | Discussion on small data transmission for RACH-based scheme | Spreadtrum Communications |
| R2-2009189 | Small Data] Agreeable details of RRC-based solution (RACH and CG) | Rapporteur (ZTE) |
| R2-2009190 | Control plane aspects of SDT | ZTE Corporation, Sanechips |
| R2-2009191 | RACH based small data transmission | ZTE Corporation, Sanechips |
| R2-2009192 | Configured grant based small data transmission | ZTE Corporation, Sanechips |
| R2-2009193 | Context fetch and data forwarding for SDT | ZTE Corporation, Sanechips |
| R2-2009316 | Discussion on RRC procedure for small data transmission | SHARP Corporation |
| R2-2009344 | Timer configuration for SDT failure detection | ETRI |
| R2-2009345 | SDT handling in RRC_INACTIVE state | ETRI |
| R2-2009347 | Differentiation and triggering of SDT procedure | Potevio |
| R2-2009350 | Discussion on aspects specific to CG based SDT | Nokia, Nokia Shanghai Bell |
| R2-2009366 | Security aspects on SDT | CATT |
| R2-2009367 | Considerations on general aspects and subsequent SDT | CATT |
| R2-2009368 | Analysis on SDT without Context relocation | CATT |
| R2-2009369 | Analysis on SDT Procedures using CG | CATT |
| R2-2009457 | RACH-based Small Data Transmission | LG Electronics Inc. |
| R2-2009458 | Coexistence of CG and RACH configuraiton for SDT | LG Electronics Inc. |
| R2-2009459 | CG resources for Small Data Transmission | LG Electronics Inc. |
| R2-2009460 | Anchor relocation for Small Data Transmission | LG Electronics Inc. |
| R2-2009490 | Security aspect on SDT procedure | Apple |
| R2-2009491 | Control plane aspects on SDT procedure | Apple |
| R2-2009492 | Context fetch procedure for SDT | Apple |
| R2-2009493 | CG based SDT procedure | Apple |
| R2-2009643 | Discussion on how to handle cell reselection during T319 for the case of SDT | ITRI |
| R2-2009646 | Discussion on RA-based Small Data Transmission | TCL Communication Ltd. |
| R2-2009649 | TAT maintenance for CG based SDT | ITL |
| R2-2009656 | Control plane issues for SDT | NEC |
| R2-2009657 | Subsequent data transmission for SDT | NEC |
| R2-2009675 | Discussion on the RRC-less SDT | Beijing Xiaomi Mobile Software |
| R2-2009799 | Details on RACH specific schemes | Nokia, Nokia Shanghai Bell |
| R2-2009872 | The basic principle for small data transmissions | Lenovo, Motorola Mobility |
| R2-2009873 | Analysis on RA selection and RNAU | Lenovo, Motorola Mobility |
| R2-2009874 | Consideration on CG based small data transmission | Lenovo, Motorola Mobility |
| R2-2009875 | Consideration on RRC-less SDT and subsequent data transmission | Lenovo, Motorola Mobility |
| R2-2009888 | Discussion on context fetch, anchor relocation and subsequent SDT in NR | Sony |
| R2-2009889 | Details of RA-based schemes for SDT in NR | Sony |
| R2-2009890 | Details of CG-based schemes for SDT in NR | Sony |
| R2-2009919 | SDT control plane aspects for RACH based schemes | Nokia, Nokia Shanghai Bell |
| R2-2009920 | Security aspects of SDT | Nokia, Nokia Shanghai Bell |
| R2-2009930 | SDT aspects common for RACH-based and CG-based SDT scheme | Huawei, HiSilicon |
| R2-2009931 | Discussion about security aspects for small data transmission | Huawei, HiSilicon |
| R2-2009963 | Details of RACH based SDT | Ericsson |
| R2-2009964 | Details of CG based SDT | Ericsson |
| R2-2009965 | Subsequent transmissions after initial SDT | Ericsson |
| R2-2009966 | RRC aspects for SDT | Ericsson |
| R2-2009967 | Report of [Post111-e][926][SmallData] ContextFetch_email | Ericsson (rapporteur) |
| R2-2009973 | Discussion on CG-based Small Data Transmissions | NEC Telecom MODUS Ltd. |
| R2-2009978 | Support of RRC-less SDT | NEC Telecom MODUS Ltd. |
| R2-2009991 | Draft LS on Need of MAC-I for UE authentication | NEC Telecom MODUS Ltd. |
| R2-2010006 | Discussion on RACH based NR small data transmission | Qualcomm Incorporated |
| R2-2010007 | Discussion on CG based NR small data transmission | Qualcomm Incorporated |
| R2-2010008 | Control plane aspects on NR small data transmission | Qualcomm Incorporated |
| R2-2010106 | RACH-based SDT selection and configuration | InterDigital |
| R2-2010107 | CG-based SDT selection and configuration | InterDigital |
| R2-2010108 | Beam selection and maintenance for CG-based SDT | InterDigital |
| R2-2010109 | Small data transmission failure and cell reselection | InterDigital |
| R2-2010232 | 2-step RACH and 4-step RACH selection criteria for SDT | Xiaomi |
| R2-2010280 | Small data transmission with RA-based scheme | Huawei, HiSilicon |
| R2-2010281 | Small data transmission with CG-based scheme | Huawei, HiSilicon |
| R2-2010388 | SDT type selection and switch procedure | CMCC |
| R2-2010389 | Some consideration on RACH based scheme | CMCC |
| R2-2010390 | Anchor relocation and context fetch | CMCC |
| R2-2010391 | Consideration on CG based SDT | CMCC |
| R2-2010429 | Discussion on subsequent small data transmission | ASUSTeK |
| R2-2010430 | Data forwarding without UE DRB configuration | ASUSTeK |
| R2-2010431 | Discussion on initiating SDT based on radio condition | ASUSTeK |
| R2-2010432 | Association between Pre-configured PUSCH resources and beam | ASUSTeK |
| R2-2010839 | [DRAFT] LS to RAN3 on small data transmission | Ericsson |
| R2-2010841 | LS on physical layer aspects of small data transmission | RAN2 |

## 4.3	RAN3

Nothing to report

## 4.1	RAN4

Nothing to report

END