3GPP TSG-RAN Meeting #90e	RP-202576

# Electronic Meeting,  December 7 – 11, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0312 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-2 on handling of fallbacks for FR2 CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | - |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-12-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mandating additional requirements for all possible fallback band combinations for a band combination consisting of mixed contiguous and non-contiguous intra-band CA in FR2 results in an excessive number of fallback combinations. That requires that requirements for an excessive amount of fallback band combinations must be specified in 3GPP and also complied to by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For FR2 intra-band CA combinations with multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, the following three changes are implemented: 1. Remove the requirement of direct fallback to single FR2 carrier 2. Introduce an applicability rule for Rx requirements (impacting clauses 7.5A, 7.5D, 7.6A, 7.6D) for fallbacks with multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination 3. Introduce an applicability rule for Tx requirements (impacting clauses 6.4A.2.3, 6.4D.2, 6.5A, and 6.5D) for fallbacks with multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, under the assumption that such CA combinations are limited to a single operator in a regional deployment Requirements in other core specifications are not affected. Impact analysis Impacted functionality: Requirements for band combinations. Inter-operability: No inter-operability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This specification is not aligned with other specifications (38.331 and 38.306) with regards to which fallback band combinations are supported and requirements for excessive number of fallback band combinations must be complied to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RP-201873 was postponed during the RAN #89e meeting |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigurationDedicated in the PCell and SCells for NR SA.

For FR2 intra-band CA combinations with multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, the UE RF requirements in clauses 7.5A, 7.5D, 7.6A, and 7.6D of only the highest order combination apply for its qualified fallbacks with the same number of multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, and the UE shall meet the out-of-gap requirements for its fallbacks.

For FR2 intra-band CA combinations with multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, in the present version of the specification, and under the assumption that such CA combinations are limited to a single operator in a regional deployment, the UE RF requirements in clauses 6.4A.2.3, 6.4D.2, 6.5A, and 6.5D of only the highest order combination apply for its qualified fallbacks with the same number of multiple sub-blocks, where at least one of the sub-blocks consists of a contiguous CA combination, and the UE shall meet the out-of-gap requirements for its fallbacks.

## <End of Changes>