3GPP TSG-RAN Meeting #90-e		                                                       RP-202732

Electronic Meeting, 7-11 December 2020

# Title:	About fixed and moving platform mounted device for NTN

# Source:	Hughes Network Systems, Thales, Intelsat, ESA

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	9.8.6

Release	Rel-17

# Introduction

As per TR 38.821 [1], it is assumed that both handheld devices (including smartphones) and other (fixed/moving platform mounted) devices are supported. The assumption and solutions developed in [1] have considered outdoor handheld (pedestrian) UEs or VSAT (vehicular relay) and UE on moving platforms. Moreover, the NTN reference scenarios as listed in chapter 4.2 of [1] consider two types of NTN UEs:

- UE with omni-directional antenna, typically handheld devices

- UE with directive antenna, typically mounted on a building or on a moving platform (aircraft, train, vessels, vehicles)

# Fixed and moving platform mounted device

UE with directive antenna can be categorized as “fixed or mounted” with the following capabilities:

- Moving platforms (aircraft, ships/vessels, high-speed trains, vehicles) mounted UE on. The UE can also act as a relay to other UEs on board moving platforms.

- Fixed UE, example VSAT (Very Small Aperture Terminal).

Figure 2: Fixed and moving UE for satellite-based NR in higher frequency bands

Note 1: The user link (UE-satellite) is based on Uu NR interface in FDD mode with CP-OFDM in the downlink and DFT-s-OFDM or CP-OFDM in uplink.

Note 2: The feeder link (gateway – satellite) may operate in any frequency outside the user link frequency bands in consideration.

Note 3: It is assumed that the satellite (and gateway in cases of transparent payload) is designed to close the link budget between the gNB and UE (satellite terminal), including effects of the feeder link.

## 2.1 Moving platform mounted UE

A moving platform mounted UE has been referred to as an Earth Station on Moving Platform (ESOMP); however, recently ITU has changed the definition to Earth Station in Motion (ESIM). These devices are typically mounted on vessels, airplanes, trains or fast vehicles. The UE can also act as a relay to other UEs on board moving platforms.

ESIM (Earth stations in motion) [2] address a complex challenge – how to provide reliable and high-bandwidth connectivity to what are literally moving targets. They provide broadband communications, including Internet connectivity, to a variety of platforms in motion. As per ITU definition there are currently three types of ESIM: ESIM on aircraft (aeronautical ESIM), ESIM on ships (maritime ESIM), and ESIM on land vehicles (land ESIM). ESIM on land vehicles was also referred to as to Vehicle-Mounted Earth Stations (VMES) [6]. The VMES may transmit and receive data when the vehicle is in motion and when the vehicle is stationary.

Please note that ETSI has defined recommended UE parameters for ESOMP terminals (ESIM) and VMES for several frequency bands ([3], [4], [5], [6]).

## 2.2 Fixed UE

The UE parameter description such UE (example VSAT) is highlighted in TR38.821 [2] in Table 6.1.1.1-3: UE characteristics for system level simulations, see below.

| Characteristics | VSAT (NOTE 2)* | Handheld | Other (NOTE 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band (i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band (i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircraft, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna. |  |  |  |

TR 38.821: Table 6.1.1.1-3: UE characteristics for system level simulations

*Note also that Ku band, considering the appropriate parameters, may also be applicable for VSAT.

# Conclusions

Proposal 1: As per TR 38.821, it shall be assumed that both handheld UE (including smartphones) and other (fixed/moving platform mounted) UE are supported in the Rel-17 WI NR-NTN-solutions.

# Reference

- 3GPP TR 38.821 “Solutions for NR to support non-terrestrial networks (NTN), (Release 16)”

- https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- ETSI EN 303 978 “Satellite Earth Stations and Systems (SES); Harmonized EN for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the R&TTE Directive”

- ETSI EN 303 979 “Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”

- ETSI EN 303980 “Satellite Earth Stations and Systems (SES); Harmonised Standard for fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands covering essential requirements of article 3.2 of Directive 2014/53/EU”

- ETSI EN 302977 “Satellite Earth Stations and Systems (SES); Harmonized EN for Vehicle-Mounted Earth Stations (VMES) operating in the 14/12 GHz frequency bands covering the essential requirements of article 3.2 of the R&TTE directive”

-END-