3GPP TSG RAN Meeting #90-e						 	                 RP-202806

Electronic Meeting, December 7 - 11, 2020

Status Report to TSG

Agenda item:			9.7.8

| WI / SI Name | Study on enhancement of RAN slicing for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_Slice |  |  |  |  |
| Unique ID | 860039 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201612 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2021 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: Overall: 50% RAN2: 40% RAN3: 60% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Ningyu Chen |
|  | Company | CMCC |
|  | Email | chenningyu@chinamobile.com |
| Secondary WG |  | RAN3 |
|  | Name | Dapeng Li |
|  | Company | ZTE |
|  | Email | li.dapeng@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN2

2.1.1	Agreements

RAN2 #112-e (Nov. 2020)

General

- RAN2 common understanding is that intended slice is based on the information AS receives from NAS for the particular use case. This may be different in different cases:

- In case of cell selection/reselection, the intended slice means the allowed or requested S-NSSAI(s).

-	For the initial registration, and requesting new S-NSSAI(s): intended slices = Requested S-NSSAI(s)

-	For idle-mode mobility: intended slices = allowed S-NSSAI(s)

- In case of MO traffic, the intended slice means the S-NSSAI associated with MO traffic based on indication from NAS to AS.

FFS whether UE needs to know the intended slice for MT service.

- For MO service, UE is aware of the intended slice. For MT service, UE is unaware of the slice for the paged service in current NR spec.

- Capture the location 3&4 in the TR (check offline to have consistent wording for "location" vs. "area").

Slice based cell reselection under network control

- These issues will be studied in this SI:

- Issue 1: The UE is unaware of the slices supported on different cells or frequencies, which prevents UE from (re)select to the cell or frequency supporting the intended slice.

- Issue 2: Dedicated priorities would not be available to the UE prior to first RRC connection establishment and only remain valid before T320 expires upon entering IDLE mode. In addition, dedicated priorities are discarded each time when UE entering CONNECTED mode and need to be configured again before UE leaving CONNECTED mode.

- Issue 3: Operator may require different frequency priority configurations for the specific slice in different areas, however the dedicated priority always overwrites the broadcast priorities if configured.

- Issue 4: If the serving cell is unable to support the requested slices for the subsequent access of the UE, the serving cell may bring on handover or rejection of access request. That may increase control plane signalling overhead as well as long control plane latency for the UE to access the network.

- The following solution approaches are captured in the TR and will be studied in this SI:

- Solution 1: Legacy dedicated priority via RRCRelease message.

- Solution 2: Slice related cell selection info, the slice info of serving cell and neighboring cells is provided in the system information or RRCRelease message. FFS: what information is broadcast.

- Solution 3: Slice related cell reselection info (e.g. Cell reselection priority per slice), the slice info of neighboring cells is provided in the system information or RRCRelease message. FFS: what information is broadcast.

- Solution 5: Rel-15 mechanisms such as HO, CA, DC and redirection can be used to access the intended slice in different cell

Slice based RACH configuration or access barring

- The intentions and use cases for slice-based RACH configuration are as follows:

- Intention 1: RA resource isolation. From marketing point of view, some of the industrial customers have the requirement for access resource isolation, in order to provide guaranteed RA resources for their sensitive slices.

- Intention 2: Slice access prioritization. In R15/16, all slices are sharing the same RA resources and cannot be differentiated by network side. But some slices may need to be prioritized during the RA procedure.

- The following solutions will be studied and captured in the TR 38.832:

- Solution 1: Slice-specific separate RACH resources pool can be configured per slice or per slice group, in addition to the existing common RACH resources.

- Solution 2: Slice-specific RACH parameters prioritization can be configured per slice or per slice group.

Neither solution may not be applicable to all possible slices.

2.1.2	Remaining Open issues

A post-meeting email discussion #253 is allocated to discuss on the following issues:

- Discuss the potential solutions for slice-based cell reselection and slice-based RACH configuration based on agreements on candidate solutions.

- Collect company views on schemes that should be prioritized with analysis on benefits and complexity for each solution.

2.2	RAN3

2.2.1	Agreements

RAN3 #110-e (Nov. 2020)

- Endorsed TR

R3-206735, Draft TR 38.832 0.2.0, endorsed

R3-207235, Draft TR 38.832 0.3.0, endorsed

- Scenario and solution for service continuity

Agreed TPs:

R3-206807, Agreed

R3-207136, Agreed unseen

R3-207193, Agreed unseen

R3-207106, (scenarios 3, 4, 5 only) Agreed

R3-207107, Agreed

R3-207194, Agreed unseen

R3-207195, Agreed unseen

R3-207124, Agreed

R3-207197, Agreed unseen

LS to SA2 on the slice SI progress, R3-207236, Agreed

LS Response to restricting the rate per UE per network slice, R3-207230, Agreed

2.2.2	Remaining Open issues

Evaluation of scenario and possible solutions, potential conclusion of the SI.

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#112-e (Nov. 2020)

- R2-2008732	LS on Enhancement of RAN Slicing (R3-205802; contact: Qualcomm)	RAN3

- R2-2008759	LS on Cell Configuration within TA/RA to Support Allowed NSSAI (S2-2006526; contact: ZTE)	SA2

- R2-2008857	Considerations on slice aware cell priority	KDDI Corporation

- R2-2008917	Slice based Cell Reselection under Network Control	CATT

- R2-2008949	Cell (re)selection based on preferred frequency(s) per slice	Beijing Xiaomi Software Tech

- R2-2008950	Deployment scenarios of RAN slicing based on SA2 LSout	Beijing Xiaomi Software Tech

- R2-2008963	Further discussion on RAN slicing enhancement	Qualcomm Incorporated

- R2-2009067	Considerations for Slice-based cell (re)selection	Nokia, Nokia Shanghai Bell

- R2-2009143	Discussion on slice based cell reselection	Spreadtrum Communications

- R2-2009174	Slice information for cell reselection	Samsung Electronics

- R2-2009175	RACH configuration for RAN slicing	Samsung Electronics

- R2-2009198	Consideration for slice based cell (re)selection	Intel Corporation

- R2-2009199	Consideration of Slice based RACH	Intel Corporation

- R2-2009288	5G RAN Slicing Framework During Cell Selection / Reselection Phases	MITRE Corporation, DoD, NTIA

- R2-2009423	RACH prioritisation for slices	Nokia, Nokia Shanghai Bell

- R2-2009473	Discussion on slice based cell selection and re-selection	Apple

- R2-2009474	Discussion on slice based RACH and cell barring	Apple

- R2-2009536	Discussion on slice based cell reselection under network control	China Unicom

- R2-2009542	Consideration on slice-based cell (re)selection	OPPO

- R2-2009543	Consideration on slice-based RACH	OPPO

- R2-2009644	Assistant information to enable UE fast access network slice	ITRI

- R2-2009669	Considerations on scenarios and solution space of RAN slicing enhancements	Lenovo, Motorola Mobility

- R2-2009688	Remaining issues on RACH and service continuity	vivo

- R2-2009689	Remaining issues on slice-based (re)-selection	vivo

- R2-2009806	Consideration on the slice specific RACH configuration	ZTE corporation, Sanechips

- R2-2009807	Consideration on slice specific cell selection and reselection	ZTE corporation, Sanechips

- R2-2009974	RACH enhancements to enable UE fast access to the intended slice	NEC Telecom MODUS Ltd.

- R2-2009979	Evaluation of Access delay to slice	Ericsson

- R2-2009986	Solutions for fast access to slice	Ericsson

- R2-2010063	Cell selection and reselection for RAN slicing	Google

- R2-2010065	Discussion on Network Slicing’s Impact on Cell (Re-)Selection	Convida Wireless

- R2-2010181	Slice based Cell (re)selection under network control	Huawei, HiSilicon

- R2-2010182	Slice based RACH configuration or access barring	Huawei, HiSilicon

- R2-2010183	Discussion on restricting the rate per UE per network slice	Huawei, HiSilicon

- R2-2010184	Draft reply LS on restricting the rate per UE per network slice	Huawei

- R2-2010222	Further discussion on how to decide intended slice for idle mobility	LG Electronics UK

- R2-2010223	Discussion on slice aware overload control	LG Electronics UK

- R2-2010364	Revised Work Plan for RAN Slicing	CMCC, ZTE

- R2-2010365	Draft TR 38.832	CMCC, ZTE

- R2-2010366	Report of [Post111-e][916][Slicing] Open issues for RAN slicing	CMCC

- R2-2010367	Discussion on SA2 LS and solutions for slice-based cell reselection	CMCC

- R2-2010488	Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI	Qualcomm Incorporated

- R2-2010646	Draft reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI	ZTE corporation, Sanechips

- R2-2010688	LS on Cell Configuration within TA/RA to Support Allowed NSSAI (C1-206760; contact: Nokia)	CT1

- R2-2010694	LS on restricting the rate per UE per network slice (S2-2007946; contact: Nokia)	SA2

- R2-2010695	LS Reply on Enhancement of RAN Slicing (S2-2008240; contact: ZTE)	SA2

- R2-2010987	[DRAFT] Reply LS on restricting the rate per UE per network slice	Nokia

- R2-2011102	Summary of [AT112-e][250][Slicing] LS replies to SA2 and RAN3 (Nokia)	Nokia

RAN3 #109-e (Aug. 2020)

- R3-206038	Discussion on slice re-mapping scenarios	Samsung

- R3-206039	Discussion on slice remapping solutions	Samsung

- R3-206040	pCR for TR 38.832 evaluation of slice remapping scenarios and solutions	Samsung

- R3-206238	Solution for RAN congestion and Service Continuity	Nokia, Nokia Shanghai Bell

- R3-206240	Remapping Solution involving 5GC for Service Continuity	Nokia, Nokia Shanghai Bell

- R3-206241	Data forwarding Solution for Service Continuity	Nokia, Nokia Shanghai Bell

- R3-206260	(TP for TR38.832) Additional slicing solutions	Qualcomm Incorporated

- R3-206362	Discussion on slice re-mapping principle	CATT

- R3-206363	Discussion on slice re-mapping due to slice resource shortage	CATT

- R3-206364	Discussion on slice re-mapping due to slice not supported	CATT

- R3-206430	Further discussion on the scenarios for network slice service continuity	Huawei, LGU+

- R3-206431	Evaluation of slice re-mapping policy solutions	Huawei, LGU+

- R3-206432	Solutions and Flow charts of network slice service continuity	Huawei, LGU+

- R3-206433	Discussion on slice Re-mapping at mobility	Ericsson

- R3-206434	Text Proposal for slice Re-mapping at mobility	Ericsson

- R3-206435	Analysis of KI#7 on Fast Access To Network Slices	Ericsson

- R3-206567	RAN impact of restricting the rate per UE per network slice	Nokia, Nokia Shanghai Bell

- R3-206568	Reply LS on restricting the rate per UE per network slice	Nokia, Nokia Shanghai Bell

- R3-206716	Further Consideration On RAN slicing scenario	ZTE

- R3-206717	Further Consideration On RAN slicing solution	ZTE

- R3-206735	Draft TR 38.832	CMCC, ZTE

- R3-206788	Further discussion on service continuity for slicing	CMCC

- R3-206792	Updated work Plan for RAN Slicing	CMCC, ZTE

- R3-206806	Further consideration on additional issues for slice remapping	LG Electronics

- R3-206807	(TP for 38.832) Additional issues for slice remapping	LG Electronics

- R3-206840	LS on restricting the rate per UE per network slice	SA WG2

- R3-206841	LS Reply on Enhancement of RAN Slicing	SA WG2

- R3-206867	Reponse for R3-206840	ZTE Corporation

- R3-206868	Reponse LS for R3-206840	ZTE Corporation

- R3-206901	CB: # RANSlicing1-Workplan_TRSkeleton - Summary of email discussion	CMCC - moderator

- R3-206902	CB: # RANSlicing2-Slice_Solutions_and_Evaluation - Summary of email discussion	Nokia - moderator

- R3-206903	CB: # RANSlicing3-SA2impact - Summary of email discussion	Ericsson - moderator

- R3-207041	Reponse to restricting the rate per UE per network slice	ZTE

- R3-207082	Solution for RAN congestion and Service Continuity	Nokia, Nokia Shanghai Bell

- R3-207083	Remapping Solution involving 5GC for Service Continuity	Nokia, Nokia Shanghai Bell

- R3-207106	Further discussion on the scenarios for network slice service continuity	Huawei, LGU+

- R3-207107	Evaluation of slice re-mapping policy solutions	Huawei, LGU+

- R3-207108	Solutions and Flow charts of network slice service continuity	Huawei, LGU+

- R3-207114	Evaluation of the solutions for RAN slicing enhancement	Samsung

- R3-207124	(TP for TR38.832) Additional slicing solutions	Qualcomm Incorporated

- R3-207133	Response to LS Reply on enhancement of RAN slicing	CMCC, ZTE

- R3-207135	Reponse to restricting the rate per UE per network slice	ZTE

- R3-207136	Solution for RAN congestion and Service Continuity	Nokia, Nokia Shanghai Bell

- R3-207143	CB: # RANSlicing2-Slice_Solutions_and_Evaluation - Summary of email discussion	Nokia - moderator

- R3-207146	Remapping Solution involving 5GC for Service Continuity	Nokia, Nokia Shanghai Bell

- R3-207193	Remapping Solution involving 5GC for Service Continuity	Nokia, Nokia Shanghai Bell

- R3-207194	Solutions and Flow charts of network slice service continuity	Huawei, LGU+

- R3-207195	Evaluation of the solutions for RAN slicing enhancement	Samsung

- R3-207196	Response to LS Reply on enhancement of RAN slicing	CMCC, ZTE

- R3-207197	Text Proposal for slice Re-mapping at mobility	Ericsson

- R3-207206	TR 38.832 0.3.0	CMCC, ZTE

- R3-207209	comparison of solution 37 and 22 with and without DC	Nokia

- R3-207216	(reserved)Response to LS on 5GC assisted cell selection for accessing network	Ericsson

- R3-207230	Reponse to restricting the rate per UE per network slice	ZTE

- R3-207231	Response to LS Reply on enhancement of RAN slicing	CMCC, ZTE

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template