3GPP TSG RAN Meeting #90e                         RP-20xxxx

Electronic Meeting, Dec, 2020

Agenda item:	9.1.5

Source:	CMCC

Title:	Email discussion for [90E][48][ATG]

Document for:	Information

0. Introduction

This is an email discussion summary for RAN4 R17 non-spectrum proposal on ATG.

The target of this email discussion is to generate an agreeable WID for ATG and to identify core set of functions that brings the most added value with reasonable work effort

- Rel-17 WI on ATG for NR

Related contributions in RAN#90

RP-202348	Email discussion summary for RAN4 R17 proposal on ATG

RP-202316	Motivation for new WI on air-to-ground network for NR

RP-202317	New WID on air-to-ground network for NR

RP-202692	Views on Rel-17 RAN4-led non-spectrum WIs and Sis

1.1 Objective 1: RF requirements

Specify features to core specifications of RF requirements for coexistence between ATG and IMT terrestrial network [RAN4]

- Identify key characteristics where it is absolutely necessary to differentiate ATG BS and UEs from ground based BS and UEs

  - Aim to reuse existing requirements for BS and UE where possible.

- Study and specify the framework how ATG core requirements are defined.

  - This includes identifying whether the requirements are captured within the existing specifications or new specifications are created.

  - Determine whether conducted, OTA or both types of requirement are required for both the BS and UE

- Identify the FR1 potential band(s) to be used as example for ATG

- Perform RF1co-existence evaluation for ATG network (e.g. ACLR, ACS)

- Specify new UE/BS type(s) for ATG network if necessary

  - Taking into account identified differences between ATG and ground based systems

- Specify RF requirements for ATG UE/BS

  - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

- Specify test procedures for ATG BS conformance testing

  - Determine at an early phase whether conducted, OTA or both types of testing are needed

Q1: Companies are invited to share views on objectives

| Company | Do you agree to include objective 1 in Release 17 ATG WI? | Any wording change suggestion on objective 1? |
| --- | --- | --- |

1.2 Objective 2: RRM core requirements

- Indentify and specify RRM core requirements for ATG, starting once the Rel-17 NTN WI has progressed sufficiently and taking into account the decisions/outcome of Rel-17 NTN work item.

  - RRM core requirements for ATG UE. [RAN4]

    - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects.

Q2: Companies are invited to share views on objectives

| Company | Do you agree to include objective 2 in Release 17 ATG WI? | Any wording change suggestion on objective 2? |
| --- | --- | --- |

1.3 Objective 3: RRM/Demod performance

- Indentify and specify RRM/Demod performance requirements for ATG, starting once the Rel-17 NTN WI has progressed sufficiently and taking into account the decisions/outcome of Rel-17 NTN work item.

  - RRM performance requirements and test cases for ATG UE type. [RAN4]

  - Demodulation performance requirements and test cases for ATG UE/BS. [RAN4]

Q3: Companies are invited to share views on objectives

| Company | Do you agree to include objective 3 in Release 17 ATG WI? | Any wording change suggestion on objective 3? |
| --- | --- | --- |

- Final scoping round summary