3GPP RAN 5G-ACIA Evaluations Week 1

October 12th – 16th 2020

Source:	Moderator (Ericsson)

Title:	Agreements on URLLC features and simulation assumptions for 5G-ACIA

Document for:	Discussion, Decision

# 1	Introduction

AT RAN#89, the following was agreed in RP-202069 on providing evaluations for 5G-ACIA:

- Start an offline email-based activity to provide evaluation results for 5G-ACIA

- One company volunteers as moderator

  - Proposes a work plan to follow

  - Ericsson is willing do this

- Discussions are on the RAN1_NR reflector

  - Email activity only during short periods (< week) distributed across the time allocated to the activity

  - No email activity in weeks before/during/after RAN1 meetings or RAN defined inactive periods

  - All companies should strive to limit email activity as much as possible

  - Outcome of the offline discussion will directly go to RAN without need for discussion in RAN1 nor need for LS from RAN1 to RAN

- Target completion by RAN#91

- At RAN#91, RAN will decide on a response LS to 5G-ACIA

The moderator made the following proposal on a timeline:

- 12-16 October 2020

  - Discussion on which URLLC features to include in the evaluations and simulation assumptions

- 14-18 December 2020

  - First round of simulation results

- 22-26 February 2021

  - Second round of simulation results

- 8-12 March 2021

  - Finalization of the report to RAN#91

A summary of the inputs provided by companies with first proposals for agreements was provided with companies adding their proposals[9]. Updated proposals were provided in[11]. Final proposals were made in [12].

In this contribution the final agreements are listed.

# 2	Simulation assumptions

The final agreements each parameter is listed in the table.

| Parameters | 5G-ACIA LS | Agreement |
| --- | --- | --- |
| Factory hall size | 120x50 m | As in 5G-ACIA LS |
| Room height | 10 m | As in 5G-ACIA LS |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. | According to proposed layout below |
| BS/TRP antenna height | 1.5 m for InF-SL and InF-DL 8m for InF-SH and InF-DH | As in 5G-ACIA LS |
| Layout – BS/TRP deployment | Depending on the number of TRPs | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| Channel model | UC-2: InF-DH > InD-DL > InF-SH > InF-SL | Mandatory: InF-DH Optional: InD-DL, InF-SH, InF-SL |
| Carrier frequency and simulation bandwidth | TDD 4 GHz: 100 MHz 30 GHz: 160 MHz | As in 5G-ACIA LS |
| TDD DL-UL configuration | Simulation company should report the used DL-UL configuration. | Companies should report the used DL-UL configuration. 1:1 DL-UL configuration is recommended. |
| Number of UEs per service area | Up to 50 per service area, e.g., 10, 20, 40, and 50 | As in 5G-ACIA LS |
| UE distribution | All UEs randomly distributed within the respective service area. | As in 5G-ACIA LS |
| Message size | 48 bytes | 48 bytes |
| DL traffic model | DL traffic arrival with option-1, option-2, and option-3. | 5G-ACIA Option 1 is mandatory. Companies are also encouraged to provide results for option 3 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 and option-2 | As in 5G-ACIA LS with Option 1 as mandatory |
| CSA requirements | UC-#2: 99.9999% | UC-#2: 99.9999% |
| Performance metrics | 1) CSA: single CDF of CSA distribution of all UEs in factory hall 2) Latency: single CDF of latency distribution of all UEs in factory hall 3) Percentage of UEs satisfying requirements 4) resource utilization | As in 5G-ACIA LS with 3) and 4) as low priority Note: For metric 2) it is clarified that a packet transmission cannot be performed after the latency deadline. The collected statistics cannot exceed the latency requirement. The packets exceeding the deadline are visible in the UE packet error statistics |
| E2E latency & air interface latency | E2E latency: 1 ms for UC#2 | E2E latency: 1 ms for UC#2 Air interface latency: 1ms |
| UE speed | Linear movement | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| BS antenna mount |  | Option 1 (1 sector per BS) from 38.824 is used |

Agreements:

- The simulation assumptions given in the table are agreed

- Additional simulation parameters are taken from TR 38.824.

# 3	Features to include in simulations

For the Rel-15 baseline, the following is agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding Rel-16 features, the following is agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

- This can be revisited after the first round of simulations have been provided in December.

# 4 Conclusions

This contribution provides the outcome of the week 1 discussion for 5G-ACIA evaluations providing URLLC features and simulation assumptions

# References

RP‑201279, “LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation”, 5G-ACIA

RP-202069, “Way forward on RAN work for 5G ACIA requested simulations“, Ericsson

“Simulation Assumptions and URLLC Features for 5G-ACIA Performance Evaluation”, Ericsson

“Discussion on URLLC and IIoT features for performance evaluation in response to 5G-ACIA”, Huawei, HiSilicon

“5G-ACIA LS – Phase 1 input”, Intel Corporation

“Features and simulation assumption for 5G ACIA URLLC LS response”, Nokia, Nokia Shanghai Bell

“Features and simulation assumption for 5G ACIA URLLC LS response”, Qualcomm CDMA Technologies

“Views on URLLC features and simulation assumptions for 5G-ACIA evaluations”, ZTE

“5G-ACIA URLLC features and simulation assumptions”, vivo

“Summary of company inputs on URLLC features and simulation assumptions v6”, Moderator(Ericsson)

“Updated proposals on URLLC features and simulation assumptions v8”, Moderator(Ericsson)

“Final proposals on URLLC features and simulation assumptions”, Moderator(Ericsson)