3GPP TSG-RAN WG1 Meeting # 104-e	R1-2102155

# e-Meeting, January 25th – February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0201 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.213 on PRACH handling for NR-DC power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), MediaTek, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For NR-DC dynamic power sharing, following condition is specified in sub-clause 7.6.2. The UE does not expect to have PUSCH, PUCCH, or SRS transmissions on the MCG that - are scheduled/triggered by DCI formats in PDCCH receptions with a last symbol that is earlier by less than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG The case of PRACH triggered by PDCCH order on MCG is missing in above condition and this is not aligned with blue highlighted part of RAN1#99 agreement below Agreements: - For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 - Alt.2: T_offset <= 2*T_proc,2 - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE does not expect to have PRACH transmission on the MCG that are triggered by PDCCH order with PDCCH reception with a last symbol that is earlier by less than or equal to Toffset from the first symbol of a transmission occasion on the SCG, and overlaps with the transmission occasion on the SCG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior for case when PRACH is triggered by PDCCH order on MCG when UE is configured with dynamic power sharing for NR-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, or semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3].

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2

-	if the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG or SCG, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PSCG or PMCG as the maximum transmission power, respectively

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 only for synchronous NR-DC operation [10, TS 38.133].

If a UE

-	is provided dynamic for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, and

-	indicates a capability to support dynamic power sharing for intra-FR NR DC,

the UE determines a maximum transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by at least Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and

-	overlap with the transmission occasion on the SCG

the maximum transmission power on the SCG is determined as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'long' for the capability,

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'short' for the capability, and

-	PMCGactual is the total power for the transmissions on the MCG that overlap with the transmission occasion on the SCG where PMCGactual is determined based on transmissions configured by higher layers and on transmissions scheduled by DCI formats in PDCCH receptions with a last symbol that is at least Toffset before the first symbol of the transmission occasion on the SCG.

The UE does not expect to have PUSCH, PUCCH, PRACH, or SRS transmissions on the MCG that

-	are scheduled/triggered by DCI formats in PDCCH receptions with a last symbol that is earlier by less than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

The UE does not expect to receive a positive TPC command value in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.

The UE is not required to apply a TPC command the UE receives in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.