3GPP TSG RAN WG1 Meeting #104-e	R1-2102157

e-Meeting, January 25 – February 05, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1381 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ process number for SPS validation when multi-TB is configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), ZTE, sanechips, Ericsson, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The HARQ process number for SPS validation in current spec is 4 bits and set as ‘0000’ according to subclause 9.2 in TS 36.213. However, for TDD, when multi-TB is configured, HARQ process number in ‘Scheduling TBs for Unicast’ field should be 3 bits, which cannot be set as ‘0000’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For SPS Activation/Release validation, change the HARQ process number field to ‘Set to all '0's’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The bitwidth and value for HARQ process number is not correct for SPS validation in TDD when multi-TB is configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 9.2	PDCCH/EPDCCH/MPDCCH/SPDCCH validation for semi-persistent scheduling

<Unchanged parts are omitted>

Table 9.2-1B: Special fields for Semi-Persistent Scheduling Activation MPDCCH Validation

|  | DCI format 6-0A | DCI format 6-1A |
| --- | --- | --- |
| HARQ process number | set to '000' | Set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| TPC command for scheduled PUSCH | set to '00' | N/A |
| TPC command for scheduled PUCCH | N/A | set to '00' |

Table 9.2-1C: Special fields for Semi-Persistent Scheduling Release MPDCCH Validation

|  | DCI format 6-0A | DCI format 6-1A |
| --- | --- | --- |
| HARQ process number | set to '000' | Set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Repetition number | set to '00' | set to '00' |
| Modulation and coding scheme | set to '1111' | set to '1111' |
| TPC command for scheduled PUSCH | set to '00' | N/A |
| Resource block assignment | Set to all '1's | Set to all '1's |

<Unchanged parts are omitted>