3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210312

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1443 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC test case 8.1.3.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang Linktester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At time T1, the entry condition 1 for event B2 Mp + Hys < Tresh1, where Mp=-85+(-156)=71, the actual value Hys is equal to 2, Tresh1 is equal to 73. Thus, the margin is 0 and the power level is not rearonable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell power of NR Cell 1 was changed to -85 at T1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE fails NR5GC-IRAT test case 8.1.3.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Fu Wang
	wangfu@datangmobile.cn

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_2_2_TestBody |
| --- | --- |
| Reason for change | - At time T1, the entry condition 1 for event B2 Mp + Hys < Tresh1, where Mp=-85+(-156)=71, the actual value Hys is equal to 2, Tresh1 is equal to 73. Thus, the margin is 0 and the power level is not rearonable. |
| Summary of change | - Cell power of NR Cell 1 was changed to -85 at T1. |
| TTCN module | RRC_Measurement_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_1_3_2_2_TestBody() runs on NR_BASE_PTC

{

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -78; //@sic R5-204382, R5-201477 sic@

var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -85; //@sic R5-204382, R5-201477 sic@

var PhysCellId v_PhysCellId;

var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA;

var EUTRA_PhysCellId v_EUTRA_CellId;

var integer v_MeasurementBandwidth_int;

var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth;

var Hysteresis v_Hysteresis := 4;

var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1;

var RSRP_Range v_B2_Threshold1 := 73; //@sic R5-201477, R5-195346 sic@

var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 58; //@sic R5-198920, R5-195346 sic@

var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo;

var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR;

var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA;

var template (value) ReportConfigToAddModList v_ReportConfigList;

var template (value) MeasIdToAddModList v_MeasIdConfigList;

var template (omit) MeasConfig v_MeasConfig;

//Power levels of NR cell 1: Table 8.1.3.2.2.3.2-1, Table 8.1.3.2.2.3.2-2

/*         |    FR1     |    FR2

* -------------------------------

*   T0    |    -75     |    FFS

* -------------------------------

*   T1    |    -85     |    FFS

* -------------------------------

*/

v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA);

v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn;

v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId);

v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth;

v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int);

v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)};

v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T1_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)};

f_NR_SetCellPowerList(v_CellPowerList_AtT0);

//@siclog "Steps 1-2" siclog@

//The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2

//The UE transmits an RRCReconfigrationComplete message

v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)};

v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)};

v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; //@sic R5-203359 sic@

v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1};

v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); //@sic R5s191140 sic@

//f_NR_SendRRCReconfiguration(nr_Cell1,-,-,-,-, v_MeasConfig);

f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-198871 sic@

//@siclog "Steps  3" siclog@

//Check: Does the UE transmit a MeasurementReport message within the next 10s?

f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any);

//@siclog "Step  4" siclog@

//SS re-adjusts the cell-specific reference signal level according to row "T1" in Table 8.1.3.2.1.3.2-1/2.

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s200543 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s200543 sic@

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 5" siclog@

//Check: Does the UE transmit a MeasurementReport message to report event B2 with the measured RSRP value for NR Cell 1?

v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1);

f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)}), -, -, -, -, -, cr_NR_MeasQuantityResults_RsrpRsrq); //@sic R5-204437, R5s191140 sic@

}

After change

function f_TC_8_1_3_2_2_TestBody() runs on NR_BASE_PTC

{

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -78; //@sic R5-204382, R5-201477 sic@

var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -90; //@sic R5-204382, R5-201477 sic@

var PhysCellId v_PhysCellId;

var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA;

var EUTRA_PhysCellId v_EUTRA_CellId;

var integer v_MeasurementBandwidth_int;

var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth;

var Hysteresis v_Hysteresis := 4;

var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r1;

var RSRP_Range v_B2_Threshold1 := 73; //@sic R5-201477, R5-195346 sic@

var RSRP_RangeEUTRA v_B2_Threshold2EUTRA := 58; //@sic R5-198920, R5-195346 sic@

var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo;

var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR;

var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA;

var template (value) ReportConfigToAddModList v_ReportConfigList;

var template (value) MeasIdToAddModList v_MeasIdConfigList;

var template (omit) MeasConfig v_MeasConfig;

//Power levels of NR cell 1: Table 8.1.3.2.2.3.2-1, Table 8.1.3.2.2.3.2-2

/*         |    FR1     |    FR2

* -------------------------------

*   T0    |    -75     |    FFS

* -------------------------------

*   T1    |    -85     |    FFS

* -------------------------------

*/

v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA);

v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn;

v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId);

v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth;

v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int);

v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)};

v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T1_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)};

f_NR_SetCellPowerList(v_CellPowerList_AtT0);

//@siclog "Steps 1-2" siclog@

//The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for event B2

//The UE transmits an RRCReconfigrationComplete message

v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)};

v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)};

v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false))}; //@sic R5-203359 sic@

v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1};

v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); //@sic R5s191140 sic@

//f_NR_SendRRCReconfiguration(nr_Cell1,-,-,-,-, v_MeasConfig);

f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-198871 sic@

//@siclog "Steps  3" siclog@

//Check: Does the UE transmit a MeasurementReport message within the next 10s?

f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any);

//@siclog "Step  4" siclog@

//SS re-adjusts the cell-specific reference signal level according to row "T1" in Table 8.1.3.2.1.3.2-1/2.

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s200543 sic@

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s200543 sic@

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 5" siclog@

//Check: Does the UE transmit a MeasurementReport message to report event B2 with the measured RSRP value for NR Cell 1?

v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1);

f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)}), -, -, -, -, -, cr_NR_MeasQuantityResults_RsrpRsrq); //@sic R5-204437, R5s191140 sic@

}