3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210167

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1573 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a timer inside f_NR_RRCRelease_Common which checks for eventual new connection establishment attempt after the release. However in case of redirection, this timer (and waiting for its expiry) may cause an unfair test failure, because only after its expiry the NR5GC component coordinates with EUTRA and progresses with RRC connection and UE may trigger new connection attempt while still waiting for the redirection procedure to finish causing TC failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New parameter added into f_NR_RRCRelease_Common to signal removal of t_Timer from the release procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

Change 1 proposed in this document shall also requires change 2.12 from R5s210149.

## Change 1

| Function name | fl_TC_8_1_1_3_4_TestBody |
| --- | --- |
| Reason for change | There is a timer inside f_NR_RRCRelease_Common which checks for eventual new connection establishment attempt after the release. However in case of redirection, this timer (and waiting for its expiry) may cause an unfair test failure, because only after its expiry the NR5GC component coordinates with EUTRA and progresses with RRC connection and UE may trigger new connection attempt while still waiting for the redirection procedure to finish. |
| Summary of change | New parameter added into f_NR_RRCRelease_Common to signal removal of t_Timer from the release procedure. |
| TTCN module | RRC_ConnRel_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_3_4_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on NR5GC_PTC { var template (value) CellReselectionPriorities v_CellReselectionPriorities; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, -88, -88); // FR2 FFS // @sic R5s201465 sic@ //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3), cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[1].Arfcn, 5)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, omit); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 3" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 5" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Step 4"); f_NR5GC_IRAT_S1ToN1_IdleMode (nr_Cell1, noRrcConnectionRelease, false); // @sic R5-202623 sic@ //@siclog "Step 5AA - 5AB" siclog@ f_NR_RRCReconfigExistingDRBs(nr_Cell1, omit); // @sic R5-202623 sic@ // @siclog "Step 5A" siclog@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_LoopbackIPdataOnDefaultBearer(nr_Cell1, v_PDUSessionInfoList[0], "Step 5C"); // @sic R5-206332 sic@ //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA and T320 v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, // @sic R5-202623 sic@ min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 7" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Step 7")); //@siclog "Step 8" siclog@ if (not f_NR_CheckNoRRCSetupReq(nr_Cell1, f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 300.0))) { // @sic R5-202623 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change:

function fl_TC_8_1_1_3_4_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on NR5GC_PTC { var template (value) CellReselectionPriorities v_CellReselectionPriorities; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, -88, -88); // FR2 FFS // @sic R5s201465 sic@ //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3), cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[1].Arfcn, 5)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, omit); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities), -, true); // Anritsu CR //@siclog "Step 3" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 5" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Step 4"); f_NR5GC_IRAT_S1ToN1_IdleMode (nr_Cell1, noRrcConnectionRelease, false); // @sic R5-202623 sic@ //@siclog "Step 5AA - 5AB" siclog@ f_NR_RRCReconfigExistingDRBs(nr_Cell1, omit); // @sic R5-202623 sic@ // @siclog "Step 5A" siclog@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_LoopbackIPdataOnDefaultBearer(nr_Cell1, v_PDUSessionInfoList[0], "Step 5C"); // @sic R5-206332 sic@ //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA and T320 v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, // @sic R5-202623 sic@ min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities), -, true); // Anritsu CR //@siclog "Step 7" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Step 7")); //@siclog "Step 8" siclog@ if (not f_NR_CheckNoRRCSetupReq(nr_Cell1, f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 300.0))) { // @sic R5-202623 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }