3GPP TSG RAN Meeting #91-e									RP-200290

Electronic Meeting, March 16-26, 2021

## Status Report to TSG

Agenda item:			9.7.1

| WI / SI Name | Further Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FeMIMO |  |  |  |  |
| Unique ID | 860040 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202024 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 30% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#104-e, the following agreements were made:

Multi-beam enhancements:

Conclusion

On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements).

For M=1:

- DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC

For N=1:

- UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC

For M=N=1:

- Joint DL/UL TCI:  A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter.

- Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate).

For M>1:

- DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC

For N>1:

- UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC

For M>1 and/or N>1:

- Joint DL/UL TCI:  A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N.

- Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate).

Note: Other TCI types/terms such as “common TCI” are not used.

Agreement

On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D.

- Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported:

  - CSI-RS for beam management

  - CSI-RS for tracking

- FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types

Agreement

On Rel.17 unified TCI framework, the following source RS types for UL TX spatial filter are supported:

- CSI-RS for tracking

- Note: SRS for BM, SSB, and CSI-RS for BM have been agreed in RAN1#102-e

- FFS (to be decided by RAN1#104bis-e): non-BM CSI-RS other than for tracking, non-BM SRS

Agreement

On Rel.17 unified TCI framework:

- For joint and separate DL/UL TCI, DL large scale QCL properties are inferred from one (qcl-Type1) or two RSs (qcl-Type1 and qcl-Type2) analogous to Rel.15/16

- For joint DL/UL TCI, UL spatial filter is derived from the RS of DL QCL Type D

Agreement

On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives:

- Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI

  - Details on dynamic indication are FFS

  - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI

- Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling

- Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling

- Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling

  - Details on how this is signaled in relation to TCI activation are FFS

Agreement

On Rel.17 unified TCI framework, decide by RAN1#104bis-e:

- Whether DL or, if applicable, joint TCI also applies to the following signals. If not, FFS any other enhancement over Rel.15/16:

  - CSI-RS resources for CSI

  - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’)

  - CSI-RS for tracking

- Whether UL or, if applicable, joint TCI also applies to the following signals

  - Some SRS resources or resource sets for BM

Agreement

On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework:

- The setting of (P0, alpha, closed loop index) is at least associated with UL channel or UL RS

- Select or modify from one of the following alternatives by RAN1#104bis-e for PUCCH, PUSCH, and SRS separately:

  - Alt1. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state

  - Alt2. The setting of (P0, alpha, closed loop index) is included with UL or (if applicable) joint TCI state

  - Alt3. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state

  - Alt4. The setting of (P0, alpha, closed loop index) is determined as in Rel-16 without enhancement

Agreement

On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability

- Support a UE capability for the minimum value of beam application time

- FFS: the exact minimum values of beam application time supported by UE

- FFS: whether existing UE capability can be reused as this UE capability.

- FFS: whether different beam application time values are supported for uplink and downlink

- FFS: whether UE capability needs to be introduced for the maximum value of beam application time

- FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission)

- FFS: whether a UE is allowed to report more than 1 values in case of MPUE

- FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s

Conclusion

On Rel.17 enhancements to facilitate UL beam selection for MP-UE, the following terms are used at least for the purpose of discussion:

- ‘Panel activation’ (at least for DL/UL measurement): activating L out of P available UE panel(s) at least for the purpose of DL and UL beam measurements (e.g. reception of DL measurement RS, transmission of SRS)

- ‘Panel selection’ (for UL transmission): selecting 1 out of L activated UE panel(s) for the purpose of UL transmission

- Note: UE-initiated panel activation and selection have been agreed in RAN1#103-e

Agreement

On Rel.17 enhancements to facilitate MPE mitigation,

- On further enhancing the P-MPR report in Rel.16 (already agreed RAN4 framework, including triggering), down select between beam-level and panel-select reporting

- On SSBRI(s)/CRI(s) and/or indication of panel selection, focus study on the following:

  - Reporting of at least SSBRI(s)/CRI(s) to indicate gNB beam(s) that is feasible for UL transmission: additional reporting quantities are FFS

  - Reporting of at least an indicator associated with a UE ‘panel’ that is feasible for UL transmission: additional reporting quantities are FFS

- Note: Just as agreed in RAN1#103-e, the purpose is to assess whether specification is needed or not

Agreement

On Rel.17 enhancements based on the unified TCI framework, perform study and, if needed, specify the following:

- Group1: Beam management with reduced DL signaling to reduce latency

- Group2: Reducing activation delay of TCI states and PL-RSs (including other WGs, e.g. RAN4)

  - On RAN4-related matters, assessment/study phase can be done in RAN1. If RAN4-based enhancements are found necessary, a LS to RAN4 will be sent (to prepare RAN4 work)

Note: Given its dependence on the maturity compared to other issues (1 to 5), when to start the work and how much work is done on issue 6 should depend on the progress on the other issues.

Note: Aim for at most one solution for each of the group in Rel-17 to address issue 6

Agreement

On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP:

- A quality of up to K beams associated at least with non-serving cell(s) can be reported in a single CSI reporting instance

  - For each beam, the UE can report at least: (1) a Measured RS Indicator, and (2) a Beam Metric associated with the Measured RS Indicator

  - FFS: Maximum value of K

  - FFS: If K is fixed, configured, reported by UE capability, or dynamically selected

  - FFS: The type of beam metric (e.g. L1-RSRP, L3-RSRP, or hybrid L1/L3-RSRP) and related measurement behavior

  - FFS: Whether or not beam reporting associated with non-serving cell(s) can be mixed with that with serving-cell in one reporting instance

At the end of RAN1#104-e, send an LS to RAN2 with all the RAN1-related inter-cell mobility agreements done so far during Rel17. LS is endorsed in R1- 2102209.

Agreement

On Rel.17 unified TCI framework:

- Select at least one of the following alternatives by RAN1#104bis-e for path-loss measurement (PL-RS):

  - Alt1. PL-RS can be included in UL TCI state or (if applicable) joint TCI state.

    - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state.

  - Alt2. PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state

    - FFS: Exact association mechanism

    - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state

  - Alt3. The periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the periodic DL-RS used as a source RS for determining spatial TX filter is not used as PL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement (e.g. pertaining to the use for PUCCH, or using default PL-RS)

    - PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state

  - Alt4. UE calculates path-loss based on periodic DL RS configured as the source RS or a periodic QCL-Type-D/spatialRelationInfo source of the source RS in UL TCI state or (if applicable) joint TCI state

    - FFS: Whether UE can calculate path-loss based on DL periodic RS for path-loss calculation for UL RS in the UL TCI

- FFS: Application time of PL-RS

- NOTE: As in Rel-16, a UE does not expect to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions

  - FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates

Agreement

On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP:

- Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s)

  - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s)

    - FFS: Whether the measurement for SS-RSRP is limited within SMTC

    - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc.

  - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported

    - Whether the support applies to CSI-RS with or without QCL source, or both

  - FFS: The number of non-serving cell(s) for measurement/reporting

  - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated

- FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP

- FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE

- FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement

Agreement

On the Rel.17 DCI -based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e):

- Alt0: No additional DCI format is supported

- Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI

  - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy

  - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism

  - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission

- Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI

  - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy

  - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only

  - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission

- Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI

Agreement

On Rel.17 enhancement for facilitating fast uplink panel selection,

- Rel.17 TCI state update (based on MAC CE + DCI along with the necessary TCI state activation, or MAC CE only) can be used for UE UL panel selection:

- FFS : Whether specification support for this feature is necessary and if so the details of such spec support, e.g.

  - Additional spec support in TCI state definition to accommodate UL panel

  - UE reporting to facilitate UL panel selection

  - UE reporting, e.g. panel-specific report, including UE -panel state, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combination(s) of UE -panel states

  - Support for linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, etc.

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, decide in RAN1#104bis-e whether to support at least one the following (not necessarily, but can be, in one reporting instance):

- {Rel.16 P-MPR based (beam/panel-level)} + {A}, where A is either Opt1A, Opt1B, Opt1C, or Opt1D:

  - Option 1A: Virtual PHR or a modified version associated with each activated UL TCI or, if applicable, joint TCI

  - Option 1B: {SSBRI(s)/CRI(s) and/or panel indication}

  - Option 1C: {SSBRI(s)/CRI(s) and/or panel indication} + virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)

  - Option 1D: No additional reporting quantity

- {SSBRI(s)/CRI(s) and/or panel indication} + {A}, where A is either Opt2A, Opt2B, Opt2A+ Opt2B, or Option 2C

  - Option 2A: L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)

    - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel

    - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR]

    - FFS: Whether/how to enhance existing beam reporting format to support Option 2A

  - Option 2B: Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)

  - Option 2C: No additional reporting quantity

Agreement

On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered:

- Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication

- Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication

- Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16)

- Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A

- Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication

- FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

Agreement

On Rel.17 enhancement for facilitating fast uplink panel selection, for discussion purpose, a panel entity corresponds to one or more RS resources:

- For CSI/beam reporting, the RS resource is an RS associated with measurement and/or reporting

- For beam indication, the RS resource is a source RS for UL TX spatial filter information

- Note: For one RS resource, the corresponding panel entity may vary and is controlled by the UE, and whether/how to maintain a common understanding between gNB and UE can be further discussed/decided

- Note: The above does not preclude possibility that an RS resource can be mapped to multiple panels

- Note: The one or more RS resources may correspond to one or more RS resource set(s) depending on further discussion/decision

- Note: Specification should not be designed in such a way that the UE is required to disclose its antenna implementation

Agreement

On Rel.17 enhancements for L1/L2-centric inter-cell mobility,

- Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s):

  - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references

  - SSB configured for non-serving cell(s) for UL TX spatial references

  - SRS for BM configured for non-serving cell(s) for UL TX spatial references

  - FFS: whether to support CSI-RS for mobility

  - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility

  - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source

- Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS:

  - RRC configuration issues

  - Serving cell issues

  - C-RNTI issues

  - Issues related to CU-DU split

  - Inter-band CA issues

  - Inter-frequency issues

Multi-TRP enhancements for PDCCH, PUSCH, and PUCCH:

Agreement

Confirm the working assumption:

For PDCCH reliability enhancements with non-SFN schemes and Option 2 + Case 1, support Alt3 (two SS sets associated with corresponding CORESETs).

Agreement

When DL DCI is transmitted via PDCCH repetition, for PUCCH resource determination for HARQ-Ack when the corresponding PUCCH resource set has a size larger than eight, starting CCE index and number of CCEs in the CORESET of one of the linked PDCCH candidates is applied. Down-select one of the following options in RAN1 #104-bis-e

- Option 1: The one with the lowest CORESET ID is applied

- Option 2: The one with the lowest SS set ID is applied.

Agreement

For Option 2, at least for the following purposes, a reference PDCCH candidate is defined as the candidate that ends later in time among the two linked PDCCH candidates in the time domain:

- To determine the scheduling offset to identify whether a default beam should be used for PDSCH / CSI-RS reception.

- To extend the definition of in-order for PDCCH-PDSCH and PDCCH-PUSCH, i.e., PDCCH ending symbol is the last symbol of the reference PDCCH candidate in at least the following restrictions in 38.214.

  - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol I, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i.

  - For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol I, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i.

- For PUSCH preparation time (N2) and CSI computation time (Z): Last symbol of the PDCCH is based on the last symbol of the reference PDCCH candidate.

- FFS: If inter-slot PDCCH repetition is supported, for slot offset for scheduling the same PDSCH/PUSCH/CSI-RS/SRS: The slot of the reference PDCCH candidate is used as the reference slot.

Agreement

If two PDCCH candidates that are linked for repetition do not belong to the same PDCCH monitoring occasion, the earlier PDCCH monitoring occasion is used as the reference for the following:

- Definition of counter DAI / total DAI and Type-2 HARQ-Ack codebook construction.

- Determining the last DCI for PUCCH resource determination based on the PRI field of the last DCI.

Agreement

Study whether / how to resolve the following potential issues in the case of PDCCH repetition:

- Issue 1: Starting symbol for PDSCH mapping type B as well as reference symbol for SLIV (i.e., when ReferenceofSLIV-ForDCIFormat1_2 is configured).

- Issue 2: Determination of PDSCH beam when TCI field is not present in DCI (when scheduling offset is equal to or larger than timeDurationForQCL)

- Issue 3: When PDCCH repetitions are associated with different CORESETPoolIndex values, and the need to use one of them as reference for PDSCH scrambling / CRS rate matching / HARQ-Ack / etc.

  - Whether PDCCH repetition can be used with multi-DCI based multi-TRP.

- Issue 4: Whether single-TRP PDCCH repetition is supported by reusing the agreed framework.

Agreement

For PDCCH repetition, support linking two SS sets by RRC configuration:

- FFS: Whether MAC-CE can be used additionally

- When PDCCH repetition is monitored in two linked SS sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets.

- The two linked SS sets have the same SS set type (USS/CSS)

  - The two linked SS sets have the same DCI formats to monitor

- For intra-slot PDCCH repetition,

  - The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration

  - For linking monitoring occasions across the two SS sets that exist in the same slot:

    - The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set

Agreement

For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, down-select one of the following options in RAN1 #104-bis-e

- Option 1: UE reports one or more numbers as required number of BDs for the two PDCCH candidates

  - Candidate values: 2, X.

    - Where X is a value larger than 2 and equal or less than 3

    - FFS: Whether a value between 1 and 2 should be added to the candidate values

    - FFS: Other values

- Option 2: UE reports whether it supports soft-combining or not

  - If soft-combining is supported, UE further reports one or more numbers as required number of BDs for the two PDCCH candidates

    - Candidate values: 2, X.

      - Where X is a value larger than 2 and equal or less than 3

      - FFS: Whether a value between 1 and 2 should be added to the candidate values

      - FFS: Other values

- Option 3: UE reports one or more decoding assumptions out of decoding assumptions 1-4

  - Number of BDs for decoding assumptions 1:

    - Alt1: 2 BDs

    - Alt2: A value between 1 and 2 BDs

  - Number of BDs for decoding assumption 2: 2

  - Number of BDs for decoding assumption 3: 2

    - FFS: Other values

  - Number of BDs for decoding assumption 4: 3

    - FFS: Other values

- Option 4: Always 2 BDs are assumed irrespective of UE’s decoding assumption

- Option 5: Always 3 BDs are assumed irrespective of UE’s decoding assumption

- FFS: Network configuration based on the above UE capabilities for options 1-3

Note: Specification should not be designed in such a way that the UE is required to disclose it receiver implementation

Agreement

At least for FR1, if a PDSCH is scheduled by a DCI in PDCCH candidates that are linked for repetition, and the resources in the CORESET(s) containing the PDCCH candidates overlap with the resources of the PDSCH, the PDSCH is rate matched around the union of two PDCCH candidates and the corresponding DMRS.

- Note: This does not imply that two linked PDCCH candidates can / cannot be overlapping in resources, which is a separate discussion.

- FFS: The case of FR2

Agreement

When two SS sets are linked for PDCCH repetition, they do not contain individual PDCCH candidates.

- Note 1: For configuration of individual PDCCH candidates, a different SS set can be configured by network.

- Note 2: When one of the linked PDCCH candidates uses the same set of CCEs as an individual PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, Rel. 15 rule is followed wrt not counting an additional BD.

Agreement

For PDCCH repetition, two PDCCH candidates in two SS sets are linked based on

- Having the same AL and the same candidate index:

  - Two linked SS sets are configured with the same number of candidates for each AL.

Conclusion.

The agreed PDCCH repetition framework (Option 2 + Case 1 + Alt3) supports both TDM and FDM multiplexing schemes.

Agreement

For single DCI based M-TRP PUSCH repetition Type B, support the following RV mapping,

- DCI indicates the first RV for the first PUSCH actual repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH actual repetitions of different TRPs with a possibility of configuring RV offset for the starting RV for the first actual repetition towards second TRP (The same method as PDSCH scheme 4).

Agreement

Support CG PUSCH transmission towards M-TRPs using a single CG configuration.

- Use same beam mapping principals as dynamic grant PUSCH repetition scheme.

- FFS: Required changes on CG parameters (ConfiguredGrantConfig)

- The feature is UE optional

Agreement

For M-TRP PUCCH scheme 1,

- Support PUCCH formats 0 and 2 (in addition to agreed PUCCH formats 1,3,4)

Agreement

For M-TRP PUCCH scheme 1,

- For PUCCH formats 1/3/4, values for the total number of repetitions at least contain values 2, 4, and 8.

  - FFS: maximum repetition number can be extended to 16.

- For PUCCH formats 0/2, the total number of repetitions at least contain 2.

  - FFS: other values.

- RRC configured number of slots (repetitions) are applied across both TRPs (e.g if the number of repetitions given by nrofSlots in PUCCH-config is 8, per TRP limit is 4).

Agreement

To support per TRP power control for multi-TRP PUCCH schemes in FR1,

- Two sets of power control parameters are used, and each set has a dedicated value of p0, pathloss RS ID and a closed-loop index.

- FFS: details on how a PUCCH resource can be linked to one or both of the two sets of power control parameters.

- FFS: whether PUCCH resource group can be linked to power control parameter sets.

Agreement

For single-DCI based M-TRP PUSCH repetition schemes, up to two power control parameter sets (using SRI-PUSCH-PowerControl) can be applied when SRS resources from two SRS resource sets indicated in DCI format 0_1/0_2.

- FFS1: Details on linking SRI fields to two power control parameters,

  - Alt. 1: Add second sri-PUSCH-MappingToAddModList, and select two SRI-PUSCH-PowerControl from two sri-PUSCH-MappingToAddModList

  - Alt. 2: Add SRS resource set ID in SRI-PUSCH-PowerControl, and select SRI-PUSCH-PowerControl from sri-PUSCH-MappingToAddModList considering the SRS resource set ID

  - Alt. 3: Let RAN2 handle this

  - Alt.4: Add second sri-PUSCH-PathlossReferenceRS-Id/sri-P0-PUSCH-AlphaSetId/sri-PUSCH-ClosedLoopIndex in SRI-PUSCH-PowerControl.

- FFS2: Enhancements on open-loop power control parameter set indication

- FFS3: Consideration on srs-PowerControlAdjustmentStates

- FFS4: Impact of multi-TRP PUSCH repetition on PHR reporting

- FFS5: Enhancement on power control parameters per TRP when SRI(s) indication of two SRS resource sets is absent.

Working Assumption

For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats.

- The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot.

- Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT

Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation.

Conclusion

For Multi-TRP PUCCH Scheme 1/3 at least containing HARQ ACK, supporting dynamic switching between multi-TRP PUCCH scheme and single-TRP PUCCH transmission is not restricted, and can be done by associating,

- a PUCCH resource activated with one or two spatial-relation-info and PRI bit-field indicating a PUCCH resource,

- or a PUCCH resource with one or two power control parameter sets and PRI bit-field indicating a PUCCH resource

FFS: Support of dynamic switching for Scheme 2 (if the schemes supported)

Agreement

For single DCI based M-TRP PUSCH repetition schemes, in codebook based PUSCH,

- Support two SRI fields corresponding to two SRS resource sets are included in DCI formats 0_1/0_2.

  - Each SRI field indicating SRI per TRP, where the SRI field based on Rel-15/16 framework

- Support dynamic switching between multi-TRP and single-TRP operation

- FFS: Support dynamic switching the order of two TRPs

Conclusion

Strive to reuse the specification support for dynamic indication of number of repetitions introduced in the Rel-17 coverage enhancement work item for multi-TRP operation. Decide whether further enhancements for multi-TRP operation are necessary in RAN1#106bis. No further discussion on this topic until RAN1#106bis under agenda item 8.1.

Agreement

For single DCI based M-TRP PUSCH Type B repetition schemes,

- For maxRank = 2, the number of bits for the indication of PTRS-DMRS association is the same as Rel-15/16, MSB and LSB separately indicating the association between PTRS port and DMRS port for two TRPs.

- FFS: the indication of PTRS-DMRS association for maxRank > 2.

Agreement

For s-DCI based multi-TRP PUSCH repetition Type A and B, if the DCI schedules A-CSI, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the X-th PUSCH repetition corresponding to the second beam.

- For PUSCH repetition Type A, X=1 (the first PUSCH repetition corresponding to the second beam)

- For PUSCH repetition Type B, the first actual PUSCH repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam are considered,

  - The UE does not expect the first actual repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam to have a single symbol duration (similar restriction as in Rel-16 NR for the single TRP case).

  - The first actual repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam are expected to have the same number of symbols

  - FFS: X = 1 or X = the first actual repetition corresponding to the second beam that contains the same number of symbols as the first actual repetition with the first beam

- FFS: Any further restrictions/enhancements needed on supporting A-CSI multiplexing on PUSCH repetitions

- FFS: whether to support multiplexing SP-CSI/P-CSI on PUSCH repetitions towards multiple TRPs.

Agreement

Further study following aspects related to beam mapping and default behaviors for multi-TRP PUCCH/PUSCH schemes,

- Whether enhancements needed on beam mapping in case of PUCCH/PUSCH dropping due to invalid UL symbols

- Whether frequency hopping is performed among the repetitions with the same beam

- Whether defining default beam for PUSCH is needed when PUSCH scheduled by DCI format 0_0 when two spatial relation info’s are configured for a PUCCH resource

Agreement

For single DCI based M-TRP PUSCH repetition schemes, in codebook based PUSCH,

- Two TPMI fields are indicated in DCI formats 0_1/0_2.

  - The first TPMI field uses the Rel-15/16 TPMI field design (which includes TPMI index and the number of layers) of DCI format 0_1/0_2. The second TPMI field only containsindicates the second TPMI index. The same number of layers are applied as indicated in the first TPMI field.

  - FFS: Details of second TPMI field interpretation including changes expected in Tables 7.3.1.1.2-2/2A/2B/3/3A/4/4A/5/5A in 38.212

  - FFS: Interpreting TPMI fields when multi-TRP and single-TRP PUSCH repetition is applied.

- FFS: whether to support of PUSCH repetitions transmitting towards two TRPs sharing the same TPMI indicated by a TPMI field.

- FFS: The size of the second TPMI field can be equal to or smaller than the size of the first TPMI field

Agreement

For single DCI based M-TRP PUSCH repetition schemes, in non-codebook based PUSCH,

- Support two SRI field(s) corresponding to two SRS resource sets are included in DCI formats 0_1/0_2.

  - Each SRI field indicating SRI per TRP, where the first SRI field based on Rel-15/16 framework,

  - Support the same number of layers applied over repetitions

  - FFS: details of second SRI field including the specification change for Table 7.3.1.1.2-28/29/30/31 in 38.212.

- Support dynamic switching between multi-TRP and single-TRP operation

  - FFS: whether/how to use SRI field(s) and additional details of SRI field(s) interpretations

- FFS: Minimizing the DCI overhead for PUSCH repetition Type A as a result of number of layers being limited to 1 when more than one repetition is scheduled.

- FFS: Support dynamic switching the order of two TRPs

- Companies are encouraged to provide total payload size of the two SRI fields and scheduling restriction, if any

Agreement

Further study following alternatives to support per TRP closed-loop power control for PUCCH , select  from the below options during the RAN1 #104-e-bis meeting.

- Option.1: A single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for both PUCCH beams

- Option.2: A single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for one of two PUCCH beams at a slot. The TPC value may be applied for the other PUCCH beam at an another slot.

- Option 3: A second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2.

- Option 4: A single TPC field is used in DCI formats 1_1 / 1_2, and indicates two TPC values applied to two PUCCH beams, respectively.

Working assumption

For beam mapping /power control parameter set mapping for PUCCH repetitions,

- For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions).

- For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource set mapping to sub-slots.

- This working assumption is also subjected to the RAN4 LS R1-2009807 and confirmed based on the RAN4 reply.

Agreement

Further study following alternatives to support per TRP closed-loop power control for PUSCH , select from the below options during the RAN1 #104-e-bis meeting.

- Option.1: A single TPC field (the existing TPC field) is used in DCI formats 0_1 / 0_2, and the TPC value applied for both PUSCH beams

- Option.2: A single TPC field (the existing TPC field) is used in DCI formats 0_1 / 0_2, and the TPC value applied for one of two PUSCH beams at a slot.

- Option 3: A second TPC field (similar to the existing TPC field) is added in DCI formats 0_1 / 0_2.

- Option 4: A single TPC field is used in DCI formats 0_1 / 0_2, and indicates two TPC values applied to two PUSCH beams, respectively.

Multi-TRP enhancements for multi-cell mTRP:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

Multi-TRP enhancements for beam management

Agreement

For M-TRP BFR

- Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set

  - FFS: value of N (e.g. fixed in specification, or UE capability)

- FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

Agreement

For M-TRP BFR

Support 1-to-1 association between each BFD-RS set and an NBI-RS set

- FFS: Association details

Agreement

BFRQ response

- Support at least the same gNB response as in Rel.16 SCell BFR (i.e. DCI with toggled NDI scheduling a same HARQ process ID as the PUSCH carrying BFRQ MAC-CE)

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

Multi-TRP enhancements for HST-SFN:

Agreement

Scheme 1 is supported in Rel-17

- TRS is transmitted in TRP-specific / non-SFN manner

- DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner

- FFS other details

Agreement

For scheme 1 and SFN transmission of PDCCH support Variant E for QCL assumption in TCI state when TRS is used as source RS

Agreement

Two TCI states are supported for scheme 1 in FR2

Agreement

- Support MAC CE activation of two TCI states for PDCCH

- FFS other details

Conclusion

The decision on support of specification based TRP pre-compensation scheme for HST-SFN scenario to be made in RAN1#104-e-bis meeting. To facilitate RAN1 decision, companies are encouraged to provide evaluation results according to the agreed evaluation assumptions. The evaluations not compliant with agreed assumptions will not be considered by RAN1 in the decision process.

Agreement

For HST-SFN scenario:

- Support semi-static (RRC based) switching of scheme 1 (PDSCH) with 2a, 2b, 3, 4

- FFS all other details including RRC signaling, possible RAN4 impact (if any), etc.

SRS enhancements:

Agreement

For Rel-17 SRS capacity and coverage enhancement, support the following

- Increase the maximum number of repetition symbols in one slot and one SRS resource to S

  - Support at least one S value from {8, 10, 12, 14}

    - FFS other candidate values

- Support to transmit SRS only in  contiguous RBs in one OFDM symbol, where  indicates the number of RBs configured by BSRS and CSRS

  - Support at least one PF value from {2, [3], 4, 8}

    - FFS other candidate values, e.g., non-integer values for PF

  - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued.

  - No new sequence including length is introduced

  - FFS it is applicable to frequency hopping and non-frequency hopping

  - FFS detailed signaling mechanism to determine PF and the location of the  RBs

- Support Comb 8

  - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued.

- FFS whether and if needed, how to use harmonized approach to define the three supported schemes

- Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

Agreement

- For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration.

  - For 1T6R, K=6, N_max = [4], and each resource has 1 port.

  - For 1T8R, K=8, N_max = [4], and each resource has 1 port.

  - For 2T6R, K=3, N_max = [3], and each resource has 2 ports.

  - For 2T8R, K=4, N_max = [4], and each resource has 2 ports.

  - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports.

  - FFS the number of supported candidate values of N for each xTyR.

- FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources

- FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS

- Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability.

Agreement

Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples

- CAT A: Time-domain parameters

  - A-1: Indication of available slot position, i.e., the t values

  - A-2: Indication of slot offset

  - A-3: Indication of SRS symbol-level offset

  - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting

- CAT B: Frequency-domain parameters

  - B-1: Indication of a group of CCs for SRS transmission

  - B-2: Indication of frequency domain resource in a BWP for SRS transmission

  - B-3: Indication of whether DL/UL BWP is applied for SRS transmission

- CAT C: Power control parameters

  - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’

    - FFS impact on power control, impact from triggering a group of CCs for SRS

  - C-2: Indication of open loop power control parameter e.g., p0.

- CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming

- CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states

- Other examples are not precluded

Agreement

A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t.

- In DCI format 0_1/0_2 without data and without CSI request,

  - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH

  - Alt 1-2: Re-purpose unused DCI field to indicate t

  - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB

    - FFS design details with other potential field(s)

  - FFS: whether t can be slot offset

- In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH

  - Alt 2-1: t is indicated by adding a new configurable DCI field

  - Alt 2-2: t is indicated without adding DCI payload

- Note: The size of DCI payload does not change dynamically

- Note: RAN1 should strive for unified solution for different DCI formats.

- FFS: The number of RRC configured t values per SRS resource set and DCI bit field size.

Agreement

Confirm the following working assumption with modifications

An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”.

- Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot.

- FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

CSI enhancements:

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where

- W1 is a free selection matrix, with identity matrix as special configuration

  - FFS polarization-common/specific selection

- Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1

  - At least one value of Mv>1 is supported

    - Decide on the value(s) of Mv, e.g. Mv=2,  in RAN1# 104bis-e

  - Working assumption:  Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters

  - FFS candidate value(s)  of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf

- Wf can be turned off by gNB. When turned off, Wf  is an all-one vector (FFS; the length of all-one vector)

- FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay,

- W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports

- Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study).

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement:

- Option 0: No further CSI-RS enhancement as the baseline

- Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25

- Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook

- Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis

- Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s).

  - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2.

  - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses

  - N CMR pairs are higher-layer configured by selecting from all possible pairs

    - signalling mechanism can be discussed further, e.g. using a bitmap

    - FFS: Whether MAC-CE or RRC+MAC CE indication is needed

    - FFS: how to support NCJT measurement hypotheses in FR2

- Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2

- Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis

Agreement

- Strive to agree at most one of the following options, if needed

  - Option 1: Confirm the Working Assumption from RAN1 103e.

  - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT

- The time of decision is RAN1#105e (May 2021)

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options:

- Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis

  - X = 0, 1, 2

    - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups

    - Support of X=1,2 is UE optional for the UE supporting option 1

  - FFS omission of CSI associated with NCJT measurement hypothesis

- Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses

  - FFS how to report recommended measurement hypothesis associated with that CSI report

#### 2.1.2	Remaining Open issues

Multi-beam enhancements:

- Further details of unified TCI framework, including QCL rules, additional details on joint and separate DL/UL TCI, and UL PC parameters

- Further details of L1/L2-centric inter-cell mobility, including additional details on inter-cell measurement/reporting and whether/how to introduce the support for inter-cell beam indication

- Further details of DCI-based beam indication, including application time definition and whether/how to introduce the support for additional format

- Further details of the spec support of fast panel selection for UL multi-panel UEs, along with MPE mitigation

- Study on advanced beam tracking/refinement

Multi-TRP enhancements for PDCCH, PUSCH, and PUCCH:

- Detailed design for PDCCH reliability and robustness enhancements for multi-TRP including supported modes of operation such as intra-slot and/or inter-slot repetition, further ambiguity resolution related rules, decision on number of BDs for two linked PDCCH candidates, and other.

- Detailed design for PUCCH reliability enhancements, including decisions/discussion on supported schemes, beam configuration/mapping, power control aspects, and other.

- Detailed design for PUSCH reliability enhancements, including the next level of details considering CB and non-CB based PUSCH, beam configurations/mapping, power control, and other.

Multi-TRP enhancements for multi-cell mTRP:

- Detailed design on QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation

Multi-TRP enhancements for beam management:

- Detailed design of beam measurement/reporting enhancement, beam failure recovery (including partial BFR) for mTRP and study on simultaneous reception of same/different channels/RS with different QCL-typeD

Multi-TRP enhancements for HST-SFN:

- Detailed solution on QCL assumption for DMRS antenna port(s) for HST-SFN deployment

- If the benefits are shown, detailed design of QCL/QCL-like relation between DL and UL signal for HST-SFN deployment

SRS enhancements:

- Design details of aperiodic SRS triggering enhancement considering flexibility and overhead

- Detailed resource set and resources configurations for SRS antenna switching up to 8Rx

- Details to complete the three agreed schemes for SRS coverage/capacity enhancement

CSI enhancements:

- Further detailed design of CSI reporting for M-TRP/panel for more dynamic hypothesis, including related configuration, measurement behavior, reporting mechanism, UE capability design at least.

- Further detailed design of PS codebook enhancement by utilizing FDD reciprocity, including related quantization per codebook component, reporting mechanism, UE capability design at least.

## 2.2	RAN2

#### 2.2.1	Agreements

not yet started yet

#### 2.2.2	Remaining Open issues

not yet started yet

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

not yet started

#### 2.4.2	Remaining Open issues

not yet started yet

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template