3GPP TSG RAN meeting #91e										       RP-210454

Electronic Meeting, March 16-26, 2021

Status Report to TSG

Agenda item:			9.7.26

| WI / SI Name | NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_enh |  |  |  |  |
| Unique ID | 900061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202928 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 15% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Jianchi Zhu |
|  | Company | China Telecom |
|  | Email | zhujc@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

0.25 TU is added for RAN4 #98e-bis, i.e., RAN4 TU to this WI is started from RAN4 #98e-bis.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #104-e

PUSCH enhancements:

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Agreement:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

Agreement:

- The same number of PRBs per symbol is allocated across slots for TBoMS transmission.

Agreements:

- Consecutive physical slots for UL transmission can be used for TBoMS for unpaired spectrum

  - To resolve in RAN1#104b-e whether to support non-consecutive physical slots for UL transmission for TBoMS for unpaired spectrum

- Consecutive physical slots for UL transmission can be used for TBoMS for paired spectrum and the SUL band

  - FFS if non-consecutive physical slots for UL transmission are also supported for paired spectrum and the SUL band

Agreements:

For TBoMS, the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers.

- FFS: Details and further constraints on the applicability of TBoMS.

Agreements:

One or two of the following approaches will be considered as a starting point to decide howNinfo NInfo for TBoMS is calculated (aiming for down selection in RAN1 #104-bis-e):

- Approach 1: Based on all REs determined across the symbols or slots (FFS whether symbols or slots are used) over which the TBoMS transmission is allocated

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K

Note: L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Agreements:

One or two of the following options will be considered (aiming for down-selection in RAN1#104b-e) to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols or slots (FFS whether symbol or slot are used) over which the TBoMS transmission is allocated.

  - FFS: if either the number of symbols or the number of slots is used.

  - FFS: if xOverhead is separately configured from the one in Rel-15/16.

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols allocated over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed.

Agreements:

- Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Agreements:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

Agreements:

- For joint channel estimation, a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS: whether the window should be specified

  - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: the time domain window may or may not be configured.

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided.

Agreements:

- Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - Different DMRS density for different PUSCH transmissions

    - No DMRS for some PUSCH transmissions

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - DMRS equally spaced among PUSCH transmissions

    - DMRS located in special slots

    - Orphan symbol used for DMRS

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain.

  - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details.

Working assumption:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

Agreements:

- For joint channel estimation.

  - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound.

  - Companies can report other values and frequency error model.

PUCCH enhancements:

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication.

- Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

  - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc.

  - FFS: RRC signaling enhancement details

- Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI

  - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication

  - FFS whether there is a need for RRC update

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

- FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.

- Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

Agreements:

Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

- FFS: the configuration is per UE or per PUCCH resource.

- FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

- FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

Conclusion: In Rel-17, deprioritize the study of DMRS pattern/location/granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of  DMRS pattern/location/granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3.

Conclusion: For the study of enhancing inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, at least the following aspects can be considered:

- Performance tradeoff between maximizing # consecutive UL slots in one frequency hop (to achieve more DMRS bundling gain) and maximizing # hops (to achieve more diversity gain)

  - Note: the maximum # frequency hopping positions is still 2 as in Rel-15/16.

- Interaction between hopping boundary determination and TDD configuration

Conclusion: For the simulations to study the enhancement of inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, simulation assumptions in 38.830 are reused as a starting point.

Note: Additional simulation scenarios/assumptions are not precluded.

Type A PUSCH repetitions for Msg3:

Agreements:

- For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below.

  - Option1: UL grant scheduling Msg3.

    - FFS details.

    - FFS fallbackRAR UL grant.

    - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

  - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI

    - FFS details.

  - Option3: SIB1 only

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Agreements:

- For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below.

Option1: DCI format 0_0 with CRC scrambled by TC-RNTI.

FFS details.

Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI.

Option2: Can be determined based on the repetition number  for  Msg3 initial transmission

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

FFS details, e.g., signaling etc.

Agreements:

For Msg3 PUSCH repetition, the following options are considered, aiming for down-selection in RAN1#104b-e:

- Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request,

- A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- FFS details if any.

- Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request,

- gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition

- For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc.

- gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition.

- FFS details if any.

- Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS details if any.

- Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

  - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

  - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

  - FFS details if any.

- Other options are not precluded.

2.1.2	Remaining Open issues

- PUSCH enhancements

  - Detailed mechanisms for enhancements on PUSCH repetition type A

    - Increasing the maximum number of repetitions.

    - The number of repetitions counted on the basis of available UL slots.

  - Detailed mechanism(s) to support TB processing over multi-slot PUSCH

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Detailed mechanism(s) to support joint channel estimation

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation

- PUCCH enhancements

  - Detailed signaling mechanism to support dynamic PUCCH repetition factor indication

  - Detailed mechanism to support DMRS bundling across PUCCH repetitions

- Detailed mechanism(s) to support Type A PUSCH repetitions for Msg3

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The progress in RAN4 #98e meeting is summarized below:

- The Email discussion summary for [98e][155] NR_reply_LS_Part_2 was provided in R4-2103341.

- The Reply LS on PUCCH and PUSCH repetition was approved in R4-2103393.

- The Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition was approved in R4-2103363.

2.4.2	Remaining Open issues

- Conditions for joint channel estimation to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #104e:

- R1-2100914, Work plan for Rel-17 WI on NR coverage enhancements, China Telecom

- R1-2102113, FL Summary on Enhancements on PUSCH repetition type A, Moderator (Sharp)

- R1-2102241, FL summary of TB processing over multi-slot PUSCH, Moderator (Nokia, Nokia Shanghai Bell)

- R1-2102161, [104-e-NR-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH, Moderator (China Telecom)

- R1-2102226, Feature lead summary on support of Type A PUSCH repetitions for Msg3, Moderator (ZTE Corporation)

- R1-2100095, Discussion on enhanced PUSCH repetition type A, ZTE

- R1-2100172, Enhancements on PUSCH repetition type A, OPPO

- R1-2100196, Coverage enhancements for PUSCH repetition typeA	Huawei, HiSilicon

- R1-2100397, Discussion on enhancements on PUSCH repetition type A	CATT

- R1-2100457, Discussion on enhancement for PUSCH repetition type A	vivo

- R1-2100665, Enhancements on PUSCH repetition type A	Intel Corporation

- R1-2100712, Discussions on PUSCH repetition type A enhancements	LG Electronics

- R1-2100731, PUSCH repetition for coverage enhancements	InterDigital, Inc.

- R1-2100915, Enhancements on PUSCH repetition type A	China Telecom

- R1-2100942, Discussion on  enhancements on PUSCH repetition type A	NEC

- R1-2101001, Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

- R1-2101017, Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2101055, Discussion on enhancements on PUSCH repetition type A	CMCC

- R1-2101127, Enhancements on PUSCH repetiton type A	Xiaomi

- R1-2101221, Enhancements on PUSCH repetition type A	Samsung

- R1-2101327, Design Considerations for Enhancements on PUSCH repetition	Sierra Wireless, S.A.

- R1-2101395, Discussion on PUSCH repetition type A enhancement	Apple

- R1-2101407, PUSCH Repetitions for Coverage Enhancement	Indian Institute of Tech (H)

- R1-2101477, Enhancements on PUSCH repetition type A	Qualcomm Incorporated

- R1-2101520, PUSCH Repetition Type A Enhancement	Ericsson

- R1-2101545, Enhancements on PUSCH repetition type A	Sharp

- R1-2101641, Enhancements on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2101656, Enhancements on PUSCH repetiton type A	Xiaomi

- R1-2101679, Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

- R1-2101710, Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2100096, Discussion on TB processing over multi-slot PUSCH	ZTE

- R1-2100173, Supporting TB  over multi-slot PUSCH	OPPO

- R1-2100232, Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2100398, Discussion on TB processing over multi-slot PUSCH	CATT

- R1-2100458, Discussion on PUSCH TB processing over multiple slots	vivo

- R1-2100666, Discussion on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2100713, Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2100732, TB processing over multi-slot PUSCH	InterDigital, Inc.

- R1-2100743, Views on TB processing over multi-slot PUSCH	Fujitsu

- R1-2100916, Discussion on TB processing over multi-slot PUSCH	China Telecom

- R1-2100943, Discussion on TB processing over multi-slot PUSCH	NEC

- R1-2101002, Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

- R1-2101018, Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2101056, Discussion on TB processing over multi-slot PUSCH	CMCC

- R1-2101128, Joint channel estimation for PUSCH	Xiaomi

- R1-2101222, TB processing over multi-slot PUSCH	Samsung

- R1-2101328, Design Considerations for TB processing over multi-slot PUSCH	Sierra Wireless, S.A.

- R1-2101396, Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2101406, On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

- R1-2101478, TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2101521, TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2101546, TB processing over multi-slot PUSCH	Sharp

- R1-2101642, TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2101646, Discussion on TB processing over multi-slot PUSCH	MediaTek Inc.

- R1-2101680, Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2101711, Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2100097, Discussion on joint channel estimation for PUSCH	ZTE

- R1-2100174, Joint channel estimation for PUSCH	OPPO

- R1-2100233, Discussion on Joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2100399, Discussion on joint channel estimation for PUSCH	CATT

- R1-2100459, Discussion on Joint channel estimation for PUSCH	vivo

- R1-2100558, Potential techniques for PUSCH coverage enhancement	Potevio Company Limited

- R1-2100667, Discussion on joint channel estimation for PUSCH	Intel Corporation

- R1-2100714, Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2100733, Discussions on joint channel estimation for PUSCH	InterDigital, Inc.

- R1-2100797, Considerations on joint channel estimation over multi-PUSCH	Spreadtrum Communications

- R1-2100917, Discussion on joint channel estimation for PUSCH	China Telecom

- R1-2101003, Enhancements for DM-RS bundling for multiple PUSCH	Lenovo, Motorola Mobility

- R1-2101020, Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2101057, Discussion on joint channel estimation for PUSCH	CMCC

- R1-2101223, Joint channel estimation for PUSCH	Samsung

- R1-2101329, Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless, S.A.

- R1-2101397, Discussion on joint channel estimation for PUSCH	Apple

- R1-2101479, Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2101522, Joint Channel Estimation for PUSCH	Ericsson

- R1-2101547, Joint channel estimation for PUSCH	Sharp

- R1-2101643, Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2101681, Discussion on joint channel estimation for PUSCH	WILUS Inc.

- R1-2101712, Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2100098, Discussion on coverage enhancements for PUCCH	ZTE

- R1-2100175, PUCCH enhancements for coverage	OPPO

- R1-2100198, PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2100400, Discussion on PUCCH enhancements	CATT

- R1-2100460, Discussion on PUCCH enhancements	vivo

- R1-2100668, Discussion on PUCCH enhancements	Intel Corporation

- R1-2100715, Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2100747, Discussions on PUCCH enhancements	InterDigital, Inc.

- R1-2100798, Considerations on PUCCH coverage enhancement	Spreadtrum Communications

- R1-2100918, Discussion on PUCCH enhancements	China Telecom

- R1-2101021, Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

- R1-2101058, Discussion on PUCCH enhancements	CMCC

- R1-2101081, PUCCH enhancements	ETRI

- R1-2101129, PUCCH enhancement	Xiaomi

- R1-2101224, PUCCH enhancements	Samsung

- R1-2101398, PUCCH coverage enhancement	Apple

- R1-2101480, PUCCH coverage enhancements	Qualcomm Incorporated

- R1-2101523, PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2101548, Dynamic PUCCH repetition factor indication	Sharp

- R1-2101576, Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

- R1-2101626, PUCCH enhancements for coverage enhancements	NTT DOCOMO, INC.

- R1-2101682, Discussion on PUCCH enhancements for coverage enhancement	WILUS Inc.

- R1-2101713,	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2100099, Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

- R1-2100176, Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2100197, Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2100401, Discussion on Type A PUSCH repetitions for Msg3	CATT

- R1-2100461, Discussion on Type A PUSCH repetitions for Msg3	vivo

- R1-2100490, Target of PUSCH Msg.3 coverage enhancements	SoftBank Corp.

- R1-2100669, On Msg3 PUSCH repetition	Intel Corporation

- R1-2100716, Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2100748, Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2100919, Discussion on type A PUSCH repetitions for Msg3	China Telecom

- R1-2100944, Discussion on  PUSCH repetitions for Msg3	NEC

- R1-2101022, Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2101059, Discussion on type A PUSCH repetitions for Msg3	CMCC

- R1-2101082, PUSCH coverage enhancement	ETRI

- R1-2101130, Type A PUSCH repetitions for Msg3	Xiaomi

- R1-2101225, Type A PUSCH repetitions for Msg3	Samsung

- R1-2101399, Discussion on msg3 PUSCH repetition	Apple

- R1-2101481, Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2101524, Type A PUSCH Repetition for Msg3	Ericsson

- R1-2101549, Type A PUSCH repetitions for Msg3	Sharp

- R1-2101627, Type A PUSCH repetitions for Msg3 for coverage enhancements	NTT DOCOMO, INC.

- R1-2101683, Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

- R1-2101714, Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2100100, Discussion on PUSCH and Msg3 enhancements for Redcap UEs	ZTE

- R1-2100177, Other considerations for coverage enhancement	OPPO

- R1-2100402, Views on reusing PUSCH enhancements for Msg3	CATT

- R1-2100462, Enhanced Contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition			vivo

- R1-2100749, Discussion on the number of HARQ processes for VoIP 	InterDigital, Inc.

- R1-2100868, Association of dual polarized SSBs	Sony

- R1-2101226, Discussion on PRACH enhancements for msg3 improvement	Samsung

- R1-2101253, Views on handling potential overlaps with other WIs	Huawei, HiSilicon

- R1-2101525, Other Coverage Enhancements for PUSCH	Ericsson

- R1-2101715, On the scope of RAN1 LS on phase and power requirements for joint channel estimation/DM-RS bundling of PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

RAN4 #98e:

- R4-2103003	Email discussion summary for [98e][155] NR_reply_LS_Part_2	Moderator (China Telecom)

- R4-2103341	Email discussion summary for [98e][155] NR_reply_LS_Part_2	Moderator (China Telecom)

- R4-2103288	Reply LS on PUCCH and PUSCH repetition	Qualcomm

- R4-2103393	Reply LS on PUCCH and PUSCH repetition 	Qualcomm

- R4-2103289	Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition	Huawei, HiSilicon

- R4-2103363	Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition	Huawei, HiSilicon

- R4-2100159	Discussion on phase continuity for PUSCH and PUCCH repetitions for LS reply	InterDigital Communications

- R4-2100889	Discussion on LS on PUCCH and PUSCH repetition	Qualcomm Incorporated

- R4-2102347	Reply LS  on PUCCH and PUSCH repetition	Ericsson

- R4-2102630	on phase continuity for PUCCH and PUSCH repetition and reply LS	Huawei, HiSilicon

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template