3GPP TSG RAN meeting #91e										RP-210566

Electronic Meeting, March 22 - 26, 2021

## Status Report to TSG

Agenda item:			9.8.8

| WI / SI Name | NR Multicast and Broadcast Services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MBS |  |  |  |  |
| Unique ID | 860048 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201038 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 30% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhenzhen CAO |
|  | Company | Huawei |
|  | Email | caozhenzhen@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Due to COVID-19 pandemic, this WI has been rescheduled and started only from Q3/2020 which is 6 months delay compared to the original schedule agreed in RP- RP-193248. For the new schedule, TUs need to be allocated for WG meetings in Q4/2020 and onwards.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#104-e agreements

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreement:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

- FFS whether the use of a common frequency resource for multicast is optional or not

- FFS whether the common frequency resource is applicable for PTM scheme 2 (if supported) or not

Agreement:

- If Option 2B is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the starting PRB and the length of PRBs of the MBS frequency region within a dedicated unicast BWP are configured via UE-specific RRC signaling.

  - The starting PRB is referenced to one of the two options:

    - Option 1: Point A

    - Option 2: the starting PRB of the dedicated unicast BWP

  - FFS the detailed signaling

- If Option 2A is supported for common frequency resource for multicast of RRC-CONNECTED UEs, the configurations of the starting PRB and the length of PRBs of the MBS frequency resource reuse the legacy BWP configuration.

Agreement:

For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission.

- The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

Agreement:

The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- FFS whether the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Working Assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability

- The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS.

- FFS: How to allocate the total SPS configurations between MBS and unicast.

Agreement:

For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS

- FFS: The retransmission scheme(s)

- FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2

Agreement:

From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations:

- Starting PRB and the number of PRBs

- One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP)

- One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP)

- SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP)

- FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP

- FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED

- FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed

- Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2

- Note: This agreement does not negate any previous agreements made on CFR

Agreement:

For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, at least support CSS

- FFS: reuse existing CSS type(s) in Rel-15/16 or define a new Type CSS

- FFS: Two options for monitoring priority:

  - Option 1: the monitoring priority is the same as existing Rel-15/16 CSS

  - Option 2: the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

Working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on concatenating the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, one common frequency resource for group-common PDCCH/PDSCH can be defined/configured.

- FFS: whether to define/configure more than one common frequency resources

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB.

- It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes.

- FFS: association rules between SSB indexes and UE monitoring occasions.

- FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured

Agreement:

For broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs and the SCS and CP are the same.

- FFS: the case when UE-specific active BWP of RRC_CONNECTED UE does not contain the common frequency resource of RRC_IDLE/INACTIVE UEs.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported:

- [Case E] the case where a CFR is defined based on a configured BWP.

  - In particular, study the following:

    - whether a configured BWP for MBS is needed or not.

    - whether BWP switching is needed or not.

  - In this study, the configured BWP has the following properties:

    - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth.

    - The CFR has the frequency resources identical to the configured BWP.

    - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

  - Note: The configured BWP is not larger than the carrier bandwidth

- the case where the initial BWP fully contains the CFR in the frequency domain.

  - In this study the following sub-cases are considered:

    - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

    - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS.

- the case where the initial BWP has same size as the CFR in the frequency domain.

  - In this study the following two sub-cases are considered:

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS.

#### 2.1.2	Remaining Open issues

- Group scheduling mechanism for MBS in RRC_CONNECTED state, including enhancements to enable simultaneous operation with unicast reception

- Reliability improvement mechanisms for MBS in RRC_CONNECTED state

- Required changes for MBS in RRC_IDLE/ RRC_INACTIVE states

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#113-e agreements

Stage-2

Stage-2 running CR in R2-2102463 was endorsed.

Reply LS to SA2 in R2-2102480 was approved

Reliability

RAN2 confirmed the following proposals in the email discussion summary R2-2100322, by assuming that MRB may include both PTP and PTM:

    - RAN2 agrees that RAN reliability requirements for NR MBS are derived based on QoS reliability requirements configured by 5GC MB-SMF.

    - RAN2 agrees that for a given set of configured QoS parameter values, it is interpreted in the same manner by RAN for both unicast and multicast service delivery.

    - RAN2 agrees that QoS requirements are same whether gNB deliveres multicast data to UEs by using DRB associated with Unicast PDU session or by using MRB associated with MBS session.

For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast).

Idle and Inactive UEs

Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other). The ability for connected mode UEs to receive this may depend on the network provisioning of the service (e.g. which freq), UE connected mode configuration and UE capabilities.

The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2.

Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner.

Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

Assume that MBS Interest Indication is supported for UEs in connected mode for Broadcast service (assume that as usual there is no mandatory network requirement, network action is up to network).

MBS Interest Indication is NOT supported for UEs in idle/inactive mode for NR MBS delivery mode 2.

Assume that some information for purpose of service continuity can be provided for NR MBS delivery mode 2. (FFS what - need to be revisited, e.g. based on progress in other groups, e.g. USD, SAI/TMGI etc)

FFS whether support UE awareness of MBS services on frequency basis for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

FFS Support frequency prioritization during cell reselection for service continuity for NR MBS delivery mode 2 (i.e. Reuse LTE SC-PTM mechanism).

Whether UEs that receive Multicast can be released to RRC Inactive / Idle and continue receiving Multicast is Postponed. Should limit to RRC inactive in future discussions

#### 2.2.2	Remaining Open issues

- Remaining issues for multicast, including but not limited to:

  - L2 architecture, L2 reliability needed or not ;

  - Dynamic switch issues (e.g. transparent or any signaling needed);

  - Use cases of PDCP status reporting and retransmission (subjected to L2 reliability discussion);

  - Remaining issues for mobility support, including mobility between MBS-supporting gNBs, as well as between MBS-supporting and non-MBS-supporting gNBs.

- Remaining issues for broadcast, including but not limited to:

  - BCCH/MCCH based PTM configuration;

  - PTM configuration change notification.

- RAN2 aspects of group scheduling, including but not limited to:

  - DRX;

  - SPS as agreed by RAN1.

  - BWP

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#111-e agreements

In RAN3#111-e meeting, 117 contributions [147] - [263] were submitted. And the following are achieved:

- General

  - Baseline CRs endorsed.

  - One Reply LS to SA2/RAN2/SA4 agreed in R3-211296, RAN3 provides answers about SA2 questions, with the following information:

    - Ask SA2 to clarify the differences between the following terms: session start/session activation and session deactivation/session stop and their implication to NG-RAN functions.

    - Ask for confirmation from SA2 whether a UE is supposed to receive the MBS Session activation notification also when served by a non-supporting NG-RAN node.

    - Ask RAN2 and SA2 to consider how to page the UEs which are not in RRC_CONNECTED state, whether a group notification towards NG-RAN supporting MBS and, if applicable, to NG-RAN nodes not supporting MBS is needed/feasible/beneficial comparing to the legacy paging methods.

    - For Xn-based handover from MBS supporting node to non-MBS supporting node, it is assumed that switch from shared delivery to individual delivery will be take place during the path switch procedure. FFS for handover from non-MBS supporting node to MBS supporting node.

    - For lossless handover with data forwarding from source NG-RAN supporting 5MBS to the target NG-RAN not supporting 5MBS, RAN3 think that Minimization of data loss should be targeted for this scenario; by which means is still up for discussions in RAN.

    - No assistance information from the CN is foreseen at the moment.

    - Xn mobility requires the associated QoS flow information to be available before mobility to a non-MBS supporting gNB takes place. RAN3 has already agreed that the reference to the MBS Session which the UE has joined and, if applicable, the associated QoS flows, are included in a PDU Session Resources Item and maintained within the NG-RAN UE Context. The associated QoS flow information should, if applicable, be provided as early as possible, preferably at Joining.

    - No SYNC protocol is supported in release 17.

    - Baseline CRs endorsed.

- Necessary Enhancements to NG-RAN Architecture

  - General Architecture

    - One TP R3-211147 agreed to 38.300 BL CR to introduce the QoS model related aspects.

  - Session Management over NG

    - Session Management over NGNG functions support all NG-RAN signaling resulting from NAS Session Joining: i.e. joining during an activated MBS Session and joining during a deactivated MBS Session, joining while served by a RAN supporting 5MBS and joining while served by a RAN not supporting 5MBS.

    - Support of all activation scenarios, i.e. for UEs in all CM/RRC states and served by both, MBS supporting and MBS non-supporting RAN with UEs having Registration Areas covering supporting/non supporting RAN nodes; whether non-supporting RAN nodes should receive this information is FFS.

    - The reference to the MBS Session which the UE has joined and, if applicable, the associated QoS flows, are included in a PDU Session Resources Item and maintained within the NG-RAN UE Context. The associated QoS flow information should, if applicable, be provided as early as possible, preferably at Joining.

    - When an MBS session is (re-)activated, group paging may be used toward supporting nodes (to be checked against RAN2 progress)

    - Support 5GC triggered MBS Session Stop/Deactivation (pending SA2 progress)

    - The following NGAP procedures are impacted for control of MBS Session related content of a PDU Session within the UE Context data in RAN: PDU Session Resource Setup, PDU Session Resource Modify.

    - The following UE associated NGAP functions are impacted for mobility reasons for control of MBS Session related content of a PDU Session within the UE Context data in RAN: Handover Resource Allocation, Path Switch Request.

    - The following protocol principles for interworking with non-supporting nodes are proposed:

      - NGAP Session Management functions defined for joining/leaving should be defined in a way that they work in a backward compatible way with non-supporting RAN nodes

      - MBS additions to PDU Session Resource procedures should have criticality “ignore”

      - we should have explicit NG-RAN reply in PDU Session Resource SMF containers to inform the SMF whether MBS is supported

    - MBS additions in PDU Session Resource procedure should ensure for active MBS Sessions the setup of individual resources in non-supporting nodes and setup/use of shared resources in supporting nodes with the same unique protocol means.It is proposed to include basic MBS Session related information (at least MBS Session ID, associated QoS flows) in the NGAP SMF transparent containers in the PDU Session Resource messages, where appropriate.

    - An (associated) PDU Session may be associated with more than one MBS Session.

  - Dynamic Change Between PTP and PTM for UEs in RRC_CONNECTED State.

    - Summary of the email discussion provided in R3-211028, noted, not treated online due to limit of time.

  - Bearer Management over F1/E1

    - Summary of the email discussion provided in R3-211029, noted, not treated online due to limit of time.

  - Others

    - Summary of the email discussion provided in R3-211030, noted, not treated online due to limit of time.

- Mobility with Service Continuity for UEs in RRC_CONNECTED State

  - Mobility Between MBS Supporting Nodes

    - Summary of the email discussion provided in R3-211031, noted, not treated online due to limit of time.

  - Mobility Between MBS Supporting and non-MBS Supporting Nodes

    - Summary of the email discussion provided in R3-211032, noted, not treated online due to limit of time.

  - Others

    - Summary of the email discussion provided in R3-211033, noted, not treated online due to limit of time.

#### 2.3.2	Remaining Open issues

- Session management over NG

- MBS Bearer management over F1 and E1

- Mobility with Service Continuity for UEs in RRC_CONNECTED State

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#104-E contributions:

- R1-2101062	Updated NR MBS work plan	CMCC

- R1-2100048	Discussion on PDCCH aspects for group scheduling	FUTUREWEI

- R1-2100106	Discussion on mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2100144	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2100189	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2100354	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2100469	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2100510	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

- R1-2100613	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2100674	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

- R1-2100698	Views on group scheduling for NR MBS	Google Inc.

- R1-2100768	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2100805	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2100872	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2100906	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2100956	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	ETRI

- R1-2101063	Discussion on group scheduling mechanisms	CMCC

- R1-2101234	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2101359	Discussion on group scheduling mechanism for RRC_connected UEs	Apple

- R1-2101424	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

- R1-2101487	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2101579	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2101658	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

- R1-2101726	Mechanisms to support group scheduling for RRC_CONNECTED Ues	Ericsson

- R1-2101767	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101854	Summary#2 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101867	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101875	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101907	Summary#5 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101937	Summary#6 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2101974	Summary#7 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2102032	Summary#8 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2102099	Summary#9 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2102171	Summary#10 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2100049	Discussion on improving reliability for RRC_CONNECTED UEs	FUTUREWEI

- R1-2100107	Discussion on mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

- R1-2100145	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

- R1-2100190	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2100355	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2100470	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

- R1-2100511	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2100557	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio Company Limited

- R1-2100614	Discussion on HARQ operation for NR MBS reliable transmission	MediaTek Inc.

- R1-2100675	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

- R1-2100699	Views on retransmission procedure for NR MBS	Google Inc.

- R1-2100769	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

- R1-2100806	Mechanisms to improve reliability for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2100907	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

- R1-2100957	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	ETRI

- R1-2101064	Discussion on reliability improvement	CMCC

- R1-2101235	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

- R1-2101360	Discussion on MBS reliability improvement for RRC_connected UEs	Apple

- R1-2101425	On reliability enhancement for NR multicast and broadcast	Convida Wireless

- R1-2101488	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2101637	Study on the reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2101727	Discussion on reliability mechanisms for NR MBS	Ericsson

- R1-2101837	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2101906	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2101987	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2102082	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2102134	FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2100108	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

- R1-2100146	Discussion on support for IDLE and INACTIVE state UEs	OPPO

- R1-2100191	Discussion on multicast support for IDLE/INACTIVE UEs	Huawei, HiSilicon

- R1-2100356	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			CATT, CBN

- R1-2100471	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues			vivo

- R1-2100512	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

- R1-2100615	Common frequency resource for NR PTM transmission	MediaTek Inc.

- R1-2100676	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

- R1-2100770	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

- R1-2100873	Considerations on MBS functions for RRC_IDLE UEs	Sony

- R1-2100908	Basic function for broadcast/multicast	LG Electronics

- R1-2101065	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

- R1-2101236	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

- R1-2101361	Discussion on MBS for RRC_IDLE/RRC_INACTIVE  UEs	Apple

- R1-2101426	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R1-2101489	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

- R1-2101638	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

- R1-2101721	Feature lead summary on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2101876	Feature lead summary#2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2101877	Feature lead summary#3 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2102067	Feature lead summary#4 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2102068	Feature lead summary#5 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2102180	Feature lead summary#6 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2100109	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

- R1-2100357	Discussion on other issues for MBS	CATT

- R1-2100472	Other issues for Rel-17 MBS	vivo

- R1-2100909	Other aspects for MBS	LG Electronics

- R1-2101639	Effects of NR MBS on PDSCH and PDCCH	CHENGDU TD TECH LTD.

- R1-2101729	Assumptions for Performance Evaluations of NR-MBS	Ericsson

- R1-2101730	LTE SC-PTM design as a reference for NR MBS in IDLE/INACTIVE	Huawei, HiSilicon

RAN2#113-e contributions:

- R2-2100032	Response LS on RAN impact of FS_5MBS Study (R3-207059; contact: Nokia)

- R2-2100071	LS on 5MBS progress and issues to address (S2- 2009235; contact: Huawei)

- R2-2100082	Discussion on Requirement and Architecture of MBS

- R2-2100083	Reliability Improvement for PTM Transmission

- R2-2100084	Open Issues on Dynamic PTM and PTP Switch

- R2-2100085	Open Issues on Mobility with Service Continuity

- R2-2100086	Discussion on Group Scheduling

- R2-2100087	Open Issues on MBS Reception for Idle and Inactive UEs

- R2-2100130	RRC state control for MBS reception

- R2-2100131	Discussion on reliability for MBS reception

- R2-2100132	Discussion on group based scheduling for MBS

- R2-2100133	Discussion on mobility with MBS Service continuity

- R2-2100134	Discussion on MBS interesting indication and service continuity for delivery mode 2

- R2-2100135	Discussion on beam sweeping transmission for delivery mode 2

- R2-2100172	HARQ operation to improve reliability for PTM transmission

- R2-2100173	Dynamic PTM-PTP switch

- R2-2100174	L2 structure for NR MBS transmission

- R2-2100175	Common frequency resource for NR PTM transmission

- R2-2100176	PTM scheduling for NR MBS

- R2-2100177	Email Report of [Post112-e][069][MBS] Delivery mode 2

- R2-2100318	NR Multicast and Broadcast Radio Bearer Architecture aspects

- R2-2100319	NR Multicast PTM bearer RLC AM mode operation

- R2-2100320	NR Multicast-Broadcast services and configuration for UEs in different RRC states

- R2-2100321	Enhancements for supporting loss less switch between PTM and PTP RLC legs

- R2-2100322	Email discussion report for [Post112-e][071][MBS] UP Performance

- R2-2100353	MBS Protocol Architecture and Logical Channel Aggregation

- R2-2100354	L2 Retransmission (PDCP vs. RLC) for MBS

- R2-2100355	ARQ of PTM with Logical Channel Aggregation

- R2-2100356	Service Continuity during Dynamic PTM/PTP Switch with Logical Channel Aggregation

- R2-2100361	MBS MAC layer and group scheduling aspects

- R2-2100370	PDCP Operation for MBS

- R2-2100371	Miscellaneous Aspects of MBS

- R2-2100372	Handling of Measurement Gaps

- R2-2100414	NR Multicast Broadcast mobility enhancements with service continuity

- R2-2100435	Considerations on Group Scheduling and Multiplexing Aspects

- R2-2100450	Mobility with Service Continuity

- R2-2100451	NR MBS in Idle/Inactive mode

- R2-2100505	Consideration on Group Scheduling Aspects

- R2-2100506	Consideration on dynamic switch between PTP and PTM

- R2-2100630	Handling MBS during a CHO

- R2-2100631	Discussion on NR MBS solutions of mode 2 delivery

- R2-2100643	MBS split bearer configuration and PTP/PTM switching

- R2-2100644	MBS Mobility with Service Continuity

- R2-2100675	Discussion on MBS session delivery mode

- R2-2100676	Discussion on reliability of MBS transmission

- R2-2100677	Discussion on dynamic PTM PTP switch

- R2-2100678	Discussion on service continuity during mobility

- R2-2100679	MBS session in Idle and Inactive mode

- R2-2100709	DL PDCP SN alignment issue

- R2-2100760	Support of dynamic switch between PTP and PTM

- R2-2100761	Bearers for MBS Transmission

- R2-2100803	Further consideration of control plane aspects for NR MBS

- R2-2100825	The counting scheme for dynamically switching PTM and PTP

- R2-2100832	Reliability for MBS Service

- R2-2100833	Dynamic PTM PTP switch for RRC Connected UE

- R2-2100834	Lossless Handover for MBS

- R2-2100835	MBS Service Continuity for RRC Connected UE

- R2-2100836	Group Scheduling for MBS

- R2-2100837	MBS in Idle and Inactive Mode

- R2-2100898	Security keys considering PTP/PTM switch for delivery mode 1/2

- R2-2100899	Standalone MRB for delivery mode 1 and RLM

- R2-2100937	Discussion on L2 User Plane for NR MBS

- R2-2100940	Discussion on L2 user plane reliability for NR MBS

- R2-2100942	Dynamic switch between PTM and PTP with service continuity

- R2-2100944	Discussion on mobility with service continuity

- R2-2100958	RAN2 related aspects for NR MBS

- R2-2100960	Control plane for delivery mode 2 for NR MBS

- R2-2100963	Simultaneous MBS and Unicast Operation in Idle/inactive Mode

- R2-2100988	Dynamic PTM PTP switching

- R2-2100989	MBS configuration for RRC_CONNECTED

- R2-2100990	MBS in IDLEINACTIVE

- R2-2100991	Mobility with service continuity

- R2-2101006	Layer-2 Structure for MBS

- R2-2101007	MBS Radio Bearer (MRB) Type

- R2-2101008	RLC AM for PTM

- R2-2101010	Updated NR MBS workplan

- R2-2101011	Reliability enhancement for NR MBS

- R2-2101012	Support of dynamic switch between PTP and PTM

- R2-2101013	High layer aspects for group scheduling

- R2-2101049	Complexity analysis for reliability enhancement in RLC and PDCP

- R2-2101050	MBS service continuity in mobility

- R2-2101051	MBS L2 Architecture, Control Plane and SA2 LS Discussion

- R2-2101060	Considerations on measurements for NR MBS in idle/inactive

- R2-2101080	MBS Idle

- R2-2101120	Issues on MBS reliability

- R2-2101139	MBS L2 architecture

- R2-2101140	Connected Mode Mobility with Service Continuity

- R2-2101141	Discussion on MBS delivery modes

- R2-2101143	MBS dynamic switch between PTP and PTM with service continuity

- R2-2101144	HO for NR MBS

- R2-2101171	Mobility for NR MBS

- R2-2101172	Reliability and Dynamic Switch for MBS

- R2-2101173	Aspects of Group Sscheduling

- R2-2101185	Discussion on the SA2 LS and the reply LS

- R2-2101186	On the general aspects for delivery mode 1 and 2

- R2-2101187	Service continuity during inter-cell mobility

- R2-2101188	MBS configuration for delivery mode 2

- R2-2101215	General aspects of NR MBS

- R2-2101216	Consideration on MBS reliability guarantee

- R2-2101217	Mode switching for NR MBS

- R2-2101218	Lossless handover support for NR MBS

- R2-2101219	Group scheduling for NR MBS

- R2-2101220	Delivery mode 2 for NR MBS

- R2-2101316	MBS Reliability

- R2-2101317	PTM/PTP mode switching

- R2-2101372	Consideration on the MBS transmission reliability

- R2-2101373	Dynamic PTM PTP switch with service continuity

- R2-2101374	Mobility with service continuity

- R2-2101375	MBS reception in CONNECTED state

- R2-2101376	MBS reception in IDLE/INACTIVE state

- R2-2101495	NR MBS Configuration Information

- R2-2101594	PTM configuration for NR MBS

- R2-2101605	Dynamic PTM/PTP Switching

- R2-2101606	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs

- R2-2101625	Discussion on L2 architecture

- R2-2101626	Discussion on MBS Reliability issues

- R2-2101627	Discussion on Dynamic PTP and PTM switch

- R2-2101628	Discussion on Mobility with service Continuity

- R2-2101629	Discussion on delivery mode 2

- R2-2101649	On reliability enhancement for NR multicast and broadcast

- R2-2101677	PDCP retransmission for PTM

- R2-2101678	Packet loss at the PDCP reestablishment of RLC UM

- R2-2101679	Support of DAPS handover for PTM MBS

- R2-2101680	Slow-moving PDCP reception window issue

- R2-2101681	Combination of service continuity and counting for delivery mode 2

- R2-2101682	Beam association for MCCH and MCCH change notification

- R2-2101718	38.300 Running CR for MBS in NR

- R2-2101719	Discussion on SA2 LS on 5MBS Progress and Issues to address

- R2-2101720	Reply LS to SA2 on 5MBS Progress and Issues to address

- R2-2101730	Discussion on L2 architecture

- R2-2101735	Data inactivity during MBS reception

- R2-2101736	MBS and Idle and Inactive mode UEs

- R2-2101737	Multicast in Idle and Inactive

- R2-2101758	Dynamic switch between PTM and PTP for service continuity

- R2-2101759	MBS support for delivery mode 2

- R2-2101860	Discussion on overall architecture of MBS traffic delivery

- R2-2101861	Discussion on reliability improvement and UL feedback in NR multicast

- R2-2101892	Discussion on MBS Control Information Configuration

- R2-2101903	L2 architecture for delivery mode 2

- R2-2102249	Summary for MBS Group Scheduling under Agenda Item 8.1.2.4

- R2-2102253	38.300 Running CR for MBS in NR

- R2-2102307	Report of [Offline-037][MBS] MBS General (Huawei)

- R2-2102313	[Offline-038][MBS] UP Architecture Decisions

- R2-2102463	38.300 Running CR for MBS in NR

- R2-2102480	Reply LS on 5MBS progress and issues to address

- R2-2102482	Report of [Offline-037][MBS] MBS General (Huawei)

RAN3#111-e contributions:

- R3-210003	Introduction of MBS(BL CR for 38.463)	CATT

- R3-210005	Introduction of NR MBS	Lenovo, Motorola Mobility

- R3-210016	MBS BL CR for TS38.410	ZTE

- R3-210017	Introduction of NR MBS	Huawei, CMCC

- R3-210036	Reply LS on RAN impact of FS_5MBS Study	RAN WG2

- R3-210039	LS on 5MBS progress and issues to address	SA WG2

- R3-210153	RAN3 feedback on MBS issues to address in SA2 TR conclusions	Nokia, Nokia Shanghai Bell

- R3-210154	Reply LS on 5MBS progress and issues to address	Nokia, Nokia Shanghai Bell

- R3-210163	(TP for 38.300) MBS Session Deactivation	Nokia, Nokia Shanghai Bell

- R3-210164	MBS Session Activation	Nokia, Nokia Shanghai Bell

- R3-210165	(TP for 38.300) Stage 2 for MBS Session Management	Nokia, Nokia Shanghai Bell

- R3-210166	MBS Session Management: MBS Context and UE MBS Context setup (stage 3)	Nokia, Nokia Shanghai Bell

- R3-210167	(TP for 38.401) MBS F1, E1 Bearer Management	Nokia, Nokia Shanghai Bell

- R3-210168	(TP for 38.300) MBS Stage 2 for Mobility Management	Nokia, Nokia Shanghai Bell

- R3-210169	MBS Mobility User Plane Setup (stage 3)	Nokia, Nokia Shanghai Bell

- R3-210170	MBS Mobility Control Plane (stage 3)	Nokia, Nokia Shanghai Bell

- R3-210171	Support of MBS Seamless Mobility	Nokia, Nokia Shanghai Bell

- R3-210172	Introduction of NR MBS	Nokia, Nokia Shanghai Bell

- R3-210173	(TP for 38.300) Mobility from MBS supporting to non-MBS supporting nodes	Nokia, Nokia Shanghai Bell

- R3-210223	Introduction of NR MBS	Samsung

- R3-210224	TP for MBS BLCR for 38.413-Session management for broadcasting over NG	Samsung

- R3-210225	TP for MBS BL CR for 38.463-Bearer management for broadcasting over E1	Samsung

- R3-210226	TP for MBS BL CR for 38.473-Bearer management for broadcasting over F1	Samsung

- R3-210227	Data forwarding for mobility between MBS supporting nodes	Samsung

- R3-210228	Coordinated assignment of PDCP SN	Samsung

- R3-210234	TP for 38.413 on session management for broadcast service	CATT,CBN

- R3-210270	Updated NR MBS work plan	Huawei,CMCC

- R3-210271	Consideration of SA2 LS on 5MBS progress and issues to address	Huawei

- R3-210273	(TP to TS 38.410 BL CR and TS 38.413 BL CR) Multicast Session Management over NG	Huawei

- R3-210274	(TP to MBS TS38.410 BL CR) Support of Shared NG-U Transport	Huawei

- R3-210275	(TP to MBS TS38.413 BL CR) Support of Shared NG-U Transport	Huawei, CMCC

- R3-210276	(TP to TS 38.401 BL CR) Decision on PTP and PTM	Huawei

- R3-210277	(TP to TS 38.401 BL CR) Bearer management over F1 and E1	Huawei

- R3-210278	(TP to TS 38.470 and 38.460 BL CRs) Bearer management over F1 and E1 interfaces	Huawei

- R3-210279	(TP to TS 38.300 BL CR) Mobility between MBS supporting nodes	Huawei, CMCC

- R3-210280	(TP to TS 38.401 BL CR) Mobility Between MBS Supporting Nodes	Huawei

- R3-210281	(TP to TS38.300 BL CR) Consideration on DL PDCP SN synchronization	Huawei, CMCC

- R3-210282	Support of NR MBS data transmission	Huawei

- R3-210283	(TP to TS38.300 BL CR) Data forwarding between MBS supporting nodes	Huawei

- R3-210284	(TP to TS38.300 BL CR) Mobility between MBS supporting and non-supporting nodes	Huawei

- R3-210285	(TP to TS 38.300 BL CR) Service reception continuity for broadcast service	Huawei

- R3-210312	Discussion on dynamic change between PTM and PTP	CATT

- R3-210313	Discussion on multicast context establishment during Xn-based handover	CATT

- R3-210314	Possible solutions to minimise data loss	CATT

- R3-210315	TP on TS 38.300 on MBS service continuity	CATT

- R3-210316	Service continuity from non-MBS gNB toward MBS gNB	CATT

- R3-210317	Service continuity from MBS gNB toward non-MBS gNB	CATT

- R3-210367	BL CR for NR MBS for 38.413	Qualcomm Incorporated

- R3-210368	NR Multicast Session Management Procedure	Qualcomm Incorporated

- R3-210369	Data forwarding in handover with NR multicast	Qualcomm Incorporated

- R3-210381	Discussion of MSB LS from SA2(S2-2009235)	Intel Deutschland GmbH

- R3-210383	Session Management Discussion	Intel Deutschland GmbH

- R3-210461	Further Consideration on MBS context management over F1 and E1	CATT

- R3-210462	Further Consideration on MBS Transmission Area	CATT

- R3-210463	TP for 38.463 on  MBS Context Management at E1 interface	CATT

- R3-210464	MBS reception of Idle and In-active Ues	CATT

- R3-210465	Some Considerations on MBS Architecture	CATT

- R3-210512	Discussion on NR MBS architecture	CHENGDU TD TECH LTD.

- R3-210513	Dynamic switch between PTM and PTP for RRC_CONNECTED UE	CHENGDU TD TECH LTD.

- R3-210514	Mobility between MBS supporting nodes	CHENGDU TD TECH LTD.

- R3-210515	Mobility between an MBS supporting node and a NON-MBS supporting node	CHENGDU TD TECH LTD.

- R3-210517	PDCP SN Synchronization	vivo

- R3-210533	Dynamic Change Between PTP and PTM	BEIJING SAMSUNG TELECOM R&D

- R3-210534	Issues on Mobility between MBS Supporting Nodes	LG Electronics

- R3-210602	Considerations on MBS session management and TP to 38.410 38.413 BL CR	ZTE

- R3-210603	TP to 38.414 BL CR on IP multicast	ZTE

- R3-210604	Mode switching for NR MBS	ZTE

- R3-210605	TP to 38.300 BL CR on mode change between PTP and PTM	ZTE

- R3-210606	Considerations on MBS bearer management over F1 and E1 and TPs to 38.470 38.473 38.460 38.463 BL CR	ZTE

- R3-210607	TP to 38.401 38.474 MBS BL CR on supporting IP multicast over F1-U	ZTE

- R3-210608	Considerations on inter-gNB mobility for MBS	ZTE

- R3-210609	Considerations on lossless handover for NR MBS	ZTE

- R3-210610	Discussion on MBS session activation and deactivation	ZTE

- R3-210618	(TP for BL CR 38.401) Dynamic switch between PTP and PTM	Lenovo, Motorola Mobility

- R3-210619	Open Issues on MBS Bearer Management over F1/E1	Lenovo, Motorola Mobility

- R3-210620	Enhancements to support loss-less handover for NR multicast	Lenovo, Motorola Mobility

- R3-210621	(TP for BL CR 38.401) MBS Transmission Area Control between gNB-CU and gNB-DU	Lenovo, Motorola Mobility

- R3-210622	User Plane Protocol Aspects for NR MBS	Lenovo, Motorola Mobility

- R3-210623	Considerations on measurements for NR MBS in idle/inactive	Lenovo, Motorola Mobility

- R3-210632	Discussion on SA2 LS on 5MBS progress and issues to address	Ericsson

- R3-210633	[DRAFT] Reply LS on 5MBS progress and issues to address	Ericsson

- R3-210639	Session Management over NG	Ericsson

- R3-210640	[TP for BL CR TS 38.300] TP on Session Management for NR MBS	Ericsson

- R3-210641	[TP for BL CR TS 38.410] TP on Session Management for NR MBS	Ericsson

- R3-210642	[TP for BL CR TS 38.413] TP on Session Management for NR MBS	Ericsson

- R3-210643	On Support of mobility between gNBs supporting MBS	Ericsson

- R3-210644	[TP for BL CR TS 38.300] HO between supporting gNBs	Ericsson

- R3-210645	[TP for BL CR 38.410 and 38.413] HO between MBS supporting gNBs	Ericsson

- R3-210646	[TP for BL CR TS 38.423] HO between supporting gNBs	Ericsson

- R3-210647	[TP for BL CR TS 38.401] HO between supporting gNBs	Ericsson

- R3-210648	On support of mobility between MBS supporting and non-MBS supporting gNBs	Ericsson

- R3-210760	LS on MBS service continuity	CATT

- R3-210782	Issues on dynamic change between PTP and PTM	LG Electronics

- R3-210787	Introduction of NR MBS	LG Electronics

- R3-210913	Discussion on dynamic change between PTP and PTM	CMCC

- R3-210914	Discussion on MBS Bearer Management	CMCC

- R3-210915	Discussion on Mobility with Service Continuity	CMCC

- R3-210916	TP to 38.300 BLCR for mobility between MBS supporting nodes	CMCC

- R3-210920	MBS Session management over NG	CMCC

- R3-210921	(TP to TS 38.410 ) MBS session management over NG	CMCC

- R3-210922	(TP to TS 38.300 ) MBS session management over NG	CMCC, Huawei

- R3-210923	Discussion on SA2 LS on on 5MBS progress and issues to address	CMCC

- R3-210924	[Draft]Reply LS to SA2 on 5MBS Progress and Issues to address	CMCC

- R3-211026	CB: # 70_MBS_Architecture - Summary of email discussion	Nokia - moderator

- R3-211027	CB: # 71_MBS_NGsessionMgmt - Summary of email discussion	Ericsson - moderator

- R3-211028	CB: # 72_MBS_PTP-PTM_DynChg - Summary of email discussion	Samsung - moderator

- R3-211029	CB: # 73_MBS_BearerMgmt_F1_E1 - Summary of email discussion	CATT - moderator

- R3-211030	CB: # 74_MBS_Txarea - Summary of email discussion	Lenovo - moderator

- R3-211031	CB: # 75_MBS_Mobility_Supporting - Summary of email discussion	Samsung - moderator

- R3-211032	CB: # 76_MBS_Mobility_Non-supporting - Summary of email discussion	Huawei - moderator

- R3-211033	CB: # 77_MBS_SvcCont_InactiveUEs - Summary of email discussion	CATT - moderator

- R3-211034	CB: # 69_MBS_BLs - Summary of email discussion	CMCC - moderator

- R3-211077	Introduction of NR Multicast and Broadcast Services	Ericsson

- R3-211146	CB: # 68_RANimpact_of_5MBSstudy - Summary of email discussion	Huawei - moderator

- R3-211147	(TP for 38.300) QoS Model and MBS Session AMBR Enforcement	Nokia, Nokia Shanghai Bell

- R3-211296	Reply LS on 5MBS progress and issues to address	Huawei

- R3-211351	Introduction of NR MBS	Nokia, Nokia Shanghai Bell

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template