3GPP RAN TSG Meeting #91e		RP-210631

Electronic meeting, 16 – 26th March 2021

Agenda item:	9.6.1

Source:	Apple Inc.

Title:	Regulatory update for the 6GHz frequency range

WI/SI:	FS_6GHz_LTE_NR

Release:	Rel-17

Document for:	Approval

# 1	Introduction

Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations is the RAN level study item, which aims at capturing the latest information and status of the regulatory decisions for the 6GHz frequency range. In this discussion paper we present a text proposal for the corresponding TR 37.890 that captures new regulatory decisions in the ITU region 3.

# 2	Text proposal

-------------------------------------------------- TP BEGIN --------------------------------------------------

## 4.3	ITU Region 3

### 4.3.1	China

In April 2019, CCSA established a new project on technical report for the feasibility study of IMT system using 5925-7125 MHz frequency band [29].

This CCSA study includes identifying IMT parameters in 5925-7125MHz in order to develop coexistence between IMT system and incumbent services and applications.

Editor’s note: This work is expected to be finalized in 2020.

### 4.3.2	South Korea

In June 2020, Korea’s Ministry of Science and ICT (MSICT) issued an amendment of technical standards that also includes proposed rules for the 5925–7125MHz frequency band, which was finally approved in October 2020. The key points are as follows:

-	The whole range of 5925–7125MHz is allocated for the license-exempt low power indoor:

-	The occupied channel bandwidth should be less or equal 160MHz.

-	The EIRP limit is 24dBm.

-	The maximum power spectral density is 2dBm/MHz.

-	The 5925-6425MHz frequency range is allocated for the license-exempt very low power indoor and outdoor:

-	The occupied channel bandwidth should be less or equal 160MHz.

-	The EIRP limit is 14dBm.

-	The maximum power spectral density is 1dBm/MHz, -2dBm/MHz, -5dBm/MHz, -8dBm/MHz for the occupied channel bandwidth of 0.5 <= 20MHz, 20 <= 40MHz, 40 <= 80MHz, and 80 <= 160MHz respectively.

-	Unwanted emissions at frequencies outside the operational frequency range shall be below -27dBm/MHz.

-------------------------------------------------- TP END --------------------------------------------------