3GPP TSG RAN Meeting #90 	RP-210645

Electronic Meeting, December 7 - 11, 2020 	(revision of RP-202689)

{Guidance text shown in curly brackets, in italics. All guidance text is to be deleted before WID submission.

Note that this form is to be used for all types of Work Item, covering both normative work and/or studies. A Work Item covering only studies is also known as "Study Item", as stated in TR 21.900.}

# Source:	MediaTek Inc.

# Title:	New Study WID on NB-IoT/eTMC support for NTN

# Document for:	Approval

Agenda Item:	10.6.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on NB-IoT/eMTC support for Non-Terrestrial Networks

## Acronym: FS_LTE_NBIOT_eMTC_NTN

## Unique identifier: 860033

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No |  |  |  | X | X |
| Don't know | X |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 Work Item is presented or e.g. the related Study Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. List previous Work Items of earlier releases if relevant.}

{This section is mandatory to be filled out by the rapporteur.}

{Not applicable for a Study Item}

{For a Feature: list here the parent Study Item if applicable}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 750040 | Study on NR to support non-terrestrial networks | The proposed study will make use of the channel model defined by the FS_NR_nonterr_nw study. |
| 800099 | Study on solutions for NR to support non-terrestrial networks (NTN) | The proposed study will leverage solutions based on the study FS_NR_NTN_solutions that addressed key impact areas |
| 7770002 | Study on using Satellite Access in 5G (FS_5GSAT) | Feasibility Study led by SA1 and completed in SA#80 |
| 800026 | Study on architecture aspects for using satellite access in 5G (FS_5GSAT_ARCH) | Feasibility Study led by SA2. |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

IoT operation is critical in remote areas with low/no cellular connectivity for many different industries, including e.g.:

-	Transportation (maritime, road, rail, air) & logistics

-	Solar, oil & gas harvesting

-	Utilities

-	Farming

-	Environment monitoring

-	Mining etc.

The capabilities of NB-IoT and eMTC are a good fit to the above, but will require satellite connectivity to provide coverage beyond terrestrial deployments, where IoT connectivity is required. There is an urgent need for a standardized solution allowing global IoT operation anywhere on Earth, in view of other solutions already available.

It is important that satellite NB-IoT or eMTC be defined in a complementary manner to terrestrial deployments.

This study intends to reuse the NR NTN study and conclusions in TR 38.821.

## 4	Objective

This Study will evaluate and confirm solutions to address the minimum necessary specifications for NB-IoT and eMTC according to the following objectives.

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items:

-	Aspects related to random access procedure/signals [RAN1, RAN2]

-	Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

-	Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

-    Aspects related to HARQ operation [RAN2, RAN1]

-	General aspects related to timers (e.g. SR, DRX, etc.) [RAN2]

-	RAN2 aspects related to idle mode and connected mode mobility [RAN2]

-	RLF-based for NB-IoT

-	Handover-based for eMTC

-	System information enhancements [RAN2]

-	Tracking area enhancements [RAN2]

NOTE 3: 	GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

Recommendations for NB-IoT and recommendations for eMTC will be documented in the conclusions.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| 36.763 | TR | Study on NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) |  | RAN#94 | Gilles Charbit, MediaTek Inc. |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Gilles Charbit, MediaTek Inc., gilles dot charbit at mediatek dot com (RAN1)

René Faurie, Eutelsat, rfaurie-ls at sfr dot fr (RAN2)

## 7	Work item leadership

Lead: RAN1

Secondary: RAN2

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

MediaTek Inc.

Asia Pacific Telecom

BlackBerry

CATT

China Unicom

Ericsson

ESA

ETRI

Eutelsat

GLOBALSTAR Inc.

HiSilicon

Huawei

HUGHES Network Systems

III

Intelsat

InterDigital

Inmarsat

ITRI

KT Corp.

KT SAT

Lenovo

Ligado Networks

Motorola Mobility

Novamint

NTPU

Qualcomm

Sequans

Sateliot

Sierra Wireless

Sony

THALES