3GPP TSG RAN meeting #91e										RP-210678

Electronic Meeting, March 16-26, 2021

## Status Report to TSG

Agenda item: 9.2.3

| WI / SI Name | NR positioning support |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_POS |  |  |  |  |
| Unique ID | 830077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191156 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2020 | Performance part: 06/2021 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 75% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  |  |
| --- | --- | --- |
| Rapporteur | Name | Alexey Khoryaev |
|  | Company | Intel Corporation |
|  | Email | alexey.khoryaev@intel.com |
| Rapporteur | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

RAN WG1 completed work in December 2019.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN WG2 completed work in June 2020.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

RAN WG3 completed work in September 2020.

## 2.4	RAN4

#### 2.4.1	Agreements

During RAN4 #98e meeting, the group continued discussion on RRM performance requirements for NR positioning that was divided into UE and gNB parts in these two e-mail threads:

- [98e][214] NR_pos_RRM_Part_2

- [98e][215] NR_pos_RRM_Part_3

UE RRM performance requirements

The following agreements on UE RRM performance requirements were made:

- WF on UE RRM performance requirements was approved [R4-2103586]

- Accuracy requirements for RSTD measurement

  - Antenna panel assumption:

    - RAN4 not to define separate accuracy requirements for RSTD measured with same panel and with different panels and

    - The same accuracy requirements will be defined, which are based on the earlier simulation results (which did not assume different panels, according to the agreed simulation assumptions)

    - Possible margin for different group delay calibration error in different panels can be considered

  - RSTD accuracy requirements are defined to cover all BW configurations

  - Different RSTD accuracy requirements depend on the following parameters

    - PRS BW defined in number of PRBs

    - Companies are encouraged to bring PRS accuracy simulation results in RAN4 #99e for the identified set of parameters

    - PRS BW defined in number of PRBs

    - PRS SCS

    - dl-PRS-ResourceRepetitionFactor

    - dl-PRS-NumSymbols

    - dl-PRS-CombSizeN

    - Other parameters are FFS

    - Note: the full set of parameters and values were included in R4-2104046 or its revision  , which will be also used as the template used to collect the simulation results also.

  - In the next meeting, the exact accuracy requirements can be defined from the simulation results based on the parameter sets listed in R4-2104046 or its revision.

    - The simulation results can still be updated in the next meeting

    - Depending on the final required accuracy numbers, not all parameters from R4-2104046 need to be present in the final accuracy requirements

- PRS RSRP measurement accuracy requirements

  - PRS-RSRP SINR side condition #1 is [-3] dB. Companies are encouraged to bring simulation results for -3dB and -6dB SINR conditions.

  - The full set of parameters and values used for PRS RSRP simulations were included in R4-21xxxxx.

  - In the next meeting, the exact accuracy requirements can be defined from the simulation results, based on the parameter sets listed in R4-2104046

    - The simulation results can still be updated in the next meeting

    - Depending on the final required accuracy numbers, not all parameters from R4-2104046 need to be present in the final accuracy requirements

- UE Rx-Tx time difference accuracy requirements

  - SINR side conditions for #1 is [-3dB]

  - Applicable propagation channel for accuracy requirement: Follow the same principle of RSTD accuracy requirements

  - The full set of parameters and values used for UR Rx-Tx time difference simulation were included in R4-2104046 or its revision .

  - In the next meeting, the exact accuracy requirements can be defined from the simulation results based on the parameter sets listed in R4-2104046 or its revision

    - The simulation results can still be updated in the next meeting

    - Depending on the final required accuracy numbers, not all parameters from R4-2104046 need to be present in the final accuracy requirements

- Test case design

  - No additional path reporting is configured (additionalPaths is not included in the measurement request)

  - Joint configuration of PRS-RSRP with RSTD and UE Rx-Tx

gNB performance requirements

The following agreements on gNB RRM performance requirements were made:

- WF on gNB positioning measurement requirements was approved [R4-2103587].

- UL RTOA measurement accuracy requirements

  - Further identify how to derive reference time in the ideal UL-RTOA (e.g. whether it is determined by gNB’s own timing based on LMF configuration).

- Side conditions for gNB measurement accuracy

  - Accuracy requirements for gNB measurements shall be defined for the following Ês/Iot values:

    - Ês/Iot1 [dB] (High Ês/Iot) = 3 dB for FR1 and FR2

    - Ês/Iot2 [dB] (Low Ês/Iot) = -13 dB for FR1 and FR2.

  - The gNB, which declares the support of positioning measurements, needs to declare the support of measurements for at least either high Ês/Iot or low Ês/Iot side conditions.

- Beam sweeping during gNB measurement

  - gNB accuracy requirements do not mandate gNB RX beam sweeping

- SRS configurations for gNB measurement accuracy

  - gNB accuracy requirements shall be defined for group of SRS BWs

    - grouping of SRS BWs will be decided based on link simulation results

- Factors impacting SRS RSRP measurement accuracy for different gNB types

  - Baseline SRS-RSRP measurement accuracy without margin is based on link simulation results

  - RF calibration error for SRS-RSRP measurement for gNB type 1-C (X) is smaller than that for gNB types 1-O/2-O (Y) i.e. Y>X.

  - Implementation and RF margins are specific to SRS-RSRP. Values are FFS.

- Factors impacting gNB Rx-Tx measurement accuracy for different gNB types

  - Baseline timing measurement accuracy without margin is based on link simulation results

  - RF calibration error for timing measurements depends on gNB type (1-C, 1-H, 1-O, 2-O).

  - Implementation and RF margins are are FFS:

- Applicable propagation channel for gNB Rx-Tx accuracy

  - gNB Rx-Tx measurement accuracy requirements apply in AWGN

The following CRs for NR Positioning performance requirements in Rel.16 were endorsed:

- R4-2103585	Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases

- R4-2103730	draftCR to introduce accuracy requirements for RSTD measurement

- R4-2103732	CR on PRS-RSRP accuracy requirements

- R4-2103731	UE Rx-Tx measurement accuracy requirements

- R4-2104052	gNB SRS-RSRP measurement

- R4-2104053	draftCR to introduce gNB Rx-Tx time difference requirements

#### 2.4.2	Remaining Open issues

Further work is needed on performance part for the following items:

- UE positioning performance requirements

  - PRS-RSTD measurement accuracy requirements

    - Accuracy requirements with the combinations of PRS BW and other parameters

    - Applicable accuracy requirement in case of any serving cell change

    - Applicable propagation channel for accuracy requirement

    - Group delay calibration margin

  - PRS-RSRP measurement accuracy requirements

    - Accuracy requirements with the combinations of PRS BW and other parameters

    - RF calibration margin

  - UE Rx-Tx timing difference measurement accuracy requirements

    - Accuracy requirements with the combinations of PRS BW and other parameters

    - Applicability of accuracy requirements in the case of NTA_offset change

    - Applicable accuracy requirement in case of other (non-HO) serving cell changes

  - RRM test cases related to PRS-RSTD, UE Rx-Tx and PRS-RSRP

  - General PRS/SRS configuration

- gNB positioning measurement accuracy requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #98e meeting contributions

- R4-2102549	Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases	Ericsson, Intel

- R4-2103585	Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases	Ericsson, Intel

- R4-2103453	Email discussion summary: [98e][214] NR_pos_RRM_2	Moderator (Intel Corporation)

- R4-2103696	Email discussion summary: [98e][214] NR_pos_RRM_2	Moderator (Intel Corporation)

- R4-2103586	WF on NR Positioning UE performance requirements	Intel Corporation

- R4-2104046	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA	Intel Corporation

- R4-2104073	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA	Intel Corporation

- R4-2100440	Discussion on PRS RSTD accuracy requirements	CATT

- R4-2100443	Link-level simulation results of PRS RSTD measurement	CATT

- R4-2101274	Discussion on NR PRS RSTD measurement accuracy requirements	Intel Corporation

- R4-2101787	Discussion on PRS RSTD accuracy requirements	vivo

- R4-2102293	On PRS-RSTD measurement accuracy requirements	Qualcomm Incorporated

- R4-2102312	Test structure and complexity for NR RSTD test cases	Rohde & Schwarz

- R4-2102550	On RSTD measurement accuracy requirements	Ericsson

- R4-2102771	Discussion on accuracy requirements for RSTD measurement	Huawei, HiSilicon

- R4-2102772	draftCR to introduce accuracy requirements for RSTD measurement	Huawei, HiSilicon

- R4-2103730	draftCR to introduce accuracy requirements for RSTD measurement	Huawei, HiSilicon

- R4-2100061	[draft CR] Test cases for PRS-RSRP measurement accuracy	ZTE Corporation

- R4-2103733	[draft CR] Test cases for PRS-RSRP measurement accuracy	ZTE Corporation

- R4-2100441	Discussion on PRS RSRP accuracy requirements	CATT

- R4-2100444	Link-level simulation results of PRS RSRP measurement	CATT

- R4-2100446	CR on PRS-RSRP accuracy requirements	CATT

- R4-2103732	CR on PRS-RSRP accuracy requirements	CATT

- R4-2100470	CR on PRS-RSRP accuracy requirements	CATT

- R4-2101275	Discussion on NR PRS RSRP measurement accuracy requirements	Intel Corporation

- R4-2102551	On PRS-RSRP measurement accuracy requirements	Ericsson

- R4-2102773	Discussion on accuracy requirements for PRS-RSRP measurement	Huawei, HiSilicon

- R4-2102774	Further simulation results for PRS-RSRP	Huawei, HiSilicon

- R4-2100442	Discussion on UE Rx-Tx time difference accuracy requirements	CATT

- R4-2100445	Link-level simulation results for UE  Rx-Tx time difference measurement	CATT

- R4-2101276	Discussion on NR UE RX-TX time difference measurement accuracy requirements	Intel Corporation

- R4-2102294	On UE Rx-Tx measurement accuracy requirements	Qualcomm Incorporated

- R4-2102552	On UE Rx-Tx measurement accuracy requirements	Ericsson

- R4-2102553	UE Rx-Tx measurement accuracy requirements	Ericsson

- R4-2103731	UE Rx-Tx measurement accuracy requirements	Ericsson

- R4-2102775	Discussion on accuracy requirements for UE Rx-Tx time difference measurement	Huawei, HiSilicon

- R4-2100447	CR on PRS configuration for test cases	CATT

- R4-2100449	Discussion on test cases for PRS based measurement	CATT

- R4-2100471	CR on PRS configuration for test cases	CATT

- R4-2101277	Discussion on NR Positioning test cases configuration	Intel Corporation

- R4-2101278	[draftCR] CR for PRS configurations for NR Pos RRM tests	Intel Corporation

- R4-2103735	[draftCR] CR for PRS configurations for NR Pos RRM tests	Intel Corporation

- R4-2102295	On design of test cases for NR positioning	Qualcomm Incorporated

- R4-2102554	On positioning test cases	Ericsson

- R4-2102776	Discussion on RRM test case for UE positioning requirements	Huawei, HiSilicon

- R4-2100448	CR on test case for PRS-RSRP measurement requirements for FR2 in SA	CATT

- R4-2103734	CR on test case for PRS-RSRP measurement requirements for FR2 in SA	CATT

- R4-2100472	CR on test case for PRS-RSRP measurement requirements for FR2 in SA	CATT

- R4-2101279	[draftCR] CR for the test case of RSTD measurement requirements reporting in SA	Intel Corporation

- R4-2103736	[draftCR] CR for the test case of RSTD measurement requirements reporting in SA	Intel Corporation

- R4-2102555	TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA	Ericsson

- R4-2103737	TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA	Ericsson

- R4-2102777	draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA	Huawei, HiSilicon

- R4-2103739	draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA	Huawei, HiSilicon

- R4-2102556	TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA	Ericsson

- R4-2103738	TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA	Ericsson

- R4-2102778	draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA	Huawei, HiSilicon

- R4-2104042	draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA	Huawei, HiSilicon

- R4-2103454	Email discussion summary: [98e][215] NR_pos_RRM_3	Moderator (Ericsson)

- R4-2103697	Email discussion summary: [98e][215] NR_pos_RRM_3	Moderator (Ericsson)

- R4-2103587	WF on gNB positioning measurement requirements	Ericsson

- R4-2100048	Beam configuration for gNB measurement accuracy	ZTE Corporation

- R4-2100353	gNB positioning requirements	Ericsson

- R4-2101729	gNB Positioning UL SRS System simulation results for side conditions	Ericsson

- R4-2102690	On SRS configurations for gNB measurement accuracy requirements	Nokia, Nokia Shanghai Bell

- R4-2102779	Discussion on general issues for gNB positioning measurement requirements	Huawei, HiSilicon

- R4-2100450	Discussion on SRS-RSRP measurement requirements	CATT

- R4-2101737	gNB SRS-RSRP measurement analysis	Ericsson

- R4-2101760	gNB SRS-RSRP measurement	Ericsson

- R4-2104052	gNB SRS-RSRP measurement	Ericsson

- R4-2102783	Discussion on SRS-RSRP requirements	Huawei, HiSilicon

- R4-2102784	Link level simulation results for SRS-RSRP	Huawei, HiSilicon

- R4-2102785	draftCR to introduce SRS-RSRP requirements	Huawei, HiSilicon

- R4-2100451	Discussion on gNB Rx-Tx time difference measurement requirements	CATT

- R4-2101761	gNB Rx-Tx measurement analysis	Ericsson

- R4-2101790	gNB Rx-Tx measurement	Ericsson

- R4-2102691	Link simulation results for gNB Rx-Tx time difference accuracy	Nokia, Nokia Shanghai Bell

- R4-2102780	Discussion on gNB Rx-Tx time difference requirements	Huawei, HiSilicon

- R4-2102781	Link level simulation results for UL-RTOA and gNB Rx-Tx time difference	Huawei, HiSilicon

- R4-2102782	draftCR to introduce gNB Rx-Tx time difference requirements	Huawei, HiSilicon

- R4-2104053	draftCR to introduce gNB Rx-Tx time difference requirements	Huawei, HiSilicon

- R4-2100452	Discussion on UL RTOA measurement requirements	CATT

- R4-2100453	Link-level simulation results for UL timing measurement	CATT

- R4-2101801	gNB UL RTOA measurement analysis	Ericsson

- R4-2102692	On UL RTOA requirements for NR positioning	Nokia, Nokia Shanghai Bell

- R4-2102786	Discussion on UL-RTOA requirements	Huawei, HiSilicon, CMCC

- R4-2102787	draftCR to introduce UL-RTOA requirements	Huawei, HiSilicon

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template