3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2102132

# Online, Jan 25 th - Feb 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0013 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to TS 38.314 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Packet loss rate measurement in the UL per DRB per UE is calculated as number of lost packets per transmitted DRB * 106, as defined in TS 28.552. TS 28.552 – h10 --- 5.1.3.1.1 UL PDCP SDU Loss Rate c) This measurement is obtained as: 1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.. To make the definition for DL packet Uu Loss Rate clearer, the description in Table 4.2.1.5.1-2 could be updated from ‘number of lost packets per transmitted packets * 106’ to ‘number of lost packets per transmitted packets per DRB * 106’. - To align the parameter description with the definition of the L2 measurements by adding ‘per cell’. |
| Summary of change: |  | - Update ‘per transmitted packets’ to ‘per transmitted packets per DRB’ in Table 4.2.1.5.1-2. - Editorial issues. Impact analysis Impacted functionality: L2 measurements Inter-operability: - If UE implements according to the CR and the network does not, there is no impact foreseen. - If the network implements according to the CR and the UE does not, there is no impact foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The contents between different specifications are not aligned, which cause further confusions. - The editorial issues still exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.3.2 Mean number of Active UEs in the DL per DRB per cell 4.2.1.3.6 Mean number of Active UEs per cell 4.2.1.3.7 Max number of Active UEs per cell 4.2.1.3.8 Mean number of Active UEs per DRB per cell 4.2.1.5.1 Packet Uu Loss Rate in the DL per DRB per UE 4.2.1.6 Other measurements defined in TS 28.552 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

4.2.1.3.2	Mean number of Active UEs in the DL per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3.2-1: Definition for Mean number of Active UEs in the DL per DRB per cell

| Definition | Mean number of Active UEs in the DL per DRB per cell. The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement refers to UEs for which there is data available for transmission for the DL for DRBs. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.2-2 below. |
| --- | --- |

Table 4.2.1.3.2-2: Parameter description for Mean number of Active UEs in the DL per DRB per cell

| M(T,drbid,p) | Mean number of Active UEs in the DL per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i. Data available for transmission includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. |

NEXT CHANGE

4.2.1.3.6	Mean number of Active UEs per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3.6-1: Definition for Mean number of Active UEs per cell

| Definition | Mean number of Active UEs per cell. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,p)=∀iN(i)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.6-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3.6-2: Parameter description for Mean number of Active UEs per cell

| M(T,p) | Mean number of Active UEs per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.2.1.3.7	Max number of Active UEs per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3.7-1: Definition for Max number of Active UEs per cell

| Definition | Maximum number of Active UEs per cell. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,p)=maxTNi, where explanations can be found in the table 4.2.1.3.7-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3.7-2: Parameter description for Max number of Active UEs per cell

| M(T,p) | Maximum number of Active UEs per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.2.1.3.8	Mean number of Active UEs per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.3.8-1: Definition for Mean number of Active UEs per DRB per cell

| Definition | Mean number of Active UEs per DRB per cell. The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement refers to UEs for which there is data available for transmission for the UL for DRBs, or there is data available for transmission for the DL for DRBs, or both. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.8-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.3.8-2: Parameter description for Mean number of Active UEs per DRB per cell

| M(T,drbid,p) | Mean number of Active UEs per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. |

NEXT CHANGE

4.2.1.5.1	Packet Uu Loss Rate in the DL per DRB per UE

The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability.

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

NOTE 1:	Packet loss is expected to be upper bounded by the PER (packet error rate, as defined in TS 23.501 [4]) of the DRB which takes values between 10-6 and 10-2. The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been received, and thus the time for the measurement.

NOTE 2:	The granularity for Packet loss rate measurement is per DRB per UE, as defined in TS 28.552 [2].

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

NEXT CHANGE

4.2.1.6	Other measurements defined in TS 28.552 [2]

The granularity for PDCP SDU Data Volume measurement defined in TS 28.552 [2] is per DRB per UE.

The granularity for Average UE throughput measurement defined in TS 28.552 [2] is per UE and per DRB per UE.

PRB usage measurements are defined in TS 28.552 [2], i.e. DL/UL Total PRB Usage, Distribution of DL/UL Total PRB Usage. M(T), M1(T), P(T) are measured per cell. P(T) is the total available PRBs for this cell. M1(T) is the PRBs used for traffic transmission in this cell. Counting unit for PRB usage measurement is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier).

END OF CHANGES