3GPP TSG RAN Meeting #91-e																		RP-210738

Electronic Meeting, March 16 – 26, 2021

Source: 			Qualcomm Incorporated

Title: 				Discussion on NR-DC cell grouping UE capability

Document for: 	Decision

Agenda Item: 	9.12

# Introduction

RAN2, in RAN2#113e meeting, technically endorsed CRs introducing NR-DC cell grouping UE capability, which covers NR-DC band combinations with up to 5 frequency bands [1][2].

It was debated in RAN2 if such signalling is indeed future proof. RAN1 on the other hand previously recommended reusing the PUCCH grouping signalling, where grouping is done based on cell type {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2}. This solution does not impose any limitation about the number of frequency bands in NR-DC band combination [3].

RAN2 as a result sent an LS to RAN4 asking if RAN4 sees any concerns on the RAN2 technically endorsed solution [4].

# Discussion

  - Solution 1: RAN2 technically endorsed solution

The UE capability signalling technically endorsed by RAN2 is similar to that of LTE-DC, see below.

A cell grouping option is represented by a number of bits, where the left-most bit corresponds to the frequency band listed first in the band combination, etc. Value 0 indicates that the carriers of the corresponding frequency band are mapped to MCG, while value 1 indicates that the carriers of the corresponding frequency band are mapped to SCG. The UE signals the Bit-String where each bit corresponds to a cell grouping option. A bit in the bit string set to 1 indicates that the UE supports NR-DC with the cell grouping option. Nr of Frequency Bands:; 5; 4; 3; 2. Length of Bit-String:; 30; 14; 6; 2. Bit String Position; Cell grouping option (0= MCG, 1= SCG). 1; 00001; 0001; 001; 01. 2; 00010; 0010; 010; 10. 3; 00011; 0011; 011. 4; 00100; 0100; 100. 5; 00101; 0101; 101. 6; 00110; 0110; 110. 7; 00111; 0111. 8; 01000; 1000. 9; 01001; 1001. 10; 01010; 1010. 11; 01011; 1011. 12; 01100; 1100. 13; 01101; 1101. 14; 01110; 1110. 15; 01111. 16; 10000. 17; 10001. 18; 10010. 19; 10011. 20; 10100. 21; 10101. 22; 10110. 23; 10111. 24; 11000. 25; 11001. 26; 11010. 27; 11011. 28; 11100. 29; 11101. 30; 11110.

The key difference from LTE signalling is that in the new NR signalling, each bit of the bitmap corresponds to a “frequency band” in the band combination, instead of “band entry” as in LTE signalling. RAN2 has chosen this option because they recognized that the number of band entries is significantly increasing in NR, mainly because of FR2, e.g. CA_n5A-n260(8A). The endorsed new UE capability signalling therefore covers NR-DC band combinations with up to 5 frequency bands, with the restriction that band entries of the same frequency band can be assigned to only one Cell Group.

  - Solution 2: PUCCH grouping signalling solution

RAN2 agreed on the following UE capability signalling for PUCCH grouping.

twoPUCCH-Grp-ConfigurationsList-r16 SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r16)) OF TwoPUCCH-Grp-Configurations-r16 TwoPUCCH-Grp-Configurations-r16 ::= SEQUENCE { pucch-PrimaryGroupMapping-r16 TwoPUCCH-Grp-ConfigParams-r16, pucch-SecondaryGroupMapping-r16 TwoPUCCH-Grp-ConfigParams-r16 } TwoPUCCH-Grp-ConfigParams-r16 ::= SEQUENCE { pucch-GroupMapping-r16 PUCCH-Grp-CarrierTypes-r16, pucch-TX-r16 PUCCH-Grp-CarrierTypes-r16 } PUCCH-Grp-CarrierTypes-r16 ::= SEQUENCE { fr1-NonSharedTDD-r16 ENUMERATED {supported} OPTIONAL, fr1-SharedTDD-r16 ENUMERATED {supported} OPTIONAL, fr1-NonSharedFDD-r16 ENUMERATED {supported} OPTIONAL, fr2-r16 ENUMERATED {supported} OPTIONAL } maxTwoPUCCH-Grp-ConfigList-r16 INTEGER ::= 32

In this solution, if used for NR-DC cell grouping, the UE can signalling up to 32 combination of cell grouping. In one cell grouping combination, the UE signals for MCG and SCG respectively:

  - One or more carrier type(s) among {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that the UE supports in the Cell Group.

  - Carrier type(s) on which the UE supports PUCCH in the Cell Group.

Since the signalling is based on the “carrier type”, this solution does not impose any limitation on the number of frequency bands in NR-DC band combinations.

  - Comparison

Brief comparison between the solutions is provided below.

|  | Solution 1 (section 2.1) | Solution 2 (section 2.2) | Notes |
| --- | --- | --- | --- |
| Granularity for Cell Group mapping | Per frequency band | Per carrier type = {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} |  |
| Supported number of frequency bands in NR-DC band combination | 5 | No limitation |  |
| Signalling overhead per band combination | 30 bits in case of 5 frequency bands | 16 * (# of cell grouping combinations) | In solution 1, the fixed number of bits is always consumed. In solution 2, the overhead depends on the number of cell grouping combinations the UE supports. |

# Conclusion

In this document we analyzed the two solutions which have been looked at in RAN WGs. Companies are kindly requested to discuss if the technically endorsed solution by RAN2 is a viable solution, in particular the limitation that it can cover only up to “5 frequency bands” in NR-DC band combination, and the fact that the RAN2 technically endorsed solution is not easily extensible to address more than 5 frequency bands.

# References

[1]		R2-2102210		Release-16 CR to 38.331 “Introduction of Cell Grouping UE capability for NR-DC”	Qualcomm Incorporated

[2]		R2-2102210		Release-16 CR to 38.306 “Introduction of Cell Grouping UE capability for NR-DC”	Qualcomm Incorporated

[3]		R1-2009570		LS reply on cell-grouping UE capability for synchronous NR-DC	RAN1 (To RAN2)

[4]		R2-2102212		LS on Introduction of Cell Grouping UE capability for NR-DC		RAN2	 (To RAN4)