3GPP TSG-RAN5 Meeting #91-e	R5-212693

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1870 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for CA_n78C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At RAN5#90-e there were 5 CRs in R5-211115, R5-211116, R5-211117, R5-211118, R5-211119 adding column "CA Aggregated CBW [MHz]" to test frequncy tables for FR1 NR CA intra-band contiguous configurations. After clarification that principles for calculating the test frequencies for the different CCs takes the RAN4 requirement on nominal channel spacing and guardband into account it was agreed that the column was not needed. R5-21115 and R5-211119 were widthdrawn, but R5-211117, R5-211118, R5-211119 were not withdrawn. To revert the addition of the column to the frequency Table 4.3.1.1.3.78.1-1 for CA_n78C in R5-211116 there is a need to remove the column. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.3.1.1.3.78.1-1: Removed column "CA Aggregated CBW [MHz]" and Note 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant column remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.78.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.78.1	CA_n78C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.1-1: NR Intra-Band contiguous CA configuration CA_n78C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3459.33 | 630622 | 102 |  |  | 7846 | 633312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3715.02 | 647668 | 3509.64 | 633976 | 504 |  |  | 7982 | 646368 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379.98 | 625332 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.95 | 638330 | 3509.07 | 633938 | 102 |  |  | 7881 | 636672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3449.34 | 629956 | 102 |  |  | 7840 | 632736 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3695.13 | 646342 | 3489.75 | 632650 | 504 |  |  | 7968 | 645024 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389.88 | 625992 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.86 | 638324 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3439.35 | 629290 | 102 |  |  | 7833 | 632064 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3675.24 | 645016 | 3469.86 | 631324 | 504 |  |  | 7954 | 643680 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399.78 | 626652 | 3350.64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.77 | 638318 | 3488.91 | 632594 | 102 |  |  | 7867 | 635328 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3459.12 | 630608 | 102 |  |  | 7846 | 633312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3720.03 | 648002 | 3509.43 | 633962 | 504 |  |  | 7982 | 646368 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3384.96 | 625664 | 3361.02 | 624068 | 0 | 30 |  | 7753 | 624384 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.96 | 638664 | 3519.3 | 634620 | 102 |  |  | 7888 | 637344 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3454.11 | 630274 | 102 |  |  | 7843 | 633024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3709.98 | 647332 | 3499.38 | 633292 | 504 |  |  | 7975 | 645696 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.99 | 638666 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3444.12 | 629608 | 102 |  |  | 7836 | 632352 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.09 | 646006 | 3479.49 | 631966 | 504 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399.9 | 626660 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.9 | 638660 | 3504.12 | 633608 | 102 |  |  | 7878 | 636384 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3670.2 | 644680 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409.8 | 627320 | 3360.66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3579.81 | 638654 | 3493.95 | 632930 | 102 |  |  | 7871 | 635712 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3449.22 | 629948 | 102 |  |  | 7839 | 632640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3710.13 | 647342 | 3489.63 | 632642 | 504 |  |  | 7968 | 645024 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3404.88 | 626992 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.86 | 639324 | 3529.2 | 635280 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3444.21 | 629614 | 102 |  |  | 7836 | 632352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3479.58 | 631972 | 504 |  |  | 7961 | 644352 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409.92 | 627328 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3524.01 | 634934 | 102 |  |  | 7891 | 637632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3434.22 | 628948 | 102 |  |  | 7829 | 631680 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3459.51 | 630634 | 504 |  |  | 7947 | 643008 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.98 | 639332 | 3514.2 | 634280 | 102 |  |  | 7885 | 637056 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3424.23 | 628282 | 102 |  |  | 7822 | 631008 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.12 | 644008 | 3439.62 | 629308 | 504 |  |  | 7933 | 641664 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429.9 | 628660 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3504.03 | 633602 | 102 |  |  | 7878 | 636384 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3439.14 | 629276 | 102 |  |  | 7832 | 631968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.23 | 646682 | 3469.65 | 631310 | 504 |  |  | 7954 | 643680 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3424.77 | 628318 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.76 | 639984 | 3539.1 | 635940 | 102 |  |  | 7902 | 638688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.18 | 646012 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429.81 | 628654 | 3400.65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3599.79 | 639986 | 3533.91 | 635594 | 102 |  |  | 7898 | 638304 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3424.14 | 628276 | 102 |  |  | 7822 | 631008 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.11 | 644674 | 3439.53 | 629302 | 504 |  |  | 7933 | 641664 |  | 2 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439.89 | 629326 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.88 | 639992 | 3524.1 | 634940 | 102 |  |  | 7891 | 637632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| +100 |  |  |  |  | & | Mid | 3500.01 | 633334 | 3414.15 | 627610 | 102 |  |  | 7815 | 630336 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3650.04 | 643336 | 3419.46 | 627964 | 504 |  |  | 7919 | 640320 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449.97 | 629998 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.97 | 639998 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.3.78.2	CA_n78B