3GPP TSG-RAN WG4 Meeting #99-e	R4-2109723

# Electronic Meeting, 19th to 27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0544 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-3 for missing MSD due to cross band and MSD due to receiver harmonic mixing for combos with n46 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce missing MSD due to cross band and MSD due to receiver harmonic mixing for combos with n46 according to R4-2109705. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce missing MSD due to cross band isolation requirements: DC_40_n78 that has been specifiec in TS38.101-1 Table 7.3A.6-1 for same frequency-range band combination - Introduce missing MSD due to receiver harmonic mixing requirements: DC_48_n46 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MSD due to receiver harmonic mixing requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

Table 7.3B.2.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n461 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | 22,3 | 28 | 28 | 28 | 28 |  |  |  |  |  |  |  |
| n46 |  | 482,4 |  | 22.6 |  | 19.5 |  | 17.8 |  | 16.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: The requirements for a victim (lower) band apply for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. . Note 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-4: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |

Table 7.3B.2.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band |  | SCS of UL band (kHz) |  | 5 MHz (LCRB) |  | 10 MHz (LCRB) |  | 15 MHz (LCRB) |  | 20 MHz (LCRB) |  | 25 MHz (LCRB) |  | 40 MHz (LCRB) |  | 50 MHz (LCRB) |  | 60 MHz (LCRB) |  | 80 MHz (LCRB) |  | 90 MHz (LCRB) |  | 100 MHz (LCRB) |  |
| n46 |  | 2 |  | 15 |  | 25 |  | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n46 |  | 48 |  | 15 |  | 12 |  | 25 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <Unchanged part omitted>

##### 7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.3.4-1 and Table 7.3B.2.3.4-1a with uplink configuration of the agressor band specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 | [3] | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | [1.4] |  |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  | 6.6 |  |  |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | [6.1] | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | [0.7] | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | 8.3 |  |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 | [3.1] | [3.1] | 3.1 | 3.1 | 3.1 |  | 3.1 |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 | [8.3] | [8.3] | [8.3] | [8.3] |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | [0.6] |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | 401 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 | [7.3] | [6.5] | 6.3 | 5.3 | 4.5 | [4.3] | 4.0 | 3.9 | 3.8 |
| n79 | 426 | 2.6 | 2.6 | 2.6 | 2.6 |  |  |  |  |  |  |  |  |  |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | - | - | - | 7 | - | - | 5.7 | - | 5.1 | - | 4.7 | - | - |
| n46 | 48 | 13.3 | 10.4 | 8.8 | 7.8 | - | - | 7.8 | 7 | 6.5 | - | 5.7 | 5.4 | 5.1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n41 | 3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | [25] |  |  |  |  |  |  |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 |  |  |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | [100] | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n3 | 11 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  | [50] | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 | [25] | [25] | 25 |  |  |  |  |  |  |
| n3 | 41 | 15 | 25 | 502 | 502 | 502 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 15 | 25 | 50 | 75 | 75 | [75] | [100] | 100 | 100 | 100 |  | 100 |  |  |
| n38 | 1 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 7 | 30 | 216 | 216 | 216 | 216 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| 41 | n3 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | [100] |  |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n77 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 | [100] | [100] | 100 | 100 | 100 | [100] | 100 | 100 | 100 |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 | [100] | [100] | 100 | 100 | 100 | [100] | 100 | 100 | 100 |
| n79 | 42 | 30 | 2705 | 2705 | 2705 | 2705 |  |  |  |  |  |  |  |  |  |
| n84 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 |  | 216 |  |  |
| n46 | 48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation “LCRB” value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 5: The requirements only apply for UEs supporting inter-band ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <End of Changes>