3GPP TSG-RAN4 Meeting #99-e	R4-2107753

# Online, , 19th May 2021 - 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0351 | rev | 1 | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | P_cmax fix for the CA applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Pcmax CA correction: In previous meeting changes were done to the same section but what remains incorrect is that sentence: “The UE maximum configured power PCMAX in a transmission occasion is determined by the UL grants for carrier f(c) of each serving cell c”. It is confusing since it implies there are many grants but only one cell to look at for determining Pcmax. Pcmax for UE is defined above “total configured maximum output power PCMAX”. Pcmax for CA is determined by looking at all grants for all cells with simultaneous transmission occasion since MPR also is defined for CA so that alloction on all CC’s is used to determine the MPR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “each” is replaced by “all” and cell is changed to its plural cells to clarify that UE uses grants for all cells to determine Pcmax. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE calculation for Pcmax and therefore reference for PHR reportting is not correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of changes >

### 6.2A.4	Configured transmitted power for CA

A UE configured with carrier aggregation can configure its maximum output power for each uplink activated serving cell c and its total configured maximum output power PCMAX. The definition of the configured UE maximum output power PCMAX,f,c for each carrier f of a serving cell c is used for power headroom reporting for carrier f of serving cell c only and is in accordance with that specified in clause 6.2.4 with parameters MPR, A-MPR and P-MPR replaced with those specified in clause 6.2A.2, 6.2A.3 and 6.2.4, respectively. The UE maximum configured power PCMAX in a transmission occasion is determined by the UL grants for carriers f of all serving cells c with non-zero granted power in the respective reference point.

PCMAX is calculated under the assumption that power spectral density for each RB in each component carrier is the same.

The configured UE maximum output power PCMAX shall be set such that the corresponding measured total peak EIRP PUMAX is within the following bounds

PPowerclass – MAX(MAX(MPR, A-MPR)  + ΔMBP,n, P-MPR) – MAX{T(MAX(MPR, A-MPR)),T(P-MPR)} ≤ PUMAX ≤ EIRPmax

with PPowerclass the peak EIRP  as specified in clause 6.2A.1, EIRPmax the applicable maximum EIRP as specified in clause 6.2A.1, MPR as specified in clause 6.2A.2, A-MPR as specified in clause 6.2A.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1, P-MPR the power management term for the UE as described in 6.2.4. The measured configured power PUMAX for carrier aggregation is defined as

PUMAX=10log10c,f(c)pUMAX,f,c

where  pUMAX,f,c is the linear value of the measured power PUMAX,f,c for carrier f=f(c) of serving cell c. The measured total radiated power PTMAX for carrier aggregation is defined as

PTMAX=10log10c,f(c)pTMAX,f,c

where pTMAX,f,c is the linear value of the measured total radiated power PTMAX,f,c for carrier f = f(c) of serving cell c. The total radiated power PTMAX is bounded by

PTMAX ≤ TRPmaxwhere TRPmax the maximum TRP for the UE power class as specified in clause 6.2A.1.

The tolerance T(ΔP) for applicable values of ΔP (values in dB) is specified in Table 6.2A.4-1.

Table 6.2A.4-1: PUMAX tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3A.1 |  |  |

< end of changes >