3GPP TSG-RAN WG4 Meeting #99-e	R4-2110300

# Online, 19th– 27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2014 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maintaining condition requirements for UE power class 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN4, power class 5 has been introduced for FR2 new FWA UE. The minimum RP values for UE power class 5 are missing in SRS-RSRP measurement condition requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update Table B.2.7-2 for adding the minimum RP values for UE power class 5 in SRS-RSRP measurement condition requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SRS-RSRP measurement condition requirements UE power class 5 will be missing in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## B.2.7	Conditions for SRS-RSRP measurements

This clause defines the following conditions for SRS-RSRP measurement and corresponding procedures performed based on SRSs: SRS_RP and SRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.7-1 for FR1 NR cells.

The conditions are defined in Table B.2.7-2 for FR2 NR cells.

Table B.2.7-1: Conditions for SRS-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SRS_RP |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSRS |  |  | dB |
|  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |
| Conditions | NR_TDD_FR1_A | -120 | -117 | -114 | 1 |
|  | NR_TDD_FR1_C | -119 | -116 | -113 |  |
|  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 |  |
|  | NR_TDD_FR1_E | -118 | -115 | -112 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.7-2: Conditions for SRS-RSRP measurements in FR2

| Parameter |  | Angle of arrival |  | NR operating bands |  | Minimum SRS_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSSRS |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSSRS = 60 kHz |  |  |  |  |  |  |  |  | SCSSRS = 120 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 |  | 3 | 4 |  |  | 5 | 1, 2, 3, 4, 5 |  |  |  |
| Conditions |  | Rx Beam Peak |  | n257 |  | -124.5 |  | -119.0 |  | -115.3 | -124.0 |  |  | -119.6 | (Value for SCSSRS = 60 kHz) +3dB |  |  | ≥1 |
|  |  |  |  | n258 |  | -124.5 |  | -119.0 |  | -115.3 | -124.0 |  |  | -119.8 |  |  |  |  |
|  |  |  |  | n260 |  | -121.5 |  |  |  | -112.7 | -122.0 |  |  |  |  |  |  |  |
|  |  |  |  | n261 |  | -124.5 |  | -119.0 |  | -115.3 | -124.0 |  |  |  |  |  |  |  |
|  |  | Spherical coverage Note 1 |  | n257 |  | -116.5 |  | -108.0 |  | -104.4 | -115.0 |  |  | -111.6 | (Value for SCSSRS = 60 kHz) +3dB |  |  | ≥1 |
|  |  |  |  | n258 |  | -116.5 |  | -108.0 |  | -104.4 | -115.0 |  |  | -111.8 |  |  |  |  |
|  |  |  |  | n260 |  | -113.5 |  |  |  | -100.1 | -110.0 |  |  |  |  |  |  |  |
|  |  |  |  | n261 |  | -116.5 |  | -108.0 |  | -104.4 | -115.0 |  |  |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Change 1>