3GPP TSG RAN meeting #92e										       RP-211126

Electronic Meeting, June 14 - 18, 2021

Status Report to TSG

Agenda item:			9.7.1.5

| WI / SI Name | NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_enh |  |  |  |  |
| Unique ID | 900061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210855 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 40% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Jianchi Zhu |
|  | Company | China Telecom |
|  | Email | zhujc@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #104b-e

PUSCH enhancements:

Agreement:

Non-consecutive physical slots for UL transmission can be used to transmit TBoMS at least for unpaired spectrum.

- How TBoMS is transmitted over non-consecutive physical slots for UL transmission for unpaired spectrum is to be discussed further.

- Whether and how non-consecutive physical slots for UL transmission can be used to transmit TBoMS for paired spectrum and SUL band as well, is to be discussed further.

Working Assumption

The concept of transmission occasion for TBoMS (TOT) is utilized for the purpose of discussion, where a TOT is constituted of time domain resources which may or may not span multiple slots

- FFS: details, whether multiple slots which constitute a TOT are consecutive or non-consecutive physical slots for UL transmissions

- FFS: other details.

- FFS: whether such concept will be specified or not.

Agreements:

For the definition of a single TBoMS, down select among the following options:

- Option 1: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using a single RV.

  - FFS: whether and how the single RV is rate matched across the TOT, e.g., continuous rate-matching across the TOT, rate matched for each slot and so on.

- Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

  - FFS: how RV index is refreshed within the TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

- Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

  - FFS: whether and how RV index is refreshed within one TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- FFS: the exact TBS determination procedure.

- FFS: whether a single TBoMS can be repeated or not.

- FFS: other implications, e.g., power control, collision handling and so on.

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS whether or not to further consider impacting of timing advance

Agreements:

- A new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17.

Agreements:

- For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

Conclusion:

- For optimization of DMRS granularity in time domain with joint channel estimation, the proponents are encouraged to provide more simulation results in next meeting

Agreements:

- For the time domain window for joint channel estimation, down select on the following two options:

  - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

  - Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

Agreement:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over back-to-back PUSCH transmissions with different TBs

Type A PUSCH repetitions for Msg3:

Agreement: For Msg3 PUSCH repetition, support the following modified Option 2-1.

- Option 2-1: For UE requested Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can request Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.).

- Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual

- FFS details if any.

Agreements: For the determination of RV for Msg3 PUSCH repetition,

- RV of the first repetition is determined in the same way as legacy.

  - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission.

  - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission.

- FFS determination of the RV sequence.

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted.

- FFS additionally using MAC RAR for indication.

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3.

- FFS: the determination of available slots.

RAN1 #105-e

PUSCH enhancements:

Agreement:

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Agreement:

- Down-selection in RAN1#106-e:

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

Agreement:

In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17.

- {20, 24, 28}

Agreement:

- Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition.

  - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

Agreement:

- If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Agreement:

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

- Alt 1-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- Alt 1-B’ consisting of two steps

  - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

  - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

- Alt 2-A consisting of a single step

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Alt 2-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

    - FFS timeline for the dynamic signalling

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Working assumption:  Agreement:

For TBS determination of TBoMS:

- NohPRB is configured by xOverhead and represents the overhead per slot.

- NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated.

Note: xOverhead configuration is as per Rel-15/16.

Agreement:

The following 2 options for time domain resource determination for TBoMS are considered for down-selection during RAN1 #105-e:

- Option 1: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- Option 2: Time domain resource determination for TBoMS can be performed via PUSCH repetition Type A like TDRA or via PUSCH repetition Type B like TDRA.

  - The use of PUSCH repetition Type B like TDRA for time domain resource determination is according to an additional UE capability for a TBoMS capable UE.

  - FFS DMRS pattern for PUSCH repetition Type B like TDRA

Working assumption

A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission

- FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc.

- FFS: whether such concept will be specified or not.

Agreement:

- The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e)

  - Option 3, if a design based on single RV is adopted.

  - Option 4, if a design based on different RVs is adopted.

- FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc.

- The single RV is not constrained to have only the same coded bits in each slot or in each TOT

- The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on.

Agreement:

Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- FFS: details

- FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case)

Working assumption

Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Agreement:

The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected:

- Option a: Rate-matching is performed per slot;

- Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT;

- Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS

Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X.

Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

Agreement:

Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.

- FFS: details.

Agreement:

The following approach is used to calculate NInfo for TBoMS:

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K.

L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Agreement:

- Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Agreement:

- Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Agreement: Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

Draft LS to RAN4 is approved, with final LS in R1-2106212.

Agreement:

- Optimization of DMRS granularity in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

Agreement:

- For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

- Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Agreement:

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Agreement:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitons of the same TB, down select one of the following alternatives for the time domain window.

- Alt 1: All the repetitions are covered by one single time domain window

  - The start of the window is the first PUSCH transmission

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum

  - FFS: frequency hopping and precoder cycling

- Alt 2: All the repetitions are covered by one or multiple time domain windows

  - For the start of each window,

    - The start of the first window is the first PUSCH transmission.

    - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum

  - For the length of each window,

    - FFS Each window consists of at least two adjacent physical slots for UL transmission.

    - The length of each window is no longer than the maximum duration.

    - FFS: how to determine the length of each window

    - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum.

  - FFS: frequency hopping and precoder cycling

- Other alternatives are not precluded.

PUCCH enhancements:

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements.

- Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

  - FFS: RRC signaling enhancement details

Conclusion: For PUCCH repetitions, the following use cases are deprioritized in RAN1 work on PUCCH DMRS bundling

- Use case 1: back-to-back PUCCH repetitions within one slot.

- Use case 2: non-back-to-back PUCCH repetitions within one slot.

  - Use case 2a: no uplink transmission in the middle of two PUCCH repetitions

  - Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions

Type A PUSCH repetitions for Msg3:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

Working assumption:

- Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission

  - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission.

    - TDRA information field with introducing a new TDRA table including the repetition factors.

    - MCS information field

    - TPC information field

    - CSI request information field

    - FDRA information field

- The total size of RAR UL grant does not change.

- Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not.

- FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Conclusion:

- Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

2.1.2	Remaining Open issues

- PUSCH enhancements

  - Detailed mechanisms for enhancements on PUSCH repetition type A

    - Increasing the maximum number of repetitions.

    - The number of repetitions counted on the basis of available UL slots.

  - Detailed mechanism(s) to support TB processing over multi-slot PUSCH

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Detailed mechanism(s) to support joint channel estimation

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions

      - Potential optimization of DMRS location in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation

- PUCCH enhancements

  - Detailed signaling mechanism to support dynamic PUCCH repetition factor indication

  - Detailed mechanism to support DMRS bundling across PUCCH repetitions

- Detailed mechanism(s) to support Type A PUSCH repetitions for Msg3

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The progress in RAN4 #98e-bis meeting is summarized below:

- The email discussion summary for [98-bis-e][140] NR_cov_enh was provided in R4-2105477.

- The reply LS on PUCCH and PUSCH repetition was approved in R4-2105417.

- The way forward on phase continuity and power consistency for PUCCH and PUSCH repetition was approved in R4-2105418.

The progress in RAN4 #99e meeting is summarized below:

- The email discussion summary for [99-e][147] NR_cov_enh was provided in R4-2107957.

- The reply LS on PUCCH and PUSCH repetition was approved in R4-2107880.

- The way forward on phase continuity and power consistency for PUCCH and PUSCH repetition was approved in R4-2107881.

2.4.2	Remaining Open issues

- Conditions for joint channel estimation to keep power consistency and phase continuity (with certain tolerance) to be investigated and specified if necessary by RAN4.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #104b-e:

- R1-2103876	FL summary of TB processing over multi-slot PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

- R1-2104102	Final FL summary of TB processing over multi-slot PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

- R1-2103808	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2104006	[104b-e-NR-R17-CovEnh-02] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2103829	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

- R1-2104101	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

- R1-2104119	Reply LS on PUCCH and PUSCH repetition	RAN1, Qualcomm

- R1-2102314	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2102408	Issues for TB over multi-slot PUSCH	OPPO

- R1-2102498	Discussion on TB processing over multi-slot PUSCH	ZTE

- R1-2102535	Discussion on PUSCH TB processing over multiple slots	vivo

- R1-2102644	Discussion on TB processing over multi-slot PUSCH	CATT

- R1-2102691	Discussion on TB processing over multi-slot PUSCH	MediaTek Inc.

- R1-2102718	Views on TB processing over multi-slot PUSCH	Fujitsu

- R1-2102861	Discussion on TB processing over multi-slot PUSCH	China Telecom

- R1-2102894	Discussion on TB processing over multi-slot PUSCH	CMCC

- R1-2102913	On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

- R1-2102993	TB processing over multi-slot PUSCH	Xiaomi

- R1-2103008	TB processing over multi-slot PUSCH	InterDigital, Inc.

- R1-2103043	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2103117	Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2103179	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2103208	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2103252	TB processing over multi-slot PUSCH	Samsung

- R1-2103381	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2103445	TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2103461	Design Considerations for TB Processing over Multi-Slot PUSCH	Sierra Wireless, S.A.

- R1-2103480	TB processing over multi-slot PUSCH	Sharp

- R1-2103514	Discussion on TB processing over multi-slot PUSCH	NEC

- R1-2103588	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2103616	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

- R1-2103625	Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2103700	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2102313	Discussion on Joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2102409	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2102465	Consideration on joint channel estimation over multi-PUSCH	Spreadtrum Communications

- R1-2102499	Discussion on joint channel estimation for PUSCH	ZTE

- R1-2102536	Discussion on Joint channel estimation for PUSCH	vivo

- R1-2102645	Discussion on joint channel estimation for PUSCH	CATT

- R1-2102692	Discussion on joint channel estimation for PUSCH	MediaTek Inc.

- R1-2102862	Discussion on joint channel estimation for PUSCH	China Telecom

- R1-2102895	Discussion on joint channel estimation for PUSCH	CMCC

- R1-2102994	Joint channel estimation for PUSCH	Xiaomi

- R1-2103009	Discussions on joint channel estimation for PUSCH	InterDigital, Inc.

- R1-2103044	Discussion on joint channel estimation for PUSCH	Intel Corporation

- R1-2103118	Discussion on joint channel estimation for PUSCH	Apple

- R1-2103180	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2103253	Joint channel estimation for PUSCH	Samsung

- R1-2103312	UE configuration for enhanced JCE in TDD	Sony

- R1-2103382	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2103446	Joint Channel Estimation for PUSCH	Ericsson

- R1-2103458	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2103460	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless, S.A.

- R1-2103481	Joint channel estimation for multi-slot PUSCH	Sharp

- R1-2103589	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2103617	Enhancements for joint channel estimation for multiple PUSCH	Lenovo, Motorola Mobility

- R1-2103626	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2103701	Discussion on joint channel estimation for PUSCH	WILUS Inc.

- R1-2102315	Discussion on Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2102410	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2102466	Discussion on type A PUSCH repetitions for Msg3	Spreadtrum Communications

- R1-2102500	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

- R1-2102537	Discussion on Type A PUSCH repetitions for Msg3	vivo

- R1-2102646	Discussion on Type A PUSCH repetitions for Msg3	CATT

- R1-2102863	Discussion on type A PUSCH repetitions for Msg3	China Telecom

- R1-2103772	Discussion on type A PUSCH repetitions for Msg3	CMCC

- R1-2102995	Type A PUSCH repetition for Msg3	Xiaomi

- R1-2103010	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2103045	On Msg3 PUSCH repetition	Intel Corporation

- R1-2103119	Discussion on Msg3 Coverage Enhancement	Apple

- R1-2103181	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2103209	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2103254	Type A PUSCH repetitions for Msg3	Samsung

- R1-2103329	Type A PUSCH repetitions for Msg3	ETRI

- R1-2103383	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2103447	Type A PUSCH Repetition for Msg3	Ericsson

- R1-2103482	Type A repetition for msg3 PUSCH	Sharp

- R1-2103515	Discussion on  PUSCH repetitions for Msg3	NEC

- R1-2103590	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2103618	Type A PUSCH repetition for Msg3	Lenovo, Motorola Mobility

- R1-2103627	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2103702	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

RAN1 #105-e:

- R1-2105992	FL Summary on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2106155	FL Summary on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2106199	FL Summary on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2106279	FL Summary on Enhancements on PUSCH repetition type A	Moderator (Sharp)

- R1-2105996	FL summary of TB processing over multi-slot PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

- R1-2106250	FL summary #2 of TB processing over multi-slot PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

- R1-2106251	Final FL summary of TB processing over multi-slot PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

- R1-2105979	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2106152	[105-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2106212	LS on joint channel estimation for PUSCH and PUCCH	RAN1, China Telecom

- R1-2106014	FL summary of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2106154	FL summary #2 of PUCCH coverage enhancement	Moderator (Qualcomm)

- R1-2106007	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

- R1-2106247	Final feature lead summary on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

- R1-2104240	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2104291	Discussion on enhancements on PUSCH repetition Type A for low latency requirement	NICT, TOYOTA MOTOR CORPORATION

- R1-2104330	Discussion on enhanced PUSCH repetition type A	ZTE

- R1-2104376	Discussion on enhancement for PUSCH repetition type A	vivo

- R1-2104537	Discussion on enhancements on PUSCH repetition type A	CATT

- R1-2104625	Discussion on enhancements on PUSCH repetition type A	CMCC

- R1-2104685	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2104792	Enhancements on PUSCH repetition type A	OPPO

- R1-2104846	Enhancements on PUSCH repetition type A	China Telecom

- R1-2104859	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2104919	Enhancements on PUSCH repetition type A	Intel Corporation

- R1-2105119	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2105146	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2105255	Discussion on PUSCH repetition type A	NEC

- R1-2105325	Enhancements on PUSCH repetition type A	Samsung

- R1-2105488	Discussions on PUSCH repetition type A enhancements	LG Electronics

- R1-2105511	Design considerations for PUSCH repetition Type A Enhancements	Sierra Wireless, S.A.

- R1-2105575	Enhancements on PUSCH repetition type A	Xiaomi

- R1-2105640	Enhancements on PUSCH repetition type A	Sharp

- R1-2105652	PUSCH Repetition Type A Enhancement	Ericsson

- R1-2105711	Enhancement on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2105773	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

- R1-2105877	Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

- R1-2105901	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2104242	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

- R1-2104297	On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

- R1-2104331	Discussion on TB processing over multi-slot PUSCH	ZTE

- R1-2104377	Discussion on PUSCH TB processing over multiple slots	vivo

- R1-2104436	Discussion on TB processing over multi-slot PUSCH	Spreadtrum Communications

- R1-2104538	Discussion on TB processing over multi-slot PUSCH	CATT

- R1-2104626	Discussion on TB processing over multi-slot PUSCH	CMCC

- R1-2104686	TB processing over multi-slot PUSCH	Qualcomm Incorporated

- R1-2104793	Issues for TB over multi-slot PUSCH	OPPO

- R1-2104847	Discussion on TB processing over multi-slot PUSCH	China Telecom

- R1-2104860	TB processing over multiple slots	InterDigital, Inc.

- R1-2104920	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

- R1-2105064	Views on TB processing over multi-slot PUSCH	Fujitsu

- R1-2105120	Discussion on TB processing over multi-slot PUSCH	Apple

- R1-2105147	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

- R1-2105256	Discussion on TB processing over multi-slot PUSCH	NEC

- R1-2105326	TB processing over multi-slot PUSCH	Samsung

- R1-2105968	Discussion on TB processing over multi-slot PUSCH	MediaTek Inc.

- Revision of R1-2105384

- R1-2105489	Discussions on TB processing over multi-slot PUSCH	LG Electronics

- R1-2105510	Design Considerations for TB processing over multi-slot PUSCH	Sierra Wireless, S.A.

- R1-2105576	Discussion on TB processing over multi-slot PUSCH	Xiaomi

- R1-2105641	TB processing over multi-slot PUSCH	Sharp

- R1-2105653	TB Processing over Multi-Slot PUSCH	Ericsson

- R1-2105712	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

- R1-2105774	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

- R1-2105878	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

- R1-2105902	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2104241	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2104332	Discussion on joint channel estimation for PUSCH	ZTE

- R1-2104378	Discussion on Joint channel estimation for PUSCH	vivo

- R1-2104437	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2104539	Discussion on joint channel estimation for PUSCH	CATT

- R1-2104627	Discussion on joint channel estimation for PUSCH	CMCC

- R1-2104687	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2104794	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2104848	Discussion on joint channel estimation for PUSCH	China Telecom

- R1-2104861	Joint channel estimation for PUSCH	InterDigital, Inc.

- R1-2104882	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

- R1-2104921	Discussion on joint channel estimation for PUSCH	Intel Corporation

- R1-2105121	Discussion on joint channel estimation for PUSCH	Apple

- R1-2105176	Joint channel estimation for PUSCH	Sony

- R1-2105327	Joint channel estimation for PUSCH	Samsung

- R1-2105394	Discussion on joint channel estimation for PUSCH	MediaTek Inc.

- R1-2105397	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2105490	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2105509	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless, S.A.

- R1-2105577	Joint channel estimation for PUSCH	Xiaomi

- R1-2105642	Joint channel estimation for multi-slot PUSCH	Sharp

- R1-2105654	Joint Channel Estimation for PUSCH	Ericsson

- R1-2105713	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2105775	Enhancements for joint channel estimation for multiple PUSCH	Lenovo, Motorola Mobility

- R1-2105879	Discussion on joint channel estimation for PUSCH	WILUS Inc.

- R1-2105903	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2104243	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

- R1-2104333	Discussion on coverage enhancements for PUCCH	ZTE

- R1-2104379	Discussion on PUCCH enhancements	vivo

- R1-2104438	Discussion on PUCCH enhancements	Spreadtrum Communications

- R1-2104540	Discussion on PUCCH enhancement	CATT

- R1-2104628	Discussion on PUCCH enhancements	CMCC

- R1-2104688	PUCCH enhancements	Qualcomm Incorporated

- R1-2104795	PUCCH enhancements for coverage	OPPO

- R1-2104849	Discussion on PUCCH enhancements	China Telecom

- R1-2104862	Discussions on PUCCH enhancements	InterDigital, Inc.

- R1-2104922	Discussion on PUCCH enhancements	Intel Corporation

- R1-2105122	PUCCH coverage enhancement	Apple

- R1-2105149	Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

- R1-2105224	PUCCH enhancements	ETRI

- R1-2105257	Discussion on PUCCH enhancements	NEC

- R1-2105328	PUCCH enhancements	Samsung

- R1-2105491	Discussions on coverage enhancement for PUCCH	LG Electronics

- R1-2105578	PUCCH coverage enhancement	Xiaomi

- R1-2105643	PUCCH coverage enhancement	Sharp

- R1-2105655	PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

- R1-2105714	PUCCH enhancements	NTT DOCOMO, INC.

- R1-2105776	Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

- R1-2105904	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2104244	Msg3 repetition for coverage enhancement	Huawei, HiSilicon

- R1-2104334	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

- R1-2104380	Discussion on Type A PUSCH repetitions for Msg3	vivo

- R1-2104439	Discussion on type A PUSCH repetitions for Msg3	Spreadtrum Communications

- R1-2104541	Discussion on Type A PUSCH repetitions for Msg3	CATT

- R1-2104629	Discussion on type A PUSCH repetitions for Msg3	CMCC

- R1-2104689	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

- R1-2104796	Type A PUSCH repetitions for Msg3 coverage	OPPO

- R1-2104850	Discussion on type A PUSCH repetitions for Msg3	China Telecom

- R1-2104863	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

- R1-2104923	On Msg3 PUSCH repetition	Intel Corporation

- R1-2105123	Discussion on Msg3 Coverage Enhancement	Apple

- R1-2105150	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

- R1-2105225	Type A PUSCH repetitions for Msg3	ETRI

- R1-2105329	Type A PUSCH repetitions for Msg3	Samsung

- R1-2105492	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

- R1-2105579	Discussion on Type A PUSCH repetition for Msg3	Xiaomi

- R1-2105644	Type A repetition for msg3 PUSCH	Sharp

- R1-2105656	Type A PUSCH Repetition for Msg3	Ericsson

- R1-2105715	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

- R1-2105777	Type A PUSCH repetitions for Msg3	Lenovo, Motorola Mobility

- R1-2105880	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

- R1-2105905	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

- R1-2104335	Performance impacts of residual frequency error for joint channel estimation of PUSCH and PUCCH transmissions	ZTE

- R1-2104381	Enhanced Contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition			vivo

- R1-2104542	Views on reusing PUSCH enhancements for Msg3	CATT

- R1-2104797	Other considerations for coverage enhancement	OPPO

- R1-2104864	Deterministic PUSCH repetition for coverage enhancements	InterDigital, Inc.

- R1-2105330	Discussion on PRACH enhancements for msg3 beam improvement	Samsung

- R1-2105523	Other issues for coverage enhancement	Huawei, HiSilicon

- R1-2105657	Other Coverage Enhancements for PUSCH	Ericsson

- R1-2105906	Implications of DMRS bundling for PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

RAN4 #98e-bis:

- R4-2104580	Discussion on phase continuity and power consistency for UL repetition	MediaTek inc.

- R4-2104955	On non-zero gap between adjacent transmissions for PUCCH and PUSCH repetition		China Telecom

- R4-2105417	Reply LS  on PUCCH and PUSCH repetition	Qualcomm

- R4-2105418	Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition	Huawei, HiSilicon

- R4-2105477	Email discussion summary for [98-bis-e][140] NR_cov_enh	Moderator (Huawei)

- R4-2106918	Discussion on phase continuity for PUSCH and PUCCH repetitions 	InterDigital Communications

- R4-2107273	on phase continuty for multiple transmissions	HiSilicon Technologies Co. Ltd

- R4-2107366	Phase continuity over the gap in tx		Qualcomm Incorporated

RAN4 #99e:

- R4-2108800	Phase continuity with the other channels in the gap	Qualcomm Incorporated

- R4-2109012	Views on phase continuity and power consistency for PUSCH and PUCCH repetition	Sony

- R4-2109263	Further discussion on phase continuity for LS reply	InterDigital Communications

- R4-2109581	On phase continuity and power consistency for PUCCH and PUSCH repetition	China Telecom

- R4-2109743	Phase continuity and power consistency for PUSCH and PUCCH repetition	Nokia, Nokia Shanghai Bell

- R4-2110611	Discussion on reply LS on NR coverage enhancement	ZTE Corporation

- R4-2110612	Discussion on phase discontinuity and power inconsistency tolerance across different repetitions	ZTE Corporation

- R4-2111156	Further analysis on PUSCH/PUCCH repetition impacts	MediaTek Inc.

- R4-2111194	Reply LS to RAN1 latest question on phase discontinuity	Ericsson

- R4-2111195	Simulation assumption for phase tolerance for PUSCH PUCCH repeition	Ericsson

- R4-2111385	on phase continuty for multiple transmissions	Huawei, HiSilicon

- R4-2111386	simulation assumption for phase tolerance Cov_enh	Huawei, HiSilicon

- R4-2107880	Reply LS on PUCCH and PUSCH repetition	Qualcomm Incorporated

- R4-2107881	WF on phase continuity and power consistency for PUCCH and PUSCH repetition	Huawei, HiSilicon

- R4-2107957	Email discussion summary for [99-e][147] NR_cov_enh		Moderator (China Telecom)