3GPP TSG RAN Meeting #92-e									RP-211185

Electronic Meeting, June 14-18, 2021

## Status Report to TSG

Agenda item:			9.7.1.1

| WI / SI Name | Further Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FeMIMO |  |  |  |  |
| Unique ID | 860040 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202024 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 50% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#104bis-e, the following agreements were made:

Multi-beam enhancements:

Conclusion

On Rel.17 unified TCI framework, at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC, there is no consensus in supporting non-BM CSI-RS other than for tracking and non-BM SRS as source RS types for UL TX spatial filter reference

No further discussion in Rel-17

Agreement

On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 1:

- Opt 1-1A: Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request)

- Opt 1-1B: UE-initiated beam selection/activation based on beam measurement (without beam indication or activation from NW)

- Opt 1-2: Semi-static NW-configured beam selection (without beam indication and measurement/reporting)

- Opt 1-3: SSB grouping to reduce beam training

- Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency

- Note: Aim for at most one solution for Group 1 in Rel-17 to address issue 6

Agreement

On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 2:

- Opt 2-1A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking

- Opt 2-1B: Latency reduction for MAC CE based PL-RS activation

- Opt 2-1C: Latency reduction for MAC CE based PUCCH resource/resource group activation

- Opt 2-2: Direct SCell TCI state activation

- Opt 2-3: Replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update

- Opt 2-4: One-shot timing update for TCI state update

- Note: Aim for at most one solution for Group 2 in Rel-17 to address issue 6

- Note: At least for Opt 2-1A/B, 2-2, and 2-4, RAN2 and RAN4 will at least have to be involved (some may be exclusively RAN2 and/or RAN4 work)

Agreement

For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment:

- Use ACK/NACK mechanism analogous to that for SPS PDSCH release with both type-1 and type-2 HARQ-ACK codebook:

  - Upon a successful reception of the beam indication DCI, the UE reports an ACK

    - Note that upon a failed reception of the beam indication DCI, a NACK can be reported.

    - For type-1 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined based on a virtual PDSCH indicated by the TDRA field in the beam indication DCI, based on the time domain allocation list configured for PDSCH

    - For type-2 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined according to the same rule for SPS release

  - The ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, or provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1-2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI

- When used for beam indication:

  - CS-RNTI is used to scramble the CRC for the DCI

  - The values of the following DCI fields are set as follows:

    - RV = all ‘1’s

    - MCS = all ‘1’s

    - NDI = 0

    - Set to all ‘0’s for FDRA Type 0, or all ‘1’s for FDRA Type 1, or all ‘0’s for dynamicSwitch (same as in Table 10.2-4 of TS38.213)

    - FFS: Whether HPN is also used

- Use the existing TCI field (always present) to signal the following: 1) Joint DL/UL TCI state, 2) DL-only TCI state (for separate DL/UL TCI), 3) UL-only TCI state (for separate DL/UL TCI)

  - FFS: Whether both DL TCI and UL TCI states can be signaled in one instance of beam indication DCI

  - FFS: Relation with joint vs separate TCI (DL and/or UL) switching, including M/N>1 if supported

- In addition, use the following DCI fields as the fields are being used in Rel-16:

  - Identifier for DCI formats

  - Carrier indicator

  - Bandwidth part indicator

  - TDRA

  - Downlink assignment index (if configured)

  - TPC command for scheduled PUCCH

  - PUCCH resource indicator

  - PDSCH-to-HARQ_feedback timing indicator (if present)

- The remaining unused DCI fields and codepoints are reserved in R17

- Support UE to report whether or not to support TCI update by DCI format 1_1/1_2.

  - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional

  - FFS: How to handle the case when there is only UL data

  - FFS: The case for UE being indicated with separate UL TCI in DCI format 1_1/1_2 with DL assignment

- FFS: When more than one TCI codepoints are activated by MAC CE, the activated TCI state(s) for the lowest codepoint is/are applied

  - Support of this feature is UE optional

  - The “lowest codepoint” function can be configured on or off.

  - FFS: Interaction with the DCI based beam update if needed, whether/how to support the case with M or N > 1 if supported

- Note: This agreement on DCI beam indication design is not to be used to be against the support of the cases of M/N>1. The support of M/N>1 will be separately discussed and not dependent on the decision here.

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options:

- Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version

  - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported.

  - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting

  - FFS: how to determine the virtual PHR or the modified version.

- Opt 1D. {Rel.16 P-MPR based (beam/panel-level)}

  - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting

- Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)

  - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel

  - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR]

  - FFS: Whether/how to enhance existing beam reporting format to support Option 2A

  - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting

  - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated

  - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added

FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting

FFS: If differential report is supported when multiple UL beams are reported in the same report

Agreement

On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP,

- On the value of K (defined in RAN1#104-e as the number of beam qualities associated at least with non-serving cell(s) can be reported in a single CSI reporting instance),

  - For the supported maximum value(s) of K, down-select at least one from the following candidates {4, 8, 16}

  - FFS: whether the maximum value of K is a UE capability

- Periodic, semi-persistent, and aperiodic reporting (and the respective measurements) are supported.

  - Note: Semi-persistent and aperiodic reporting (and their respective measurements) are NW-initiated

Agreement

On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/reporting, down select and/or modify from the following candidates:

- Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance

  - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW

    - FFS: How to inform through CSI/beam reporting framework

  - FFS: Detailed design of the correspondence including the conveyed information

  - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16)

- Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports

  - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID

  - Note: The association between the new panel ID and the panel entity is determined by the UE

- Opt1-3: No additional specification support

- The duration in which the above panel entity reference is valid and the respective setting are FFS

Note: “panel entity” is only used for discussion purpose

Agreement

On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, for each of PUSCH, PUCCH, and SRS, in RAN1#105-e, further discuss to down-select or combine from the following alternatives:

- AltA. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state

- AltB. The setting of (P0, alpha, closed loop index) is also included with UL or (if applicable) joint TCI state

- AltC. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state

Note: It has been agreed that the setting of (P0, alpha, closed loop index) is associated with UL channel or UL RS (therefore the setting is channel- and signal-specific).

Agreement

On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP,

- In one reporting instance, depending on NW configuration, beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell

  - FFS: whether this applies to periodic, semi-persistent, and/or aperiodic

  - FFS: How to report the K beams and corresponding qualities if the Tx power among the non-serving cell and with serving-cell is not the same

  - Note: The supported numbers of non-serving cells (in terms of measurement/reporting) have not yet been decided. The above description doesn’t imply only one non-serving cell is allowed to be configured for measurement. Nor does this imply that only one non-serving cell is allowed in one reporting instance.

Agreement

On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, for L1-RSRP measurement and at least aperiodic reporting, investigate and, if needed, specify MAC CE based dynamic activation/deactivation of a subset of higher-layer-configured measurement for non-serving cell SSBs

Agreement

On Rel.17 unified TCI framework, in RAN1#105-e, further discuss to down select or combine from the following three alternatives for PL-RS (note: the text below is based on the agreed description in RAN1#104-e):

- AltA. PL-RS can be included in UL TCI state (or, if applicable, joint TCI state).

  - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state.

- AltB. PL-RS can be associated with (but not included in) UL TCI state (or, if applicable, joint TCI state)

  - FFS: Exact association mechanism

  - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state

- AltC. UE calculates path-loss based on periodic DL RS configured as the source RS for determining spatial TX filter in UL or (if applicable) joint TCI state

  - FFS: If a PL RS is not included in or associated with the UL TCI state (or, if applicable, joint TCI state), whether the UE can estimate path-loss based on the PL-RS of an UL RS provided in an UL TCI state (or, if applicable, joint TCI state) as a source RS for determining the spatial TX filter.

In addition:

- FFS (to be decided in RAN1#105-e) whether a fallback scheme is needed and, if so, the details

- FFS: Support additional UE capability to report whether above PLRS determination mechanism is supported

- Note: As agreed in RAN1#104-e, the total number of maintained PL-RSs per CC is no more than 4

- FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates based on UE capability

- FFS: UE capability for maximum number of active PL-RS across CCs per band

Agreement

On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following:

- UE reporting of panel-specific information as a UE capability, for example:

  - Information related to the total number of DL/UL panel entities

  - Information related to the number of (max) antenna ports/layers per panel entity

  - Information related to the maximum number of resources per panel entity for SRS BM

  - Information related to panel selection delay

  - Information related to panel activation delay

- UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling

- UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only

  - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling)

- UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status

- Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation

- Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed

Agreement

On Rel.17 enhancements for MPUE, for codebook based UL transmission, decide by August RAN1 meeting whether to support CB-based SRS resources with different numbers of ports

- FFS details (e.g. per resource or per resource set)

- Note: the above is not for Rel-16 full power transmission but for Rel-17 panel-specific UL transmission

- FFS: non-codebook based UL transmission for MPUE

- FFS whether existing BWP switch based mechanism (discussed previously in Rel-16 power saving WI) can serve such purpose

Multi-TRP enhancements for PDCCH, PUSCH, and PUCCH:

Agreement

When DL DCI is transmitted via PDCCH repetition, for PUCCH resource determination for HARQ-Ack when the corresponding PUCCH resource set has a size larger than eight, starting CCE index and number of CCEs in the CORESET of one of the linked PDCCH candidates is applied, and option 2 is supported

- Option 2: The one with the lowest SS set ID is applied.

- FFS: Support of Option 2 does not mean PDCCH repetition based on two linked search space set within one CORESET is supported

Agreement

For PDSCH rate matching around the scheduling DCI in the case of PDCCH repetition, the previous agreement for FR1 also applies to FR2.

Agreement

For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support

- UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates

  - Candidate values: 2, 3.

- FFS: Default behaviour

- FFS: Whether one of the candidate values imply that UE supports soft combining

- FFS: Whether additional candidate values are supported (e.g. non-integer numbers)

- FFS: RRC configuration based on reported UE capability

Agreement

If a PDSCH with mapping Type B is scheduled by a DCI in PDCCH candidates that are linked for repetition

- For the purpose of the earliest time that the PDSCH can be scheduled as well as for the purpose of the reference symbol for SLIV (when UE is configured with ReferenceofSLIV-ForDCIFormat1_2, and when receiving the PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0), a reference candidate is used. Select one among the following:

  - Alt1: The candidate that starts later in time

  - Alt3: The candidate that starts earlier in time

- FFS: How to define d1,1 d1,1for PDSCH processing time in this case

Agreement

If a PDSCH is scheduled by a DCI in PDCCH candidates (the first PDCCH candidate associated with a first CORESET and the second PDCCH candidate associated with a second CORESET) that are linked for repetition,

- Working assumption: The UE expects the same configuration for the first and second CORESETs wrt presence of TCI field in DCI.

- If the TCI field is not present in the DCI, and the scheduling offset is equal to or larger than timeDurationForQCL if applicable, PDSCH QCL assumption is based on the CORESET with lower ID among the first and second CORESETs

- FFS: Whether additional options are needed (e.g. to enable SDM/FDM/TDM PDSCH schemes w/o TCI field in the DCI)

Agreement

For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs

- FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs

- Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.

Agreement

When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, for the purpose of BD counting and interpretation of a detected DCI, select one option among the following in RAN1#105-e:

- Option 1: The individual candidate is not counted for monitoring

  - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate).

- Option 2: The candidate in a higher SS set ID is not counted for monitoring

  - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules).

  - FFS: Impact to the other linked PDCCH candidate

- Option 3: The candidate associated with SS set(s) with lower priority is not counted for monitoring, where for two linked SS sets, the priority is according to one of the two SS sets with a lower SS set ID

  - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules).

  - FFS: Impact to the other linked PDCCH candidate

- FFS: Whether a max limit on number of such overlaps is needed.

Additional specification support may be introduced for the purpose of resolving ambiguity (if any) for interpretation of the detected DCI. For example,

- Distinguished by different RNTIs defined for the linked candidate versus the individual candidate

- Distinguished by aggregation level restrictions that can be expected by the UE in the case of overlap

Agreement

For PDCCH repetition with two linked candidates, if due to Rel. 15/16 procedures, one of the linked candidates is not monitored (is dropped), select one option from Options 1 and 2 in RAN1#105-e:

- Option 1: UE still monitors the linked candidate that is not dropped and interprets the DCI based on Rel. 17 PDCCH rules (wrt reference PDCCH candidate)

- Option 2: Even the candidate that is not dropped is not monitored (Both linked candidates are dropped if at least one of them is dropped)

- FFS: Which of the following Rel. 15/16 rules are applicable for this purpose:

  - Case 1: Overlap with SSB

  - Case 2: Overlap with rate matching resources: RateMatchPattern, lte-CRS-ToMatchAround, or LTE-CRS-PatternList-r16, availableRB-SetPerCell-r16

  - Case 3: Due to TDD DL/UL related conflicts: Overlap with semi-static / dynamic UL symbols or overlap with PRACH

  - Case 4: QCL-TypeD prioritization rule among CORESETs result in one of the linked candidates not being monitored

  - Case 5: Overbooking results in one of the linked candidates not being monitored

  - Case 6: Overlap with reserved PRB(s) and OFDM symbol(s) indicated by DCI format 2_1 where UE may assume no transmission intended for the UE

  - Other cases are not precluded

- FFS: Whether there is an impact to BD count

Agreement

For the case of multi-TRP, to support per-TRP power control in FR1, the linking of PUCCH resource with [one or] two power control parameter sets, the following is supported

- MAC-CE indicates RRC IE that configures power control parameter sets (p0, pathloss RS ID, and a closed-loop index).

  - The exact design of RRC IE is up to RAN2 but from RAN1 point of view, one possible example is to reuse PUCCH-SpatialRelationInfo except for the referenceSignal

Note: It is common understanding in RAN1 that one PUCCH resource can be linked to one power control parameter set.

Conclusion

With reference to the normative work on NR-feMIMO:

Related to the support of switching gap between UL transmissions towards two TRPs in RAN1 specifications, there is no consensus in RAN1 to specify symbol gap(s) for the following cases

- PUSCH Type A

- PUCCH scheme 1

- PUSCH Type B

- PUCCH scheme 3

The above applies for the case included in the LS from RAN4 in R1-2102297.

Agreement

When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting,

- Option 1

  - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15).

  - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam.

- Option 2:

  - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact)

- Option 3:

  - Frequency hopping is performed on slot level as in Rel-15 (no spec impact).

Agreement

When SRS resources from two SRS resource sets indicated in DCI format 0_1/0_2, for linking SRI fields to two power control parameters, it is up to RAN2 to finalize the RRC details related to linking. RAN1 identified that the following options could be used.

- Alt. 1: Add second sri-PUSCH-MappingToAddModList, and select two SRI-PUSCH-PowerControl from two sri-PUSCH-MappingToAddModList

- Alt. 2: Add SRS resource set ID in SRI-PUSCH-PowerControl, and select SRI-PUSCH-PowerControl from sri-PUSCH-MappingToAddModList considering the SRS resource set ID

Agreement

For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting.

- Option 1:  Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted)

- Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them

  - FFS: How to select the PHR for reporting.

- Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs

- Option 5: No changes to legacy PHR reporting

Agreement

When MAC-CE indicates a PL-RS ID for one or more SRI IDs, it also indicates whether the SRI IDs are associated with the first or the second SRS resource set.

Agreement

For multiplexing A-CSI on two PUSCH repetitions in the case of multi-TRP PUSCH repetition,

- For S-DCI based multi-TRP PUSCH repetition Type B, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the first (X = 1) PUSCH repetition corresponding to the second beam.

  - The UE is expected to follow the above operation for multiplexing A-CSI on two PUSCH repetitions only if

    - the first actual repetition corresponding to the first beam and the first actual repetition corresponding to the second beam does not have the same number of symbols, and

    - UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions.

  - When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16.

- The content for the two A-CSI should be the same

- Note: RAN1 has the assumption on CSI timelines are followed as rel-15/16, including UE shall expect the timeline for the first A-CSI meets Z and Z’ requirement

- FFS: For s-DCI based multi-TRP PUSCH repetition Type A and B, support multiplexing of A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH.

  - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions.

  - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation).

Working Assumption

For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition,

- Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation

  - FFS: Whether the new field is 1 bit or 2 bits

Working Assumption

For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first KxKx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx)(2N2- Kx) codepoint(s) are reserved.

Agreement

For the indication of open-loop power control parameter (OLPC) in DCI format 0_1/0_2, support enhanced open-loop power control parameter (OLPC) set indication by indicating per-TRP OLPC set.

- FFS: Details of indication.

Agreement

For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. For each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky KyTPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved.

- How to describe/capture this in 38.212 is up to the editor.

Agreement

Confirm the following Working Assumption:

For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions.

- FFS: Applicability of mapping patterns for different beam switching gaps

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows,

  - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions.

  - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions.

Agreement

Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED):

- For beam mapping /power control parameter set mapping for PUCCH repetitions,

  - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions).

  - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots.

  - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

Agreement

Confirm the following working assumption (with removing the last bullet):

For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams.

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- FFS: Support of half-half mapping.

- FFS: Additional considerations on mapping patterns (including required beam switching gaps)

Agreement

For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting,

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs.

- Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP.

- Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP

  - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports.

  - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

Agreement

For type 1 or type 2 CG based multi-TRP PUSCH repetition,

- Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’

- For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'.

- For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI.

- FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1

- FFS3: Details on RV mapping.

- FFS4: Possible transmission occasion for initial transmission

- FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'.

Multi-TRP enhancements for multi-cell mTRP:

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

Agreement

For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex

- Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex

Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e

Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

Multi-TRP enhancements for beam management

Agreement

For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

Agreement

On CMR resource configuration for beam reporting option 2, adopt the following alternative:

- Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

  - FFS: extension to aperiodic CMR resource setting

- Each reported beam pair in a single CSI-report consists of M = 2 SSB-RI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset.

- Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above.

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement

Adopt the following beam failure detection criteria for each BFD-RS set

- The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold

  - X is max{minimal periodicity of BFD RS in the set, 2ms}

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Agreement

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs,

- Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

  - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

  - Alt-3: Leave it up to UE implementation

- Note: PUCCH-SR resource is PUCCH resource carrying SR

- FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

Agreement

On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”:

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported

  - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset.

- FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

Multi-TRP enhancements for HST-SFN:

Agreement

Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission

- The corresponding MAC CE includes at least the following fields

  - Serving cell ID

  - CORESET ID

  - Two TCI state IDs

- FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE

- FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17. LS is endorsed in R1-2104064

Agreement

Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both:

- UL RS based Doppler estimation by gNB

  - FFS: Details including UL RS enhancement

- DL RS based Doppler feedback by UE

  - FFS: Details

  - FFS: Whether UE capability needs to be introduced

- Whether to support one or both will be decided later

Agreement

- Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - This feature is UE optional

- FFS all other details including RRC signalling, possible RAN4 impact (if any), etc.

Working Assumption

All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1

Agreement

Support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a

- FFS: Whether dynamic switching is additionally supported

For future meeting:

Companies to consider Proposal #3-8a in FL summary (R1-2104020) for future meetings.

Companies to consider Proposal #3-10 in FL summary (R1-2104020) for future meetings.

Agreement

Scheme 1 for PDSCH is identified by

- New RRC parameter and the number of TCI states indicated by DCI

  - FFS RRC configuration details, e.g., per BWP or per CC

  - FFS whether or not restriction to a single CDM group for DM-RS is also supported

SRS enhancements:

Agreement

For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values

- N_symbol = 8, R = {1, 2, 4, 8}

- N_symbol = 12, R = {1, 2, [3], 4, 6, 12}

- FFS the following configurations

  - N_symbol = 10, R = {1, 2, 5, 10}

  - N_symbol = 14, R = {1, 2, 7, 14}

- FFS options to reduce SRS BW for R>1

Agreement

On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values

- 1T6R: N_max = 3

- 1T8R: N_max = 4

- 2T6R: N_max = 3

- 2T8R: N_max = 4

- [4T8R: N_max = 2]

- The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS.

- FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed

Agreement

For RB-level partial frequency sounding (RPFS) in Rel-17

- The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1}

  - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern

- Support to determine PF and Noffset at least via RRC configuration per SRS resource.

  - FFS whether to introduce DCI and/or MAC CE in addition

Working Assumption

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Agreement

On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e

- Alt 1: All the non-zero integer values <= N_max are supported for N

- Alt 2: Support N=N_max only

- Alt 3: Support specific N values <= N_max

- FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot

- FFS: whether different alternatives may be selected for different xTyR configuration

Agreement

Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points

- Alt 1: The maximum number of CSs for Comb-8 is 6

- Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Agreement

- Up to 4 “t” values can be configured per SRS resource set.

Agreement

- For RPFS in Rel-17, support PF = {2, 4}.

- FFS  3, 8, 12, 16 or fractional numbers

- Support at least one of the following alternatives (to be decided in RAN1#105-e)

  - Alt 1: 1PFmSRS, BSRS is an integer value

  - Alt 2: 1PFmSRS, BSRS is an integer value with minimum value 4

  - Alt 3:  is a multiple of 4

  - Alt 4: Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2

Agreement

On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2

Agreement

For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e)

- Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence

- Alt 2: Truncate from legacy length-12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS

Agreement

For antenna switching, support one of the following

- Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS

- Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set

  - Note: the two SP-SRS resource sets are not activated at the same time.

- FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed

- FFS whether configurations on SRS repetitions have impact

- FFS relevant UE capability design

CSI enhancements:

Agreement

For rank=1, polarization-common based free-selection should be supported for W1W1.

- FFS: Whether there is a need to restrict the number of CSI-RS ports for which this is supported

Agreement

Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4:

- {1, 1}, {1, 2}, {2,1}, {2,2}

- FFS: CBSR and/or RI restrictions per TRP or across TRPs

Agreement

With regarding to the maximal values of Nmax for N, Ks,max for Ks:

- Support of Nmax=2 is a UE optional feature

- Support of Ks,max=X is a UE optional feature

  - X can be up to 8 and other candidate values can be discussed as part of UE features

- FFS: Default value of Nmax, Ks,max

- FFS: Which combinations of N<=Nmax, Ks<=Ks,max are supported

Agreement

At least for rank 1, combinatorial coefficient is used for port selection for W1.

- FFS when Wf is turned off

Agreement

Confirm following working assumption of Wf for R17 PS CB

- Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters.

Agreement

With regarding to possible restriction between K1 and K2

- Alt 2: No restriction as long as K1+K2=Ks

Agreement

The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource.

Agreement

For the UE be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses (i.e. Option 2),

- Alt 1: Single CRI is reported whereas CRI bit size depends on total number of valid CMR pairs for NCJT measurement hypothesis and valid CMRs for single-TRP measurement hypotheses.

  - FFS further mapping mechanism between each CRI codepoint and Single-TRP/NCJT measurement hypothesis.

Agreement

A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications:

- Within CSI part 1

  - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details

  - FFS whether RI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead

  - FFS whether additional field is needed, at least for Option 2

- Within CSI part 2:

  - FFS further compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses

Agreement

Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses, study following Alternatives and down-select one Alternative in RAN1#105-e:

- Alt 1: It is feasible for FR1 but not for FR2.

- Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2.

Agreement

Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis, study following Alternatives and down-select one Alternative in RAN1#105e:

- Alt 2: It is feasible for FR1 but it is not for FR2. For FR2, the UE is expected to have different NZP CSI-RS resources configured for all CMRs of Single-TRP and NCJT measurement hypotheses respectively.

- Alt 3: It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

Agreement

For the UE configured to report X CSIs (at least when X>0) associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis, study following issues for potential CSI omission/priority/updating rules:

- Issue 1: Prioritize CSI with different measurement hypotheses within the single CSI report, when the UE is configured with CSI Option 1 with X=1 or 2.

- Issue 2: Omission of NCJT CSI in CSI part 2 depending on the corresponding CRI or RI or CQI in CSI part 1.

Agreement

For the UE configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis (i.e. Option 1),

- Alt 1: X+1 CRIs are reported, whereas X CRIs are for single-TRP measurement hypotheses and one CRI is for NCJT measurement hypothesis.  Each CRI bit size depends on the corresponding number of either valid CMR pairs for NCJT measurement hypothesis or valid CMRs for single-TRP measurement hypotheses

- FFS: Whether the X+1 CRIs are reported jointly as one CSI report or as separate CSI reports.

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, study following aspects:

- whether to support dynamic updating, e.g. by MAC-CE,  for CMR pairs for NCJT measurement hypotheses, and/or CMRs for Single-TRP measurement hypotheses, and/or TCI states in CMRs, and/or the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report

- whether additional high layer signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses

- For CMRs configured in the CSI-RS resource set, whether support high layer signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis

For future meetings:

Companies to study whether a CSI-IM can be referred by both NCJT and Single-TRP measurement hypotheses. Consider following Alternatives and FR1/FR2 differentiation:

- Alt 1: CSI-IM can be shared by both NCJT and Single-TRP measurement hypotheses.

- Alt 2: A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis

Agreement

Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM, study following Alternatives and down-select one Alternative in RAN1#105e:

- Alt 1: Yes, it is supported, subject to limitations, e.g. N=1 CMR pair and Ks=2 CMR resources

- Alt 2: No, it is not supported

Agreement

At least for rank 1, regarding the value(s) of K1 for port selection matrix W1 in NP*K1, study and down-select from the following candidate values of K1 and the maximal value of P in RAN1 105e

- K1 in {2,4,8,12,16,24,32} with K1 <= P

- The maximal value of P as Pmax, e.g.  32

- FFS: possible parameter combinations/dependence for K1 with other PS CB parameters, e.g. whether different candidate values of K1 should be configured for different ranks (if rank>1 is supported).

- FFS: Whether any value of K1 up to P can be supported for some codebook parameters

- Note: for Polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations.

Note: for polarization-specific based free-selection, it means select K1 ports out of P ports

Note: P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study)

Agreement

A bitmap for indication non-zero coefficients should be supported for W2 with a compression coefficient beta<=1 whereas

- FFS values of beta < =1, e.g. 1/8, 1/4, 1/2, 3/4, 1

- FFS: whether/how such a bitmap can be absent for specific codebook configuration parameters

- FFS: whether a bitmap is polarization-common or polarization-specific whereas polarization-specific bitmap is the baseline

- FFS: possible parameter combinations/dependence for beta with other PS CB parameters

Agreement

At least for rank 1, the FD bases used for Wf quantitation are limited within a single window/set with size N configured to the UE, study and down-select one Alternative in RAN1 105e:

- Alt 1: FD bases in the window must be consecutive from an orthogonal DFT matrix

- Alt 2: FD bases in the set can be consecutive/non-consecutive, and are selected freely by gNB from an orthogonal DFT matrix

- FFS: Applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv

- FFS: Whether this applies when Wf is turned OFF

Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank.

Agreement

At least for rank 1, for relationship between N and Mv, study and down-select one Alternative from following in RAN1#105e

- Alt 1: N= Mv always

- Alt 2: N >= Mv and FFS candidate value(s) of N, e.g. 2, 4

- FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv

- FFS: Whether this applies when Wf is turned OFF

Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/ not supporting additional alternatives for higher rank.

Agreement

At least for rank 1, regarding the value(s) of R for Rel-17 PS codebook enhancement, study and down-select one or more than one Alternative (or a subset of corresponding values) in RAN1 105e:

- Alt 0:  R < 1 (e.g. 1/4, 1/2)

- Alt 1: R=1

- Alt 2: R=1 and 2

- Alt 3: R=1,2, 4, and 8

- Alt 4: R= {1,2,…, D*NPRBSB} whereas D is the density of CSI-RS in frequency domain

- FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv

- FFS: Whether this applies when Wf is turned OFF

Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank.

Agreement

For the quantization of W2 coefficient, study following Alternatives with Alt 1 as the baseline:

- Alt1: Reusing Rel-16 quantization mechanism for Rank 1 at least, which can be summarized as following:

  - An indicator for the strongest coefficient

  - Two polarization-specific reference amplitudes:

    - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

    - for the other polarization, reference amplitude is quantized to 4 bits

  - For coefficients other than the strongest coefficient

    - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - phase is quantized to 16PSK

- Alt1-1: the ref amplitude = 0 reserved in R16 can be replaced with a new value, e.g. (1/2)^(1/8), (1/2)^(3/8)

- Alt2-0: Individual amplitude (e.g. 3 or 4 bits with Rel15/16 amplitude codebooks) and phase (e.g. 16PSK) quantization

  - FFS: amplitude codebook is uniform in db or linear scale

  - FFS: support a strongest coefficient indicator, and individual quantization for other non-zero coefficients.

- Alt2-1: ref amp (e.g. 4 bits), Individual amplitude (e.g. 3 bits) and phase (e.g. 16PSK) quantization for each non-zero coefficient

  - FFS: amplitude codebook is uniform in db or linear scale

  - FFS: reference amplitude is polarization specific or polarization common, and corresponding codebook

- Note: Other quantization schemes or enhancement on top of Alt 1 or Alt 2 are not precluded.

Agreement

For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, down-select ONE option for CSI-RS configurations associated with Rel-17 PS codebook, from Option 0 (No further enhancement), Option 1 (i.e. lower CSI-RS density) and Option 3 (i.e. configuring multiple CSI-RS resources)

- If there is no consensus in RAN1#105e, Option 0 is by default.

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, an NCJT CSI hypothesis based on a pair of CMRs assumes to occupy two CPUs, two active NZP CSI-RS resources, and a number of active ports corresponding to both CMRs.

- If a NZP CSI-RS resource is referred X times by CMR pairs for NCJT measurement hypothesis and CMR for Single-TRP measurement hypothesis, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times for active resources and active ports.

- Note: For above CSI computation, UE assumes PDSCH transmission is single-DCI based multi-TRP scheme(s). FFS: Multi-DCI based multi-TRP scheme

In RAN1#105-e, the following agreements were made:

Multi-beam enhancements:

Agreement

On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication.

- Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability

- FFS: Application time and whether additional offset is needed for the application time in case of cross carrier beam indication and common TCI state ID update across a set of configured CCs if CCs have different SCSs

- FFS: Whether inter-cell beam switching needs higher X/Y values than intra-cell

- FFS: Whether application time can be indicated/determined dynamically for different scenarios, e.g. cross CC, inter-cell, inter-panel without reverting previous RAN1 agreements

Agreement

For M=N=1, on Rel-17 unified TCI, for separate DL/UL TCI, one instance of beam indication using DCI formats 1_1/1_2 (with and without DL assignment) can be used as follows:

- One TCI field codepoint represents a pair of DL TCI state and UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state and UL TCI state.

- One TCI field codepoint represents only a DL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state, and keeps the current UL TCI state.

- One TCI field codepoint represents only an UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding UL TCI state, and keeps the current DL TCI state.

FFS: the cases of M or N>1

Agreement

On Rel.17 enhancements to facilitate advanced beam refinement/tracking, focus study (including down-selection) and, if needed, specification effort on the following options:

- Group 1: Aim for at most one solution for Group 1 in Rel-17 to address issue 6

  - Opt 1-A. UE-initiated beam selection/activation based on beam measurement and/or reporting (without beam indication or activation from NW)

  - Opt 1-B. Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request)

  - Opt 1-C. Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency

- Group 2: Aim for at most one solution for Group 2 in Rel-17 to address issue 6

  - Opt 2-A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking

  - Opt 2-B: Latency reduction for MAC CE based PL-RS activation

  - Opt 2-C: One-shot timing update for TCI state update

Agreement

On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state.

- Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability

  - Note: The term “beam misalignment” is for discussion purpose only

- Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2

- The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state

- The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability

- FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration

- FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS

- Note: PL-RS is assumed to be periodic

Agreement

On Rel.17 unified TCI framework,

- Any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool)

  - Note: This does not imply that all such DL RSs necessarily share a same TCI state

  - The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH

- FFS: Whether some SRS resources or resource sets for BM can be configured as a target signal/channel of a Rel-17 UL TCI (hence the Rel-17 UL TCI state pool)

- Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL/UL TCI (this issue is still TBD)

Agreement

On Rel.17 unified TCI framework, discuss and decide by RAN1#106-e (August 2021)

- Whether each of the following DL RSs can share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC

  - CSI-RS resources for CSI

  - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’)

  - CSI-RS for tracking

  - DMRS(s) associated with non-UE-dedicated reception on PDSCH and all/subset of CORESETs

- Whether some SRS resources or resource sets for BM can share the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC

Agreement

On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), discuss and down-select by RAN1#106-e (August 2021) between the following two alternatives:

- Alt1. Rel-15/16 TCI state update signaling/configuration mechanism(s) are reused to update/configure the Rel-17 TCI state

- Alt2. Rel-17 TCI state update signaling/configuration mechanism(s) are used, e.g. with Rel-17 MAC-CE/DCI-based beam indication for Rel-17 joint/separate TCI

Note: The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH

Note: For some channels/signals, only one of the above two alternatives may apply (to be discussed).

Agreement

On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP,

- Support at least K=4, where K is defined as the number of beams associated at least with non-serving cell(s) reported in a single CSI reporting instance

  - The maximum value of supported K is a UE capability

  - K is configured by NW based on the UE capability

  - FFS: The support of K=8 and 16

    - For K>4, the maximum number of beams associated with one cell is 4

- FFS: Support L1-based event-driven reporting based on Rel-16 SCell BFR framework or analogous to L3-based event-driven reporting, including the definition of L1-based event, if needed

Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies

Agreement

On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, decide by RAN1#106-e whether to support the following RS types as measurement RS or not:

- CSI-RS for mobility/RRM associated with a non-serving cell

- CSI-RS for BM associated with a non-serving cell

- CSI-RS for tracking associated with a non-serving cell

Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies

Note: An RS is associated with a non-serving cell means that it is either configured for a non-serving cell or configured for a serving cell but is QCLed with a non-serving cell SSB

Working Assumption

On Rel.17 beam indication enhancements for L1/L2-centric inter-cell mobility, support the following:

- Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation)

  - FFS (to be decided in RAN1#106-e): Whether this also applies to PDSCH/PUSCH associated with UE-dedicated CORESETs only or additional target channels (e.g. UE-dedicated PDCCH/PUCCH)

  - FFS: Whether the above is supported only for joint TCI, or both joint TCI and separate DL/UL TCI (including that, if separate DL/UL TCI is supported, the DL TCI and UL TCI associated with a same cell)

  - FFS: Whether to support activation of TCI states for more than one cells simultaneously

  - FFS: Whether down-selection between MAC-CE only based and MAC-CE+DCI-based beam indication scheme is necessary

- The DL QCL and UL spatial relation rules already agreed for intra-cell scenario

- FFS: The use of SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference for UE-dedicated PDSCH

  - FFS (to be decided in RAN1#106-e): Whether this also applies to UE-dedicated PDCCH

  - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

  - FFS (to be decided in RAN1#106-e): Whether SSB associated with a physical cell ID different from that of the serving cell can also be used as a direct QCL reference (source RS) for UE-dedicated PDCCH/PDSCH

Agreement

On Rel.17 unified TCI framework, for common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of configured CCs/BWPs

- The source RS determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for a target CC can be configured in the target CC or other CC

- For intra-band CA, the following configurations can be supported without additional QCL rules:

  - One source RS across CCs can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs

  - One source RS per CC can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs, and the CC-specific source RSs are further associated with a same QCL-TypeD RS

- “A set of configured CCs/BWPs” includes all the BWPs in the set of configured CCs

Working Assumption

For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs:

- RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16

  - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported

- RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC

  - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured

  - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC

- When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies

- Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1

- FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band

- FFS: How to define reference BWP/CC

Agreement

On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework,

- For each of PUSCH and PUCCH, the setting of (P0, alpha, closed loop index) can be associated with UL or (if applicable) joint TCI state per BWP.

  - In this case, multiple settings are configured. Each setting can be associated with at least one TCI state, and, for a given TCI state, only one setting for PUSCH and only one setting for PUCCH can be associated at a time.

  - (Working Assumption) In this case, for each of the PUSCH and PUCCH, each of the activated UL or (if applicable) joint TCI states is associated with one of the settings.

- If not associated, for each of the PUSCH and PUCCH, the setting(s) of (P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or (if applicable) joint TCI states

- FFS: If the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state.

- FFS: (to be decided in RAN1#106-e) whether to configure the same setting of (P0, alpha, closed loop index) per TCI state across channels and apply a channel dependent component, or configure a channel dependent setting of (P0, alpha, closed loop index) per TCI state

Agreement

On Rel-17 unified TCI, in RAN1#106-e, for M>1 and/or N>1:

- Identify and agree on use cases

- Decide whether to support M>1 and/or N>1, and if so, the maximum value of M and/or N

- If supported, identify feasible candidate schemes for beam indication signaling mechanism (including TCI state activation)

Note:

- Previous agreement in RAN1#104b-e that remaining unused DCI fields and codepoints are reserved in R17 are not to be reverted.

- The use case of simultaneous UL transmission from multiple UE panels are not to be considered in Rel-17 as it is out of scope

Conclusion

On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling

- There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based)

For future meetings:

Continue to study necessary enhancements to optimize transmission from UEs with different number of max number of UL MIMO layers per panel entity

Multi-TRP enhancements for PUSCH and PUCCH:

Agreement

For indicating per-TRP OLPC set in DCI format 0_1/0_2, if two SRI fields present in the DCI,

- Use the existing field (1 bit) for OLPC set indication and a second p0-PUSCH-SetList-r16.

  - if value of the field equals to ‘0’, the UE determine value of P0 from SRI-PUSCH-PowerControl with a sri-PUSCH-PowerControlId value mapped to the SRI field value corresponding to each TRP.

  - if value of the field equals to ‘1’, the UE determine value of P0 from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value corresponding to each TRP.

Agreement

For s-DCI based multi-TRP PUSCH repetition Type A and B, support transmitting A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH.

- The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions.

- The UE is expected to follow the above operation for transmitting A-CSI on two PUSCH repetitions only if

  - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation).

  - For PUSCH repetition Type A and B, UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions.

- When the UE does not follow the above operation, UE transmits A-CSI only on the first PUSCH repetition similar to Rel. 15/16.

- Note: The scheduling offset for the first A-CSI should meet the Z and Z’ requirement

Agreement

For s-DCI based multi-TRP PUSCH repetition Type A, the UE is expected to multiplex A-CSI on two PUSCH repetitions only if UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions.

- When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16.

Agreement

For multi-TRP PUCCH (scheme 1 and 3) and PUSCH (Type A and B) repetition, when the number of repetitions is equal to two, the first and second transmission occasion shall be associated with two TRPs, respectively (two UL beams or Power control parameter sets), regardless of the configured mapping pattern.

- Note: For M-TRP PUSCH type B, the number of repetitions refers to ‘nominal’ repetition.

Agreement

The following working assumption is confirmed.

For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first Kx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx) codepoint(s) are reserved.

Agreement

For type 2 CG based multi-TRP PUSCH repetition:

- The first (legacy) RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ are associated with the first SRS resource set.

- The second (new) RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ are associated with the second SRS resource set.

- Applying the first, second, or both first and second RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ is determined from the new DCI field (for dynamic switching) of the activating DCI similar to the case of DG-PUSCH.

Agreement

Confirm the Working Assumption (with supporting two bits for the new field).

- For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition,

  - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation.

  - The new field is 2 bits

Agreement

For the new field in the DCI for dynamic switching, support Alt.1 (modified).

Alt.1

- Support 2 bits with the following combinations.

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field (2nd field is unused) |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 1st SRI/TPMI field (2nd field is unused) |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: FFS 2nd SRI/TPMI field: FFS | Both 1st and 2nd SRI/TPMI fields |

- The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set.

  - For codebook and non-codebook usage, respectively

- The same number of SRS resource shall be configured in the two SRS resource sets.

Agreement

For SP-CSI report on mTRP PUSCH repetition Type A and B activated by a DCI, further study the use of a similar mechanism to A-CSI multiplexing on M-TRP PUSCH without a TB, which includes the following,

- When SP-CSI multiplexed on m-TRP PUSCH, SP-CSI multiplexed on the two repetitions associated with the two TRPs, and the number of repetitions is always assumed to be 2, regardless of the value indicated.

- Reuse similar conditions (e.g. UCIs other than the A-CSI are not multiplexed, same number for first actual repetitions, the content of the CSI is the same) to support SP-CSI multiplexing on m-TRP PUSCH as defined in A-CSI multiplexing on M-TRP PUSCH.

Agreement

Confirm the working assumption with removing brackets on [consecutive] and adding UE capability.

- For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats.

  - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot.

  - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT

- Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation.

- This feature is optional.

Conclusion

For multi-TRP PUCCH schemes, only one ‘twoPUCCH-PC-AdjustmentStates’ parameter is configured for both TRPs, and the parameter is shared across both TRPs, which means there will be two closed loops in total (no RAN1 spec impact).

For future meetings:

Further study the enhancements needed on grouping of PUCCH resources for Rel-17 multi-TRP PUCCH repetition

Agreement

- To support per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2, a second TPC field can be configured via RRC.

- When the second field is configured by RRC, a second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2 (option 3).

  - Each TPC field is for each closed-loop index value respectively

    - FFS: Whether or not the mapping between the TPC field and the PUCCH transmissions is needed

- When the second field is not configured by RRC, a single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for the closed loop index(es) for the scheduled PUCCH

- To support per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, adopt the same solution as with M-TRP PUCCH schemes.

  - FFS: any additional considerations

- Support UE to report the capability on whether it supports the second TPC field

- Note1: Per TRP closed-loop power control is only applicable when the “closedLoopIndex” values are not the same for TRPs.

Agreement

For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP. Select one from the following in RAN1 #106-e meeting,

- Alt.1

  - The first P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the first SRS resource set.

  - The second P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the second SRS resource set.

  - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

- Alt.2

  - The first set of values {the first value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if  twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2.

  - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

- Alt.3

  - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the first set of values {the first value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the first SRS resource set and closed-loop index l = 0} is used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControlassociated with the second SRS resource set and closed-loop index l = 1 if  twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise} is used for TRP2.

  - Otherwise, the first set of values {the first value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id=0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if  twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2.

  - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

For further study in future meetings:

For PHR reporting related to M-TRP PUSCH repetition, study following aspects related to option 4,

- Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.

- FFS1: How the PHRs are calculated for reporting (actual PHR or virtual PHR)

- FFS2: How the PHRs are calculated for reporting for other CCs if the multi-cell PHR MAC CE is applied.

- FFS3: Required changes to triggering conditions including the required higher layer parameters (e.g.,’phr-PeriodicTimer’, ‘phr-ProhibitTimer’, ‘phr-Tx-PowerFactorChange’ as TRP specific).

- FFS4: Report P-MPR and MPE per TRP within the same MAC-CE extension.

Note: Down-selection between Options 1-5 will be based on this study as well as the trade-off between benefit versus UE complexity.

Multi-TRP enhancements for beam management

Agreement

For CMR configuration for option 2, adopt

- Alt-1: “set”

Agreement

The bitwidth of each SSBRI/CRI is determined based on the number of SSB/CSI-RS resources in the associated CMR resource set

- FFS: specify the association between SSBRIs/CRIs in a reported group and CMR resource sets

Agreement

- For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17.

  - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how

- The number of beam groups (N) reported in a single CSI-report

  - Alt1: The value of N is configured by RRC signalling

Agreement

Select one of the following alternatives with possible modification in RAN1#106-e

- Alt 2.5.2 A:

  - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 B:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 C:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 D:

  - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

Multi-TRP enhancements for HST-SFN:

Agreement

Confirm the following working assumption from RAN1#104b-e:

All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1.

Agreement

UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with scheme 1 PDSCH by RRC , but not capable to support dynamic switching between scheme 1 and single-TRP by TCI state field in DCI Format 1_1/1_2

Agreement

For specification based TRP-based frequency offset pre-compensation scheme

- Support dynamic (DCI -based) switching with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - This feature is UE optional

  - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with TRP-based frequency PDSCH by RRC , but not capable to support dynamic switching between TRP-based frequency and single-TRP by TCI state field in DCI Format 1_1/1_2

- Support semi-static (RRC based) switching with Rel-16 schemes 1a, 2a, 2b, 3, 4

- Support semi-static (RRC based) switching with Rel-17 scheme 1 (PDSCH)

Agreement

Enhanced MAC CE signaling is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Working Assumption

For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) are supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states.

- FFS: Additional support of Variant B

Agreement

- For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type

  - FFS rule or signalling to determine which TCI state with dropped QCL parameters

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.

  - FFS whether this restriction is per UE or per CC

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs.

  - FFS whether this restriction is per UE or per CC

Agreement

Enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is identified by the number of TCI states activated per CORESET and RRC parameter

- FFS: Configuration detail of RRC parameter

  - Including whether the same RRC parameter is used for PDCCH and PDSCH

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP -based pre-compensation) is configured and a CORESET is activated with two TCI states and UE is configured with enableTwoDefaultTCI-States and time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, down-select rule to determine default beam(s) for Rel-17 SFN PDSCH reception in RAN1#106-e:

- Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a

- Alt 2: Introduce new rules to determine TCI states based on two TCI state(s) of the CORESET

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD

- Down-select one alternative for implicit configuration

  - Alt 1-2: RS of CORESETs with both single and two TCI states are used

  - Alt 1-3: RS of CORESETs with only two TCI states are used

- Down-select one alternative for explicit configuration

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

- Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3

CSI enhancements:

Agreement

For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32.

Conclusion

At least for rank 1, no further restriction or condition is applied for polarization-common based free-selection and combinatorial coefficient based port selection for W1.

Working Assumption

At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1

- FFS: Further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization

- FFS: Whether to introduce thresholds for N3 and/or P

Agreement

A polarization-specific bitmap for indication non-zero coefficients should be supported for W2.

Agreement

For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- Two polarization-specific reference amplitudes:

  - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

  - for the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size)

- For coefficients other than the strongest coefficient

  - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size)

  - phase is quantized to 16PSK

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4)

  - Alt 3: it is replaced as (1/2)^(3/8)

Note: whether/how SCI is supported for R17 codebook will be discussed separately

Agreement

Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis:

- It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

Conclusion

Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM

Alt 2: No, it is not supported

Agreement

At least for rank 1, candidate values of K1 for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}.

- Note: for polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations

Agreement

Further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined

- based on trade-off among UPT performance, feedback overhead, and complexity

- based on all supported ranks

- Limit total number of parameter combinations comparable to Rel-16 eType II

- Exact parameters (e.g. with 2 or 3 parameters) within each combination are FFS

- Other parameterizations of codebook parameter (e.g. alpha with K1= Alpha*# of CSI-RS ports and Alpha <=1) are not excluded

Agreement

At least for rank 1 and 2, for the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {[1/4], 1/2, 3/4, 1}

- Note: [1/4] means that 1/4 is also a candidate value for the discussion on reduction of parameter combinations, but has a lower priority compared to other beta values

Agreement

For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook

- FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2

- FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8

Agreement

Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses

- Alt 1: It is feasible for FR1 but not for FR2.

Agreement

A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis:

- One-to-one mapping between M+N CSI-IM resources versus M NZP CSI-RS resources for single-TRP measurement hypothesis and N NZP CSI-RS resource pairs for NCJT measurement hypothesis configured in a CSI-RS resource set.

  - FFS the value/definition of M

Note: it is possible to configure the same value of CSI-IM resource ID for both NCJT and Single-TRP measurement hypotheses in FR1 and FR2, subject to QCL-Type D consistency between measurement hypotheses of the shared CMR in FR2

Agreement

For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max,

- Alt 1: Ks,max = 4

- Alt 2: Ks,max = 2

- Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1

- Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature.

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, study whether/how to support following dynamic updating on, e.g. by MAC-CE

- Alt 1: CMR pairs for NCJT measurement hypotheses

- Alt 2: CMRs for Single-TRP measurement hypotheses

- Alt 3: TCI states in CMRs

- Alt 4: the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report

Conclusion:

There is no consensus to go with either of the following options in RAN1 #105e:

- Option 1: Confirm the Working Assumption from RAN1#103e

- Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT

Agreement

For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact:

- CRI codepoint mapping order with CMRs and CMR pairs

- Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement

- Whether/how to enhance the CSI updating rule to address CPU overbooking

- Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation

- Whether/how to support wideband CSI report

Agreement

At least for rank 1 and for Mv>1, Minit for the single window with size N is fixed to be 0

Conclusion

For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, there is no consensus of further enhancement for CSI-RS configurations associated with Rel-17 PS codebook.

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, down-select one or more Alts in RAN1#106-e:

- Alt 2: additional RRC signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. Additional RRC signaling may select {#0,#3} (if sharing is allowed), or {#1, #3} (if not allowed), or select any from the set for single-TRP measurement hypotheses.

- Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. If gNB enables the sharing, {#0, #1, #2, #3} are for single-TRP measurement. If gNB disable the sharing, {#1, #3} are for single-TRP measurement hypotheses.

- Alt 4: CMR sharing between single-TRP measurement hypothesis and NCJT measurement hypothesis is realized by configuring the same value of CMR ID for single-TRP CMR and NCJT CMR pair.

  - Example: When the UE supports sharing, for a given set of {{#0, #0}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypotheses, the rest {#0, #3} are for single-TRP measurement hypotheses. The CMRs for STRP can be updated by re-configuring the CSI resource set.

Note that above examples are only for the purpose of illustrating/discussing Alternatives.

Agreement

For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s):

- Enabling/Disabling PMI, RI sharing via higher-layer configuration

- Dynamic indication of PMI, RI sharing in the CSI report

- FFS: other details

- FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied

For future RAN1 meeting:

For a CSI report setting with Option 1 and X=1 or 2, study prioritizing CSI associated with reported CSI hypotheses within a CSI Reporting Setting

- FFS potential impact for UCI payload generation

- FFS whether/how to update CSI priority formula, and additional specification impact due to updated formula

- FFS whether/how to update CSI omission rules for Part 2 CSI based on prioritized CSI

- FFS: whether the X+1 CSI hypotheses per CSI Reporting Setting are mapped to a single CSI report or X+1 CSI reports

- Companies are encouraged to discuss and justify purposes of prioritizing CSI associated with reported CSI hypotheses.

Agreement

At least for rank 1 and 2 and Mv > 1, for relationship between N and Mv, study and down-select one alternative from following in RAN1#106-e

- Alt 1: N= Mv always, no UE reporting of Wf

- Alt 2-1: N >= Mv, Wf  is layer-common and reported by UE for N>Mv.

- Alt 2-2: N >= Mv, Wf is layer-specific and reported by UE for N>Mv.

Note: Wf is layer-common for N=Mv

Note: For all alternatives, a layer-common window/set of size N is configured.

Agreement

Support rank 2 for Rel-17 codebook

Agreement

For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e:

- Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed

- Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed.

- Alt 3: Keep both Wf OFF and Wf ON with Mv=1.

  - If PMI format is SB, Wf  is an all-one vector of length N3

    - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e

  - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar

    - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e

- Note: N3 = NCQISubband*R.

- FFS: the case when no SB size is configured.

For future RAN1 meeting:

Study whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- FFS: applicable conditions of being absent, .e.g. Mv=1 and Beta =1 for rank 1 or higher ranks

- FFS: additional impact for reporting mechanism when/how the bitmap is absent

- Note: The principle of UE determining the real number of NZC (same as Rel-15 and Rel-16) is unchanged in Rel-17

- based on trade-off among UPT performance, feedback overhead and complexity

#### 2.1.2	Remaining Open issues

Multi-beam enhancements:

- Remaining details of unified TCI framework as well as DCI-based and MAC-CE-based beam indication, including TCI update/configuration for ‘other’ target RSs, the support for M and/or N>1, TCI state pool designs, and finer details on joint and separate DL/UL TCI

- Remaining details of inter-cell beam management to facilitate L1/L2-centric inter-cell mobility, including additional details on inter-cell measurement/reporting and inter-cell beam indication

- Remaining details of the spec support of fast panel selection for UL multi-panel UEs, along with MPE mitigation

- Study on advanced beam tracking/refinement

Multi-TRP enhancements for PDCCH, PUSCH, and PUCCH:

- Remaining details on PDCCH reliability and robustness enhancements for multi-TRP including supported modes of operation such as intra-slot and/or inter-slot repetition, further ambiguity resolution related rules, details of BD counting with overlapping PDCCH candidates, and other.

- Identifying open issues and additional enhancements on multi-TRP PUCCH reliability enhancement, and finalizing details.

- Remaining details on multi-TRP PUSCH reliability enhancements, including open issues and additional enhancements related to DG PUSCH, CG PUSCH, beam configurations/mapping, power control, and other.

Multi-TRP enhancements for multi-cell mTRP:

- Remaining details on QCL/TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation

Multi-TRP enhancements for beam management:

- Detailed design of beam measurement/reporting enhancement, beam failure recovery (including partial BFR) for mTRP and study on simultaneous reception of same/different channels/RS with different QCL-typeD

Multi-TRP enhancements for HST-SFN:

- Remaining details on QCL assumption for DMRS antenna port(s) for HST-SFN deployment

- If the benefits are shown, detailed design of QCL/QCL-like relation between DL and UL signal for HST-SFN deployment

SRS enhancements:

- Remaining details of aperiodic SRS triggering enhancement considering flexibility and overhead

- Remaining details on resource set and resource configurations for SRS antenna switching up to 8Rx

- Remaining details to complete the three agreed schemes for SRS coverage/capacity enhancement

CSI enhancements:

- Remaining detailed design of CSI reporting for M-TRP/panel for more dynamic hypothesis, including related configuration, measurement behavior, reporting mechanism, UCI design, UE capability design at least.

- Remaining detailed design of PS codebook enhancement by utilizing FDD reciprocity for rank 1 and higher rank, including related quantization per codebook component, reporting mechanism, UE capability, UCI design at least.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#113bis-e meeting

L1/L2 centric inter-cell mobility

R2-2104632	Summary of email discussion [AT113bis-e][035][feMIMO] L1L2 Centric Mobility	Samsung

DISCUSSION

- AGREEMENTS

The term “non-serving cell(s)” seems to cause confusion, and should be changed (to be consistent with the current RAN2 definitions).

RAN2 further study the impact on L1/L2 centric mobility for inter-cell multi-TRP-like model and inter-cell HO-like model.

Chair: while unclear, there seems to be support for: RRC provides the pre-configured configuration of “the candidate cell for L1/L2 centric mobility” (FFS if > 1), and L1/L2 signaling can be used/feasible for the dynamic switching of the pre-configured value.

Continue by long email discussion, to better understand impact in R2, pave the way for potential high level decisions, and get replies and Q to R1 LS

RAN2#114-e meeting

TCI states indication for PDCCH

[AT114-e][035][feMIMO] TCI states indication for PDCCH (Intel)

- AGREEMENTS

[035] An enhanced MAC CE is introduced for PDCCH activating two TCI states.

[035] The enhanced MAC CE includes 1) serving cell ID, 2) CORESET ID and 3) Two TCI state IDs.

[035] The enhanced MAC CE is based on option 1 but the CRs will be discussed after RAN1 send further details.

[035] no reply LS out is required in this meeting.

L1/L2 centric inter-cell mobility

R2-2106314	Summary of email discussion [Post113bis-e][061][feMIMO] InterCell mTRP and L1/L2 mobility (Samsung)	Samsung	report	NR_feMIMO-Core

- AGREEMENTS

RRC provides the configuration for “the cells for L1/L2 centric mobility”, and L1/L2 signaling can be used/feasible for the dynamic usage/switching of the configured value.

R2 didn’t see a problem with using different C-RNTIs for different cells. Different C-RNTI seems more natural in a mobility scenario. No conclusion in R2 for mTRP scenario.

RRC configurations of the cells for L1/L2 centric mobility, including C-RNTI, are configured by RRC.

RAN2 prefer to restrict the scope of the deployment only for intra-DU case in Rel-17.

RAN2 assumes to prioritize intra-frequency case in Rel-17, but RAN2 follows the RAN4 decision to support inter-frequency case.

Use P1 and P2 as baseline for further discussion, aiming to reply to the LS. (P1 seems to be too detailed need generalizing).

R2-2106664	[AT114-e][036][feMIMO] InterCell mTRP and L1/L2 mobility	Samsung

DISCUSSION

- AGREEMENTS

RAN2 confirm the simplified procedures on the inter-cell multi-TRP-like model as a baseline RAN2 understanding:

- Scenario 1: Inter-cell multi-TRP-like model

- 1. UE receives from serving cell, configuration of SSBs of the TRP with different PCI for beam measurement, and configurations needed to use radio resources for data transmission/reception incl resources for differet PCI.

- 2. UE performs beam measurement for the TRP with different PCI and report it to serving cell.

- 3. Based on the above reports, TCI state(s) associated to the TRP with different PCI is activated from the serving cell (by L1/L2 signaling).

- 4. UE receives and transmits using UE-dedicated channel on TRP with different PCI.

- 5. UE should be in coverage of a serving cell always, also for multi-TRP case, e.g. UE should use common channels BCCH PCH etc. from the serving cell (as in legacy).

RAN2 confirm the simplified procedures on the L1L2 mobility model as a baseline RAN2 understanding:

- Scenario 2: L1L2 mobility model (i.e. with serving cell change)

- 1. UE receives from serving cell, configuration of SSBs of the cell with different PCI for beam measurement/ serving cell change.

- 2. UE performs beam measurement for the cell with different PCI and report it to serving cell.

- 3. Serving cell configuration for cell with other PCI is provided to the UE by RRC (pre-configuration for serving cell change, FFS if this step is same as 1).

- 4. Based on the above reports, TCI states for cell with different PCI is activated along with the serving cell change (by L1/L2 signaling). FFS if this is multiple steps.

- 5. UE changes the serving cell and starts receiving/transmitting using the pre-configured UE-dedicated channel and TCI states.

Ask R1 to confirm that L1L2 mobility is assumed to be based on L1 measurements (not in R2 scope)

R2 assumes for now that L1L2 mobility model includes Pcell mobility and possibly also Scell mobility (FFS).

R2 assumes that for both multi-TRP and mobility scenarios, single protocol stack can be assumed (intra-DU)

#### 2.2.2	Remaining Open issues

L1/L2 centric inter-cell mobility

- RRC pre-configurations on the dedicated channels for the neighbor TRP/Cell.

- New L1/L2 signaling design for TCI state update between serving cell and neighbor cell.

- Handling of MAC entities at the change of TRP/Cell e.g. UL timing management, etc.

- Further details to support TCI state update with serving cell change e.g. how L1/L2 signaling is triggered, how to ensure reliability and robust for the L1/L2-triggered serving cell change, etc.

Enhanced TCI states indication for PDCCH

- Further details to support additional functionalities for enhanced TCI states indication for PDCCH based on RAN1 decision, e.g. simultaneous activation of TCI states for the RRC configured set of the serving cells, etc.

Further issues will be identified from RAN1 decision

- TRP specific BFR, PHR operation

- RRC configuration support for RAN1 parameters

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #98bis-e meeting

RF related

Agreement

- Second reply LS on beam switching gaps for Multi-TRP UL transmission was approved in R4-2105436

RRM related

Agreement:

- WF on FeMIMO L1/L2-Centric Inter-Cell Mobility was approved in R4-2105838, captured as below:

RAN4 understanding on FeMIMO L1/L2-Centric inter-cell mobility

- Operations in L1/L2-Centric inter-cell mobility that RAN4 needs to evaluate

  - Based on RAN1/RAN2 agreement, RAN4 may need to consider and evaluate the impact of at least the followings from RAN4 perspective

    - Measurement on NSC and reporting of the measurement; and

    - Operations for indicating a TCI state referring to RSs on the NSC; and

    - FFS: Transmission schemes for enabling inter-cell multi-TRP operations

      - Note: Transmission schemes are discussed in the following items.

- Identified impacts on RAN4 for further discussion

  - For L1/L2-Centric inter-cell mobility, RAN4 shall discuss on the impacts with regard to following issues:

  - UE L1-measurement on NSC for beam measurements and reporting;

    - Whether to enhance UE capability due to L1 measurements on NSC

  - Requirement for TCI state switching considering TCI related operations pertained to RS(s) on NSC;

  - UE BM capability and timing requirement for

    - UE simultaneous “reception and transmission” or simultaneous reception only from multiple SC, after L1/L2-Centric inter-cell mobility operation , if RAN1 supports this feature;

    - UE CA scenario, if RAN1 considers the case that UE is configured with multi CCs;

  - Other issues are not precluded.

- RAN4 assumptions for L1/L2-Centric inter-cell mobility scenarios

  - RAN4 considers the feature of L1/L2-Centric inter-cell mobility under the assumptions that:

  - Focus on both FR2 and FR1

    - Both SC and NSC are in the same FR;

  - Study on non-collocated and collocated SC and NSC case

  - SC and NSC are

    - in the same frequency layer

      - The SSBs of NSC have the same center frequency and SCS as the SSBs of the serving cell (RAN1 agreement)

    - not in the same frequency layer, if supported by RAN1

      - CA scenarios (e.g. intra-band and inter-band), if defined and supported by RAN1

      - Other inter-frequency case, if any supported by RAN1

  - Other options are not precluded.

RAN4 Reply to RAN1 LS R1-2102248

- What to include in the reply LS to RAN1

  - Identified RAN4 impacts in the contexts of RAN1 LS Q5 and Q6 that includes but not limited to

    - Impacts due to measuring inter-frequency and/or inter-band NSC L1 resources

      - Use of MG

      - Different SMTCs between SC and NSC

      - Reference timing for measuring NSC L1 resources (if SSB based search is needed)

      - Other issues if any

    - Impacts due to TCI state switching for NSC

  - And RAN4 asks RAN1 to clarify, if needed,

    - Which of the following(s) and what else are under consideration by RAN1 for enabling inter-cell multi-TRP operations

      - Simultaneous transmission

      - Multi-TRP

      - Regular CA

      - Other options if any

RAN4 #99-e meeting

RF related

Agreement

- WF on impact to RF requirements for FeMIMO was approved in R4-2107882.

  - No transmission requirements will be specified for multi-panel UE in Rel-17 FeMIMO WI

  - RAN4 will further study if any impact to the reception requirements for multi-panel UE in Rel-17 FeMIMO WI considering RAN1 status

  - RAN4 will further study the UE RF requirements impact for MPE mitigation. Companies are encouraged to provide the initial analysis for UE RF impact for MPE in the next RAN4 meeting considering the latest RAN1 progress considering RAN1 status

  - No UE RF impact for CSI enhancement and link recovery for FR2 serving cells in Rel-17 FeMIMO WI

RRM related

Agreement

- WF on work plan for Rel-17 FeMIMO RRM was approved in R4-2108358

WF for prioritized objective

  - In RAN4 #99e meeting, companies provide the input for overall RRM impact for feMIMO. Among the objectives, RAN4 will firstly focus on objectives below

    - 1-a Unified TCI for DL and UL

    - 1-b: L1/L2 centric inter-cell mobility

    - 1-c: Beam indication signaling medium

  - Other objectives can be discussed up to RAN1/2 discussion status

WF for L1/L2 centric non-serving cell measurement

  - In RAN4 #99e meeting, RAN4 discussed the UE behavior, performance and UE complexity for supporting non-serving cell measurement. RAN4 will further discuss the non-serving cell measurement in the context of L1/L2 centric inter-cell mobility considering the followings in the future RAN4 meetings:

    - For the case when the measurement RS from the non-serving cell is within SMTC, in FR1, legacy measurement behavior based on L3 measurement may be reused from RAN4 perspective.

    - RAN4 will continue to evaluate the impact on a longer measurement delay due to sharing of SMTC window by L1 and L3 measurements.

    - For the case when the measurement RS from the non-serving cell is outside SMTC and within SMTC, performance degradation or increased UE complexity may be expected

    - To consider concurrent serving cell PDSCH/PDCCH reception and neighbor SSB based L1 measurement outside SMTC, RAN4 needs more time to study on the impacts on performance degradation and UE complexity.

    - Note: existing UE feature simultaneousRxDataSSB-DiffNumerology is aimed for concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception.

- Response LS on L1/L2 centric inter-cell mobility was agreed in R4-2108356

- Response LS on timing assumption for inter-cell DL measurement was agreed in R4-2108044

#### 2.4.2	Remaining Open issues

- Specify core requirements associated with items specified by RAN1

- Investigate if the requirements on link recovery procedure is suitable for FR2 serving cells

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template