3GPP TSG-RAN Meeting #92 Electronic	RP-211219

Electronic meeting, 14 – 18 June 2021

Agenda item:	9.7.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	WID update proposals for support of reduced capability NR devices

WID/SID:	NR_redcap – Release 17

Document for:	Approval

# 1	Introduction

The Rel-17 WID on support of reduced capability NR devices was revised in RAN#91-e in [1]. The revised WID included RAN2 study phase, which has now ended. During the RAN2 discussions it became apparent that companies did not have common understanding on all objectives. Therefore, in this contribution we discuss how the WID objectives could be clarified to enable better progress for the work. In the end of the document we propose updated WID objectives.

# 2	Discussion

The RAN2 study phase for RRM measurement relaxation criteria has now ended. In our view it would be important to define clear and concise objectives for the normative work following the study phase conclusions.

In our view it would also be beneficial to define even clearer objectives for some of the items to help the progress of the normative work. For instance, identification, access and camping restrictions were not handled in RAN2#113e-bis as some companies and people felt that RAN1 should progress these topics first although RAN2 was indicated as the first WG for the objectives.

Some other objectives like specific cell barring and intra-frequency reselection created discussion in RAN2 and we see that it would be beneficial to further clarify the objectives to ensure common understanding of the objectives.

# 4	Proposal

To help the progress of the normative work for Rel-17 RedCap we propose to update the RedCap WID objectives as follows.

## 4	Objective

### 4.1	Objective of Core part WI

This WI has the following objectives:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]:

  - Reduced maximum UE bandwidth:

    - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz.

    - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

  - Reduced minimum number of Rx branches:

    - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

  - Maximum number of DL MIMO layers:

    - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

    - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

  - Relaxed maximum modulation order:

    - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE.

    - No other relaxations of maximum modulation order are specified for a RedCap UE.

  - Duplex operation:

    - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1]

  - The existing UE capability framework is used; changes to capability signalling are specified only if necessary.

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and Msg3, and Msg A, including the ability for the Msg1 indication to be configurable by the network. [RAN2, RAN1]

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency/frequencies/PLMN or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

- Specify necessary updates of UE capabilities (38.306) and RRC parameters (38.331). [RAN2]

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]:

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4.

  - CN configures eDRX for RRC_Idle and RAN for RRC_Inactive.

- RRM relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected:

  - Specify RSRP/RSRQ and beam-level based RRM measurement relaxation criteria for RRC_Idle/Inactive/Connected  [RAN2]

    - Enabling/disabling of RRM relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM relaxation.

  - Specify RRM measurement relaxation [RAN4]

  - No RRM relaxations are specified for the serving cell.

- Specify RAN4 core requirements for the above.

Notes:

- Uplink coverage enhancement solutions specified in the NR Coverage Enhancement WI (NR_cov_enh) shall be assumed to be available also to RedCap UEs by default (with small modifications for RedCap UEs if found necessary).

- Power saving enhancement solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) shall be assumed to be available also to RedCap UEs by default.

- Rel-15 SSB bandwidth is reused and L1 changes minimized.

- The work defined as part of this WI is not to overlap with LPWA use cases.

- Coexistence with non-RedCap UEs is to be ensured.

- This WI focuses on SA mode and single connectivity with operation in a single band at a time.

# References

[1] RP-210918, Revised WID on support of reduced capability NR devices, Nokia, Ericsson