3GPP TSG-RAN WG1 Meeting #105-e	R1-2106360

# E-meeting, May 10 – May 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0241 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling collision between PUCCH/PUSCH and semi-static DL symbols and SSBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm), Samsung, Nokia, DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-5-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to capture the agreement made in RAN1 #104e for handling collisions with SSB and DL symbols and intra-UE multiplexing, for two additional scenarios 1) the PUCCH/PUSCH are configured channels and 2) all PUCCH/PUSCH are of the same priority. These scenarios were missed from the current specification. Agreement To address collision with semi-static DL symbols and SSB, the following easy way is suggested: - Step1: Perform intra UE prioritization (including multiplexing, overriding) according to related working assumption in 102 e-meeting and produce final PUCCHs/PUSCHs. - Step 2: Final PUCCHs/PUSCHs is cancelled by semi-static DL symbols and SSB symbols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture the UE behavior that handling of collisions with semi-static DL symbols and SSBs is done after applying intra-UE mulitplexing/prioritization for the scenarios where 1) the PUCCH/PUSCH are configured channels and 2) all PUCCH/PUSCH are of the same priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential misunderstanding between gNB and UE as it would not be clear whether the UE would apply the handling of collisions with SSB and DL symbols before or after intra-UE mulitplexing/prioritization for the two scenarios listed above. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------Start of Text Proposal ------------------------------------------------

9          UE procedure for reporting control information

***Unchanged parts are omitted***

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clauses 9.2.5 and 9.2.6.

***Unchanged parts are omitted***

In the remaining of this Clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission as described in clause 11.1. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH.

*** Unchanged text is omitted ***

### ----------------------------------------------------- End of text proposal ----------------------------------------------------