3GPP TSG-RAN WG1 Meeting #105-e              	R1-2106374

e-Meeting, May 19 - 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0244 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction for sub-slot based PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, sub-slot based transmission was adopted for some UCIs on PUCCH. According to the running specification, all “slot” in Clause 9 should be interpreted as “sub-slot” when a UE is provided subslotLength-ForPUCCH. However, interpreting “slot” as “sub-slot” in some parts of Clause 9 is not correct, e.g. those only related to the slot-based UCI such as SR, CSI, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when a UE is provided subslotLength-ForPUCCH, the slot length of subslotLength-ForPUCCH symbols is only applied for an PUCCH resource of a PUCCH transmission with HARQ-ACK information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of mis-interpretation of slot length when a UE is provided subslotLength-ForPUCCH, which leads to wrong PUCCH resource used for transmission of some UCIs in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding, which has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

<Unchanged parts are omitted>

In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLength-ForPUCCH.

<Unchanged parts are omitted>