3GPP TSG RAN meeting #92-e									RP-211266

Electronic Meeting, June 14-18, 2021

## Status Report to TSG

Agenda item:			9.6.3

| WI / SI Name | Study on XR (Extended Reality) evaluations for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_XR_eval |  |  |  |  |
| Unique ID | 860062 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210406 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2021 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 60 % | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eddy (Hwan-Joon) Kwon |
|  | Company | Qualcomm |
|  | Email | eddykwon@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements on traffic model and other evaluation assumptions

Agreement

In addition to the response LS from RAN1#104-bis-e in April 2021 to SA2 and SA4 (cc: RAN2) in R1-2104117, RAN1 would like to provide the following information in response to the LS from SA4, based on additional evaluation results: even though RAN1 hasn’t performed evaluations with the exact parameters (e.g. in RAN1 evaluations, data rate higher than 45Mbps was not considered and simulation was frame based) presented by SA4 (5QIs), it is RAN1 understanding that these values can be supported by NG-RAN.

- LS is endorsed in R1-2106149

Agreement

PDB value of the stream in UL AR aggregating streams of scene, video, data, and audio, i.e., Option 2, Stream 2 in Option 1, and Stream 2 in Option 3.

- 30ms (baseline), 10/15/60ms (optional)

Agreement

For DL video stream, separate packet arrivals in time for dual-eye buffer can be optionally evaluated, based on the single stream model by doubling the packet arrival rate and halving the packet size compared to the single stream, while all other parameters (e.g., jitter, PDB) are the same as for single stream.

- For companies who are evaluating separate packet arrivals in time for dual-eye buffer in addition to single stream (baseline), it is recommended to evaluate at least the following scenarios in the table.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Application | AR/VR 30Mbps |  |  |
| --- | --- | --- | --- |
| Traffic model | Single stream for dual-eye buffer | Separate packet arrival for dual-eye buffer |  |
| Data rate (Mbps) | 30 | 30 |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 62500 | 31250 | Data rate / FPS / 8 [bytes] |
| STD of packet size (Bytes) | 6563 | 3281 | 10.5% x mean packet size |
| Max packet size (Bytes) | 93750 | 46875 | 150% x mean packet size |
| Min packet size (Bytes) | 31250 | 15625 | 50% x mean packet size |
| Packet arrival interval (ms) | 1000/60 | 1000/120 |  |
| PDB (ms) | 10 |  |  |

Agreement:

When companies are submitting evaluation results to RAN1, it is recommended to submit results at least the following parameters in the below table.

- Note 1: This is only intended to have more results from more companies at least for the corresponding configuration. RAN1 agreements regarding baseline vs. optional for simulation scenarios, configurations, parameters, remain the same.

- Note 2: Companies are encouraged to submit results for other baseline/optional configurations as much as they can.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Agreement:

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of α, A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream | RI=R*αN-1+α |  | RP=R*N-1N-1+α |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

Agreement

Confirm the 2-symbol gap at the end to third “D” slot of DDDUU for FR1/FR2.

- Applies only for Option 2

Agreement

UE with transmit power less than 0 dBm is considered for power consumption evaluation, adopt option 2 as baseline, i.e. the power model of 0 dBm for UE with transmit power less than 0 dBm.

- Option 1 can be optionally evaluated

- Note: Above is not intended to introduce new power class

Agreement

For FR2, it is up to company to report the UE UL power consumption model.

#### 2.1.2	Remaining Open issues

In accordance to the SID:

- There are some open issues on XR/CG traffic model, KPIs, other evaluation methodologies, e.g., parameters for evaluation of multi-stream evaluation for DL video stream that is an optional evaluation scenario. In addition, evaluation methodology for XR coverage evaluation was not agreed in RAN1#105-e and RAN1#106-e will continue to discuss it.

- Carry out performance evaluations towards characterization of identified KPIs.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#105-e tdoc list

Traffic Model

R1-2104207	XR traffic model	FUTUREWEI

R1-2104238	Traffic model for XR and Cloud Gaming	Huawei, HiSilicon

R1-2104395	Remaining issues on traffic models of XR	vivo

R1-2104502	XR traffic model	CATT

R1-2104555	On Traffic Model for XR study	Nokia, Nokia Shanghai Bell

R1-2104701	Remaining Issues on XR Traffic Models	Qualcomm Incorporated

R1-2104745	Discussion on the XR traffic models for evaluation	OPPO

R1-2104934	Traffic Model for XR	Intel Corporation

R1-2105134	Considerartions on XR traffic model	Apple

R1-2105181	Considerations on XR traffic model	Sony

R1-2105342	Traffic Models for XR	Samsung

R1-2105376	Traffic Model for XR and CG	MediaTek Inc.

R1-2105443	Discussion on traffic models for XR evaluation	LG Electronics

R1-2105499	Discussion on UL traffic models	InterDigital, Inc.

R1-2105547	Discussion on remaining issues of traffic Model for XR services	Xiaomi

R1-2105603	Remaining Issues of XR Traffic Model	ZTE, Sanechips

R1-2105726	Discussion on traffic model for XR	NTT DOCOMO, INC.

R1-2105829	Traffic model for XR	Ericsson

Evaluation Methodology

R1-2104208	XR evaluation methodology	FUTUREWEI

R1-2104239	Evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

R1-2104396	Discussion on evaluation methodologies for XR	vivo

R1-2104499	Evaluation methodology and performance index for XR	CATT

R1-2104556	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

R1-2104702	Remaining Issues on Evaluation Methodology for XR	Qualcomm Incorporated

R1-2104746	Discussion on the XR evaluation methodology	OPPO

R1-2104935	Evaluation Methodology for XR	Intel Corporation

R1-2105135	Remaining issues in XR evaluation methodology	Apple

R1-2105343	Evaluation methodology and KPIs for XR	Samsung

R1-2105377	On Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2105444	Discussion on evaluation methodologies for XR	LG Electronics

R1-2105500	Discussion on additional issues on XR Evaluations Methodology and KPI	InterDigital, Inc.

R1-2105548	Discussion on remaining issues of evaluation methodology for XR services	Xiaomi

R1-2105604	Further Discussion on XR Evaluation Methodology	ZTE, Sanechips

R1-2105727	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

R1-2105830	Evaluation methodology for XR	Ericsson

Initial Performance Evaluation Results

R1-2104209	XR initial evaluations	FUTUREWEI

R1-2104397	Initial performance evaluation results on XR	vivo

R1-2104500	Evaluation results of XR performance	CATT

R1-2104557	Performance results in indoor hotspot and dense urban deployments of CG and VR applications			Nokia, Nokia Shanghai Bell

R1-2104703	Initial Evaluation Results for XR Capacity and UE Power Consumption	Qualcomm Incorporated

R1-2104747	Evaluation results for XR evaluation	OPPO

R1-2104936	Initial results for XR	Intel Corporation

R1-2105136	Performance evaluation on XR	Apple

R1-2105392	Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

R1-2105501	Initial Performance Evaluation Results on XR	InterDigital, Inc.

R1-2105521	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2105605	Performance Evaluation Results for XR	ZTE, Sanechips

R1-2105664	XR Initial Performance Results	AT&T

R1-2105831	Initial XR performance evaluation results	Ericsson

R1-2105963	Initial results for XR	Intel Corporation

Others

R1-2104398	Challenges and potential enhancements of XR	vivo

R1-2104501	Potential area of NR enhancement for the support of XR services	CATT

R1-2104558	Enhancements for better XR support over NR	Nokia, Nokia Shanghai Bell

R1-2104704	Potential Enhancements for XR	Qualcomm Incorporated

R1-2104748	Discussion on the  support of XR/CG service in sidelink-unlicensed	OPPO

R1-2105137	Considerations on potential enhancements for XR	Apple

R1-2105344	Potential enhancements for better XR support in NR	Samsung

R1-2105502	Discussion on potential enhancements for XR	InterDigital, Inc.

R1-2105522	Challenges and potential enhancements for XR and Cloud Gaming	Huawei, HiSilicon

R1-2105549	Discussion on potential enhancement for supporting XR	Xiaomi

R1-2105606	Further Discussion on Capacity and Power Working Areas for XR	ZTE, Sanechips

R1-2105832	Discussion on enhancements for XR	Ericsson

RAN1#104-e tdoc list

Traffic Model

R1-2100207	Discussion on applications and traffic model for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101137	Traffic Model for XR and CG	MediaTek Inc.

R1-2100055	XR traffic model	FUTUREWEI

R1-2100132	Discussion on the XR traffic models for evaluation	OPPO

R1-2100361	XR traffic model	CATT

R1-2100476	Discussion on traffic models of XR	vivo

R1-2100528	Discussion on Traffic Model for XR evaluations	ZTE , Sanechips

R1-2100555	Discussion on traffic model for XR study	LG Electronics

R1-2100571	Discussion on XR applications and traffic models	InterDigital, Inc.

R1-2100680	On traffic model for XR	Intel Corporation

R1-2100724	On Traffic Model for XR study	Nokia, Nokia Shanghai Bell

R1-2100775	XR Traffic Model Considerations	AT&T

R1-2100879	Discussion on XR Applications and Evaluation Assumptions	Sony

R1-2101101	Discussion on Traffic model for XR evaluation	Xiaomi

R1-2101240	XR Applications and Traffic Models	Samsung

R1-2101314	Traffic model for XR	Ericsson

R1-2101365	Views on XR traffic models	Apple

R1-2101493	XR Traffic Models	Qualcomm Incorporated

R1-2101635	Discussion on traffic model for XR	NTT DOCOMO, INC.

Evaluation Methodology

R1-2100477	Discussion on evaluation methodologies of XR	vivo

R1-2100056	XR evaluation methodology	FUTUREWEI

R1-2100133	Discussion on the XR evaluation methodology	OPPO

R1-2100242	Discussion on evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

R1-2100362	Evaluation methodology and performance index for XR	CATT

R1-2100529	On XR Evaluation Methodology	ZTE , Sanechips

R1-2100556	Discussion on evaluation assumption for XR study	LG Electronics

R1-2100572	Discussion on Evaluation Methodology for XR	InterDigital, Inc.

R1-2100586	On Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2100681	On evaluation methodology for XR	Intel Corporation

R1-2100725	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

R1-2100776	XR Evaluation Assumptions	AT&T

R1-2101102	Discussion on evaluation methodology for XR services	Xiaomi

R1-2101241	XR Evaluation Methodology and KPIs	Samsung

R1-2101315	Evaluation methodology for XR	Ericsson

R1-2101366	Views on XR evaluation methodology	Apple

R1-2101494	Evaluation Methodology for XR	Qualcomm Incorporated

R1-2101636	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

Initial Performance Evaluation Results

Placeholder only – no email/GTW discussion

R1-2100363	Evaluation results of XR performance	CATT

R1-2100478	Initial performance evaluation results of XR	vivo

R1-2100530	Preliminary Performance Evaluation Results for XR	ZTE , Sanechips

R1-2100587	Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

R1-2100682	Initial results for XR	Intel Corporation

R1-2100726	Initial Performance Evaluation Results	Nokia, Nokia Shanghai Bell

R1-2101068	Discussion on XR evaluation	CMCC

R1-2101255	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101316	Initial XR performance evaluation results	Ericsson

R1-2101495	Initial XR Performance Evaluation Results	Qualcomm Incorporated

Others

R1-2100134	Discussion on the  support of XR/CG service in sidelink-unlicensed	OPPO

R1-2100364	Potential area of NR enhancement for the support of XR services	CATT

R1-2100479	Challenges and potential enhancements of XR	vivo

R1-2100531	Considerations on XR specific enhancements	ZTE , Sanechips

R1-2100573	Discussion on potential enhancements for supporting XR	InterDigital, Inc.

R1-2101103	Discussion on potential enhancement for supporting XR	Xiaomi

R1-2101269	Other aspects for XR and Cloud Gaming	Huawei, HiSilicon

R1-2101317	Mobility and XR applications	Ericsson

R1-2101367	Views on potential enhancements for XR	Apple

RAN1#103-e tdoc list

Applications, Traffic Model and Evaluation Methogology

R1-2007698	Discussion on XR applications, traffic model and evaluation methodologies	vivo

R1-2007843	XR use cases, evaluation methodologies and traffic model	CATT

R1-2007976	Discussion on applications, traffic model and evaluation methodology for XR	ZTE

R1-2008037	Discussion on XR evaluation and Challenges for NR	CMCC

R1-2008198	Applications, Evaluation Methodology, and KPIs for XR	Samsung

R1-2008311	XR evaluations for NR: Applications and Evaluation Methodology	AT&T

R1-2008454	XR Applications, Traffic Model and Evaluation Methodology	Apple

R1-2008818	Discussion on traffic models and evaluation assumptions for XR	InterDigital, Inc.

R1-2008896	Applications, Traffic Model and Evaluation Methodology for XR evaluations for NR	Nokia, Nokia Shanghai Bell

R1-2008939	Discussion for study in XR evaluation for NR	LG Electronics

R1-2008967	On Applications, Traffic Model, and Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2009006	Scenarios, Traffic Model and EVM for XR	Intel Corporation

R1-2009041	Discussion on XR application and evaluation methodology	Xiaomi

R1-2009087	XR use cases, traffic modelling and performance measure	Ericsson

R1-2009198	Discussion on study on XR evaluations for NR	NTT DOCOMO, INC.

R1-2009280	Evaluation Methodology for XR	Qualcomm Incorporated

Others

R1-2007699	Performance evaluation of XR traffic	vivo

R1-2007842	Potential area of NR enhancement for the support of XR services	CATT

R1-2008316	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2008455	Views on enhancements for XR	Apple

R1-2008819	Discussion on potential enhancements for XR	InterDigital, Inc.

R1-2009289	Initial XR performance evaluations	Ericsson

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template