3GPP TSG RAN WG #92e                                                         RP-211306

e-Meeting, June 14th – 18th, 2021

Source: 	Intel Corporation

Title: 	NR Positioning Deployment Scenarios for V2X and Public Safety

Agenda item:	9.6.2

Document for: 	Discussion and Decision

# Introduction

The RAN WG study item related to NR positioning support for in-coverage, partial coverage and out-of-coverage scenarios was approved with the following objectives [1]:

This study item includes the following objectives for positioning use cases, requirements, and scenarios for the UE in in-coverage, partial coverage, and out-of-coverage as per the network coverage definition in the current specifications: - Identify the positioning use cases and requirements for V2X and public safety, based on the existing 3GPP work and input from industry fora. - Identify potential deployment and operation scenarios Note: Studying the feasibility of the identified requirements is not in the scope of this SI. Note: The work in this SI shall take into account the outcomes and progresses of NR positioning study in both Rel-16 and Rel-17. Note: Work plan - RAN#91: TR skeleton, initial input on Objective 1 - RAN#92: Finalize Objective 1, initial input on Objective 2 - RAN#93: Finalize Objective 2, approve the TR (SI target completion)

In this contribution, we provide our inputs on deployment and operation scenarios for V2X and public safety. This document is a revision of our contribution to the previous meeting [2]. Here, we have removed content on requirements, use cases (that were already agreed) and rather continue discussion on remaining details of deployment / operation scenarios.

# Deployment / Operation Scenarios

In our view, the following aspects should be considered in the RAN study for V2X and public safety positioning use cases:

## Spectrum Considerations

V2X positioning services

According to the LS from 5GAA [4]-[6], “without any preference on a specific band, the ITS-dedicated, the licensed (including mmWave band) and the unlicensed spectrum band were considered for V2X positioning”.

From technical perspective, mmWave bands are very attractive for positioning applications. However as of now mmWave bands are not settled/harmonized worldwide for ITS applications (except Europe where band 63,72-65,88 GHz is available for ITS). Further work aiming alignment of mmWave bands allocation for ITS applications seems needed.

Analyzing V2X support in licensed FR2 bands, it can be noticed that up to date V2X support in FR2 licensed band is not defined by 3GPP at least in terms of RAN4 RF requirements.

Furthermore, the first generation of NR-V2X assumes operation in ITS band at low frequencies (i.e. band n47, 5855 – 5925 MHz). The amount of ITS spectrum in low frequencies is quite limited and vary across different regions. It is expected to be in the range from 10 to 40 MHz (in the best-case scenario) depending on specific geographical region. Available amount of spectrum in ITS band may be a limiting factor for positioning accuracy. Therefore, unlicensed bands (already supported for NR-U Uu-based communication) can be also considered for positioning support using either Uu or PC5 radio-interfaces and are attractive from the bandwidth consideration perspective.

The access to licensed spectrum for V2X services is up to operators and thus may not be guaranteed (i.e. to be enabled by operators). As of today, the band n38 is supported for NR-V2X including sidelink and can be available in some regions.

Based on discussion, it seems natural to consider with higher priority the following candidate spectrum options for V2X positioning:

- PC5 radio-interface (sidelink):

  - ITS band (n47)

  - Licensed bands with V2X support (e.g. n38)

  - Unlicensed bands in low frequency range (around 5.9 GHz) currently supported by NR-U framework for Uu based communication

- Uu radio-interface (downlink/uplink):

  - Licensed bands

  - Unlicensed bands in low frequency range (around 5.9 GHz) currently supported by NR-U framework for Uu based communication.

The licensed FR2 and mmWave ITS bands are also attractive options for V2X sidelink positioning and can be considered for technology evolution in future releases.

Public safety positioning services

The use of licensed band for public safety positioning seems to be a primary candidate option. In particular, band n14 seems to be a valid option based on inputs provided to 3GPP RAN WG4 [9]. The unlicensed bands can be also potentially considered and may be beneficial, if the allocated amount of licensed spectrum is not sufficient to meet positioning performance targets for PS use cases.

  - Discuss and decide on spectrum related aspects for NR positioning work targeting V2X and PS use cases

  - For support of V2X positioning services using PC5 radio-interface (sidelink), consider the following candidates in low frequency bands for Rel.18 work

    - ITS band (frequency band n47)

    - Licensed bands with V2X support for sidelink (e.g. frequency band n38 or others if introduced at a later stage)

    - Unlicensed bands in low frequency range (5 GHz and 6 GHz) currently supported for NR-U, subject to regulation considerations

  - For support of V2X positioning services using Uu radio-interface (downlink / uplink), consider the following candidates for Rel.18 work

    - Licensed bands where positioning service is enabled

    - Unlicensed bands in low frequency range (5 GHz and 6 GHz) currently supported for NR-U, subject to regulation considerations

  - For support of public safety positioning services using Uu or PC5 interface, consider licensed bands (e.g. frequency band n14) in low frequency range with higher priority

## Deployment Scenarios

Considering specific V2X / PS service requirements, the support of NR-RAT dependent and RAT-independent positioning for in-coverage, partial-coverage and out-of-network coverage scenarios should be provided.

## Node Types

For V2X and PS use cases, different node types can be considered in practical deployments. In general, we do not see the need to have any restrictions for existing node types. In addition, we think that nodes with known coordinates and antenna orientation can be considered in these scenarios. In Rel.17 work on NR positioning enhancements the positioning reference units/nodes (PRUs) were considered. In particular, the support of positioning reference UEs should be considered on top of gNB/TRP/IAB nodes. Discussion on specific positioning solutions utilizing different node types is the scope of corresponding WGs.

## Operation Scenarios

In terms of operation scenarios, in our view the following RAT dependent positioning options need to be supported for V2X / PS use cases:

- Scenario 1: PC5 (sidelink) based positioning (relative or absolute) for V2X / PS use cases

  - Radio-layer measurements over PC5 links are used to perform UE positioning either at UE or network side. This framework needs to be introduced.

- Scenario 2: Uu (downlink / uplink) based positioning (relative or absolute) for V2X/PS use cases

  - Radio-layer measurements over Uu links are used to perform UE positioning either at UE or network side. This framework is already supported and necessary optimizations, if any, for V2X/PS can be further considered.

- Scenario 3: Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

  - Radio-layer measurements over Uu and PC5 links are used to perform UE positioning either at UE or network side. This framework needs to be integrated and supplement existing NR positioning functionality.

- Scenario 4: Combination of NR RAT-dependent and RAT-independent solutions

  - Radio-layer measurements and RAT-independent information is utilized for positioning (e.g. GNSS, IMUs, etc.)

In terms of coordinate estimation, both UE assisted solutions (where coordinate is determined at network side) and UE based solutions (where coordinate is determined at UE side) should be supported.

  - The following deployment scenarios are supported for V2X/PS use cases: in-coverage, partial coverage, out-of-coverage

  - The following operation scenarios are supported for V2X/PS use cases:

    - PC5 (sidelink) based positioning (relative or absolute) for V2X/PS use cases

    - Uu (downlink / uplink) based positioning (relative or absolute) for V2X/PS use cases

    - Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

    - Combination of NR RAT-dependent and RAT-independent solutions

## Radio-Interfaces

NR RAT-dependent positioning solutions can be based on Uu radio-interface, PC5 radio-interface and their combination. The support of RAT dependent positioning framework based on NR Uu radio-interface was specified in the Rel.16 and is being enhanced in the Rel.17 for I-IoT use cases aiming to reduce latency and improve accuracy. The NR positioning framework for PC5 radio-interface (relative and absolute positioning) in application to V2X / PS use cases should be a focus of the Rel.18 work on NR positioning. The enhancements of Uu interface targeting V2X / PS positioning services need to be also analyzed and specified if any specific optimizations for V2X / PS use cases are identified on top of the Rel.17 design.

  - Considering that support of NR PC5 sidelink positioning is not defined yet and importance of V2X / PS use cases, the RAN work should focus on support of sidelink positioning, not precluding potential enhancements of NR Uu downlink / uplink positioning

## Antenna System Considerations

In 5GAA LS, the support of distributed antenna system was considered. Support of advanced distributed antenna systems can be beneficial in terms of performance for NR positioning and communication frameworks and is equally valuable / applicable for Uu and PC5 radio-interfaces. If such systems are considered, the functional split in both RF and BB processing need to be discussed and evaluated with many implementation-based options possible. In general, design and support of such systems can be left up to implementation, while impact on specification, if any, may need to be further analyzed. At the same time, considering previous inputs on minimum number of RX antennas for V2X use cases it does not seem to be a priority (optimal starting point) for enabling V2X positioning frameworks in NR system. Therefore, we think that the following two alternatives need to be discussed in RAN:

- Alt.1 – As a starting point, do not focus on support / potential optimizations for distributed antenna systems in the upcoming release, i.e. Rel.18

  - This alternative is motivated by at least the following considerations:

    - NR positioning framework for V2X is not expected to rely on the presence of distributed antenna system and centralized processing in any vehicle and especially UEs (e.g. VRUs).

    - Support of distributed antenna systems seems an optimization and can be done by implementation. The specification work in this direction does not seem to be an urgent and needs more discussion / extra work to identify potential specification impact.

- Alt.2 – Request RAN WGs (RAN1/RAN2/RAN4) to analyze benefits and impact on specification from the support of distributed antenna systems in the upcoming release, i.e. Rel.18

  - This alternative is motivated by at least the following considerations:

    - Considering that distributed antenna systems is expected to provide performance benefits, the early identification of potential specification impacts can simplify its future integration and potentially resolve some of the technical issues. Note that it needs to be further discussed whether benefits can be extracted w/o further specification efforts on DAS support.

    - Up to date the whole NR V2X framework is transparent to the support of distributed antenna system. Therefore, such systems should be discussed in a more general context including Uu and PC5 communication, that may significantly increase the scope of discussion and further increase workload at WG level.

Based on discussion of the above alternatives, considering amount of work and possibility of implementation-based solutions, we prefer to start the work based on Alt.1 and would like to invite more views from automotive sector and other companies on this aspect.

  - Analyze potential impact on specification and benefits of DAS support for Uu and PC5 links only if strong demand and urgency are indicated by automotive sector

## Cooperativeness

Cooperative solutions among vehicles using RAT-dependent and RAT-independent technologies (e.g. GNSS) are beneficial and seems to be important for analysis. In general, this aspect can be left for the discussion at WG level since it seems to be a part of specific solution domain. Further analysis may be needed in terms of specific impact on specification and radio-layers, if any.

# Conclusions

In this contribution, we provide our views on deployment/operation scenarios for NR V2X and PS positioning. In our view, sidelink positioning can further improve NR positioning framework and facilitate accurate positioning for emerging V2X and public safety use cases in various deployment scenarios.

In summary, we have the following proposals:

  - Discuss and decide on spectrum related aspects for NR positioning work targeting V2X and PS use cases

  - For support of V2X positioning services using PC5 radio-interface (sidelink), consider the following candidates in low frequency bands for Rel.18 work

    - ITS band (frequency band n47)

    - Licensed bands with V2X support for sidelink (e.g. frequency band n38 or others if introduced at a later stage)

    - Unlicensed bands in low frequency range (5 GHz and 6 GHz) currently supported for NR-U, subject to regulation considerations

  - For support of V2X positioning services using Uu radio-interface (downlink / uplink), consider the following candidates for Rel.18 work

    - Licensed bands where positioning service is enabled

    - Unlicensed bands in low frequency range (5 GHz and 6 GHz) currently supported for NR-U, subject to regulation considerations

  - For support of public safety positioning services using Uu or PC5 interface, consider licensed bands (e.g. frequency band n14) in low frequency range with higher priority

  - The following deployment scenarios are supported for V2X/PS use cases: in-coverage, partial coverage, out-of-coverage

  - The following operation scenarios are supported for V2X/PS use cases:

    - PC5 (sidelink) based positioning (relative or absolute) for V2X/PS use cases

    - Uu (downlink / uplink) based positioning (relative or absolute) for V2X/PS use cases

    - Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

    - Combination of NR RAT-dependent and RAT-independent solutions

  - Considering that support of NR PC5 sidelink positioning is not defined yet and important for V2X / PS use cases, the RAN work should focus on support of sidelink positioning, not precluding potential enhancements of NR Uu downlink / uplink positioning

  - Analyze potential impact on specification and benefits of DAS support for Uu and PC5 links only if strong demand and urgency are indicated by automotive sector

# References

| - [1] | - RP-201384, “New SID: Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases ”, LGE, FirstNet, July, 2020 |
| --- | --- |
| - [2] | - RP-210468, NR Positioning Use Cases, Requirements, Scenarios for V2X and Public Safety, Intel Corporation, June 2020 |
| - [2] | - R1-201390, “LS on 3GPP RAN study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases”, RAN WG, July 2020 |
| - [3] | - 5GAA LS / S1-210018, LS reply to 3GPP RAN on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases |
| - [4] | - 5GAA TR, C-V2X Use Cases and Service Level Requirements Volume I |
| - [5] | - 5GAA TR, C-V2X Use Cases and Service Level Requirements Volume II |
| - [6] | - 3GPP SA1 TS 22.186, Enhancement of 3GPP support for V2X scenarios, v16.2.0 |
| - [7] | 5GAA LS / RP-200546 LS on the 3GPP work on the NR sidelink, June, 2020 |
| - [8] | - SAE LS / RP-200567 LS on Sidelink Positioning for advanced V2X applications, June 2020 |
| - [9] | - R4-2016464, NR Sidelink Operating Bands, AT&T, FirstNet |
| - [10] | - RP-201484, Positioning use cases and requirements for public safety, FirstNet, AT&T |
| - [11] | - 3GPP TR, Study on NR positioning enhancements, V1.1.0 |