3GPP TSG RAN meeting #92e										RP-211428

Electronic Meeting, June 14-18, 2021

## Status Report to TSG

Agenda item:			9.7.4.9

| WI / SI Name | Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_RRM_enh2 |  |  |  |  |
| Unique ID | 890157 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202874 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: 09/2022 |  |
| Overall Completion level | Study Item: NA | Core part: 30% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Corporation |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Qiuge Guo |
|  | Company | CATT |
|  | Email | guoqiuge@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #98bis-e meeting

(1) SRS antenna port switching

The WF R4-2105786 on SRS antenna port switching was agreed.

- Issue 1-1-1: whether delay requirement would be defined in RRM for SRS antenna port switching

  - Agreements:

    - Do not define SRS antenna port switching delay requirement in RRM

    - FFS whether and how to define the scheduling restriction before and after SRS transmission for the cell with SRS antenna port switching

      - There are no further scheduling restrictions for SRS symbols in addition to the restrictions defined in RAN1 specifications

- Issue 1-1-2: RAN4 defines the requirement only for SRS antenna port switching in FR1 or in both FR1 and FR2

  - Agreement

    - Define the RRM requirements at SRS antenna switching only for FR1 unless the transient period in FR2 gets clarified in RF session (the scope of “RRM requirements” here depends on the conclusions from issue 1-1-1)

- Issue 1-2-1: Interruption requirement applicability

  - Agreement: The interruption requirement should be defined based on the band combination capability reported by UE, i.e., txSwitchImpactToRx or txSwitchWithAnotherBand.

    - FFS: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand.

- Issue 1-2-3: Would the interruption requirement based on different SCS?

  - Agreement

    - Interruption requirement is based on the aggressor CC and victim CC SCS.

(2) Handover with PSCell

The WF R4-2105787 and LS R4-2105830 on handover with PSCell was agreed.

- Issue 2-2-1: timeline for HO with PSCell

  - Agreements:

    - Timeline for HO with PSCell

      - Option 1 (Xiaomi, Apple, OPPO): PCell HO and PSCell addition is performed in a sequential order.

      - Option 2 (CATT, CMCC, Huawei, MTK, QC, ZTE, NEC, Ericsson): PCell HO and PSCell addition is performed in parallel.

      - Option 3 (NTT DOCOMO, Intel, OPPO, Nokia, Ericsson, NEC): Some of procedures of HO with PSCell should be able to be performed in parallel, but RACH processing is performed in a sequential order (RACH procedure of PSCell will happen after the RACH procedure of PCell).

      - Other options are not precluded

    - Send LS to RAN2 to clarify possible restrictions on parallel or sequential RACH processing from RAN2 perspective

- Issue 2-2-2: starting point of the delay requirement for HO with PSCell

  - Agreement

    - For delay requirement of HO with PSCell, the starting point is the end of the last TTI containing the RRC command of handover with PSCell.

- Issue 2-2-6: RRC processing delay for HO with PSCell

  - Agreement: RAN4 waits for the reply LS from RAN2 on RRC processing delay for HO with PSCell

- Issue 2-5: Failure case definition for HO with PSCell

  - Agreement: RAN4 waits for the reply LS from RAN2 before any decision

(3) PUCCH SCell activation/deactivation requirements

The WF R4-2105788 on PUCCH SCell activation/deactivation requirements was agreed.

- Issue 1-1-3: Whether the beam information (SSB index) of PUCCH Scell is needed to be indicated to NW?

  - If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed.

  - If the target PUCCH Scell is unknown cell in FR2:

    - If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

    - If there is no active serving cell on that FR2 band, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- Issue 1-1-6: Known/unknown condition for PUCCH SCell activation?

  - The known and unknown condition for Scell activation can be reused for PUCCH Scell.

- Issue 1-4-1: Interruption requirements for PUCCH Scell activation in valid TA case

  - Reuse the existing requirement for Scell activation in Rel-15.

- Sub-topic 1-6 Interruption requirements for PUCCH Scell deactivation

  - Reuse the existing requirement for Scell deactivation in Rel-15.

- Sub-topic 1-7 PUCCH SCell deactivation delay requirement for activated PUCCH SCell with multiple Scells

  - Reuse the SCell deactivation delay requirement for activated SCell with multiple downlink SCells specified in section 8.3.8 of TS 38.133, which is (( THARQ+ 3ms)/ NR slot length).

RAN4 #99-e meeting

(1) SRS antenna port switching

The WF R4-2108343 on SRS antenna port switching was agreed.

- Issue 1-1-3: RAN4 requirement scope with LTE SRS antenna port switching

  - Agreement: LTE SRS antenna port switching is out of scope of this R17 FeRRM WI, and no need to discuss.

- Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

  - Agreement: No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - The above-mentioned NR measurement requirements includes serving/neighboring cell L3 measurement requirements

      - FFS on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements

- Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC

  - Agreement

    - In EN-DC and NE-DC operation,

      - NR SRS antenna switching colliding with E-UTRA measurement

        - Interruptions on E-UTRA measurement in the interrupted carrier group are allowed due to NR SRS antenna switching, but NOT allowed due to NR SRS antenna switching for the carriers not in the interrupted carrier group.

        - Additional delay can be expected on E-UTRA measurement in the interrupted carrier group when UE is configured to perform NR SRS antenna switching.

        - NR SRS antenna switching is allowed to be dropped when colliding with E-UTRA measurement in the interrupted carrier group.

- Issue 1-2-5: Impact of SRS antenna port switching to timing requirements

  - Agreement:

    - New option 4: No need to consider impact to timing requirements (non-positioning related) for SRS antenna switching.

- Issue 1-2-6: Impact of SRS antenna port switching to positioning related requirements

  - Agreement: The impact of SRS antenna switching on positioning related measurement will not be discussed in this Rel-17 FeRRM.

- Issue 1-3-3: whether same interruption requirement applies to different SRS antenna port switching patterns

  - Agreement: use same set of requirements for different SRS antenna switch patterns.

(2) Handover with PSCell

The WF R4-2108045 on handover with PSCell was agreed.

- Tprocessing includes both software processing time and RF warm up time.

- It depends on RAN2 reply whether RACH processing can be performed in parallel or not and it can be further discussed

- Considering the reply LS from RAN2, RRC processing delay for HO with PSCell is:

  - NR SA to EN-DC : 50ms

  - EN-DC to EN-DC: 20ms

  - NE-DC to NE-DC: 16ms

  - NR-DC to NR-DC: 16ms

  - Note: RRC processing delay is defined in RAN2 specification.

- PCC could be scheduled for UE when Pcell HO is completed but PSCell addition is not completed

- PCell HO and PSCell addition, except RACH procedure, are performed in parallel for at least some of procedures

  - FFS condition of parallel processing in Issue 2-2-1a

(3) PUCCH SCell activation/deactivation requirements

The WF R4-2108345 on PUCCH SCell activation/deactivation requirements was agreed.

- Issue 1-1-1: The ending point of PUCCH SCell activation procedure for valid TA case?

  - For valid TA case, the ending point of PUCCH SCell activation should be the point when UE transmit valid CSI report on target PUCCH SCell

- Issue 1-2-1: Whether the beam information (SSB index) of PUCCH SCell is needed to be indicated to NW for unknown cell in FR2?

  - Agreements

    - If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed.

      - If the target PUCCH Scell is unknown cell in FR2:

        - If there is at least one active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

        - Otherwise, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

    - Note: When capturing the agreements in the specification, the text on known and unknown condition shall be aligned with existing SCell activation requirements.

- Issue 1-2-2: Whether the beam information (SSB index) of PUCCH SCell is needed to be indicated to NW for unknown cell in FR1?

  - Agreements:

    - If it is contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 Scell activation), no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

    - Otherwise, need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA.

- Issue 1-2-4: Which cell is the L1-RSRP reporting transmitted for PUCCH SCell activation?

  - Agreements:

    - L1-RSRP report is one kind of CSI report, and the same solution of CSI report in issue 1-2-3 can be applied.

- Issue 1-2-5: Whether the UL spatial relation is needed for PUCCH Scell activation?

  - Agreements:

    - The UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation for valid case.

      - The UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation in FR2 only.

        - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated Scell shall be defined in the baseline FR2 Scell activation delay part (Tactivate_basic). Details are FFS.

    - FFS: whether the UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation for invalid case.

- Issue 1-5-2: the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell (i.e. T1)

  - Agreements:

    - T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213

#### 2.4.2	Remaining Open issues

(1) SRS antenna port switching (open issues in R4-2108343)

- Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

- Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC

- Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Issue 1-2-7: Impact of SRS antenna port switching to CSF and other RS

- Issue 1-3-1: Interruption requirement applicability

- Issue 1-3-2: Interruption requirement for UE with or without per-FR MG capability

- Issue 1-3-4: Would the interruption requirement differentiate between sync and async cases?

- Issue 1-3-5: txSwitchImpactToRx for intra-band case

- Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

- Issue 1-4-3: details of the interruption time in FR1

- Issue 1-4-4: Interruption requirement proposals

- Issue 1-5-1: LS to other groups

(2) Handover with PSCell (open issues in R4-2108045)

- Issue 2-1-1: Scenarios for RRM requirement of HO with PSCell

- Issue 2-1-2: NR-DC and NE-DC mode in HO with PSCell

- Issue 2-1-2a: Requirements for Rel-16 FR1+FR1 NR-DC

- Issue 2-2-1a: Condition of parallel processing

- Issue 2-2-1b: Whether requirements for sequential processing are needed if parallel processing is only possible under certain condition

- Issue 2-2-2: Parallel processing for HO with PSCell

- Issue 2-2-3: UE SW processing and RF warm-up(if needed) time for HO with PSCell

- Issue 2-2-5: Ending point of the delay requirement for HO with PSCell

- Issue 2-2-6: Optimisation for the case when PSCell is not changed during HO with PSCell

- Issue 2-2-8: Delay requirement design if parallel processing is assumed

- Issue 2-2-9: Delay requirement design if sequential processing is assumed

- Issue 2-3-2: Interruption requirement for HO with PSCell

- Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell

- Issue 2-4-3: RACH occasion on NR-U CC for HO with PSCell

(3) PUCCH SCell activation/deactivation requirements (open issues in R4-2108345)

- Issue 1-1-2: The ending point of PUCCH SCell activation procedure for invalid TA case?

- Issue 1-2-3: How to indicate the beam information for PUCCH SCell activation (The procedure for beam indication for PUCCH SCell activation)?

- Issue 1-2-5a: Whether the UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation for invalid case?

- New Issue 1-2-6: Whether the beam information (L1-RSRP measurement result) of PUCCH SCell for TCI determination is needed or not for unknown cell?

- Sub-topic 1-3 UE capability for PUCCH SCell activation requirements

- Sub-topic 1-4 PUCCH Scell activation delay requirement for valid TA case

- Issue 1-5-1: The PUCCH SCell activation requirements for invalid TA case

- Issue 1-5-1a: Whether to define separated requirements for downlink actions and uplink actions?

- Issue 1-5-3: the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs (i.e. T2)

- Issue 1-5-4: the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated (i.e. T3)

- Sub-topic 1-6 Interruption requirements for PUCCH SCell activation in invalide TA case

- Sub-topic 1-7 Applicability of PUCCH SCell activation requirements

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #98bis-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2104564 | Discussion on PUCCH SCell activation and deactivation | MediaTek inc. |
| R4-2104565 | Discussion on SRS antenna port switching | MediaTek inc. |
| R4-2104633 | Regarding PUCCH SCell activation and deactivation | vivo |
| R4-2104685 | Discussion on RRM requirements for handover with PSCell | Xiaomi |
| R4-2104686 | Discussion on SCell activation and deactication requirements for PUCCH Scell | Xiaomi |
| R4-2104694 | Discussion on SRS antenna switching interruption requirements | Xiaomi |
| R4-2104758 | Further discussion on SRS antenna port switching | CATT |
| R4-2104759 | Further discussion on HO with PSCell | CATT |
| R4-2104760 | Further discussion on PUCCH SCell activation_deactivation | CATT |
| R4-2104831 | On SRS antenna port switching | Apple |
| R4-2104832 | On RRM requirement for handover with PSCell | Apple |
| R4-2104833 | On PUCCH SCell activation and deactivation | Apple |
| R4-2104909 | SRS antenna switching discussion | Qualcomm, Inc. |
| R4-2104932 | Views on Procedures of HO with PSCell | NTT DOCOMO, INC. |
| R4-2104943 | Discussion on HO with PSCell | CMCC |
| R4-2104944 | Discussion on PUCCH SCell activation/deactivation | CMCC |
| R4-2104945 | Discussion on SRS antenna port switching | CMCC |
| R4-2104979 | Discussion on SRS antenna port switching | NEC |
| R4-2104980 | Discussion on PSCell HO | NEC |
| R4-2104981 | Discussion on PUCCH SCell activation | NEC |
| R4-2104991 | Discussion on interruption due to SRS antenna port switching | LG Electronics Inc. |
| R4-2105104 | Discussions on PUCCH SCell Activation/Deactivation delay requirements | NTT DOCOMO, INC. |
| R4-2105686 | Email discussion summary for [98-bis-e][216] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2105687 | Email discussion summary for [98-bis-e][217] NR_RRM_enh2_2 | Moderator (CATT) |
| R4-2105786 | WF on further RRM enhancement for NR and MR-DC – SRS antenna port switching | Apple |
| R4-2105787 | WF on further RRM enhancement for NR and MR-DC – Handover with PSCell | Apple |
| R4-2105788 | WF on further RRM enhancement for NR and MR-DC – PUCCH SCell activation/deactivation requirements | CATT |
| R4-2105818 | Email discussion summary for [98-bis-e][216] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2105819 | Email discussion summary: [98-bis-e][216] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2105830 | LS on RACH procedure for HO with PSCell | Ericsson |
| R4-2106408 | Discussion on the activation delay for deactivated PUCCH SCell | Nokia, Nokia Shanghai Bell |
| R4-2106409 | Discussion on the interruption requirements at SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2106462 | Discussion about SRS antenna port switching | Intel Corporation |
| R4-2106463 | Discussion about HO with PSCell | Intel Corporation |
| R4-2106532 | RRM requirements for SRS ant port switch | OPPO |
| R4-2106533 | RRM requirements for HO with PSCell | OPPO |
| R4-2106534 | RRM requirements for PUCCH SCell ActivationDeactivation | OPPO |
| R4-2106881 | On RRM requirements for SRS antenna port switching | Ericsson |
| R4-2106882 | On handover with PSCell | Ericsson |
| R4-2106883 | On SCell (de)activation with PUCCH | Ericsson |
| R4-2106924 | Discussion on handover with PSCell | ZTE Corporation |
| R4-2106986 | Discussion on requirements for SRS antenna switching | Huawei, HiSilicon |
| R4-2106987 | Discussion on requirements for HO with PSCell | Huawei, HiSilicon |
| R4-2106988 | Discussion on requirements for PUCCH SCell activation | Huawei, HiSilicon |
| R4-2107079 | Discussion on RRM requirements for SRS antenna port switching | vivo |
| R4-2107080 | Discussion on RRM requirements for HO with PSCell | vivo |
| R4-2107123 | Discussion on HO with PSCell | MediaTek inc. |
| R4-2107224 | discussion on HO with PSCell | Nokia, Nokia Shanghai Bell |
| R4-2107249 | Discussion on timelines of HO with PSCell | Qualcomm CDMA Technologies |
| R4-2107290 | Discussion on PUCCH SCell Activation | Qualcomm Incorporated |

RAN4 #99e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2108045 | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo |
| R4-2108148 | Email discussion summary: [99-e][224] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2108149 | Email discussion summary: [99-e][225] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2108150 | Email discussion summary: [99-e][226] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2108343 | WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple |
| R4-2108344 | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | Vivo |
| R4-2108345 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2108396 | Email discussion summary: [99-e][224] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2108397 | Email discussion summary: [99-e][225] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2108398 | Email discussion summary: [99-e][226] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2108768 | Discussion on handover with PSCell | ZTE Corporation |
| R4-2108970 | Discussion on PUCCH SCell Activation | Qualcomm Incorporated |
| R4-2109050 | Further discussion on SRS antenna port switching | CATT |
| R4-2109051 | Further discussion on HO with PSCell | CATT |
| R4-2109052 | Further discussion on PUCCH SCell activation_deactivation | CATT |
| R4-2109243 | Discussion on SRS antenna port switching | Intel Corporation |
| R4-2109244 | Discussion on HO with PSCell | Intel Corporation |
| R4-2109247 | Further discussion on SRS antenna switching RRM requirements | Xiaomi |
| R4-2109250 | Further discussion on RRM requirements for handover with PSCell | Xiaomi |
| R4-2109251 | Further discussion on SCell activation and deactication requirements for PUCCH SCell | Xiaomi |
| R4-2109308 | On SRS antenna port switching | Apple |
| R4-2109309 | On RRM requirement for handover with PSCell | Apple |
| R4-2109310 | On PUCCH SCell activation and deactivation | Apple |
| R4-2109510 | Discussion on HO with PSCell | CMCC |
| R4-2109518 | Discussion on PUCCH SCell activation/deactivation | CMCC |
| R4-2109520 | Discussion on SRS antenna port switching | CMCC |
| R4-2109545 | Discussions on PUCCH SCell Activation/Deactivation delay requirements | NTT DOCOMO, INC. |
| R4-2109548 | Discussion on the activation delay for deactivated PUCCH SCell | Nokia, Nokia Shanghai Bell |
| R4-2109549 | Discussion on the interruption requirements at SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2109563 | On SRS antenna switching | Qualcomm, Inc. |
| R4-2109617 | On PUCCH SCell activation and deactivation | vivo |
| R4-2109631 | Discussion on PUCCH SCell activation and deactivation | MediaTek inc. |
| R4-2109632 | Discussion on SRS antenna port switching | MediaTek inc. |
| R4-2109717 | Discussion on interruption due to SRS antenna port switching | LG Electronics Inc. |
| R4-2109732 | Further views on timeline assumptions for HO with PSCell | Qualcomm CDMA Technologies |
| R4-2109885 | Discussion on HO with PSCell | MediaTek inc. |
| R4-2109890 | Discussion on SRS antenna port switching | NEC |
| R4-2109891 | Discussion on PSCell HO | NEC |
| R4-2109892 | Discussion on PUCCH SCell activation | NEC |
| R4-2110061 | RRM requirements for SRS ant port switch | OPPO |
| R4-2110062 | RRM requirements for HO with PSCell | OPPO |
| R4-2110063 | RRM requirements for PUCCH SCell ActivationDeactivation | OPPO |
| R4-2110147 | Views on HO with PSCell | NTT DOCOMO, INC. |
| R4-2110343 | Discussion on requirements for SRS antenna switching | Huawei, HiSilicon |
| R4-2110344 | Discussion on requirements for HO with PSCell | Huawei, HiSilicon |
| R4-2110345 | Discussion on requirements for PUCCH SCell activation | Huawei, HiSilicon |
| R4-2110971 | On handover with PSCell | Ericsson |
| R4-2110972 | On SCell (de)activation with PUCCH | Ericsson |
| R4-2110976 | On RRM requirements for SRS antenna port switching | Ericsson |
| R4-2111042 | discussion on HO with PSCell | Nokia, Nokia Shanghai Bell |
| R4-2111264 | Discussion on RRM requirements for SRS antenna port switching | vivo |
| R4-2111265 | Discussion on RRM requirements for HO with PSCell | vivo |

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template