3GPP TSG RAN Meeting #92e									        RP-211520

Electronic Meeting, June 14 - 28, 2021

## Status Report to TSG

Agenda item:			9.2.4

| WI / SI Name | Integrated Access and Backhaul for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB |  |  |  |  |
| Unique ID | 820070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210442 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2020 | Performance part: 06/2021 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 100% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #105-e

Corrections to Rel-16 IAB were not discussed during this meeting.

RAN1 #104bis-e

Conclusion

RAN1 observations on Rel-16 IAB:

- The RAN1#98: “The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies)” is not explicitly captured in either RAN1 or RAN3 specifications.

- It is RAN1’s common understanding that the following interpretation of 38.213 captures the intended behavior in the usage of soft resources configured for a given IAB-DU cell in case the IAB-DU cell and IAB-MT CC are not on the same carrier frequency:

Agreement

Inform RAN3 that based on the above observations, providing H/S/NA slot configurations per (IAB-DU cell, collocated IAB-MT’s serving cell) pair is not necessary in Rel-16 and the existing per IAB-DU cell granularity is sufficient. LS is endorsed in R1-2104058.

Agreement

Text proposal for TS38.213 is endorsed in R1-2104059 (TS38.213, Rel-16, CR#0219, Cat. F).

#### 2.2.2	Remaining Open issues

100% of the items defined in the RAN1 WID objectives have been accomplished. RAN1 has completed the WI.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #113e-bis

Agreements on stage-2 correction:

R2-2103557	Clarification on IP packet type in DedicatedInfoF1c, Nokia, Nokia Shanghai Bell, CR, Rel-16, 37.340, 16.5.0, 0258, -, F, NR_IAB-Core

- Agreements on BAP corrections:

R2-2104560  Miscellaneous corrections on BAP transmitting operation and default routing   Huawei, HiSilicon (Rapporteur),CR, Rel-16, 38.340, 16.4.0, 0015, 1, F, NR_IAB-Core

- Agreements on RRC corrections

R2-2104561  Miscellaneous corrections for TS 36.331 on F1 over LTE for IAB, Huawei, HiSilicon, CR, Rel-16, 36.331, 16.4.0, 4633, -,  F,  NR_IAB-Core

R2-2104597  IAB LTE Changes,Samsung Electronics GmbH, CR, Rel-16, 36.331, 16.4.0, 4649, -, F, NR_IAB-Core

R2-2104173  Missing smtc3 for smtc restriction with ssbFrequency, Samsung R&D Institute UK, CR, Rel-16, 38.331, 16.4.1, 2558, -, F, NR_IAB-Core

Merged with Rapporteur CR

R2-2104562	Miscellaneous corrections on BH RLC channel management for IAB-MT, Huawei, HiSilicon, CR, Rel-16, 38.331, 16.4.1, 2557, 1, F, NR_IAB-Core

RAN2 #114e

Agreements on stage-2 correction:

R2-2105474   Clarification on IP packet type in DedicatedInfoF1c Nokia, Nokia Shanghai Bell, CR, Rel-16, 37.340, 16.5.0, 0258, 1, F, NR_IAB-Core, R2-2103557

R2-2106459   Missing IAB SA mode for QoS description, Samsung, CR, Rel-16, 38.300, 16.5.0, 0386, 2, F, NR_IAB-Core, R2-2104647

R2-2105356   Corrections on stage-2 description for IAB, vivo, CR, Rel-16, 38.300, 16.5.0, 0375, -, F, NR_IAB-Core

1st change only, incoporated into Rapporteur CR R2-2106655

R2-2106206	Miscellaneous corrections on BAP transmitting operation and default routing, Huawei, HiSilicon, CR, Rel-16, 38.340, 16.4.0, 0015, 2, F, NR_IAB-Core, R2-2104560

R2-2106764	Corrections to the handling of unknown, unforeseen, and erroneous protocol data, Ericsson, AT&T, CR, Rel-16, 38.340, 16.4.0, 0018, 1, F, NR_IAB-Core

R2-2106209	Correction for TS38.304 on power class for cell selection of IAB, Huawei, HiSilicon, CR, Rel-16, 38.304, 16.4.0, 0209, -, F, NR_IAB-Core

[024] Adopt Option 1 in R2-2106209 for the IAB-MT power related changes provided in RAN4 LS (R2-2008444) as a baseline. The Option adopted for TS 38.304 should also be used for TS 36.304.

Agreements on LS out:

R2-2106726   LS to RAN4 on power class and P-max for IAB-MT cell selection, RAN2, LS out, Rel-16, NR_IAB-Core, To:RAN4

- Agreements on user plane corrections

R2-2106321	CR for not transmitting only padding and padding BSR with eLCID, Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, LG, ZTE, MediaTek, Intel, CR, Rel-16, 38.321, 16.4.0, 1118, -, F, NR_IAB-Core

- Agreements on RRC corrections

R2-2106774	Miscellaneous corrections on IAB, vivo, CR, Rel-16, 38.331, 16.4.1, 2619, 1, F, NR_IAB-Core

#### 2.2.2	Remaining Open issues

100% of the items defined in the RAN2 WID objectives have been accomplished. RAN2 has completed the WI.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #112e

Agreements on corrections:

| R3-212173 | Miscellaneous corrections on IAB in TS 38.473 (ZTE) | CR0763r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-212798 | Correction on BH RLC CH configured for BAP control PDU (Huawei, CATT, Qualcomm , Lenovo, Motorola Mobility,Nokia, Nokia Shanghai Bell) | CR0770r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| R3-212411 | Correction on gNB-DU Resource Configuration (Huawei, China Unicom) | CR0771r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| R3-212799 | Correction on UL BH information configuration for DRBs support CA based duplication (Huawei, Qualcomm, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell) | CR0772r, TS 38.473 v16.5.0, Rel-16, Cat. F |

#### 2.3.2	Remaining Open issues

100% of the items defined in the RAN3 WID objectives have been accomplished. RAN3 has completed the WI.

## 2.4	RAN4

#### 2.4.1	Agreements

IAB core maintenance related agreements:

RAN4 #98-Bis-e

- The following document was endorsed.

- R4-2107135: Side conditions for IAB-MT RRM test cases in TS 38.174

The following CRs for 38.174 with corrections have been endorsed:

R4-2107228 - CR on Rx Charateristic other related requirements

R4-2106042 - Draft CR to TS 38.174 – corrections to general and transmitter part

R4-2106043 - Draft CR to TS 38.174 – corrections to receiver part

The following CRs for 38.809 have been endorsed:

R4-2106041 - Draft CR for TR 38.809: IAB-MT EVM measurement

RAN4#99-e

- The following document has been endorsed

- R4-2111300: Side conditions in IAB-MT RRC connection mobility requirements in TS 38.174

  - Corrections on side conditions SSB Es/Iot and SSP_RP for IAB-MT testing

The following CRs for 38.174 with corrections have been endorsed:

R4-2108536 - Draft CR for TS 38.174: IAB-MT EVM measurement

R4-2108537 - CR to TS 38.174 – corrections to general and transmitter part

R4-2108538 - CR to TS 38.174 – corrections to receiver part

R4-2108539 - CR on the further clear up the IAB specification

All the changes were captured in big CRs for TS 38.174 and TR 38.809:

R4-2108097 - Big CR for update Core part of TS 38.174

R4-2110000 - Big CR for update on TR38.809

IAB conformance testing related agreements:

RAN4 #98-Bis-e

Following agreements were reached and captured in WFs:

R4-2106044 : WF on Test Models

TDD pattern for IAB test models is agreed as follow:

- Apply BS FR1 TDD configuration for IAB-MT in FR1 (Baseline)

  - Further consider TDD pattern which has similar percentile of the DL and UL slots

- It is FFS for BS FR2 TDD configuration considering available DL/UL slots during tests with following options:

- Option 1: re-use of BS FR2 TDD configurations

- Option 2: TDD patten which has similar percentile of the DL and UL slots

RB allocation for IAB-MT emission testing is agreed as follow:

- It is FFS the following options:

- Option 1: Full RB allocation (as BS emission testing)

- Option 2: Both single and full RB allocation (as UE emission testing)

- Option 3: Single RB allocation (more stringent case for IAB testing)

R4-2106045 : WF on Test Configurations

Proposals to be considered for test configurations:

1) To re-use NRTC1, NRTC2, NRTC3, NRTC4 and NRTC5 for both IAB-DU and IAB-MT as baseline.

2) To place both DL and UL transmission for IAB-DU and IAB-MT in the same TC.

3) To allocate single PRB within channel bandwidth for IAB-MT transmission with 5/10 dB lower PSD than the maximum available PSD.

4) To allocate the channel bandwidth for IAB-DU and IAB-MT in the same frequency range (simultaneous operation in SDM/TDM mode) or different frequency ranges (simultaneous operation in FDM/SDM/TDM mode).

R4-2106046 : WF on MU value tables

Proposes tentative values for the measurement uncertainties, to be confirmed in RAN4#99-e

R4-2106047 : WF on Test point reduction for shared IAB-DU and IAB-MT hardware

Sub-topic#4-1-1 : Declaring shared HW

WF:

It is declared that RF HW is the same between IAB-DU and IAB-MT

Sub-topic#4-1-2 : When Test case reduction can be used (IAB-MT and IAB-DU test results are interchangeable)

WF:

Test conditions of the test case reduction of IAB-MT and IAB-DU can be used:

- Declaration of the same output power

- Declaration of the same RF bandwidth

To further discuss:

Issue#4-1-2-1: Test Conditions of the test case reduction of IAB-MT and IAB-DU can be used:

- Declaration of the same direction

- Other OTA test conditions: e.g number of directions

Issue#4-1-2-2: Test requirement selection if the IAB-DU and IAB-MT Test tolerance is different

- In the case of different TT for IAB-DU and IAB-MT,  select the DUT with tighter test requirement

- In the case of different TT for IAB-DU and IAB-MT,  select the test requirement value with tighter TT.

Sub-topic 4-2 : Tables for test case reduction

The principle is to have a clear view on which test case can be reduced and when the test results is interchangeable, then a simple table may be preferred with the conditions to choose the preferred DUT. Nokia table (6688) gives two way of the reducing the test cases when test results of IAB-Du and IAB-MT are interchangeable, one is to split the test case among the two DUT and the other is sufficiently to test only one (IAB-DU preferred in the table 6688). It is not clear under what circumstances the test cases can be split and under what other circumstance the DUT can be freely chosen. , so it may be good to have company option on this point for 2nd round discussion. For the table selection, Nokia table seems good fo splitting case and Ericsson table is simple for latter case.

For 2nd round discussion:

Condition for test case splitting and sufficiently test one

- Under which condition the test cases should be split and under which condition sufficiently test one (either IAB-DU or IAB-MT)

Nokia, Nokia Shanghai Bell: Condition for test case splitting and sufficiently test one: Our concerns are more on the overall test coverage rather than individual tests. Splitting tests automatically ends up making sure that functionality from both MT and DU sides are tested. However, if (almost) all tests are interchangeable, a situation is possible when practically no MT operation or no DU operation is tested. Therefore, for the approach of interchangeable tests, some side condition is needed on requiring that in the big picture both MT and DU end up tested. WF:

To be discussed in next meeting.

Sub-topic 4-3 : TP drafting text for test case reduction

Tentative agreements:

New section in chapter 4 is added to capture test reduction table:

Option 1: The section title and text in 7237 and 7238 is ok to use

Option 2: No new section, captured in existing clause

Option 3: FFS

WF:

To be discussed in next meeting.

Following TPs were agreed:

R4-2106313 - Proposal on the skeleton of TS38.176-2

R4-2106050 - TP for Annex E for conducted IAB test specification

R4-2106051 - TP for Annex G and H for OTA IAB test specification

R4-2106131 - TP to TS 38.176-1 -Clause 4.1

R4-2106054 - TP to TS38.176-1 on subclause 4.10 -5

R4-2106055 - TP to TS38.176-2 on subclause 4.10 -5

R4-2106056 - TP to TS 38.xxx-1:  Section 4.2~4.5

R4-2106057 - TP to TS 38.xxx-2:  Section 4.2~4.5

R4-2106058 - IAB conducted conformance specification skeleton

R4-2106059 - TP to TS 38.176-1  - Annex D

R4-2106062 - TP for TS 38.176-1: Transmit ON/OFF power

R4-2106064 - TP to TS 38.176-1: Output power and Unwanted emission

R4-2106065 - TP to TS 38.176-1:  TX IMD requirements

R4-2106066 - TP to TS 38.176-1 Annex A for IAB conducted test specification

R4-2106067 - TP to TS 38.176-1:  RX IMD requirements

R4-2106068 - TP to TS 38.176-1:  RX ICS requirements

R4-2106069 - TP to TS 38.176-1  - Sensitivity, clause  7.2

R4-2106070 - TP to TS 38.176-1  - Rx dynamic range, clause  7.3

R4-2106071 - TP for IBB, OBB and RX spurious of conducted receiver test

R4-2106073 - TP for TS 38.176-2:  OTA transmit ON/OFF power

R4-2106066 - TP to TS 38.176-1 Annex A for IAB conducted test specification

R4-2106077 - TP to TS 38.176-2 Annex A for IAB OTA test specification

R4-2106078 - TP to TS 38.176-2:  RX IMD requirements

R4-2106080 - TP to TS 38.176-2  - OTA Sensitivity, clause  7.2, 7.3

R4-2106081 - TP to TS 38.176-2  - OTA Rx dynamic range, clause  7.3

R4-2106082 - TP on IBB, OBB and RX spurious for OTA receiver characteristic test

R4-2106083 - TP to TS 38.146-2 Clause 4.6 Declarations for IAB radiated test specification

R4-2106084 - TP to TS 38.176-2  - Annex D&E

R4-2106076	 - TP to TS 38.xxx-2: TX IMD requirements, ZTE, Approved

The following WF was approved:

R4-2106085	 - WF on IAB EMC spatial exclusion, Ericsson, Approved

RAN4#99-e

Following TPs were agreed:

R4-2108566 - TP to TS 38.176-2 on test efficiency clause 4.13

R4-1208573 - TP to TS 38.176-1 -Clause 4.1

R4-1208574 - TP to TS 38.176-2 -Clause 4.1

R4-2108567  - IAB Common test issue on test model-Conducted

R4-2108568 - IAB Common test issue on test model-OTA

R4-2108569  - TP for TS 38.176-1: Test configurations and applicability of requirements

R4-2108570 - TP for TS 38.176-2: Test configurations and applicability of requirements

R4-2108571 - TP for TS 38.176-1: Annex B and C

R4-2108572 - TP for TS 38.176-2: Annex B and C

R4-2111397 - TS 38.176-1 -Updated TS 37.176-1

R4-2111399 - TP to TS 38.176-1 -Clean up

R4-2111177 - TP for IBB, OBB and RX spurious of conducted receiver test

R4-2111405 - TP to TS 38.176-1  - Sensitivity, clause  7.2

R4-2110944 - TS 38.176-2 v.0.1.0  - update after RAN4#98bis meeting

R4-2110608 - TP to TS 38.176-2:  RX ICS requirements

R4-2108087 - TP to TS 38.176-1  - OTA Tx dynamic range, clause  6.3

R4-2108576 - TP to TS 38.176-1 Clause 4.6 Declarations for IAB conducted test specification

R4-2108577  - TP to TS 38.176-1:  Annex G and H: In-channel TX test

R4-2108710  - TP for TS 38.176-1: Transmitted signal quality

R4-2108578  - TP to 38.176-2 Editor update - editorials

R4-2108579  - TP for TS 38.176-2: OTA transmitted signal quality

R4-2108580  - TP to TS 38.176-2: clauses 6.1, 6.2, 6.3 and 6.7

R4-2108582  - TP to TS 38.176-2  - OTA Tx dynamic range, clause  6.4

R4-2108088    - TP to TS 38.176-2  - OTA Tx dynamic range, clause  6.4

R4-2108583  - TP to TS 38.176-2 Clause 4.6 Declarations for IAB radiated test specification

R4-2108584  - TP to TS38.176-2 on Annex I and Annex K

R4-2108585  - TP to TS 38.176-2:  Annex L and M: In-channel TX test

R4-2108586  - TP to TS 38.176-2 – Clause 3

R4-2108587  - IAB-MT specific declaration FR2

All the TPs will be compiled into Big TPs for the new IAB conformance specifications and will be presented in RAN#92-e for one step approval:

R4-2108095 - Big TP to TS 38.176-1 on RF part

R4-2108096 - Big TP to TS 38.176-2 on RF part

IAB RRM related agreements:

RAN4#98-Bis-e

- The following WF agreements have been reached

R4-2105847: WF on test cases for IAM-MTs

  - Do not define test cases for WA IAB-MTs.

  - Do not define test cases of timing advance for both LA IAB-MTs and WA IAB-MTs.

  - Minimum SSB_RP for IAB-MT RRM requirement is derived from the reference sensitivity (REFSENS) defined for LA and WA IAB-MT classes and for all IAB types (1-H, 1-O and 2-O).

  - The CSI-RS RLM tests for IAB-MT are based on the same methodology used in corresponding CSI-RS based RLM tests defined for the UE in TS 38.133

- The following CR’s have been agreed for Rel-16 IAB RRM test:

R4-2106951: Draft CR on maintenance for IAB-MT RRM test cases

R4-2105730: Big CR: IAB-MT RRM test cases in 38.174

R4-2106952: Draft CR on test cases for RRC release with redirection for IAB-MT

R4-2105733: [draft CR] Test cases for timing for IAB-MT

R4-2107137: CSI-RS based RLM tests for LA IAB-MT in TS 38.174

R4-2105731: [draft CR] Test cases for Beam Failure Detection and Link Recovery with CSI-RS in FR1

R4-2105732: [draft CR] Test cases for Beam Failure Detection and Link Recovery with SSB in FR1

R4-2105734: Draft CR to introduce test cases for BFD and LR based on SSB in FR2 for IAB-MT

R4-2105735: draftCR on test cases for CSI-RS based BFD and LR for IAB-MTs

- RAN4#99-e

- The following CR’s have been agreed for Rel-16 IAB RRM test:

R4-2111299: Draft Big CR: IAB-MT RRM test cases in 38.174

R4-2111302: Correction to CSI-RS based RLM tests for LA IAB-MT in TS 38.174

- The following WF agreements have been reached

R4-2102825: WF for Rel-16 IAB_RRM test

  - In CSI-RS RLM tests for IAB-MT, test times are defined without any update to the core requirements for FR2.

  - Keep previous agreements on the core requirements for CSI-RS based RLM in FR2

Rel-16 NR IAB Demodulation related agreements:

RAN4#98-bis-e

- R4-2106172	WF on Rel-16 NR IAB demodulation requirements, Nokia, Approved.

RAN4#99-e

- R4-2108606	draftCR to 38.174: IAB-MT and IAB-DU performance requirements, Intel Corporation, Endorsed.

- R4-2108607	TP to TS 38.176-1: FRC and PRACH test preambles, Intel Corporation, Approved.

- R4-2108605	TP to TS 38.176-2: Demodulation manufacturer declarations, Intel Corporation, Approved.

- R4-2109211	Big TP to TS 38.176-1: IAB demodulation performance requirements, Intel Corporation, Approved.

- R4-2108589	WF on Rel-16 NR IAB demodulation requirements, Nokia, Approved.

- R4-2108590	pCR on IAB conducted conformance testing (Manufacturer declarations) to TS 38.176-1, Huawei, HiSilicon, Approved.

- R4-2108591	pCR on IAB radiated conformance testing (FRCs and PRACH test preambles) to TS 38.176-2, Huawei, HiSilicon, Approved.

- R4-2108592	pCR to 38.176-1: Introduction of annexes on test tolerance, test setup and propagation conditions for performance requirements, Ericsson, Approved.

- R4-2108593	Draft CR to 38.174: FRCs and PRACH preambles, Ericsson, Endorsed.

- R4-2108594	draftTP to TS 38.176-2 IAB-DU performance requirements and parts of DU and MT appendix, Nokia, Nokia Shanghai Bell, Approved.

- R4-2111396	bigTP draft to TS 38.176-2 Demodulation performance, Nokia, Nokia Shanghai Bell, Approved.

- R4-2108595	Draft CR to 38.174: Introduction of IAB-DU performance requirements, Ericsson, Endorsed.

- R4-2108596	draftTP to TS 38.176-1 IAB-DU performance requirements, Nokia, Nokia Shanghai Bell, Approved.

- R4-2108604	Big CR on IAB-MT demodulation in TS 38.174, Huawei, HiSilicon, Agreed.

- R4-2108597	pCR on IAB-MT conducted conformance testing (CSI reporting and Interworking) to TS 38.176-1, Huawei, HiSilicon, Approved.

- R4-2108598	draftCR on IAB-MT conducted performance requirements (General and Demodulation) in TS 38.174, Huawei, HiSilicon, Endorsed.

- R4-2108599	pCR on IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2, Huawei, HiSilicon, Approved.

- R4-2108600	pCR to 38.176-2: Introduction of CSI-RS performance tests and requirements, Ericsson, Approved.

- R4-2108601	pCR to 38.176-1: IAB-MT performance tests, Ericsson, Approved.

- R4-2108602	TS 38.174 draftCR CSI reporting radiated performance requirements, Nokia, Nokia Shanghai Bell, Approved.

#### 2.4.2	Remaining Open issues

- 100% of the items defined in the RAN4 Core Part objectives have been accomplished. 100% of the items defined in the RAN4 Performance Part objectives have been accomplished.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#104bise:

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2102294 | LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | RAN3, Ericsson |
| R1-2102588 | Correction on search space definition for Type-3 CSS and USS | CATT |
| R1-2102927 | Draft Reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | vivo |
| R1-2103136 | Reply to RAN3 LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | Qualcomm Incorporated |
| R1-2103210 | Draft reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | Samsung |
| R1-2103284 | About reply LS on Granularity of the HSNA Slot Configurations for the IAB-DU | ZTE, Sanechips |
| R1-2103322 | Draft reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | ETRI |
| R1-2103498 | Clarification on availability indication for IAB-DU cell | ZTE, Sanechips |
| R1-2103628 | Draft reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | LG Electronics |
| R1-2103713 | H/S/NA configuration per pair of IAB-DU cell and IAB-MT’s serving cell | Ericsson, AT&T |
| R1-2103753 | Discussion on LS on IAB-DU H/S/NA Configuration per Cell | Huawei, HiSilicon |
| R1-2103769 | About reply LS on Granularity of the HSNA Slot Configurations for the IAB-DU | ZTE, Sanechips |
| R1-2103797 | H/S/NA configuration per pair of IAB-DU cell and IAB-MT’s serving cell | Ericsson, AT&T |
| R1-2103993 | Session notes for 7.2.3 (Maintenance of Integrated Access and Backhaul for NR) | Ad-hoc Chair (Samsung) |
| R1-2104058 | Reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | RAN1, Ericsson |
| R1-2104059 | Conditions for IAB-DU soft resource availability | Moderator (AT&T), Qualcomm, Ericsson |

RAN2#113bise:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2102764 | Stop ongoing Random Access procedure due to pre-emptive BSR | CATT |
| R2-2102800 | Clarification on DLInformationTransfer and ULInformationTransfer | CATT |
| R2-2103137 | Correction on IAB in TS 38.306 | ZTE, Sanechips |
| R2-2103293 | CR for not transmitting only padding and padding BSR with eLCID | Samsung |
| R2-2103557 | Clarification on IP packet type in DedicatedInfoF1c | Nokia, Nokia Shanghai Bell |
| R2-2103558 | Clarification on IP packet type in DedicatedInfoF1c | Nokia, Nokia Shanghai Bell |
| R2-2103598 | Transfer of F1C traffic over LTE leg in IAB - Option A | Samsung Electronics GmbH |
| R2-2103601 | Transfer of F1C traffic over LTE leg in IAB - Option B | Samsung Electronics GmbH |
| R2-2103935 | Handling of Unknown and Reserved Values in the BAP Header | Ericsson, AT&T |
| R2-2103937 | Clarification to BAP address field description in the BAP-RoutingID IE | Ericsson |
| R2-2104165 | Miscellaneous corrections on BAP transmitting operation and default routing | Huawei, HiSilicon (Rapporteur) |
| R2-2104166 | Miscellaneous corrections for TS 36.331 on F1 over LTE for IAB | Huawei, HiSilicon |
| R2-2104167 | Miscellaenous corrections on BH RLC channel management for IAB-MT | Huawei, HiSilicon |
| R2-2104172 | Missing IAB SA mode for QoS description | Samsung R&D Institute UK |
| R2-2104173 | Missing smtc3 for smtc restriction with ssbFrequency | Samsung R&D Institute UK |
| R2-2104177 | Correction on ULInformationTransfer failure handling for IAB in 36.331 [Opt A] | Samsung R&D Institute UK |
| R2-2104178 | Correction on ULInformationTransfer failure handling for IAB in 36.331 [Opt B] | Samsung R&D Institute UK |
| R2-2104513 | Discussion summary (Phase 1) - [AT113bis-e][022][NR16] IAB LTE Changes (Samsung) | Samsung |
| R2-2104560 | Miscellaneous corrections on BAP transmitting operation and default routing | Huawei, HiSilicon (Rapporteur) |
| R2-2104561 | Miscellaneous corrections for TS 36.331 on F1 over LTE for IAB | Huawei, HiSilicon, Samsung Electronics GmbH |
| R2-2104562 | Miscellaenous corrections on BH RLC channel management for IAB-MT | Huawei, HiSilicon |
| R2-2104597 | IAB LTE changes | Samsung |
| R2-2104647 | Missing IAB SA mode for QoS description | Samsung R&D Institute UK |

RAN2#114e:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2105356 | Corrections on stage-2 description for IAB | vivo |
| R2-2105357 | Corrections on BAP Control PDU operations | vivo |
| R2-2105358 | Miscellaneous corrections on IAB | vivo |
| R2-2105360 | Capability of supporting one-octet eLCID in IAB - Option A | vivo |
| R2-2105361 | Capability of supporting one-octet eLCID in IAB - Option B | vivo |
| R2-2105362 | Capability of supporting one-octet eLCID in IAB - Option B | vivo |
| R2-2105474 | Clarification on IP packet type in DedicatedInfoF1c | Nokia, Nokia Shanghai Bell |
| R2-2105602 | IAB LTE changes | Samsung Electronics GmbH |
| R2-2105645 | Resolving ambiguity in use of BAP routing ID | Samsung Electronics GmbH |
| R2-2105749 | Clarification on MAC PDU assembly with eLCID | Huawei, HiSilicon |
| R2-2105875 | Handling of erroneous data at BAP layer | Samsung Electronics GmbH |
| R2-2106027 | Corrections to the handling of unknown, unforeseen, and erroneous protocol data | Ericsson, AT&T |
| R2-2106028 | Handling of Unknown and Reserved Values in the BAP Header | Ericsson, AT&T |
| R2-2106031 | Clarification to transmission of padding and padding BSR with eLCID in IAB | Ericsson, Apple |
| R2-2106206 | Miscellaneous corrections on BAP transmitting operation and default routing | Huawei, HiSilicon |
| R2-2106207 | Miscellaenous corrections on BH RLC channel management for IAB-MT | Huawei, HiSilicon |
| R2-2106208 | Miscellaneous corrections on F1 over LTE for IAB | Huawei, HiSilicon, Samsung |
| R2-2106209 | Correction for TS38.304 on power class for cell selection of IAB | Huawei, HiSilicon |
| R2-2106210 | Correction for TS36.304 on power class for cell selection of IAB | Huawei, HiSilicon |
| R2-2106218 | Correction on BAP handling for the hybrid release IAB deployment | Huawei (Rapporteur), HiSilicon |
| R2-2106219 | Discussion on extension principles for mixed deployment of IAB node in different releases | Huawei, HiSilicon |
| R2-2106221 | Enhancements for topology-wide fairness, multi-hop latency and congestion mitigation | Huawei, HiSilicon |
| R2-2106321 | CR for not transmitting only padding and padding BSR with eLCID | Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, LG, ZTE, MediaTek, Intel |
| R2-2106459 | Missing IAB SA mode for QoS description | Samsung |
| R2-2106724 | Correction for TS38.304 on power class for cell selection of IAB | Huawei, HiSilicon |
| R2-2106725 | Correction for TS36.304 on power class for cell selection of IAB | Huawei, HiSilicon |
| R2-2106726 | LS to RAN4 on power class and P-max for IAB-MT cell selection | RAN2 |
| R2-2106763 | Summary of [AT114-e][019][NR16] BAP (Ericsson) | Ericsson |
| R2-2106764 | Corrections to the handling of unknown, unforeseen, and erroneous protocol data | Ericsson, AT&T, Samsung, Huawei, Nokia |
| R2-2106774 | Miscellaneous corrections on IAB | vivo |

RAN3#112e:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-212173 | Miscellaneous corrections on IAB in TS 38.473 | ZTE |
| R3-212410 | Correction on BH RLC CH configured for BAP control PDU | Huawei, CATT, Qualcomm , Lenovo, Motorola Mobility,Nokia, Nokia Shanghai Bell |
| R3-212411 | Correction on gNB-DU Resource Configuration | Huawei, China Unicom |
| R3-212412 | Correction on UL BH information configuration for DRBs support CA based duplication | Huawei, Qualcomm, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell |
| R3-212594 | Reply LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU | RAN1 |

RAN4#98bise:

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2104482 | On IAB test cases | ZTE Corporation |
| R4-2104659 | Draft CR to 38.174: Introduction of IAB-DU performance requirements | Ericsson |
| R4-2104660 | pCR to 38.176-1: Introduction of annexes on test tolerance, test setup and propagation conditions for performance requirements | Ericsson |
| R4-2104661 | Draft CR to 38.174: FRCs and PRACH preambles | Ericsson |
| R4-2104662 | pCR to 38.176-2: Introduction of CSI-RS performance tests and requirements | Ericsson |
| R4-2104663 | pCR to 38.176-1: IAB-MT performance tests | Ericsson |
| R4-2104664 | IAB-DU remaining issues | Ericsson |
| R4-2104665 | IAB-MT remaining issues | Ericsson |
| R4-2104666 | IAB-MT simulation results | Ericsson |
| R4-2104781 | Discussion on open issues of IAB-MT EVM measurement | CATT |
| R4-2104782 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT |
| R4-2104783 | Draft CR for TR 38.809: IAB-MT EVM measurement | CATT |
| R4-2104784 | Draft CR for TS 38.174: Correction of IAB-DU class definition | CATT |
| R4-2104785 | TP for TS 38.176-1: Test configurations and applicability of requirements | CATT |
| R4-2104786 | TP for TS 38.176-2: Test configurations and applicability of requirements | CATT |
| R4-2104787 | TP for TS 38.176-1: Transmit ON/OFF power | CATT |
| R4-2104788 | TP for TS 38.176-1: Transmitted signal quality | CATT |
| R4-2104789 | TP for TS 38.176-1: Annex B and C | CATT |
| R4-2104790 | TP for TS 38.176-2: OTA transmit ON/OFF power | CATT |
| R4-2104791 | TP for TS 38.176-2: OTA transmitted signal quality | CATT |
| R4-2104792 | TP for TS 38.176-2: Annex B and C | CATT |
| R4-2104928 | [draft CR] Test cases for Beam Failure Detection and Link Recovery with CSI-RS in FR1 | ZTE Corporation |
| R4-2104929 | [draft CR] Test cases for Beam Failure Detection and Link Recovery with SSB in FR1 | ZTE Corporation |
| R4-2104930 | [draft CR] Test cases for timing for IAB-MT | ZTE Corporation |
| R4-2104960 | Further discussion on IAB RI testing with spatial exclusions | ZTE Corporation |
| R4-2105036 | TP to TS38.176-1 on subclause 4.10 -5 | Samsung |
| R4-2105037 | TP to TS38.176-2 on subclause 4.10 -5 | Samsung |
| R4-2105038 | View on Local Area IAB-MT power control testing | Samsung |
| R4-2105729 | Draft CR on maintenance for IAB-MT RRM test cases | Huawei, HiSilicon |
| R4-2105730 | Big CR: IAB-MT RRM test cases in 38.174 | Ericsson |
| R4-2105731 | [draft CR] Test cases for Beam Failure Detection and Link Recovery with CSI-RS in FR1 | ZTE |
| R4-2105732 | [draft CR] Test cases for Beam Failure Detection and Link Recovery with SSB in FR1 | ZTE |
| R4-2105733 | [draft CR] Test cases for timing for IAB-MT | ZTE |
| R4-2105734 | Draft CR to introduce test cases for BFD and LR based on SSB in FR2 for IAB-MT | Huawei, HiSilicon |
| R4-2105735 | draftCR on test cases for CSI-RS based BFD and LR for IAB-MTs | Nokia, Nokia Shanghai Bell |
| R4-2106040 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT |
| R4-2106041 | Draft CR for TR 38.809: IAB-MT EVM measurement | CATT |
| R4-2106042 | Draft CR to TS 38.174 – corrections to general and transmitter part | Nokia, Nokia Shanghai Bell |
| R4-2106043 | Draft CR to TS 38.174 – corrections to receiver part | Nokia, Nokia Shanghai Bell |
| R4-2106048 | TP for TS 38.176-1: Test configurations and applicability of requirements | CATT |
| R4-2106049 | TP for TS 38.176-2: Test configurations and applicability of requirements | CATT |
| R4-2106050 | TP for Annex E for conducted IAB test specification | Ericsson |
| R4-2106051 | TP for Annex G and H for OTA IAB test specification | Ericsson |
| R4-2106052 | IAB Common test issue on test model-Conducted | Ericsson |
| R4-2106053 | IAB Common test issue on test model-OTA | Ericsson |
| R4-2106054 | TP to TS38.176-1 on subclause 4.10 -5 | Samsung |
| R4-2106055 | TP to TS38.176-2 on subclause 4.10 -5 | Samsung |
| R4-2106056 | TP to TS 38.xxx-1: Section 4.2~4.5 | ZTE Corporation |
| R4-2106057 | TP to TS 38.xxx-2: Section 4.2~4.5 | ZTE Corporation |
| R4-2106058 | IAB conducted conformance specification skeleton | Huawei |
| R4-2106059 | TP to TS 38.176-1 - Annex D | Huawei |
| R4-2106060 | TP to TS 38.176-1 - Tx dynamic range, clause 6.3 | Huawei |
| R4-2106061 | TP to TS 38.176-2 - OTA Tx dynamic range, clause 6.3 | Huawei |
| R4-2106062 | TP for TS 38.176-1: Transmit ON/OFF power | CATT |
| R4-2106063 | TP for TS 38.176-1: Transmitted signal quality | CATT |
| R4-2106064 | TP to TS 38.176-1: Output power and Unwanted emission | Nokia, Nokia Shanghai Bell |
| R4-2106065 | TP to TS 38.xxx-1: TX IMD requirements | ZTE Corporation |
| R4-2106066 | P to TS 38.176-1 Annex A for IAB conducted test specification | Nokia, Nokia Shanghai Bell |
| R4-2106067 | TP to TS 38.xxx-1: RX IMD requirements | ZTE Corporation |
| R4-2106068 | TP to TS 38.xxx-1: RX ICS requirements | ZTE Corporation |
| R4-2106069 | TP toTS 38.176-1 - Sensitivity, clause 7.2 | Huawei |
| R4-2106070 | TP to TS 38.176-1 - Rx dynamic range, clause 7.3 | Huawei |
| R4-2106071 | TP for IBB, OBB and RX spurious of conducted receiver test | Ericsson |
| R4-2106072 | TP to TS 38.176-1 Clause 4.6 Declarations for IAB conducted test specification | Nokia, Nokia Shanghai Bell |
| R4-2106073 | TP for TS 38.176-2: OTA transmit ON/OFF power | CATT |
| R4-2106074 | TP for TS 38.176-2: OTA transmitted signal quality | CATT |
| R4-2106075 | TP to TS 38.176-2: Output power and Unwanted emission | Nokia, Nokia Shanghai Bell |
| R4-2106076 | TP to TS 38.xxx-2: TX IMD requirements | ZTE Corporation |
| R4-2106077 | TP to TS 38.176-2 Annex A for IAB OTA test specification | Nokia, Nokia Shanghai Bell |
| R4-2106078 | TP to TS 38.xxx-2: RX IMD requirements | ZTE Corporation |
| R4-2106079 | P to TS 38.xxx-2: RX ICS requirements | ZTE Corporation |
| R4-2106080 | TP to TS 38.176-2 - OTA Sensitivity, clause 7.2, 7.3 | Huawei |
| R4-2106081 | TP to TS 38.176-2 - OTA Rx dynamic range, clause 7.3 | Huawei |
| R4-2106082 | TP on IBB, OBB and RX spurious for OTA receiver characteristic test | Ericsson |
| R4-2106083 | TP to TS 38.146-2 Clause 4.6 Declarations for IAB radiated test specification | Nokia, Nokia Shanghai Bell |
| R4-2106084 | TP to TS 38.176-2 - Annex D&E | Huawei |
| R4-2106085 | WF on IAB EMC spatial exclusion | Ericsson |
| R4-2106086 | Draft CR to TS 38.175 on IAB EMC test configurations and performance requirements | Ericsson, ZTE |
| R4-2106131 | TP to TS 38.176-1 -Clause 4.1 | Huawei |
| R4-2106306 | Draft CR to TS 38.174 – corrections to general and transmitter part | Nokia, Nokia Shanghai Bell |
| R4-2106307 | Draft CR to TS 38.174 – corrections to receiver part | Nokia, Nokia Shanghai Bell |
| R4-2106313 | Proposal on the skeleton of TS38.176-2 | Nokia, Nokia Shanghai Bell |
| R4-2106314 | TP to TS 38.176-1 Clause 4.6 Declarations for IAB conducted test specification | Nokia, Nokia Shanghai Bell |
| R4-2106315 | TP to TS 38.176-1: Output power and Unwanted emission | Nokia, Nokia Shanghai Bell |
| R4-2106316 | TP to TS 38.176-1 Annex A for IAB conducted test specification | Nokia, Nokia Shanghai Bell |
| R4-2106317 | TP to TS 38.176-2 Annex A for IAB OTA test specification | Nokia, Nokia Shanghai Bell |
| R4-2106318 | TP to TS 38.146-2 Clause 4.6 Declarations for IAB radiated test specification | Nokia, Nokia Shanghai Bell |
| R4-2106319 | TP to TS 38.176-2: Output power and Unwanted emission | Nokia, Nokia Shanghai Bell |
| R4-2106321 | Further considerations on IAB test configurations with TP to 38.176-1 and 38.176-2. | Nokia, Nokia Shanghai Bell |
| R4-2106322 | Further considerations on TDD pattern for IAB test models | Nokia, Nokia Shanghai Bell |
| R4-2106433 | Views on IAB-DU demodulation performance requirements | Intel Corporation |
| R4-2106434 | Views on IAB-MT demodulation performance requirements | Intel Corporation |
| R4-2106438 | draftCR to 38.174: IAB-MT and IAB-DU performance requirements | Intel Corporation |
| R4-2106439 | TP to TS 38.176-1: FRC and PRACH test preambles | Intel Corporation |
| R4-2106440 | TP to TS 38.176-2: Demodulation manufacturer declarations | Intel Corporation |
| R4-2106441 | Big TP to TS 38.176-1: IAB demodulation performance requirements | Intel Corporation |
| R4-2106510 | Discussion on Performance criteria for transient phenomena for IAB | Ericsson |
| R4-2106511 | Draft CR to TS 38.175 on IAB EMC test configurations and performance requirements | Ericsson, ZTE |
| R4-2106512 | Discussion on Spatial Exclusion for IAB EMC RI test | Ericsson |
| R4-2106513 | Draft CR to TS 38.175 on Spatial Exclusion for IAB EMC Radiated Immunity test | Ericsson |
| R4-2106571 | On IAB-MT demodulation requirements | Nokia, Nokia Shanghai Bell |
| R4-2106595 | TP to TS 38.xxx-1: Section 4.2~4.5 | ZTE Corporation |
| R4-2106596 | TP to TS 38.xxx-2: Section 4.2~4.5 | ZTE Corporation |
| R4-2106597 | TP to TS 38.xxx-1: TX IMD requirements | ZTE Corporation |
| R4-2106598 | TP to TS 38.xxx-2: TX IMD requirements | ZTE Corporation |
| R4-2106599 | TP to TS 38.xxx-1: RX IMD requirements | ZTE Corporation |
| R4-2106600 | TP to TS 38.xxx-2: RX IMD requirements | ZTE Corporation |
| R4-2106601 | TP to TS 38.xxx-1: RX ICS requirements | ZTE Corporation |
| R4-2106602 | TP to TS 38.xxx-2: RX ICS requirements | ZTE Corporation |
| R4-2106667 | EVM measurement procedure considerations | Nokia, Nokia Shanghai Bell |
| R4-2106668 | IAB RF conformance testing coverage considerations | Nokia, Nokia Shanghai Bell |
| R4-2106669 | Remaining test setup considerations | Nokia, Nokia Shanghai Bell |
| R4-2106777 | On IAB-DU demodulation requirements | Nokia, Nokia Shanghai Bell |
| R4-2106778 | draftTP to TS 38.176-2 IAB-DU performance requirements and parts of DU and MT appendix | Nokia, Nokia Shanghai Bell |
| R4-2106779 | draftCR to TS 38.174 CSI reporting radiated performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2106812 | Discussion on NR IAB-DU demodulation performance requirements | Huawei, HiSilicon |
| R4-2106813 | Discussion on NR IAB-MT demodulation performance requirements | Huawei, HiSilicon |
| R4-2106814 | Simulation results for NR IAB-MT demodulation performance requirements | Huawei, HiSilicon |
| R4-2106815 | Updated simulation assumptions for NR IAB-MT demodulation requirements | Huawei, HiSilicon |
| R4-2106816 | Summary of simulation results for NR IAB-MT demodulation requirements | Huawei, HiSilicon |
| R4-2106817 | Big CR on IAB-MT demodulation in TS 38.174 | Huawei, HiSilicon |
| R4-2106818 | Draft CR on IAB-MT conducted performance requirements (General and Demodulation) in TS 38.174 | Huawei, HiSilicon |
| R4-2106819 | pCR on IAB conducted conformance testing (Manufacturer declarations) to TS 38.176-1 | Huawei, HiSilicon |
| R4-2106820 | pCR on IAB-MT conducted conformance testing (CSI reporting and Interworking) to TS 38.176-1 | Huawei, HiSilicon |
| R4-2106821 | pCR on IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2 | Huawei, HiSilicon |
| R4-2106822 | pCR on IAB radiated conformance testing (FRCs and PRACH test preambles) to TS 38.176-2 | Huawei, HiSilicon |
| R4-2106950 | Discussion on RRM performance requirements for IAB | Huawei, HiSilicon |
| R4-2106951 | Draft CR on maintenance for IAB-MT RRM test cases | Huawei, HiSilicon |
| R4-2106952 | Draft CR on test cases for RRC release with redirection for IAB-MT | Huawei, HiSilicon |
| R4-2106953 | Draft CR to introduce test cases for BFD and LR based on SSB in FR2 for IAB-MT | Huawei, HiSilicon |
| R4-2107046 | IAB-MT conformance Test about EVM Equalizer | Keysight Technologies UK Ltd |
| R4-2107050 | IAB-MT conformance Test setup MU | Keysight Technologies UK Ltd |
| R4-2107094 | bigTP draft to TS 38.176-2 Demodulation performance | Nokia, Nokia Shanghai Bell |
| R4-2107095 | IAB conducted conformance specification skeleton | Huawei |
| R4-2107096 | IAB - discussion on MU values | Huawei |
| R4-2107097 | TP to TS 38.176-1 -Clause 4.1 | Huawei |
| R4-2107098 | TP to TS 38.176-1 - Tx dynamic range, clause 6.3 | Huawei |
| R4-2107099 | TP to TS 38.176-2 - OTA Tx dynamic range, clause 6.3 | Huawei |
| R4-2107100 | TP toTS 38.176-1 - Sensitivity, clause 7.2 | Huawei |
| R4-2107101 | TP to TS 38.176-2 - OTA Sensitivity, clause 7.2, 7.3 | Huawei |
| R4-2107102 | TP to TS 38.176-1 - Rx dynamic range, clause 7.3 | Huawei |
| R4-2107103 | TP to TS 38.176-2 - OTA Rx dynamic range, clause 7.3 | Huawei |
| R4-2107104 | TP to TS 38.176-1 - Annex D | Huawei |
| R4-2107105 | TP to TS 38.176-2 - Annex D&E | Huawei |
| R4-2107133 | Big CR: IAB-MT RRM test cases in 38.174 | Ericsson |
| R4-2107134 | Analysis of side conditions for IAB-MT RRM test cases | Ericsson |
| R4-2107135 | Side conditions for IAB-MT RRM test cases in TS 38.174 | Ericsson |
| R4-2107136 | Analysis of CSI-RS based RLM tests for LA IAB-MT | Ericsson |
| R4-2107137 | CSI-RS based RLM tests for LA IAB-MT in TS 38.174 | Ericsson |
| R4-2107220 | draftCR on test cases for CSI-RS based BFD and LR for IAB-MTs | Nokia, Nokia Shanghai Bell |
| R4-2107225 | IAB-MT EVM procedure | Ericsson |
| R4-2107226 | CR on Tx signal quality requirements | Ericsson |
| R4-2107227 | CR on Tx Power related requirements | Ericsson |
| R4-2107228 | CR on Rx Charateristic other related requirements | Ericsson |
| R4-2107229 | IAB Common test issue on test model-Conducted | Ericsson |
| R4-2107230 | IAB Common test issue on test model-OTA | Ericsson |
| R4-2107231 | On IAB-MT dynamic range and power control test for conduct test | Ericsson |
| R4-2107232 | On IAB-MT dynamic range and power control test for OTA test | Ericsson |
| R4-2107233 | TP for Annex E for conducted IAB test specification | Ericsson |
| R4-2107234 | TP for Annex G and H for OTA IAB test specification | Ericsson |
| R4-2107235 | TP for IBB, OBB and RX spurious of conducted receiver test | Ericsson |
| R4-2107236 | TP on IBB, OBB and RX spurious for OTA receiver characteristic test | Ericsson |
| R4-2107237 | On IAB test case reduction for IAB Conducted conformance test | Ericsson |
| R4-2107238 | On IAB test case reduction for IAB OTA conformance test. | Ericsson |
| R4-2107251 | draftTP to TS 38.176-1 IAB-DU performance requirements | Nokia, Nokia Shanghai Bell |

RAN4#99e:

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2108478 | CR to TS 38.175: IAB EMC test configurations and performance requirements (updated) | Huawei |
| R4-2109016 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT |
| R4-2109017 | TP for TS 38.176-1: Test configurations and applicability of requirements | CATT |
| R4-2109018 | TP for TS 38.176-2: Test configurations and applicability of requirements | CATT |
| R4-2109019 | TP for TS 38.176-1: Transmitted signal quality | CATT |
| R4-2109020 | TP for TS 38.176-1: Annex B and C | CATT |
| R4-2109021 | TP for TS 38.176-2: OTA transmitted signal quality | CATT |
| R4-2109022 | TP for TS 38.176-2: Annex B and C | CATT |
| R4-2109207 | Views on IAB-MT demodulation performance requirements | Intel Corporation |
| R4-2109208 | draftCR to 38.174: IAB-MT and IAB-DU performance requirements | Intel Corporation |
| R4-2109209 | TP to TS 38.176-1: FRC and PRACH test preambles | Intel Corporation |
| R4-2109210 | TP to TS 38.176-2: Demodulation manufacturer declarations | Intel Corporation |
| R4-2109211 | Big TP to TS 38.176-1: IAB demodulation performance requirements | Intel Corporation |
| R4-2109651 | CR to TS 38.175: Radiated emission, ancillary equipment | ZTE Corporation |
| R4-2109831 | IAB RF conformance test efficiency improvement | Nokia, Nokia Shanghai Bell |
| R4-2109832 | TP to TS 38.176-1 Clause 4.6 Declarations for IAB conducted test specification | Nokia, Nokia Shanghai Bell |
| R4-2109833 | TP to TS 38.176-2 Clause 4.6 Declarations for IAB radiated test specification | Nokia, Nokia Shanghai Bell |
| R4-2109999 | TP to TS38.176-2 on Annex I and Annex K | Samsung |
| R4-2110000 | Big CR for update on TR38.809 | Samsung |
| R4-2110042 | Discussion on the definition of Exclusion Bands and Spatial Exclusion for IAB EMC nodes | Ericsson |
| R4-2110043 | CR on exclusion bands and spatial exclusion for IAB EMC Radiated Immunity testing | Ericsson |
| R4-2110137 | CR to TS 38.174 – corrections to general and transmitter part | Nokia, Nokia Shanghai Bell |
| R4-2110138 | CR to TS 38.174 – corrections to receiver part | Nokia, Nokia Shanghai Bell |
| R4-2110139 | TDD pattern for IAB test models | Nokia, Nokia Shanghai Bell |
| R4-2110140 | Discussion on IAB test configurations with TPs to 38.176-1 and 38.176-2 | Nokia, Nokia Shanghai Bell |
| R4-2110142 | TP to TS 38.176-2: clauses 6.1, 6.2, 6.3 and 6.7 | Nokia, Nokia Shanghai Bell |
| R4-2110335 | CR on maintenance of side conditions for IAB-MT R16 | Huawei, HiSilicon |
| R4-2110336 | CR on maintenance on sharing factor of RLM and link recovery for IAB-MT R16 | Huawei, HiSilicon |
| R4-2110337 | Draft CR on maintenance of IAB test cases | Huawei, HiSilicon |
| R4-2110420 | IAB-MT conformance Test about EVM annex text | Keysight Technologies UK Ltd |
| R4-2110537 | pCR on IAB conducted conformance testing (Manufacturer declarations) to TS 38.176-1 | Huawei, HiSilicon |
| R4-2110538 | pCR on IAB radiated conformance testing (FRCs and PRACH test preambles) to TS 38.176-2 | Huawei, HiSilicon |
| R4-2110539 | Big CR on IAB-MT demodulation in TS 38.174 | Huawei, HiSilicon |
| R4-2110540 | Discussion on NR IAB-MT demodulation performance requirements | Huawei, HiSilicon |
| R4-2110541 | Updated simulation results on NR IAB-MT demodulation performance requirements | Huawei, HiSilicon |
| R4-2110542 | Updated simulation assumptions for NR IAB-MT demodulation requirements | Huawei, HiSilicon |
| R4-2110543 | Summary of simulation results for NR IAB-MT demodulation requirements | Huawei, HiSilicon |
| R4-2110544 | pCR on IAB-MT conducted conformance testing (CSI reporting and Interworking) to TS 38.176-1 | Huawei, HiSilicon |
| R4-2110545 | CR on IAB-MT conducted performance requirements (General and Demodulation) in TS 38.174 | Huawei, HiSilicon |
| R4-2110546 | pCR on IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2 | Huawei, HiSilicon |
| R4-2110578 | IAB-MT conformance Test setup MU | Keysight Technologies UK Ltd |
| R4-2110717 | Draft CR to 38.174: Introduction of IAB-DU performance requirements | Ericsson |
| R4-2110721 | pCR to 38.176-2: Introduction of CSI-RS performance tests and requirements | Ericsson |
| R4-2110722 | pCR to 38.176-1: Introduction of annexes on test tolerance, test setup and propagation conditions for performance requirements | Ericsson |
| R4-2110723 | Draft CR to 38.174: FRCs and PRACH preambles | Ericsson |
| R4-2110724 | pCR to 38.176-1: IAB-MT performance tests | Ericsson |
| R4-2110725 | General issues for IAB specifications | Ericsson |
| R4-2110726 | IAB-MT related proposals | Ericsson |
| R4-2110944 | TS 38.176-2 v.0.1.0 - update after RAN4#98bis meeting | Nokia, Nokia Shanghai Bell |
| R4-2110945 | TP to 38.176-2 Editor update - editorials | Nokia, Nokia Shanghai Bell |
| R4-2111025 | On IAB-MT demodulation requirements | Nokia, Nokia Shanghai Bell |
| R4-2111027 | On IAB-MT demodulation requirements | Nokia, Nokia Shanghai Bell |
| R4-2111174 | IAB Common test issue on test model-Conducted | Ericsson |
| R4-2111175 | On IAB-MT dynamic range and power control test for conduct test | Ericsson |
| R4-2111176 | On IAB-MT dynamic range and power control test for OTA test | Ericsson |
| R4-2111177 | TP for IBB, OBB and RX spurious of conducted receiver test | Ericsson |
| R4-2111178 | TP on IBB, OBB and RX spurious for OTA receiver characteristic test | Ericsson |
| R4-2111179 | On IAB test case reduction for IAB Conducted conformance test | Ericsson |
| R4-2111180 | On IAB test case reduction for IAB OTA conformance test. | Ericsson |
| R4-2111181 | IAB-MT specific declaration FR1 | Ericsson |
| R4-2111182 | IAB-MT specific declaration FR2 | Ericsson |
| R4-2111183 | CR on the further clear up the IAB specification | Ericsson |
| R4-2111205 | IAB Common test issue on test model-OTA | Ericsson |
| R4-2111237 | TS 38.174 draftCR CSI reporting radiated performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2111299 | Draft Big CR: IAB-MT RRM test cases in 38.174 | Ericsson |
| R4-2111300 | Side conditions in IAB-MT RRC connection mobility requirements in TS 38.174 | Ericsson |
| R4-2111301 | Further analysis of CSI-RS based RLM tests for LA IAB-MT | Ericsson |
| R4-2111302 | Correction to CSI-RS based RLM tests for LA IAB-MT in TS 38.174 | Ericsson |
| R4-2111348 | draftTP to TS 38.176-2 IAB-DU performance requirements and parts of DU and MT appendix | Nokia, Nokia Shanghai Bell |
| R4-2111350 | draftTP to TS 38.176-1 IAB-DU performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2111396 | bigTP draft to TS 38.176-2 Demodulation performance | Nokia, Nokia Shanghai Bell |
| R4-2111397 | TS 38.176-1 -Updated TS 37.176-1 | Huawei |
| R4-2111398 | Discussion on Test models and Test configurations | Huawei |
| R4-2111399 | TP to TS 38.176-1 -Clean up | Huawei |
| R4-2111400 | TP to TS 38.176-1 -Clause 4.1 | Huawei |
| R4-2111401 | TP to TS 38.176-2 -Clause 4.1 | Huawei |
| R4-2111402 | Discussion on IAB-MT TX dynamic range testing | Huawei |
| R4-2111403 | TP to TS 38.176-1 - OTA Tx dynamic range, clause 6.3 | Huawei |
| R4-2111404 | TP to TS 38.176-2 - OTA Tx dynamic range, clause 6.4 | Huawei |
| R4-2111405 | TP to TS 38.176-1 - Sensitivity, clause 7.2 | Huawei |
| R4-2111406 | TP to TS 38.176-2 - OTA Sensitivity, clause 7.2, 7.3 | Huawei |
| R4-2111407 | Discussion on MU values | Huawei |
| R4-2111466 | Draft CR to TS 38.175: IAB EMC test configurations and performance requirements (updated) | Huawei |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template