3GPP TSG-RAN5 Meeting #92-e	R5-214450

# Online, , 16th Aug - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0188 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IMS over 5GS test case 7.33 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Steps 22 and 25 send timer tag in the Supported header of 200 OK response for re-INVITE. However based on RFC 4028 cl 9.1 the “timer” support should be indicated in ‘Require’ Header field. [RFC 4028, cl.9.1] If the refresher parameter in the Session-Expires header field in the 2xx response has a value of ’uac’, the UAS MUST place a Require header field into the response with the value ’timer’. This is because the uac is performing refreshes and the response has to be processed for the UAC to know this. - Also, 100rel cannot be required for re-INVITE which is not a session modification request. - RFC 4028 is not clear on whether and where the UAS, when setting the refresher to ‘uas’, signals the timer tag, in the Supported or Require header. - Minor improvements needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced Supported header by Require header. - Used new condition A32 from R5-214531 (CR #1466) - Removed requirement on Supported header from step 19. - Also, for 200 OK for INVITE, referred to TS 34.229-5 generic procedure rather than the default message in TS 34.229-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UEs can fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 34.229-1 CR 1466 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to R5s210678. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 7.33	Session Timer / MT Call / Remote end chooses UE as refresher / 5GS

7.33.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being configured to use Session Timer and being configured to not use preconditions }

ensure that {

when { UE receives INVITE for voice call with Session-Expires value 1800 and refresher set uas and remote UE not supporting UPDATE }

then { UE continues setup of voice call and finally sends 200 OK for INVITE with Session-Expires being 1800 and setting refresher to uas }

}

(2)

with { Voice call having been set up }

ensure that {

when { 900 seconds have passed }

then { UE sends re-INVITE to refresh the session }

}

(3)

with { UE having refreshed the session }

ensure that {

when { Another 900 seconds have passed }

then { UE sends another re-INVITE to refresh the session }

}

7.33.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.2A.1.1]

A UE supporting RFC 4028 [58], when it receives a 422 (Session Interval Too Small) to an INVITE request where the response contains a Min-SE header field, shall retry the request in accordance with RFC 4028 [58] subclause 7.4.

[TS 24.229 clause 5.2.7.2]

When the P-CSCF receives from the UE an INVITE request, the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it.

[TS 24.229 clause 5.2.7.3]

When the P-CSCF receives an INVITE request destined for the UE the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it in order to make it work.

[TS 24.229 clause 5.4.5.3]

If the S-CSCF requested the session to be refreshed periodically, and the S-CSCF got the indication that the session will be refreshed, when the session timer expires, the S-CSCF shall delete all the stored information related to the dialog.

7.33.3	Test description

7.33.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use Session Timer

-	UE is configured to not use preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.33.3.2	Test procedure sequence

Table 7.33.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 9 | SS sends INVITE with Session-Expires value set to 1800 and refresher set to uas. | <-- | INVITE |  |  |
| 10-17 | Steps 2-8A of Annex A.5.2 happen. | - | - |  |  |
| 18 | Void | - | - | - | - |
| 19 | UE send 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uas. | --> | 200 OK | 1 | P |
| 20 | SS sends ACK. (Step 12 of Annex A.5.1) | <-- | ACK |  |  |
| 21 | 900 seconds after step 20, UE sends an INVITE request to refresh the session. | --> | INVITE | 2 | P |
| 22 | SS sends 200 OK for INVITE. | <-- | 200 OK |  |  |
| 23 | UE sends ACK. | --> | ACK |  |  |
| 24 | 900 seconds after step 23, UE sends an INVITE request to refresh the session. | --> | INVITE | 3 | P |
| 25 | SS sends 200 OK for INVITE. | <-- | 200 OK |  |  |
| 26 | UE sends ACK. | --> | ACK |  |  |
| 27-28 | SS releases the call. (Steps 1-2 of Annex A.8) | - | - |  |  |

7.33.3.3	Specific message contents

Table 7.33.3.3-1: INVITE (step 9, table 7.33.3.2-1)

| Derivation Path: Step 1 of Annex A.5.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Allow |  | INVITE, ACK, OPTIONS, CANCEL, BYE |  | RFC 3261 [6] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uas |  |  |

Table 7.33.3.3-2: 200 OK (step 19, table 7.33.3.2-1)

| Derivation Path: Annex A.5.2, Step 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uas |  |  |

Table 7.33.3.3-3: INVITE (steps 21 and 24, table 7.33.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, Conditions A32, A26 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac |  |  |
| Content-Type |  |  |  | RFC 3261 [6] |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  | RFC 3261 [6] |
| value |  | length of message-body |  |  |
| Message-body |  | Session description: v=0 o=(origin) [Note 1] s=(session name) c=IN (addrtype) (connection-address for UE) if present b=AS: (bandwidth-value) Time description: t= (start-time) (stop-time) Media description: Any media, if present. Note 1: Same origin as in last SDP sent by the UE. |  | RFC 4566 [38] |

Table 7.33.3.3-4: 200 OK (steps 22 and 25, table 7.33.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, Conditions A2, A11, A20 and A22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  | timer |  | RFC 4028 [37] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac |  |  |