3GPP TSG-RAN5 Meeting #92-e	R5-215367

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1408 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core spec alignment to add CCR configuration for EN-DC event triggered |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CCR configuration is missing for some EN-DC test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CCR has been added to the test configuration parameters table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will not be aligned with latest configuration in TS 38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.1, 4.6.2.2, 4.6.2.5, 4.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-2112613 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 4.6.2	Inter-frequency measurements

#### 4.6.2.0	Minimum conformance requirements for Inter-frequency measurements

The requirements in Section 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Sections 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in Sections 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in Sections 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3 and table 9.3.4-4.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1 and table 9.3.5-2.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in section 9.1.5.2 for measurement conducted within measurement gaps.

Table 9.3.4-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133-f50, clause 9.3.5]

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in subclauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively,  as shown in table 9.3.5-1 and 9.3.5-2:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

[TS 38.133, clause 9.3.6.3]

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in event triggered measurement reports shall meet the requirements in sections 10.1.4.1, 10.1.5.1, 10.1.9.1, 10.1.10.1, 10.1.14.1 and 10.1.15.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 × TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. Both Tidentify_inter_without_index and Tidentify_inter_with_index are defined in clause 9.3.4. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4 and then triggers the measurement report as per TS 38.331 [2], the event triggered measurement reporting delay shall be less than T SSB_measurement_period_inter  defined in clause 9.3.5 provided the timing to that cell has not changed more than  3200 Tc while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

#### 4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX

4.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within EN-DC inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

4.6.2.1.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

4.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.4.6.2.1.

4.6.2.1.4	Test description

4.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.2.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.6.2.1.4.1-2. Test environment parameters are given in Table 4.6.2.1.4.1-3.

Table 4.6.2.1.4.1-1: EN-DC FR1-FR1 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.6.2.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.6.2.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The target NR cell3 has the same SCS, BW and duplex mode as NR serving cell2 |  |

Table 4.6.2.1.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1 | 1 |  |

Table 4.6.2.1.4.1-3: Test Environment test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1. Message contents are defined in clause 4.6.2.1.4.3.

2.  Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 are set according to Annex C.1.2 and Annex C.1.3. Cell 3 is switched off during the initial connection setup.

4.6.2.1.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 4.6.2.1.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 4.6.2.1.4.1-2 is provided for UE that support per-FR gap. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 4.6.2.1.4.1-2. T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.6.2.1.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delay measured from the beginning of time period T2 is less than 920 ms for Test 1 and 760 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8. Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 4.6.2.1.4.1-2 as appropriate.

4.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.6.2.1.4.3-1: Common Exception messages for Additional EN-DC FR1-FR1 event triggered reporting tests in non-DRX test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE for Test 1 Table H.3.4-4 with Condition gapFR1 for Test 2 Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 for Test 1 Table H.3.4-5 with Condition Pattern #4 and gap offset = 9 for Test 2 |
| Specific message contents exceptions for Test Configuration 4.6.2.1-1 and 4.6.2.1-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 4.6.2.1-2 and 4.6.2.1-5 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 4.6.2.1-3 and 4.6.2.1-6 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 4.6.2.1.4.3-2: SchedulingRequest-Config for E-UTRAN PCell

| Derivation Path: 36.508 [25], Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 23 |  | TDD |
| } |  |  |  |
| } |  |  |  |

4.6.2.1.5	Test requirement

Table 4.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.2.1.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  | Config 1,4 | FDD |  |  |  |
|  |  | Config 2,3,5,6 | TDD |  |  |  |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| BWP BW | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | Config 3,6 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Initial DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| TRS configuration |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  | NA |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  | NA |  |
| PDSCH Reference measurement channel |  | Config 1,4 | SR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | SR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CCR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CCR 2.1 TDD |  |  |  |
| SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 2,5 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 3,6 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |  |  |  |
|  |  | Config 3,6 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 | dBm/SCS | Config 1,2,4,5 | -98 |  | -98 |  |
|  |  | Config 3,6 | -95 |  | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity | -91 |
|  |  | Config 3,6 | -91 | -91 | -Infinity | -88 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 | -64.59 | -70.05 | -62.26 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 760 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX

4.6.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX within EN-DC inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

4.6.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

4.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.4.6.2.2.

4.6.2.2.4	Test description

4.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.2.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.6.2.2.4.1-2. Test environment parameters are given in Table 4.6.2.2.4.1-3.

Table 4.6.2.2.4.1-1: EN-DC FR1-FR1 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.6.2.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.6.2.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The target NR cell3 has the same SCS, BW and duplex mode as NR serving cell2 |  |

Table 4.6.2.2.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX | ms | Config 1,2,3,4,5,6 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in A.5 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1.1 | 11 | 1.1 | 11 |  |

Table 4.6.2.2.4.1-3: Test Environment parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1. Message contents are defined in clause 4.6.2.2.4.3.

2. Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 and are set according to Annex C.1.2 and Annex C.1.3. Cell 3 is switched off during the initial connection setup.

4.6.2.2.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 4.6.2.2.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 4.6.2.2.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 4.6.2.2.4.1-2. T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.6.2.2.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1, 10240 ms for Test 2, 1080 ms for Test 3 and 10240 ms for Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 4.6.2.2.4.1-2 as appropriate.

4.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.6.2.2.4.3-1: Common Exception messages for Additional EN-DC FR1-FR1 event triggered reporting with SSB test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE for Test 1 and Test 2 Table H.3.4-4 with Condition gapFR1 for Test 3 and Test 4 Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.4-5 with Condition Pattern #4 and gap offset = 9 for Test 3 and Test 4 |
| Specific message contents exceptions for Test Configuration 4.6.2.2-1 and 4.6.2.2-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 4.6.2.2-2 and 4.6.2.2-5 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 4.6.2.2-3 and 4.6.2.2-6 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 4.6.2.2.4.3-2: SchedulingRequest-Config for E-UTRAN PCell

| Derivation Path: 36.508 [25], Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 23 |  | TDD |
| } |  |  |  |
| } |  |  |  |

4.6.2.2.5	Test requirement

Table 4.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.2.2.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  | Config 1,4 | FDD |  |  |  |
|  |  | Config 2,3,5,6 | TDD |  |  |  |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| BWP BW | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | Config 3,6 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Initial DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  | NA |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  | Config 1,4 | SR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | SR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CCR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CCR 2.1 TDD |  |  |  |
| SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 2,5 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 3,6 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |  |  |  |
|  |  | Config 3,6 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 | dBm/SCS | Config 1,2,4,5 | -98 |  | -98 |  |
|  |  | Config 3,6 | -95 |  | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity | -91 |
|  |  | Config 3,6 | -91 | -91 | -Infinity | -88 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 | -64.59 | -70.05 | -62.26 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 4.6.2.3	Void

#### 4.6.2.4	Void

#### 4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection

4.6.2.5.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event in non-DRX within EN-DC inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

4.6.2.5.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

4.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.4.6.2.3.

4.6.2.5.4	Test description

4.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.2.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.6.2.5.4.1-2. Test environment parameters are given in Table 4.6.2.5.4.1-3.

Table 4.6.2.5.4.1-1: EN-DC FR1-FR1 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.6.2.5 -1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.5 -2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.5 -3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.6.2.5 -4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.5 -5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.5 -6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The target NR cell3 has the same SCS, BW and duplex mode as NR serving cell2 |  |

Table 4.6.2.5.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1.1 | 1 |  |

Table 4.6.2.5.4.1-3: Test Environment parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1. Message contents are defined in clause 4.6.2.5.4.3.

2. Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 and are set according to Annex C.1.2 and Annex C.1.3. Cell 3 is switched off during the initial connection setup.

4.6.2.5.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCel in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 4.6.2.5.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 4.6.2.5.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 4.6.2.5.4.1-2. T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.6.2.5.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message  ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than 1040 ms for Test 1 and 880 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On  according to TS 38.508-1 [14] clause 4.5.)

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 4.6.2.5.4.1-2 as appropriate.

4.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.6.2.5.4.3-1: Common Exception messages for Additional EN-DC FR1-FR1 event triggered reporting tests in non-DRX with SSB test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE for Test 1 Table H.3.4-4 with Condition gapFR1 for Test 2 Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 for Test 1 Table H.3.4-5 with Condition Pattern #4 and gap offset = 9 for Test 2 |
| Specific message contents exceptions for Test Configuration 4.6.2.5-1 and 4.6.2.5-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 4.6.2.5-2 and 4.6.2.5-5 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 4.6.2.5-3 and 4.6.2.5-6 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 4.6.2.5.4.3-2: SchedulingRequest-Config for E-UTRAN PCell

| Derivation Path: 36.508 [25], Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 23 |  | TDD |
| } |  |  |  |
| } |  |  |  |

4.6.2.5.5	Test requirement

Table 4.6.2.5.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.2.5.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  | Config 1,4 | FDD |  |  |  |
|  |  | Config 2,3,5,6 | TDD |  |  |  |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| BWP BW | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | Config 3,6 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| Initial DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  | Config 1,4 | TRS.1.1 FDD |  | NA |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  | NA |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  | Config 1,4 | SR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | SR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 | CR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 | CCR.1.1 FDD |  | - |  |
|  |  | Config 2,5 | CCR.1.1 TDD |  |  |  |
|  |  | Config 3,6 | CCR 2.1 TDD |  |  |  |
| SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 2,5 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  | Config 3,6 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
|  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |  |  |  |
|  |  | Config 3,6 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 | dBm/SCS | Config 1,2,4,5 | -98 |  | -98 |  |
|  |  | Config 3,6 | -95 |  | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity | -91 |
|  |  | Config 3,6 | -91 | -91 | -Infinity | -88 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 | -64.59 | -70.05 | -62.26 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 880 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection

4.6.2.6.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX within EN-DC inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

4.6.2.6.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

4.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A. 4.6.2.6.

4.6.2.6.4	Test description

4.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.2.6.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.6.2.6.4.1-2. Test environment parameters are given in Table 4.6.2.6.4.1-3.

Table 4.6.2.6.4.1-1: EN-DC FR1-FR1 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 4.6.2.6-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.6-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.6-3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.6.2.6-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.6.2.6-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.6.2.6-6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The target NR cell3 has the same SCS, BW and duplex mode as NR serving cell2 |  |

Table 4.6.2.6.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN TDD carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX | ms | Config 1,2,3,4,5,6 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 2,3,5,6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1.3 | 13.5 | 1.3 | 13.5 |  |

Table 4.6.2.6.4.1-3: Test Environment parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT Part. and A.3.1.8.4 for TE Part |  |  |

1. Message contents are defined in clause 4.6.2.6.4.3.

2. Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in different frequencies. Cell 2 is the PSCell and Cell 3 is the target cell. The power levels and settings for Cell 2 and are set according to Annex C.1.2 and Annex C.1.3. Cell 3 is switched off during the initial connection setup.

4.6.2.6.4.2	Test procedure

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 4.6.2.6.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 4.6.2.6.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 4.6.2.6.4.1-2. T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 4.6.2.6.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3 embedded in E-UTRA RRC message ULInformationTransferMRDC. If the overall delays measured from the beginning of time period T2 is less than 1280 ms  for Test 1, 13440 ms  for Test 2, 1280 ms  for Test 3 and 13440 ms  for Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8. Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.]

9. The SS shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If either of the reconfiguration in step 7 or step 9 fails, SS switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 4.6.2.6.4.1-2 as appropriate.

4.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.6.2.6.4.3-1: Common Exception messages for Additional EN-DC FR1-FR1 event triggered reporting in DRX with SSB test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and with Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 Table H.3.4-4 with Condition gapUE for Test 1 and Test 2 Table H.3.4-4 with Condition gapFR1 for Test 3 and Test 4 Table H.3.4-5 with Condition Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.4-5 with Condition Pattern #4 and gap offset = 9 for Test 3 and Test 4 |
| Specific message contents exceptions for Test Configuration 4.6.2.6-1 and 4.6.2.6-4 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 4.6.2.6-2 and 4.6.2.6-5 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 4.6.2.6-3 and 4.6.2.6-6 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 4.6.2.6.4.3-2: SchedulingRequest-Config for E-UTRAN PCell

| Derivation Path: 36.508 [25], Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 23 |  | TDD |
| } |  |  |  |
| } |  |  |  |

4.6.2.6.5	Test requirement

Table 4.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.2.6.5-1: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |  |
| Duplex mode |  | Config 1,4 | FDD |  |  |  |  |
|  |  | Config 2,3,5,6 | TDD |  |  |  |  |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW | MHz | Config 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 | SR.1.1 FDD |  | - |  |  |
|  |  | Config 2,5 | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 | SR 2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 | CR.1.1 FDD |  | - |  |  |
|  |  | Config 2,5 | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 | CR 2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 | CCR.1.1 FDD |  | - |  |  |
|  |  | Config 2,5 | CCR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 | CCR 2.1 TDD |  |  |  |  |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3,6 | TDDConf.2.1 |  |  |  |  |
| Initial DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  |  |  |  |
| Dedicated DL BWP |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  |  |
| Dedicated UL BWP |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  |  |
| TRS configuration |  | Config 1,4 | TRS.1.1 FDD |  |  | N/A |  |
|  |  | Config 2,5 | TRS.1.1 TDD |  |  | N/A |  |
|  |  | Config 3,6 | TRS.1.2 TDD |  |  | N/A |  |
| SSB parameters |  | Config 1,4 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  | Config 2,5 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  | Config 3,6 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | Config 1,4 | SMTC.2 |  |  | SMTC.5 |  |
|  |  | Config 2,3,5,6 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |  |  |  |  |
|  |  | Config 3,6 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 | dBm/SCS | Config 1,2,4,5 | -98 |  | -98 |  |  |
|  |  | Config 3,6 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity |  | -91 |
|  |  | Config 3,6 | -91 | -91 | -Infinity |  | -88 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity |  | 7 |
|  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity |  | 7 |
| IoNote3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 | -64.59 | -70.05 |  | -62.26 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 | -58.49 | -63.94 |  | -56.15 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 4.6.2.7	Void

#### 4.6.2.8	Void

< End of changes >