3GPP TSG-RAN5 Meeting #92-e	R5-216334

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0162 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add applicabilities for test cases 8.1.1.4.7, 8.1.1.4.8 and 8.1.1.4.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add applicabilities for the new test cases - RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band Contiguous CA - RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band non-Contiguous CA - RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Inter-band CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicabilities for test cases 8.1.1.4.7-9 have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-214515 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

<Start of first modified section>

# 4	Recommended Test Case Applicability

## 4.0	Introduction

The applicability of each individual test is identified in subclause 4.1. This is just a recommendation based on the purpose for which the test case was written.

<<<Skipped part >>>

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 8 | RRC |  |  |  |
| 8.1 | NR RRC |  |  |  |
| 8.1.1 | RRC connection management procedures |  |  |  |
| 8.1.1.1 | Paging |  |  |  |
| 8.1.1.1.1 | RRC / Paging for connection / Multiple paging records | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.1.2 | RRC / Paging for connection / Shared network environment | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2 | RRC connection establishment |  |  |  |
| 8.1.1.2.1 | RRC connection establishment / Return to idle state after T300 expiry | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.2 | Void |  |  |  |
| 8.1.1.2.3 | RRC connection establishment / RRC Reject with wait time | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.4 | RRC connection establishment / Extended and spare fields in SI | Rel-15 only | C21 | UEs supporting 5G Core |
| 8.1.1.3 | RRC release |  |  |  |
| 8.1.1.3.1 | RRC connection release / Redirection to another NR frequency | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.2 | RRC connection release / Redirection from NR to E-UTRA | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 8.1.1.3.3 | RRC connection release / Success / With priority information | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.4 | RRC connection release / Success / With priority information / E-UTRA | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 8.1.1.3.5 | Void |  |  |  |
| 8.1.1.3.6 | Void |  |  |  |
| 8.1.1.3.7 | RRC connection release / Success / Deprioritisation / T325 expiry | Rel-15 | C133 | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| 8.1.1.4 | RRC resume |  |  |  |
| 8.1.1.4.1 | RRC resume / Suspend-Resume / RNA update / Success | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.2 | RRC resume / Suspend-Resume / RRC setup / T319 expiry | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.3 | Void |  |  |  |
| 8.1.1.4.7 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band Contiguous CA | Rel-16 | Cnn1 | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE |
| 8.1.1.4.8 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band non-Contiguous CA | Rel-16 | Cnn2 | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE |
| 8.1.1.4.9 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Inter-band CA | Rel-16 | Cnn3 | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE |
| 8.1.2 | RRC reconfiguration |  |  |  |

<End of modified section>