3GPP TSG RAN meeting #93-e								              RP-211871

Electronic Meeting, September 13-17, 2021

## Status Report to TSG

Agenda item:	9.3.2.7 - NR small data transmissions in INACTIVE state [RAN2 WI: NR_SmallData_INACTIVE]

| WI / SI Name | NR small data transmissions in INACTIVE state |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: - |
| Acronym | NR_SmallData_INACTIVE |  |  |  |  |
| Unique ID | 860051 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210870 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: Overall: 50% RAN2: 60% RAN3: 10% RAN4: 0% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Eswar Vutukuri |
|  | Company | ZTE Corporation |
|  | Email | eswar.vutukuri@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

- RAN1#106-e

Agreement

- Each N of consecutive SSB indexes associated to one CG configuration are mapped to valid CG PUSCH resources

  - first, in increasing order of DMRS resource indexes, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index

  - second, in increasing order of CG period indexes in the association period

- The mapping ratio N is explicitly signalled and the association period is implicitly derived

  - FFS candidate value set of mapping ratio, and whether it is configured per CG configuration or per cell

  - The SSB to CG PUSCH association period is the duration of multiple of CG periods depending the smallest time duration in the set determined by the CG period such that all SSBs associated with the CG configuration are mapped at least once to CG PUSCH resources.

  - An association pattern period includes one or more association periods and is determined so that a pattern between CG PUSCH occasions and SS/PBCH block indexes associated with the CG configuration repeats at most every 640 msec.

- Note: The mapping ordering and steps may be revisited if multiple CG PUSCH occasions in one CG period is supported

Agreement

Support multiple DMRS resources per CG configuration when single layer PUSCH transmission is assumed, and each DMRS resource could be mapped to the same or different SSB(s)

- FFS if multi-layer PUSCH transmission is supported for CG-SDT

- FFS any limitation on the DMRS configuration if multiple CG PUSCH occasions per CG period is supported

Agreement

- The following PUSCH occasion validation rule is applied for CG-SDT

  - for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon

    - if a UE is provided tdd-UL-DL-ConfigurationCommon, the valid PO is the PO in UL part in a slot, or at least Ngap symbols after the end of the DL part in a slot or after the end of the SSB in a slot

    - if a UE is not provided tdd-UL-DL-ConfigurationCommon, the valid PO does not precede a SS/PBCH block in the PUSCH slot, starts at least Ngap symbols after a last SS/PBCH block symbol

    - Ngap is provided in Table 8.1-2 in TS 38.213

  - FFS if any validation rule following the CG-PUSCH in RRC connected state is applicable, and whether and how to handle the overlapping between CG-PUSCH occasions for CG-SDT and any valid PRACH occasion or MsgA PUSCH occasion.

- FFS the rule for paired spectrum, and whether/how to support CG-SDT for UEs operating in Type-A HD-FDD.

Conclusion:

RAN1 cannot reach consensus on the following options for the SSB subset for RSRP based TA validation. Ask RAN2 if they can do the down-selection.

- Option 1: Within a set of SSBs configured per CG configuration

- Option 2: Within a set of SSBs configured for all CG configurations

- Option 3: Within a set of all SSBs actually transmitted as indicated in SIB1

- Option 4: Highest N SSBs of all SSBs actually transmitted as indicated in SIB1

Agreement:

- For RA-SDT, when PRACH occasions are separate between SDT and non-SDT, PRACH resource configurations/parameters for 4-step RACH and/or 2-step RACH should be re-used as much as possible for 4-step RACH and/or 2-step RACH based SDT, respectively.

  - Note: It is up to RAN2 discussion on the RO configuration for RA-SDT in separate ROs.

- For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, at least following parameters can be configured, including 4-step RACH and/or 2-step RACH based SDT operation.

  - Number of contention-based preambles for SDT per SSB per valid RO

  - Note: whether starting position of the preambles for SDT per SSB per valid RO needs to be configured for RA-SDT in shared ROs is up to RAN2 discussion.

- For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, a PRACH mask can be configured to indicate a subset of ROs for RA-SDT.

- For RA-SDT in shared ROs and separate ROs with non-SDT, the power control parameters follow those for non-SDT,

  - i.e. preambleReceivedTargetPower and power ramping setting follow those for non-SDT.

Conclusion:

- Further discuss on the case when ROs are shared between SDT and non-SDT, but different RACH types have separate ROs after RAN2’s decision

LS in:

- R1-2106405, Reply LS to RAN1 on physical layer aspects of small data transmission	from RAN2

LS out:

- R1-2108533, Reply LS on physical layer aspects of small data transmission 	to RAN2

- R1-2108649, LS on the TA validation and mapping details for CG-SDT		to RAN2

#### 2.1.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#115-e

- Data volume used for SDT selection criteria is calculated as the total sum of Buffer Size across SDT RBs (i.e. same approach as BSR)

- At initiation of SDT procedure, the PDCP status report is not triggered even if the RB is configured with statusReportRequired

- If ROHC is configured, the area scope of ROHC continuity is specified in the specification, i.e. gNB configuration is not needed

- For SDT procedure selection, Same data volume threshold is used for CG-SDT and RA-SDT

- The BSR configuration used for SDT can be different from the BSR configuration used in RRC_CONNECTED.

  - [CB] FFS Whether the BSR configuration used for SDT is configured by gNB or used from default configuration needs further discussion. (gNB 10 / default 11)

- Legacy PHR triggers are applied for SDT

- DL SPS is not supported for SDT

- DataInactivityTimer is not supported for SDT.

- RLC polling is supported for SDT.

- The UE performs RLC re-establishment implicitly, i.e. without explicit indication for RLC re-establishment, when the UE initiates SDT procedure.

- At initiation of SDT procedure, the RRC indicates to the PDCP to disable the PDCP status report, e.g. by de-configuring statusReportRequired (i.e. UE internally indicates).  FFS how PDCP status reporting is enabled.

- The LCP priority of PHR MAC CE in SDT is same as in RRC_CONNECTED, i.e. the PHR MAC CE in SDT is prioritized over SDT data

- During the SDT procedure, all the triggered PHRs are cancelled if all SDT data are included in the UL grant, if there is NO room in the MAC PDU to fit the PHR.

- Working assumption: LCH restrictions can be applied, re-using existing signalling.  It is up to gNB how restrictions are configured and MAC applies current specification rules.   Revisit next meeting if we have technical issues.

- No new solution is defined to prevent data loss or duplication for the scenario where the anchor relocation is required in the middle of an SDT session, i.e. network can release UE back into RRC_INACTIVE

- PDCP entities of only the non-SDT RBs are re-established (i.e. not for the SDT RBs) when the UE moves from RRC_INACTIVE with SDT session ongoing to RRC CONNECTED.

- Events that trigger a termination or failure of an ongoing SDT session 1) cell reselection, 2) expiry of the SDT failure detection timer, 3) when Max retx is reached in RLC.  RLC AM max retransmission functionality remains unchanged.

- When a UE detects a failure of an ongoing SDT session, UE transitions autonomously into RRC_IDLE (as baseline solution).   If time allows or have a ready solution we can consider further optimizations.

- SDT related RACH resources are configured via system information, i.e., SIB1

- Explicit indication (other than RA-SDT configuration) to enable/disable RA-SDT is not supported

- At least the following parameters can be RA-SDT specific.

  - SSB selection related parameters, i.e., rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB.

  - Power control related parameters, i.e., preambleReceivedTargetPower/gA-PreambleReceivedTargetPower, powerRampingStep/msgA-PreamblePowerRampingStep,  msg3-DeltaPreamble/msgA-DeltaPreamble.

  - Preamble group related parameters, i.e., msg3-DeltaPreamble/msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT and 4-step RA-SDT.

- For shared ROs case, all the following configurations can be allowed: (28/28)

  - 4-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

  - 2-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

  - 2-step RA-SDT shares ROs with 4-step RA-SDT and/or 4-step RA and/or 2-step RA.

- For the RA-SDT preamble group selection, the UE should consider SDT data size plus MAC subheader in addition to CCCH SDU size plus MAC subheader and pathloss, same in legacy.  FFS whether any additional things on top of legacy criteria is needed.

- The fallbackRAR reception as legacy 2-step RACH is supported in 2-step RA-SDT, i.e., fallback from 2-step RA-SDT to 4-step RA-SDT when fallbackRAR is received

- As legacy, UE can be configured to switch from 2-step RA-SDT to 4-step RA-SDT after N times of MsgA transmission

- Send an LS to RAN1 to provide overall relevant agreements.  Check if the PUCCH resources used for HARQ-ACK during subsequent SDT transmissions (applicable for both RA and CG).  Ask if other L1 PHY resources may be needed for subsequent SDT transmission, for example RAN2 thinks we can use the common resources (PDCCH and PUCCH) for RA and ask if we need others.

  - Add that RAN2 discussed RA-SDT configuration on non-initial BWP.  There was a large number of companies supporting and other companies expressed concerns on complexity and paging monitoring.  Ask RAN1 if they have any concerns from their side.  NOTE that RAN2 agreed for CG-SDT we already agreed to dedicated BWP and why we decided to support it.

- UE suspends all UL transmissions and triggers RACH if any UL transmission is needed (same as in connected mode) when TAT expires during RA-SDT procedure

- RA-SDT can be configured on initial BWP.  FFS for non-initial BWP

- RA prioritization related parameters cannot be configured for RA-SDT, i.e., powerRampingStepHighPriority, scalingFactorBI

- UE selects any SSBs if there is no qualified SSB for RA-SDT, like in legacy.  No optimizations are considered.

- Switching from SDT to non-SDT via RAR/fallbackRAR/DCI sent by network is not supported for RA-SDT

- No new timer (other than the SDT failure detection timer) is introduced to control the PDCCH monitoring during subsequent transmissions in RA-SDT

- If none of the SSBs’ RSRP is above the RSRP threshold of CG-SDT criteria in the type selection phase, UE should select RA-SDT if RA-SDT criteria is met

- MAC PDU rebuilding is not required (unless we find a case that is needed)

- During subsequent CG transmission phase (i.e. after the UE has received response from NW) UE can initiate at least legacy RACH procedure (e.g. trigger due to no UL resources).  No MAC PDU rebuilding is required.  FFS if the RA-SDT RA resources can be used for subsequent data.

- At least the following conditions are agreed:

  - No qualified SSB when the evaluation is performed;

  - When TA is invalid;

  - When SR is triggered due to lack of UL resource

- UE should release CG-SDT resource (if stored) when UE initiates RRC resume procedure from another cell which is different from the cell in which the RRCRelease is received.

- The C-RNTI previously configured in RRC_CONNECTED state is used for UE to monitor PDCCH in CG-SDT.

- CS-RNTI based dynamic retransmission mechanism can be reused for CG-SDT.  FFS whether CS-RNTI is the same one as the one previously configured in RRC_CONNECTED or a new CS-RNTI one is provided to the UE

- During the subsequent new CG transmission phase, for the purpose of CG resource selection, UE re-evaluates the SSB for subsequent CG transmission.  FFS what happens if no SSBs are valid or if no sample is available

- From RAN2 perspective, at least the following parameters should be included in the CG-SDT configuration. FFS whether these parameters are common for multiple CG-SDT configurations or per CG-SDT configuration.

  - The new TA timer in RRC_INACTIVE;

  - The RSRP change threshold for TA validation mechanism in SDT (details dependent on RAN1);

  - The SSB RSRP threshold for beam selection (i.e. UE selects the beam and associated CG resource for data transmission).

In addition to the above, RAN2 has handled the following LSs for SDT:

LS in:

- R2-2106923	LS on the physical layer aspects of small data transmission	from RAN1

- R2-2106931	Reply LS on small data transmission, 					from RAN3

- R2-2109065	Reply LS on Small Data transmission, 					from SA3

LS out:

- R2-2109222,	LS on agreements related to SDT, 						To RAN1

Latest endorsed versions of the running CRs:

- R2-2107478	RRC Running CR for SDT	ZTE Corporation (rapporteur)

- R2-2107494	Running MAC CR for small data	Huawei, HiSilicon

- R2-2108242	Stage-2 running CR Introduction of SDT	Nokia, Nokia Shanghai Bell

#### 2.2.2	Remaining Open issues

- Stage-2 design work for RACH and CG based solutions for small data in INACTIVE

- Stage-3 work

- See the WI objectives for general overview of remaining open issues

## 2.3	RAN3

#### 2.3.1	Agreements

- RAN3#113-e

#### Nothing to report as SDT was not discussed at this meeting

#### 2.3.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 2.4	RAN4

#### 2.4.1	Agreements

[General]

None (NR_SmallData_INACTIVE was not discussed yet)

#### 2.4.2	Remaining Open issues

- See the WI objectives for general overview of remaining open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### N/A

#### 3.1.1	Agreements with cross-TSG impacts

N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

N/A

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

RAN1#106-e

| R1-2106458 | Physical layer aspects of CG-SDT | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2106683 | Discussion on physical layer aspects of small data transmission | Spreadtrum Communications |
| R1-2106765 | Physical layer aspects for NR small data transmissions in INACTIVE state | Ericsson |
| R1-2106788 | Physical layer aspects of small data transmission | Sony |
| R1-2106855 | Discussion on physical layer aspects for NR small data transmissions in INACTIVE state | Samsung |
| R1-2106926 | Discussion on remaining issues on small data transmission | CATT |
| R1-2107007 | Discussion on the remaining physical layer issues of small data transmission | ZTE, Sanechips |
| R1-2107075 | Physical layer aspects of small data transmission | InterDigital, Inc. |
| R1-2107139 | Discussion on RAN1 Aspects for NR small data transmissions | NEC |
| R1-2107309 | Draft Reply to RAN2 LS on Physical Layer Aspects of SDT | Qualcomm Incorporated |
| R1-2107433 | Discussion on physical layer aspects of small data transmission | LG Electronics |
| R1-2107566 | Discussion on physical layer aspects of small data transmission | Intel Corporation |
| R1-2107707 | Discussion on physical layer aspects of small data transmission | Apple |
| R1-2107971 | Discussion on RAN1 impacts for small data transmisison | vivo |
| R1-2107972 | Draft reply LS on physical layer aspects of small data transmission | vivo |
| R1-2108089 | On physical layer aspects of small data transmission | Nokia, Nokia Shanghai Bell |
| R1-2108532 | Email discussion on [106-e-NR-R17-SDT-02] | Moderator (vivo) |
| R1-2108561 | Summary on the physical layer aspects of small data transmission | Moderator (ZTE) |

## 4.2	RAN2

###### RAN2#115-e

| R2-2106923 | LS on the physical layer aspects of small data transmission (R1-2106335; contact: ZTE) | RAN1 |
| --- | --- | --- |
| R2-2106931 | Reply LS on small data transmission (R3-212820; contact: Ericsson) | RAN3 |
| R2-2107002 | User Plane Common Aspects of RACH and CG based SDT | Samsung Electronics Co., Ltd |
| R2-2107003 | Control Plane Common Aspects of RACH and CG based SDT | Samsung Electronics Co., Ltd |
| R2-2107004 | RACH configuration for Small Data Transmission. | Samsung Electronics Co., Ltd |
| R2-2107005 | Further Details of RACH bsaed Small Data Transmission | Samsung Electronics Co., Ltd |
| R2-2107006 | Details of Configured Grant based Small Data Transmission | Samsung Electronics Co., Ltd |
| R2-2107053 | Further Discussion on User Plane Aspect for Small Data Transmission | vivo |
| R2-2107054 | Discussion on RRC-Controlled Small Data Transmission | vivo |
| R2-2107055 | Handling of non-SDT Data Arrival | vivo |
| R2-2107056 | Supporting Small Data Transmission via RA Procedure | vivo |
| R2-2107057 | Supporting Small Data Transmission via CG PUSCH | vivo |
| R2-2107245 | Discussion on the remianing issues of SDT modelling | OPPO |
| R2-2107246 | Discussion on user plane issues of SDT | OPPO |
| R2-2107247 | Discussion on control plane issues of SDT | OPPO |
| R2-2107248 | Discussion on RACH-based SDT | OPPO |
| R2-2107249 | Discussion on CG-based SDT | OPPO |
| R2-2107292 | Report of email discussion [Post114-e][507][SData] Non-SDT data arrival handling | Intel Corporation |
| R2-2107293 | Control Plane leftover issues on SDT mechanism | Intel Corporation |
| R2-2107294 | Expected duration and applicable features for SDT procedure | Intel Corporation |
| R2-2107295 | User Plane leftover issues on SDT mechanism | Intel Corporation |
| R2-2107296 | RACH leftover issues on SDT mechanism | Intel Corporation |
| R2-2107297 | CG-SDT leftover aspects | Intel Corporation |
| R2-2107354 | Discussion on RACH-based SDT | Spreadtrum Communications |
| R2-2107440 | Discussion on CG-SDT Request by UE | NEC Telecom MODUS Ltd. |
| R2-2107463 | Issues of the Subsequent Data Transmission | FGI, Asia Pacific Telecom |
| R2-2107464 | Switching during a SDT procedure | FGI, Asia Pacific Telecom |
| R2-2107465 | PDCCH monitoring in RA-SDT | FGI, Asia Pacific Telecom |
| R2-2107478 | RRC Running CR for SDT | ZTE Corporation (rapporteur) |
| R2-2107486 | Summary: [Post114-e][505][SData] RRC/MAC modeling and RRC running CR (ZTE) | ZTE Corporation (Rapporteur) |
| R2-2107487 | Common aspects for UP for SDT | ZTE Corporation, Sanechips |
| R2-2107488 | Common aspects for CP for SDT | ZTE Corporation, Sanechips |
| R2-2107489 | Open issues for RA-SDT | ZTE Corporation, Sanechips |
| R2-2107490 | Open issues for CG-SDT | ZTE Corporation, Sanechips |
| R2-2107491 | Control plane common aspects for SDT | Huawei, HiSilicon |
| R2-2107492 | CG-based schemes for SDT | Huawei, HiSilicon |
| R2-2107493 | Discussion on the NAS aspects of Small Data | Huawei, HiSilicon |
| R2-2107494 | Running MAC CR for small data | Huawei, HiSilicon |
| R2-2107495 | Remaining issue for MAC spec | Huawei, HiSilicon |
| R2-2107496 | Summary of [Post114-e][506][SData] Running MAC CR (Huawei) | Huawei, HiSilicon |
| R2-2107580 | Power Saving for SDT | Apple |
| R2-2107581 | Non-SDT handling during the SDT procedure | Apple |
| R2-2107582 | Control plane aspects on the SDT procedure | Apple |
| R2-2107583 | RACH specific SDT procedure | Apple |
| R2-2107584 | CG specific SDT procedure | Apple |
| R2-2107659 | Handling of SDTF detection timer | Fujitsu |
| R2-2107660 | RAN paging reception and response during SDT | Fujitsu |
| R2-2107661 | PDCCH monitoring and SDT-TAT | Fujitsu |
| R2-2107778 | User plane aspects of SDT | NEC |
| R2-2107779 | Control plane aspects of SDT | NEC |
| R2-2107780 | Aspects specific to RACH based schemes | NEC |
| R2-2107788 | Discussion on beam selection aspect for CG-SDT | PANASONIC R&D Center Germany |
| R2-2107844 | User plane aspects of small data transmission | InterDigital, Europe, Ltd. |
| R2-2107850 | CG-based SDT selection and configuration | InterDigital, Europe, Ltd. |
| R2-2107866 | Consideration on switching to non-SDT procedure | LG Electronics Inc. |
| R2-2107867 | Consideration on open issues of CG-SDT | LG Electronics Inc. |
| R2-2107868 | Consideration on security issue on CCCH-based approach | LG Electronics Inc. |
| R2-2107898 | The UP common issues for small data transmissions | Lenovo, Motorola Mobility |
| R2-2107899 | Discussion on CP data transmission over SDT | Lenovo, Motorola Mobility |
| R2-2107900 | Consideration on CG based small data transmission | Lenovo, Motorola Mobility |
| R2-2107930 | Report of [Post114-e][508][SData] Open issues for CG-SDT | Qualcomm Incorporated |
| R2-2107991 | UP common aspects of SDT | Qualcomm Incorporated |
| R2-2107992 | CP common aspects of SDT | Qualcomm Incorporated |
| R2-2107993 | Open issues for RACH based SDT | Qualcomm Incorporated |
| R2-2107994 | Open issues for CG based SDT | Qualcomm Incorporated |
| R2-2108006 | Discussion on some FFSes | Potevio Company Limited |
| R2-2108009 | Paging reception during SDT | Nokia, Nokia Shanghai Bell |
| R2-2108010 | Aspects specific to CG based SDT | Nokia, Nokia Shanghai Bell |
| R2-2108055 | User Plane aspects of SDT in NR | Sony |
| R2-2108056 | Discussion on subsequent SDT in NR | Sony |
| R2-2108057 | Discussion on context fetch and anchor relocation | Sony |
| R2-2108058 | RACH-based SDT in NR | Sony |
| R2-2108059 | CG-based SDT in NR | Sony |
| R2-2108085 | RACH based small data transmission | Ericsson |
| R2-2108086 | Details of CG based SDT | Ericsson |
| R2-2108087 | Common aspects for SDT | Ericsson |
| R2-2108088 | SDT Faliure Handling | Ericsson |
| R2-2108089 | CP aspects for SDT | Ericsson |
| R2-2108199 | Small data transmission with RA-based schemes | Huawei, HiSilicon |
| R2-2108200 | User plane common aspects for SDT | Huawei, HiSilicon |
| R2-2108242 | Stage-2 running CR Introduction of SDT | Nokia, Nokia Shanghai Bell |
| R2-2108243 | Details of RACH specific schemes | Nokia, Nokia Shanghai Bell |
| R2-2108261 | SDT control plane aspects | Nokia, Nokia Shanghai Bell |
| R2-2108262 | RRC procedure for SDT | Nokia, Nokia Shanghai Bell |
| R2-2108327 | SDT cell re-selection | Convida Wireless |
| R2-2108506 | Consideration on control plane issues | CMCC |
| R2-2108507 | Discussion on RA-SDT | CMCC |
| R2-2108508 | UP common issues of SDT | CMCC |
| R2-2108509 | Consideration on CG-SDT | CMCC |
| R2-2108591 | Paging reception during SDT | Nokia, Nokia Shanghai Bell |
| R2-2108630 | Discussion on CG small data transmission | Google Inc. |
| R2-2108665 | Untreated proposal from [Post113-e][503] | InterDigital |
| R2-2108680 | Consideration on PDCP protocol in SDT | CATT |
| R2-2108681 | Consideration on UP common aspects of SDT | CATT |
| R2-2108682 | Consideration on CP issues | CATT |
| R2-2108683 | Transition from SDT to RRC_CONNECTED | CATT |
| R2-2108684 | Analysis and views on CG-SDT | CATT |
| R2-2108702 | Discussion on RA-based small data transmission | Google Inc. |
| R2-2108710 | BSR and PHR for SDT procedure | ASUSTeK |
| R2-2108711 | Discussion on fallback to non-SDT | ASUSTeK |
| R2-2108712 | Discussion on PDCCH monitoring for RA-SDT | ASUSTeK |
| R2-2108713 | Discussion on RA configuration reception | ASUSTeK |
| R2-2108714 | Discussion on CS-RNTI for CG-SDT | ASUSTeK |
| R2-2108729 | Remaining untreated proposals from [AT113bis-e][501] UP SDT open issues | LG Electronics Inc. (Rapporteur) |
| R2-2108730 | Remaining UP issues in SDT | LG Electronics Inc. |
| R2-2108731 | Non-SDT data arrival handling | LG Electronics Inc. |
| R2-2108788 | Discussion on the data volume computation | Xiaomi Communications |
| R2-2108789 | Handling of MAC CE | Xiaomi Communications |
| R2-2108790 | Paging reception during SDT | Xiaomi Communications |
| R2-2108791 | RACH failure in subsequent data transmission phase | Xiaomi Communications |
| R2-2108792 | Remaining issues of CG SDT in RAN2 | Xiaomi Communications |
| R2-2108816 | Handling of abrupt termination for SDT | ZTE Wistron Telecom AB |
| R2-2108916 | Report of [AT115e][502][SData] Summary of RA aspects | OPPO (Rapporteur) |
| R2-2109039 | [Pre115-e][501][SData] Summary of 8.6.2 User Plane remaining open issues | LG Electronics Inc. (Rapporteur) |
| R2-2109065 | Reply LS on Small data transmissions (S3-213034; contact: InterDigital) | SA3 |
| R2-2109079 | [AT115-e][501][SData] Summary of UP SDT open issues | LG Electronics Inc. (Rapporteur) |
| R2-2109222 | LS on agreements related to SDT | RAN2 |

## 4.3	RAN3

RAN3#113-e

Nothing to report

## 4.1	RAN4

Nothing to report

END