3GPP TSG RAN meeting #93-e									RP-212170

Electronic meeting, Sep. 13-16, 2021

## Status Report to TSG

Agenda item: 9.3.4.7

| WI / SI Name | RF requirements enhancement for NR frequency range 1 (FR1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR1_enh |  |  |  |  |
| Unique ID | 890062 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210899 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 60% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Zhangqian |
|  | Company | Huawei, HiSilicon |
|  | Email | Zhangqian67@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #106-e:

Agreement

For SUL and UL CA option 1, if 1Tx-2Tx UL Tx switching or 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured, the switching period is only applicable when the UL transmissions are switched between band A and band B.

Agreement

For inter-band UL CA, if 1Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1 Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on at least one carrier on band B.

    - If the current state of Tx chains is 0 Tx on band A and 2Tx on band B, the next UL transmission has a 1-port transmission on the carrier on band A.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

Agreement

For inter-band UL CA, if 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured:

- For option 2 of mapping between UL transmission ports and Tx chain

  - The switching period is only applicable in the following cases:

    - If the current state of Tx chains is 1Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on the carrier on band A or at least one carrier on band B.

    - If the current state of Tx chains is 0Tx on band A and 2Tx on band B, the next UL transmission has a 1-port or 2-port transmission on the carrier on band A.

    - If the current state of Tx chains is 2Tx on band A and 0Tx on band B, the next UL transmission has a 1-port or 2-port transmission on at least one carrier on band B.

  - For other cases, the state of Tx chains of last UL transmission is assumed.

Agreement

Down select one of the following options in RAN1#106b-e:

- Option 1: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then

  - 1Tx on carrier 1 and 1Tx on carrier 2 is assumed if the carrier is configured with uplinkTxSwitchingPeriodLocation as true.

  - the state of Tx chains supporting 2Tx transmission is assumed on the carrier if the carrier is configured with uplinkTxSwitchingPeriodLocation as false.

- Option 2: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then the state of Tx chains supporting 2Tx transmission on the carrier is assumed.

- Option 3: For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, then 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

Agreement

Down select one of the following options in RAN1 #106bis-e

- Option 1:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

    - 2Tx-2Tx switching mode: when the maximum number is 2 for all uplinks configured with uplinkTxSwitching

    - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

    - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

      - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

    - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode.

      - FFS how to determine the number of antenna ports for SRS resources.

- Option 2:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

#### 2.1.2	Remaining Open issues

• 2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

- Update RAN1 uplink switching for carrier aggregation and supplementary uplink

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #115-e:

- No need to introduce Rel-17 UE capability of DL interruption for 2Tx-2Tx switching. The Rel-16 UE capability of DL interruption for 1Tx-2Tx switching applies to 2Tx-2Tx switching as well.

- To introduce Rel-17 per-band pair UE capability to indicate a different switching time for 2Tx-2Tx switching for a given BC (Option 1).

- The Rel-16 filter uplinkTxSwitchRequest-r16 can be reused to request Rel-17 UL Tx switching UE capability.

- For R17 1Tx-2Tx/2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA, RAN2 takes the following way-forward as RAN2 understanding.

  - Way-forward: the UE should report corresponding CA bandwidth class and UL MIMO layers in the UL featureSetPerCCs for 2 continuous CCs on band B in the legacy way. No new UE capability is needed specific to the case with 2CCs on band B.

- Running CRs for TS 38.306 and TS 38.331 on Tx switching are under discussion

#### 2.2.2	Remaining Open issues

• 2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

-	Update the RRC configuration if needed

-	Update the UE capabilities if needed

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.1.1	Agreements

RAN4 #100-e meeting

- 2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

  - WF on UL MIMO coherence for Rel-17 Tx switching was approved, in which the UE capability for UL MIMO coherence on 3CC 1Tx-2Tx was concluded, while 2Tx-2Tx still open.

  - WF on FR1 RF enhancement RRM was approved, the agreements on DL interruption requirement for Rel-17 Tx switching was captured.

- HPUE for TDD intra-band contiguous UL CA

  - The big CR for PC2 contiguous CA R4-2114956 was agreed.

  - The agreements for PC2 contiguous CA with 2Tx was captured in WF R4-2114948.

- HPUE for TDD intra-band non-contiguous UL CA

  - WF R4-2114948 was approved, the MPR requirement for PC2 NC CA with 2LO architecture was agreed and captured.

- Intra-band UL contiguous CA for UL MIMO(n41C and n78C)

  - WF R4-2114951 was approved, agreement on MPR and other requirements are captured.

  - Big CR R4-2114953 was agreed on PC3 contiguous CA+MIMO.

#### Remaining Open issues

- UL MIMO configuration for SUL band configurations

  - Any issues need to be improved.

- 2Tx switching between carrier 1 and carrier 2, Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

  - UL MIMO coherence capability for Rel-17 Tx switching, and corresponding RAN2 spec impact if needed.

  - Specify RRM requirements for Tx switching in TS 38.133

  - Update RAN1 and RAN2 specs to capture the impact from introducing new switching requirement. Any issues need to be improved.

- HPUE for TDD intra-band contiguous UL CA

  - Evaluate MPR requirement based on 2Tx architecture.

  - Specify AMPR requirements for example BC if any.

  - CR on HPUE intra-band contiguous UL CA with 2Tx.

- HPUE for TDD intra-band non-contiguous UL CA

  - Conclude on MPR and corresponding requirements based on 1LO RF architecture.

  - Specify AMPR requirements for example BC if any.

  - CR on HPUE intra-band non-contiguous UL CA

- Intra-band UL contiguous CA for UL MIMO(n41C and n78C)

  - Specify PC2 contiguous CA+MIMO requirements

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #100-e:

[1] R4-2114727	Email discussion summary for [100-e][127] NR_RF_FR1_enh_Part_1_HPUE_ULMIMO , Huawei, HiSilicon

[2] R4-2115027	Email discussion summary for [100-e][127] NR_RF_FR1_enh_Part_1_HPUE_ULMIMO, Huawei, HiSilicon

[3] R4-2114754	LS on signalling for intra-band CA with UL-MIMO, OPPO

[4] R4-2114948	WF on PC2 intra-band UL NC CA and contiguous CA with 2Tx architecture, Skyworks

[5] R4-2114949	WF on solution for Scell dropping issue, Huawei, HiSilicon

[6] R4-2114950	LS on scell dropping issue of CA, Huawei, HiSilicon

[7] R4-2114951	WF on contiguous CA for UL-MIMO , vivo

[8] R4-2114728	Email discussion summary for [100-e][128] NR_RF_FR1_enh_Part_2,China Telecom

[9] R4-2115028	Email discussion summary for [100-e][128] NR_RF_FR1_enh_Part_2 ,China Telecom

[10] R4-2114957	WF on UL-MIMO coherence for Rel-17 Tx switching, China Telecom

[11] R4-2112228	Discussion on UL MIMO coherence for Rel-17 Tx switching, China Telecom

[12] R4-2112824	Draft LS on modification of Pcmax for UL CA with uplink Tx switching capability, Ericsson

[13] R4-2114493	on intra-band contiguous CA MPR requirement, Huawei, HiSilicon

[14] R4-2114956	CR on PC2 intra-band UL contiguous CA RF requirements, Huawei, HiSilicon, Skyworks

[15] R4-2112022	PC2 Intra-band UL NC CA MPR Simulations with 1PA, Nokia

[16] R4-2112893	MPR requirements and other remaining issues for PC2 intra-band NC CA UE, LGE

[17] R4-2114954	on intra-band UL NC CA architecture and MPR, Huawei, HiSilicon

[18] R4-2114571	Proposal for non-baseline PC2 NC UL CA architectures, Skyworks

[19] R4-2112324	Capability signaling for combining UL-MIMO and UL intra band contiguous CA, ZTE

[20] R4-2113024	Discussion on Intra-band UL contiguous CA for UL-MIMO Capability Signaling, vivo

[21] R4-2113898	R17 FR1 UL CA with MIMO and draft LS, OPPO

[22] R4-2114953	CR on contiguous CA with UL MIMO for power class 3, Huawei, HiSilicon, China Telecom

[23] R4-2114491	on CA+MIMO RF requirement, Huawei, HiSilicon

[24] R4-2114564	2Tx PC2 contiguous UL CA back-off evaluation for TxD and UL MIMO, Skyworks

[25] R4-2114692	2Tx PC2 contiguous UL CA back-off evaluation for TxD and UL MIMO, Skyworks

[26] R4-2112825	Draft LS on TX switching with multiple TAG, Ericsson

RAN1 106-e:

[1] R1-2108643	[106-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching	Moderator (China Telecom)

[2] R1-2106500	Discussions on enhancements for UL Tx switching,	Huawei, HiSilicon

[3] R1-2106729	Discussion on Rel-17 UL Tx Switching,ZTE

[4] R1-2106925	Discussion on R17 enhancements for UL TX switching, CATT

[5] R1-2107122	Discussion on Rel-17 uplink Tx switching, China Telecom

[6] R1-2107211	Dsicussion on Rel-17 Tx switching enhancement, OPPO

[7] R1-2107308	Discussion on R17 UL Tx switching, Qualcomm Incorporated

[8] R1-2107388	Discussion on Rel-17 UL Tx Switching	,CMCC

[9] R1-2107970	Discussion on Rel-17 Tx switching enhancements, vivo

RAN2 115-e:

[1] R2-2109042	Summary of [AT115-e][035][NR17] TX switching, China Telecom

[2] R2-2109224	Summary of [AT115-e][035][NR17] TX switching, China Telecom

[3] R2-2108159	Draft CR to TS38.331 to support Tx switching enhancements, Huawei, HiSilicon, China Telecom, Apple, CATT

[4] R2-2108160	Draft CR to TS38.306 to support Tx switching enhancements, Huawei, HiSilicon, China Telecom, Apple, CATT

[5] R2-2106907	Reply LS on Rel-17 uplink Tx switching (R1-2104137; contact: China Telecom)

[6] R2-2106951	LS on Rel-17 Tx switching enhancements (R4-2103234; contact: China Telecom)

[7] R2-2108274	UE capability reporting and RRC configuration for Rel-17 UL Tx switching enhancements	China Telecommunication, CATT, Baicells

[8] Discussion on Rel-17 UL Tx Switching, Apple

[9] R2-2107979	UE capabilities for UL Tx switching enhancement, Ericsson

[10] R2-2108158	RAN2 impact to support R17 UL Tx switching enhancement, Huawei, HiSilicon, Apple

[11] R2-2108671	R17 TX switching enhancements, vivo

[12] R2-2106953	Reply LS on Rel-17 uplink Tx switching (R4-2107847; contact: China Telecom)

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template