3GPP TSG RAN Meeting #93-e                                           RP-212235

Electronic Meeting, September 13 - 17, 2021

Source:	CATT

Title:	Progress and scope of Rel-17 enhanced IIoT and URLLC

Agenda Item:	9.3.2.1

Document for:	Discussion and Decision

Introduction

Rel-17 enhanced IIoT and URLLC WI was approved in RAN#86 [1]. The scope was updated in [2] in RAN#88e and demodulation in performance part is added in [3] in RAN#91e.

The objectives of core part WI are as follows. In this contribution, we discuss the progress and scope of the WI based on the progress in RAN1.

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] Note: DMRS-based CSI feedback is not in scope of this WI - Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum - Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline - Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4] - RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

Discussion

In RAN1#106-e meeting, good progress has been made for Rel-17 enhanced IIoT and URLLC. However, it is still quite challenging to complete all the objectives within the remaining two RAN1 meetings in Q4, in particular for intra-UE multiplexing and prioritization with different priority, and for propagation delay compensation enhancements.

Intra-UE multiplexing and prioritization with different priority

A working assumption on framework for intra-UE multiplexing with different priorities was agreed in RAN1#106-e meeting as follows.

Working Assumption For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

RAN1 can continue discussing the details in the following two RAN1 meetings in Q4 although it is quite challenging.

For intra-UE prioritization, there was no discussion since RAN1#104-e meeting because it is pending on clarification of Rel-16 UE behavior, which is not clear yet in RAN1#106-e.

[106-e-NR-L1enh-URLLC-05] Issue#9: Remaining issues on UL prioritization and UL skipping by August 20 – Lihui (vivo) Email discussion closed without any agreement or conclusion.

Given that RAN1#106b-e meeting will only treat Rel-17 items, the Rel-16 UE behavior can be concluded in RAN1#107-e meeting at the earliest. It is impossible to finalize Rel-17 intra-UE prioritization in the same meeting. Therefore, it is proposed to remove intra-UE prioritization from Rel-17 IIoT/URLLC scope.

Proposal 1: Adopt the following WID update for Rel-17 enhanced IIoT and URLLC:

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

- Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

Propagation delay compensation enhancements

In RAN1#106-e meeting, an LS in R1-2108635 was agreed to be sent to RAN4 on TA-based propagation delay compensation enhancements. Meanwhile, RAN1 has also started discussion on RTT-based propagation delay compensation enhancements.

Given that there is no RAN4 meeting in October, the feedback from RAN4 can be received before RAN1#107-e meeting at the earliest. RAN1 can continue discussing RTT-based propagation delay compensation enhancements in RAN1#106b-e meeting and discuss how to proceed in RAN1#107-e based on the feedback from RAN4 if any.

Proposal 2: RAN1 continues discussing propagation delay compensation enhancements in Q4 and no RAN intervention is needed in RAN#93-e.

Conclusion

In this contribution, we discussed the progress and scope of Rel-17 enhanced IoT and URLLC with the following proposals.

Proposal 1: Adopt the following WID update for Rel-17 enhanced IIoT and URLLC:

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

- Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

Proposal 2: RAN1 continues discussing propagation delay compensation enhancements in Q4 and no RAN intervention is needed in RAN#93-e.

Reference

- RP-193233, New WID on enhanced Industrial Internet of Things (IoT) and URLLC support, Nokia, Nokia Shanghai Bell, RAN#86

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN#88e

- RP-210854, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN#91e