3GPP TSG RAN Meeting #93-e	RP-212414

Electronic Meeting, September 13 - 17, 2021

Title: 	Discussion on CFR configuration for Rel-17 MBS

Source: 	ZTE, Nokia, Nokia Shanghai Bell, Qualcomm, vivo, Convida Wireless

Agenda item:	9.3.2.4

Document for:	Discussion/Decision

Introduction

One of the objectives of Rel-17 MBS WI is to support broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [1]. RAN1 has agreed that UE needs to receive broadcast/multicast within a common frequency resource (CFR). It is clear that CFR has to be confined within the active bandwidth part for UEs under RRC_CONNECTED state. However,  companies didn’t reach consensus on whether support only Case C is enough, or whether there is the need to support also the CFR size smaller or larger than the SIB1 configured initial DL BWP for UEs in RRC_IDLE/ RRC_INACTIVE states. For example, for a carrier with 100MHz carrier bandwidth, CORESET#0=20MHz, SIB1 configured initial DL BWP=50MHz, companies have different views on whether the CFR size always has to be 20MHz (Case A) and 50MHz (Case C), or it can also be other sizes, e.g., 40MHz (Case D) or 100MHz (Case E).

Companies have been discussing this issue in RAN1 for at least 4 meetings for more than half a year. Considering that there are only two RAN1 meetings left for Rel-17, it is beneficial if RANP can have guidance on this issue. Thus, companies can save time in working group meeting to discuss further on how to support these different cases instead of discussing whether to support these cases again.

Discussion

The detailed definition of Case C, Case D, Case E as well as Case A and Case B can be found in Appendix 5.1 and some figures in Appendix 5.2 show examples of each case and the difference between these cases.

Companies couldn’t converge on the support of Case C, Case D and Case E. Some companies suggested to only support Case C but not support Case D and Case E. In this case, network has to configure the CFR with the same size as the initial DL BWP configured by SIB-1, which is too restrictive. Meanwhile, if network configured a small initial DL BWP by SIB-1 in legacy networks, networks may have to upgrade the unicast scheduling/configuration in order to support large bandwidth demanding MBS services in these networks, which will delay the deployment of MBS. Moderator summarized the potential cons of only supporting Case C as follows [2].

    - Rel-17 MBS UEs (both MBS UE and non-MBS UE) should still use legacy initial BWP

    - Changing the frequency resources of the initial BWP also changes the frequency resources of legacy Rel-15/Rel-16 UEs as well as Rel17 non-MBS UE in RRC connected (if configured).

    - For broadcast services requiring large bandwidths requires (e.g. high bit rates) with corresponding large bandwidth could have an impact on the power consumption of legacy and Rel17 UEs not receiving MBS broadcast services.

Observation 1: Only supporting Case C but not supporting Case D and Case E is too restrictive for MBS networks. It also requires networks to upgrade legacy scheduling/configuration for unicast, which will delay the commercial progress of MBS.

On the other hand, Case D and E can provide network flexibility by configuring the CFR with smaller or larger size than the initial DL BWP configured by SIB-1, there were no key issues being identified during RAN 1 discussion. Furthermore, a common configuration method can be used to configure CFR in case C, D, and E by configuring a CFR/BWP via SIB signalling or MCCH. And thus, by supporting case C, D, and E via the common design, the network is able to minimize the impacts on legacy scheduling/configuration for unicast and it can also take the flexibility to configure the CFR depending on the practical requirements. And no extra specification work is required to support the three cases with common configuration framework.

Observation 2: A common configuration method can be used to configure CFR in case C, D, and E.

In RAN1#106-e meeting, after extensive discussion, companies proposed the following proposal as a compromised solution. However, it was still objected by one company in the end.

Proposal: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state. - Support Case-C - Working assumption: Support at least one of Case D and Case E. - Down-selection to be made at RAN1#106b-e - Note: Case C, D and E are defined in previous agreements Objected by Lenovo

To break the deadlock in working group discussion, it is crucial if RAN plenary can give some guidance on this issue to facilitate the working group discussion. Considering the widespread support of Case C, Case D and Case E from different operators, UE vendors and network vendors, it is proposed to support all the three cases. With this, network can choose different cases for different scenarios to accommodate different MBS requirements.

Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, Rel-17 MBS supports all of the following cases with a common configuration framework to accommodate different broadcast services with different requirements.

- The CFR is larger than the initial DL BWP configured by SIB1.

- The CFR is the same as the initial DL BWP configured by SIB1

- The CFR is larger than CORESET#0 but smaller than the initial DL BWP configured by SIB1.

Note: The CFR fully contains CORESET#0.

Conclusion

RAN1 has been discussing different cases (Case A, Case B, Case C, Case D and Case E) for CFR for Rel-17 MBS for more than four meetings. Case A has been agreed and Case B has been excluded. However, companies couldn’t converge on whether to support Case C, Case D and Case E. To break the deadlock in working group discussion, it is crucial if RAN plenary can give some guidance on this issue to facilitate the working group discussion.

From network perspective, we propose to support Case C, Case D and Case E together to avoid the impact on legacy non-MBS UEs and also to offer flexibilities for different MBS services with different requirements. Thus, the following observation and proposal are proposed.

Observation 1: Only supporting Case C but not supporting Case D and Case E is too restrictive for MBS networks. It also requires networks to upgrade legacy scheduling/configuration for unicast, which will delay the commercial progress of MBS.

Observation 2: A common configuration method can be used to configure CFR in case C, D, and E.

Proposal 1: For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state, Rel-17 MBS supports all of the following cases with a common configuration framework to accommodate different broadcast services with different requirements.

- The CFR is larger than the initial DL BWP configured by SIB1.

- The CFR is the same as the initial DL BWP configured by SIB1

- The CFR is larger than CORESET#0 but smaller than the initial DL BWP configured by SIB1.

Note: The CFR fully contains CORESET#0.

Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

R1-2108579, Feature Lead summary #6 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states, RAN1#106-e, Moderator (BBC).

Appendix

RAN1 agreements

RAN1 Agreement (RAN1#104-e):

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, further study the following cases of a configured/defined specific common frequency resource (CFR) for group-common PDCCH/PDSCH, and identify which case(s) will be supported:

- [Case E] the case where a CFR is defined based on a configured BWP.

  - In particular, study the following:

    - whether a configured BWP for MBS is needed or not.

    - whether BWP switching is needed or not.

  - In this study, the configured BWP has the following properties:

    - The configured BWP is different than the initial BWP where the frequency resources of this initial BWP are configured smaller than the full carrier bandwidth.

    - The CFR has the frequency resources identical to the configured BWP.

    - The configured BWP needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

  - Note: The configured BWP is not larger than the carrier bandwidth

- the case where the initial BWP fully contains the CFR in the frequency domain.

  - In this study the following sub-cases are considered:

    - [Case B] A CFR with smaller size than the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

    - [Case D] A CFR with smaller size than the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the frequency resources confined within the initial BWP and have the same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with smaller size than the initial BWP are needed or not for MBS.

- the case where the initial BWP has same size as the CFR in the frequency domain.

  - In this study the following two sub-cases are considered:

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

  - In particular, study the following:

    - Whether the considered two options with a CFR with the same size as the initial BWP are needed or not for MBS.

Example figures

| Case E | The CFR is larger than the initial DL BWP configured by SIB1 |
| --- | --- |
| Case C | The CFR is the same as the initial DL BWP configured by SIB1 |
| Case D | The CFR is larger than CORESET#0 but smaller than the initial DL BWP configured by SIB1 Although there are two sub-cases for Case D, Case D-1 is the common understanding and typical use case. |
| Case A | The CFR is the same as CORESET#0 Case A is the most straightforward solution, which has been approved. |
| Case B | The CFR is smaller than CORESET#0 Companies think that Case B can be realized by network scheduling/implementation, thus no need to specify it in specification. |